# General Description

## Steering Control System

The MRJ aircraft requires that one SCU will be installed in each aircraft. The SCU receives Control and Monitor lane information via the ARINC429 bus. The SCU transmits steering and flight status information to the Primary Flight Control Computer (PFCC) via (DMC-Avionics1 and DMC-Avionics2). The SCU also transmit errors, warnings and failure information to the Engine Indicating and Crew Alerting System (EICAS) within the Data Management Cabinet (DMC). The SCS turns to Free Castor Mode (Steering free mode) when a safety critical SCU failure has occurred. System troubleshooting can be accomplished by using failure message information from the DMC and the SCU in accordance with the Built In Test (BIT) result.

The SCS has a shutoff valve (SOV) which shuts off (de-Energize) the hydraulic pressure to the SCS per SCU commands. When the SOV Command is ‘ON’ or Energize, the SCV provides hydraulic pressure to the SCS, and the SCV detects it via the pressure switch. The SCU sends control commands to Electro Hydraulic Servo Valve (EHSV), which controls the hydraulic flow and direction to steering actuators. The SCS also has anti-shimmy functions, to prevent or minimize shimmy generated from the Nose Landing Gear (NLG) during taxi, takeoff, and landing. The SCS has a pressure sensor discrete to check actual pressure in the system. The SCS is configured for 1 Pilot steering handle with the option for an additional co-Pilot steering handle.

## SCU Architecture

### System Architecture

The SCU controls the Nose Wheel steering angle by using the following inputs:

- Rotary Variable Differential Transducer (RVDT) via the pilot steering handle; which provide indication of the steering angle requested by the Pilot

- Linear Variable Differential Transducer (LVDT) via the steering actuators; which provide indication of the actual steering angle (Actual Steering Angle)

- Data from ARINC429 interfaces and Analog signal which provided the Rudder operation information of the steering angle requested by the Pilot

- Data from ARINC429 interfaces indicating whether the aircraft is on the ground as well as the ground speed of the aircraft.

The SCU Control Lane provides the following control functions:

- Nose Wheel steering angle position control

- Nose Wheel steering de-activation control

The SCU Monitor Lane provides the following monitor/reporting functions:

- Nose Wheel steering angle position monitoring

- Steering Control System status monitoring

- EICAS Failure Indication for the Steering Control System

- BIT Reporting for the Steering Control System

### SCU Equipment Architecture

The SCU Equipment Architecture is based on a single Control Lane and a single Monitor Lane.

The Control Lane is responsible for the calculation of the target steering angle (TSA) via the DSP microcontroller. The Control Lane is also responsible for controlling the actuation of the Nose Wheel Steering via the Electro Hydraulic Servo Valve (EHSV).

The Monitor Lane is responsible for monitoring the Control Lane and, if necessary, will activate the Shutoff Valve (SOV) solenoid to disable steering control upon detection of a fault which may otherwise lead to hazardous or catastrophic behaviour. Additionally the Control Lane has the capability to disable steering control by de-energising the SOV if it detects faults within the Monitor Lane.

## SCU Inputs

The SCU receives the following inputs, which are routed to both the Control Lane and the Monitor Lane for independent processing:-

  - 2 x Pilot RVDT Steering Command Signal

  - 2 x Co-Pilot RVDT Steering Command Signal

  - 2 x LH LVDT Steering Position Signal

  - 2 x RH LVDT Steering Position Signal

  - 2 x Rudder Command from ACE1B via ARINC429 communication

  - 2 x Rudder Command from ACE1B via Analog Signal

  - 2 x Rudder Command from ACE2B via ARINC429 communication

  - 2 x Rudder Command from ACE2B via Analog Signal

  - 3 x Aircraft Speed from A-DMC via ARINC429 communication

  - 3 x Aircraft Speed from B-DMC via ARINC429 communication

  - Steering System Hydraulic Pressure Switch Discrete Input

  - Pilot Enable Discrete Input

  - co-Pilot Enable Discrete Input

  - 2 x Disengage Discrete Inputs

The Control Lane and the Monitor Lane uses a different ‘primary’ RVDT steering angle command input and LVDT actual steering angle input in order to provide complete independence from each other. Additionally, they also monitor the LVDT and RVDT inputs provided as the ‘primary’ signals for the opposite lane.

The Monitor Lane provides monitoring of the actual steering angle. If the actual steering angle exceeds safety limits or is significantly different to the commanded steering angle, the Monitor Lane will send a message to de-energize the SOV solenoids.  When the SOV solenoids are de-energised, the hydraulic pressure from the steering system is removed, thereby removing control of the Nose Wheel steering from the EHSV and allowing the Nose Wheel to free castor. This over-ride is required to guard against any excessive or inadvertent steering angle being commanded by the Control Lane. In addition the Monitor Lane provides BIT reporting for the Steering Control System (SCS) and EICAS indication.

Monitoring of the SCU circuits and the SCU interfaces is predominately performed in software. The software operation is monitored by a separate, independent hardware watchdog.

Communications with other aircraft systems is via ARINC 429 interfaces.

The Control Lane and the Monitor Lane communicate internally via a point-to-point serial interface

## SCU Outputs

The SCU provides the following outputs:

  - 4 x servo amplifier outputs for driving the high and low sides of the two solenoids coils within the EHSV.

  - 1 SOV solenoid output a high-side (via CL) and a low-side (via ML) solenoid driver for the SOV.

The Control Lane and Monitor Lane are configured to receive via 4 ARINC RX busses and provide 1 ARINC TX bus:

  - The primary function of the Control Lane ARINC TX bus is to provide failure indication, BIT data and information for steering control status.

  - The primary function of the Monitor Lane ARINC TX bus is to provide failure indication, BIT data, support for rigging and information for steering control status.

## Power Supply

The internal power supply receives 28V DC power from two independent busses, which is consolidated to an internal DC-DC power supply.

## Target & Actual Steering Angle

The Control Lane and the Monitor lane use a different 'primary' RVDT steering angle command inputs and LVDT actual steering angle inputs to provide independence from each other.

The steering angle feedback is provided to the SCU from a Left Hand LVDT (LH_LVDT) and a Right Hand LVDT (RH_LVDT) assembly. Each LVDT assembly comprises of two channels, channel A and channel B. The Control Lane provides the excitation signal for the primary coils for the channel A LVDTs, the secondary coils of the channel A LVDTs are monitored by both Control and Monitor lanes.  The Monitor Lane provides the excitation signal for the primary coils for the channel B LVDTs, the secondary coils of the channel B LVDTs are monitored by both Control and Monitor lanes.

- Channel A LVDTs are primary to Control Lane and secondary to Monitor Lane

- Channel B LVDTs are primary to Monitor Lane and secondary to Control Lane

This allows the 'secondary' LVDT to be used for the remainder of a flight where there is a detected failure of one of the 'primary' LVDT or RVDT inputs.

The SCU receives target steering angle information from a pilot steering handle and a co-pilot steering handle. Each handle comprises of two RVDTs, Channel A and Channel B. The Control Lane provides the excitation signal for the primary coils for the channel A RVDTs, the secondary coils of the channel A RVDTs are monitored by both Control and Monitor lanes.  The Monitor Lane provides the excitation signal for the primary coils for the channel B RVDTs, the secondary coils of the channel B RVDTs are monitored by both Control and Monitor lanes.

- Channel A RVDTs are primary to Control Lane and secondary to Monitor Lane

- Channel B RVDTs are primary to Monitor Lane and secondary to Control Lane

The SCU also receives target steering angle from two inputs from Rudder 1 and two inputs from Rudder 2. The Monitor Lane and the Control Lane will each monitor all four rudder command inputs.

The actual steering command is a function of aircraft speed. The aircraft speed is provided to the SCU via the ARINC 429 receiver channels.

## Shut Off Valve (SOV)

Both the Control and Monitor Lanes provide monitoring of the actual steering angle and if this exceeds safe limits or is significantly different to the commanded steering angle they will independently de-energise the SOV solenoid.

When the SOV is de-energised, the hydraulic pressure from the steering system is removed, thereby removing control of the Nose Wheel steering from the EHSV and allowing the Nose Wheel to free castor. This over-ride is required to guard against excessive steering angle being commanded by the control lane.

## Monitor Lane Function

In addition the Monitor Lane provides BIT reporting for the Steering Control System and EICAS indication.

Monitoring of the SCU circuits and the SCU interfaces is predominately performed in software. Software operation is monitored by a hardware watchdog.

The monitor lane performs a check on the integrity of the SOV system once every flight.

Communications with other aircraft systems is provided via ARINC 429 interfaces.

Background checks will be continuously performed on the integrity of SCU sub-systems.

## ARINC 429 Data

Both the Control Lane and Monitor Lane each receive data on four (4) ARINC 429 Receive busses and each output data on one (1) ARINC 429 Transmit bus. Data on each ARINC bus is identified in the Electrical Interface Control Document [A2].

ARINC inputs provide aircraft speed, Weight On Wheel information and Rudder Commands.

## Discrete Inputs (Switches)

The SCU receives several discrete inputs from the steering control system. These include Pilot/Co-Pilot steering handle enable inputs, Hydraulic Pressure indication, and disengage inputs from the ground maintenance panel. The discrete input interface is designed to interface with these inputs.

Figure 4.10-1: SCU Architecture

## SCU BITE

The following summarise the Built-In Test features provided by the SCU.

### Control Lane

- DSP Integrity

- Watchdog test

- RAM integrity Test

- ROM Test

- Inputs

- LVDT - open, short and reasonableness checks.  Reasonableness checks are carried out via cross-comparison.

- LH_LVDT1

- LH_LVDT2

- RH_LVDT1

- RH_LVDT2

- RVDT - open, short and reasonableness checks.  Reasonableness checks are carried out via cross-comparison. In the event of failure the secondary RVDT from the other lane will be used. Note: In the event that during PBIT the primary and secondary RVDTs for the co-pilot are deemed to be open or short circuit then it will be assumed that the system is operating in single pilot mode.

- Pilot_RVDT1

- Pilot_RVDT2

- Co_Pilot_RVDT1 (optional)

- Co_Pilot_RVDT2 (optional)

- Discrete Inputs -   Errors in discrete inputs will be deduced via cross comparison of signals.

- Pilot enable Switch

- Co_Pilot enable Switch  (optional)

- Towing Disengage Switch#1

- Hydraulic Pressure Switch  - Errors will be deduced by SOV Test

- ARINC 429 Rx Bus - Loss of data labels, loss of data bus, and reasonableness checks.  Reasonableness checks are carried out via cross comparison of data i.e. for WOW, airspeed and rudder Commands.

- ARINC_RX_DMCA1

- ARINC_RX_DMCB1

- ARINC_RX_ACE1B1

- ARINC_RX_ACE2B1

- Outputs

- EHSV Coil - coil open/short will be deduced via monitors on the EHSV drivers. These will be checked during CBIT.  In the event of failure the alternate EHSV Coil will be used.

- EHSV CoilA

- EHSV CoilB

- EHSV Drivers - driver stuck ON or stuck OFF will be deduced via monitors on the EHSV drivers. These will be checked during CBIT. In the event of failure the alternate EHSV Drivers will be used.

- EHSV Hi-side Driver #1

- EHSV Hi-side Driver #2

### Monitor Lane

- DSP Integrity

- Watchdog test

- RAM Integrity Test

- ROM Integrity Test

- Inputs

- LVDT - open, short and reasonableness checks. Reasonableness checks are carried out via cross-comparison. In the event of failure the secondary LVDT from the other lane will be used.

- LH_LVDT1

- LH_LVDT2

- RH_LVDT1

- RH_LVDT2

- RVDT - open, short and reasonableness checks. Reasonableness checks are carried out via cross-comparison. In the event of failure the secondary RVDT from the other lane will be used. Note: In the event that the primary and secondary RVDTs for the co-pilot are deemed to be open circuit then it will be assumed that the system is operating in single pilot mode.

- Pilot_RVDT1

- Pilot_RVDT2

- Co_Pilot_RVDT1 (optional)

- Co_Pilot_RVDT2 (optional)

- Discrete Inputs -  Errors in discrete inputs will be deduced via cross comparison of signals.

- Pilot enable Switch

- Co_Pilot enable Switch (optional)

- Towing Disengage Switch

- Hydraulic Pressure Switch  - Errors will be deduced by SOV Test

- ARINC 429 Rx Bus - Loss of data labels, loss of data bus, and reasonableness checks. Reasonableness checks are carried out via cross comparison of data i.e. for WOW, airspeed and rudder commands.

- ARINC_RX_DMCA2

- ARINC_RX_DMCB2

- ARINC_RX_ACE1B2

- ARINC_RX_ACE2B2

- Outputs

- SOV Coil - coil open/short will be deduced via monitors on the SOV driver. This will be checked during PBIT.

- SOV Coil

- SOV Driver - driver stuck ON or stuck OFF will be deduced via monitors on the SOV driver. These will be checked during PBIT.

- SOV Driver

- SOV enable test - performed at power-up when aircraft is on-ground and airspeed is less than 10knots. Also BIT performed during normal operation.

- SOV disable test - performed at power-up alongside SOV enable test. Also BIT performed when hydraulics are not enabled i.e. when steering has been disabled by Monitor Lane.

- Power Supply

- 28V aircraft inputs - the +28V aircraft power supplies will be monitored for availability. The SCU will operate if one or other or both +28V supplies are available.

- +28V #1

- +28V #2

- Internal Power Supply - the health of the internal supply is checked via hardware monitors on the internal supply rail.

- PSU Monitors

- Other

- Steering fault indication - The monitor lane will report a fault and will de-activate the SOV when the steering angle exceeds the maximum allowed angle for a given speed.

- Steering fault indication - The monitor lane will report a fault and will de-activate the SOV if the commanded steering angle is not consistent with the value determined by the Monitor Lane.

# CSCI Description

## CSCI Software Overview

The SCU Steering Control (STC) program has the following main functional requirements:

- Executive

o	Power-up Processing:

-	Create C-Environment

-	Initialise DSP Registers

-	Perform PBIT

-	Load Configuration Parameters from NVM

o	Runtime Processing:

-	Level-1 tasks

-	Level-2 tasks

-	Level-3 tasks

- CBIT

o	Perform CBIT on DSP resources

-	Kick Watchdog

-	Calculate Arithmetic Flash Checksum and 32-Bit CRC

-	Check Register Values

o	Perform CBIT on SCU components

-	EHSV Feedback BIT

-	LVDT/RVDT BIT

-	Disagreement BIT

-	ARINC Bus BIT

- Input Processing

o	Analogue Input Processing

-	LVDT/RVDT data

-	SHDL Logical Level Processing & Monitoring

-	SACT Logical Level Processing & Monitoring

-	Rudder Position Inputs (via external ADC)

o	Discrete Input Processing

-	Hydraulic Pressure Switch

-	Pilot SHDL Engagement Switch On and Off

-	Co_Pilot SHDL Engagement Switch On and Off

-	Towing Switch Engage

-	Towing Switch Disengage

o	ARINC 429 Input Processing

-	WOW data from DMCA and DMCB

-	Airspeed data from DMCA and DMCB

-	Rudder Pedal Command from ACE1 and ACE2

- Steering Control

o	Steering Demand Calculation

-	Pilot SHDL Steering Demand

-	Co_Pilot SHDL Steering Demand

-	SHDL Command Calculation and Limiting

-	Rudder Command Calculation

-	Steering Demand Summation

-	Steering Demand Limiting

o	Touchdown Protection

-	WOW Debouncing

-	Touchdown Protection Selection

o	Loss of Steering Detection

o	Steering Control Enable

o	Steering Control Loop

- Output Processing

o	EHSV Output Processing

o	Discrete Output Processing

-	SOV Hi-side Output Processing

o	ARINC 429 Output Processing

-	BIT Information

-	Control Status

-	Actual Steering Angle

-	Target Steering Angle

- Cross Lane Communication

o	Transmit STC messages

o	Receive SMB messages

- Rigging Function

- Engineering Access Interface

o	SCU Status Messages

### Executive

During start-up the STC Program will create the C-environment, initialise the DSP Registers and perform PBIT functions such as Watchdog test and SRAM check. Configuration parameters will then be loaded from NVM.

At runtime the scheduler will execute the required tasks for each level of processing period. ADC channels have to be sampled at the highest processing rate (8 channels every 5us). The second level of processing cycles contains the partial DFT calculations. On the third level the DFT calculations are completed. The fourth level (every 5ms) performs the ratiometric calculations for the LVDT and RVDT interfaces.

CBIT monitors and functions are being scheduled at their required cycle rate.

The scheduler will include runtime integrity checks to detect if a processing cycle overruns and has not finished by the time it is supposed to be scheduled in.

### CBIT

The flash content will be checked continuously by calculating an arithmetic checksum and a 32-bit CRC and comparing these checksums with the values stored during unit production.

DSP register settings will be checked continuously with their initialisation values.

The STC program will monitor system components such as LVDTs, RVDTs, SHDL inputs, EHSVs, switches and power supply for faults .

Received ARINC data will be monitored for validity and transmitted ARINC data will be checked by looping the data externally back into the SCU for comparison with the transmitted data.

### Input Processing

The STC Program will sample analogue input values from the LVDT and RVDT interfaces. The resulting DFT calculations will be used to determine the SHDL positions and the actual LVDT / RVDT positions SACT.

Discrete input signals will be sampled and debounced for further processing.

ARINC 429 input labels for WOW status, Airspeed and Rudder Pedal Position will be checked for validity.

### Steering Control

The STC Program will perform the Pilot and Co-Pilot S-HDL Steering command calculation and limiting and the Rudder Command calculation. This will be followed up by the Steering Demand summation and limiting function.

The steering control logic function will determine the steering error, which will be fed into a PID controller.

A Touchdown Protection function will be implemented in order to prevent steering at high speed. This function will also include WOW sensor debouncing.

### Output Processing

The EHSV will be controlled via an analogue PWM excitation signal at a frequency of ca. 50kHz. The mark/space ratio will be updated at a rate of 200Hz.

The LVDT/RVDTs require an excitation frequency of ca. 3kHz. The mark/space ratio will be updated at a rate of 200kHz.

The high-side of the SOV will be controlled via a discrete output.

The ARINC 429 outputs consist of labels for Failure Information, Control Information, Actual Steering Angle and Target Steering Angle.

### Internal Interlane Interface Communication

The Control and Monitor Lanes will exchange system status information via an SPI Bus internal to the SCU. The Control Lane will report its BIT status and Steering Angle information to the Monitor Lane.

### Rigging Function

The OMS can enter a rigging dialogue with the SCU monitor lane, which allows the Steering Handles and Nose Wheel angle inputs to be rigged and allows checking that extreme settings of the handles and actual steering angle are within tolerance. The Monitor Lane requests the Control Lane to calibrate the RVDTs and LVDTs and to store results in NVM. The zero-setting ratios are then applied as a correction to the RVDT & LVDT ratiometric values.

### Engineering Access Interface

Transmission of SCU Status Messages

## CSCI External Interfaces

Figure 5.2-1 shows the External Interfaces for the STC Program.  For a more complete description of each hardware interface, refer to Section 5.2.3.

Figure 5.2-1: External Interfaces

### Input Signals

A summary of the inputs to the STC Program is given in Table 5.2-1.

Table 5.2-1 Input Signals (Part 1)

Table 5.2-1 Input Signals (Part 2)

Table 5.2-1 Input Signals (Part 3)

Table 5.2-1 Input Signals (Part 4)

Table 5.2-1 Input Signals (Part 5)

Table 5.2-1 Input Signals (Part 6)

### Output Signals

A summary of the outputs from the STC Program is given in Table 5.2-2.

Table 5.2-2: Output Signals (part 1)

Table 5.2-2: Output Signals (part 2)

Table 5.2-2: Output Signals (part 3)

Table 5.2-2: Output Signals (part 4)

### Hardware/Software Interface and Assumptions

The STC Program utilises the SM320F2812-EP Digital Signal Processor (DSP) with a 150 MHz clock (30 MHz oscillator frequency).  Internal control registers are used to initialize the DSP functional settings and communications interface.  Periodically, the control registers will be read for feedback on the status of various hardware operations. 
Refer to Ref. C5 for additional information relating to this Interface.

#### PWM Outputs

The STC Program utilises three PWM channels from the Event A peripheral. One channel will be used for generating the LVDT/RVDT excitation signals. The second channel will be used for generating the EHSV excitation signal. The third channel will be used to provide the clock signal for the external ARINC device.

- LVDT/RVDT excitation signal: 	PWM1 from EV-A peripheral

- EHSV excitation signal: 	PWM10 from EV-B peripheral

- ARINC clock signal: 		T2PWM from EV-A peripheral

Refer to Ref. C8 for additional information relating to this Interface.

#### LVDT/RVDT Sensor Analogue Inputs

The steering angle feedback is provided to the SCU from a Left Hand LVDT (LH_LVDT) and a Right Hand LVDT (RH_LVDT) assembly. Each LVDT assembly comprises of two channels, channel A and channel B. The Control Lane provides the excitation signal for the primary coils for the channel A LVDTs, the secondary coils of the channel A LVDTs are monitored by both Control and Monitor lanes.  The Monitor Lane provides the excitation signal for the primary coils for the channel B LVDTs, the secondary coils of the channel B LVDTs are monitored by both Control and Monitor lanes.

The SCU receives target steering angle information from a pilot steering handle and a co-pilot steering handle. Each handle comprises of two RVDTs, Channel A and Channel B. The Control Lane provides the excitation signal for the primary coils for the channel A RVDTs, the secondary coils of the channel A RVDTs are monitored by both Control and Monitor lanes.  The Monitor Lane provides the excitation signal for the primary coils for the channel B RVDTs, the secondary coils of the channel B RVDTs are monitored by both Control and Monitor lanes.

#### DSP External Interface

The external interface of the DSP (XINTF) allows for mapping of external devices onto the DSP’s address space. The ARINC Bus provides the capability to communicate with other aircraft systems. The ARINC protocol which will be used is ARINC429 High Speed (Ref. C3). The SCU utilises 3 devices of type DEI-1085 ARINC 429 Enhanced Transceiver  (Ref. C11). The ARINC devices will be mapped onto the DSP’s External Interface (XINTF) Zone 1.

Refer to Ref. C7 for additional information relating to this Interface.

#### Discrete Inputs

The STC Program receives the following Open-Circuit/Ground type discrete signals via GPIO inputs:

- Pilot SHDL Engage

- Pilot SHDL Disengage

- Co_Pilot SHDL Engage

- Co_Pilot SHDL Disengage

- Towing Engage

- Towing Disengage

- EHSV1 Fail

- EHSV2 Fail

- Hydraulic Pressure

- Analog_Supplies_Ok

#### Discrete Outputs

The STC Program outputs the following Open-Circuit/Ground type discrete signals via GPIO outputs:

- LVDT/RVDT VA-VB Switch

- EHSV1 Enable/Disable

- EHSV2 Enable/Disable

- NVM SPI Enable/Disable

- Rudder1 SPI Enable/Disable

- Rudder2 SPI Enable/Disable

- McBSP SPI Enable/Disable

- Watchdog Trigger

The STC Program outputs the following 28V/Open-Circuit type discrete signals via GPIO outputs:

- SOV Energise/De-energise

#### CAN Bus Inputs/Outputs

The DSP CAN Bus will be utilised to provide an engineering interface. The CAN protocol which will be used is CAN 2.0B (Ref. C10). The STC Program will transmit messages containing the system control status data.

Refer to Ref. C9 for additional information relating to this Interface.

#### External Non-Volatile Memory Inputs/Outputs

The STC Program will utilise a Numonyx Forte Serial Flash Memory M25P16 in order to store SCU Configuration Parameters (Ref. C13). The NVM device will be connected to the DSP via McBSP SPI Bus.

Refer to Ref. C14 for additional information relating to this Interface.

#### External ADC Inputs

The STC Program will utilise two Analog Devices AD7887 devices in order to sample the analogue Rudder Pedal Position signals (Ref. C15). The ADC devices will be connected to the DSP via McBSP SPI Bus.

Refer to Ref. C14 for additional information relating to this Interface.

#### Internal Interlane Interface Inputs/Outputs

The STC Program will utilise SPI-A Bus for communicating with the Monitor Lane.

Refer to Ref. C12 for additional information relating to this Interface.

### Software/Software Interface (Internal)

The STC Program will have no internal CSCI / CSCI Interfaces.

# CSCI Software Requirements

Note: Timers in the requirements are to be interpreted in the context of the scheduling granularity and jitter. Where a time is stated as a value with a tolerance, a greater than comparison is to be interpreted as not being met for a time below the lower limit and being met when above the upper limit. Between the lower and upper limits the result is indeterminate and either result is acceptable.

## Executive - Power-up Processing

### C-Environment

| MRJ_SCU_STC_SRS_612 | During start-up the STC Program shall create the C-environment required for executing the C-functions. This includes the following tasks:  Stack Pointer initialisation,  Static Variables initialisation,  C28 Mode and Status Register initialisation,  Handover of control to the 'main' function. Justification: The STC will be programmed in 'C' and hence requires C-environment. |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

### Register Initialisation

| MRJ_SCU_STC_SRS_513 | The STC Program shall configure the DSP System Control Registers in accordance with the hardware control and protocol requirements defined in the Hardware/Software Interface Specification (HSIS) Ref. B4. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_514 | The STC Program shall configure the DSP GPIO Registers in accordance with the hardware control and protocol requirements defined in the HSIS Ref. B4. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_515 | The STC Program shall configure the DSP ADC Registers in accordance with the hardware control and protocol requirements defined in the HSIS Ref. B4. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_516 | The STC Program shall configure the DSP EV-A Registers in accordance with the hardware control and protocol requirements defined in the HSIS Ref. B4. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_877 | The STC Program shall configure the DSP EV-B Registers in accordance with the hardware control and protocol requirements defined in the HSIS Ref. B4. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_517 | The STC Program shall configure the DSP eCAN Registers in accordance with the hardware control and protocol requirements defined in the HSIS Ref. B4. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_520 | The STC Program shall configure the DSP SPI-A Registers in accordance with the hardware control and protocol requirements defined in the HSIS Ref. B4. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_525 | The STC Program shall configure the McBSP Registers in accordance with the hardware control and protocol requirements defined in the HSIS Ref. B4. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_518 | The STC Program shall configure the DSP Flash and OTP Registers in accordance with the hardware control and protocol requirements defined in the HSIS Ref. B4. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_519 | The STC Program shall configure the DSP XINTF Registers in accordance with the hardware control and protocol requirements defined in the HSIS Ref. B4. Note: Zone 1 used for ARINC, Zone 2 used for memory mapped I/O, Zones 6 and 7 not used. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_125 | The STC Program shall configure the DEI-1085 ARINC Transceiver #1 Device Control Register at address 0x4001 of Zone 1 XINTF in accordance with the hardware control and protocol requirements defined in the HSIS Ref. B4. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_522 | The STC Program shall configure the DEI-1085 ARINC Transceiver #2 Device Control Register at address 0x4011 of Zone 1 XINTF in accordance with the hardware control and protocol requirements defined in the HSIS Ref. B4. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_523 | The STC Program shall configure the DEI-1085 ARINC Transceiver #3 Device Control Register at address 0x4021 of Zone 1 XINTF in accordance with the hardware control and protocol requirements defined in the HSIS Ref. B4. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1113 | The STC Program shall perform a dummy read from Zone 2 in order to force the DSP chip select lines before a read-access from ARINC Zones 0 & 1. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_521 | The STC Program shall configure the DMCA1 ARINC 429 receiver in accordance with the HW/SW interface specification [ref B4] to keep only the labels listed for this interface in MRJ_SCU_STC_SRS_137. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_127 | The STC Program shall configure the DMCB1 ARINC 429 receiver in accordance with the HW/SW interface specification [ref B4] to keep only the labels listed for this interface in MRJ_SCU_STC_SRS_137. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_129 | The STC Program shall configure the ACE1B1 ARINC 429 receiver in accordance with the HW/SW interface specification [ref B4] to keep only the labels listed for this interface in MRJ_SCU_STC_SRS_137. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_131 | The STC Program shall configure the ACE2B1 ARINC 429 receiver in accordance with the HW/SW interface specification [ref B4] to keep only the labels listed for this interface in MRJ_SCU_STC_SRS_137. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_718 | Prior to normal processing the STC Program shall enable the T1UFINT timer interrupt and clear the first timer interrupt. Justification: First interrupt has to be disregarded as at this point there are no valid measurements to be processed (no previous ADC trigger). |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_722 | During start-up the STC Program shall initialise all unused interrupt vectors with a default Interrupt Service Routine to catch spurious interrupts and to reset the DSP. Justification: Spurious interrupts must be handled in a controlled manner. |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1260 | The STC Program shall insert a 65536 SYSCLKOUT cycles delay after writing to the PLLCR register in order for the PLL to complete the locking sequence. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

### Rigging Parameter Loading

In order to read the rigging parameters from the Numonyx NVM device, the STC Program will upon start-up configure the SPI control registers for SPI Master Mode Operation, 16-bit SPI Data Transmission/Reception and 2MHz SPI Clock output.

Note: This is defined by MRJ_SCU_STC_SRS_525.

| MRJ_SCU_STC_SRS_702 | The STC Program shall read the following parameters from NVM sector 0 and store in RAM. Justification: Required to have the values in RAM in order to add the rigging offsets during fast run-time operation. |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_709 | The STC Program shall read the following parameters from NVM sector 1 and store in RAM. Justification: Required to have the values in RAM in order to perform fast rigging data integrity check. |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

The STC Program will satisfy each of MRJ_SCU_STC_SRS_702 and MRJ_SCU_STC_SRS_709 by performing a defined sequence as follows:

1) Assert the NVM_SPI_Enable and McBSP_Enable (Chip Select Low) via GPIOs F9 and F10.

2) Transmit READ command (Op Code 0x03) followed by Address bytes and dummy bytes to SPI Tx FIFO

3) Wait until DSP transmits data

4) Deselect NVM_SPI_Enable (Chip Select High)

5) Read Data from SPI Rx FIFO

6) Assert the NVM_SPI_Enable (Chip Select Low)

7) Transmit Deep Power_Down command (Op Code 0xB9) to SPI Tx FIFO

8) Wait until DSP transmits data

9) Deselect NVM_SPI_Enable and McBSP_Enable (Chip Select High)

| MRJ_SCU_STC_SRS_710 | After reading rigging parameters from NVM, the STC Program shall check whether the parameters set read from sector 0 and stored in RAM match with the parameters set read from sector 1 and stored in RAM, and in case of mismatch repeat the reads from NVM and comparisons up to two times (i.e. up to three total read attempts, including the original read operation). Justification: Repeating the read 2 times would add about 5ms to the start-up time, which is within the software budget. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_711 | In case of a failure of the comparison of MRJ_SCU_STC_SRS_710 after the maximum amount of retries, the STC Program shall set NVM Read Failure to TRUE, and otherwise set NVM Read Failure to FALSE. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1038 | If the read value of first 2 bytes of NVM Sector 0 and NVM Sector 1 are 0xFF then the STC Program shall set the NVM Empty Flag to TRUE, and otherwise set the NVM Empty Flag to FALSE and validate the Rigging data as defined in requirements MRJ_SCU_STC_SRS_710, MRJ_SCU_STC_SRS_711, MRJ_SCU_STC_SRS_1039, MRJ_SCU_STC_SRS_1041. Justification: An NVM cell value of 0xFF at beginning of the sectors would indicate an erased NVM device. Hence SCU is not rigged, and a comparison of the two sectors would not be applicable as they don’t contain rigging values. |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1039 | The STC Program shall perform a rigging offset in-range check for the 8 LRVDT sensors as follows: IF NVM Read Failure = FALSE THEN For Pilot_RVDT1_Ratiometric_Position_Offset, Pilot_RVDT2_Ratiometric_Position_Offset, RH_LVDT1_Ratiometric_Position_Offset, RH_LVDT2_Ratiometric_Position_Offset, LH_LVDT1_Ratiometric_Position_Offset, LH_LVDT2_Ratiometric_Position_Offset: IF Sensor Rigging Offset is outside Rigging Offset Range Limit THEN In-Range Criterion Check = FAIL ELSE In-Range Criterion Check = FAIL END IF For Co-Pilot_RVDT1_Ratiometric_Position_Offset, Co-Pilot_RVDT2_Ratiometric_Position_Offset: IF Co_Pilot SHDL Fitted THEN IF Sensor Rigging Offset is outside Rigging Offset Range Limit THEN In-Range Criterion Check = FAIL ELSE In-Range Criterion Check = {No change} ENDIF ELSE In-Range Criterion Check = {No change} END IF ELSE {Rigging Offsets read from NVM Sectors 0 & 1 do not match} THEN In-Range Criterion Check = FAIL END IF Where: LVDT Rigging Offset Range Limit is ratiometric equivalent of -4 to +4 degrees. RVDT Rigging Offset Range Limit is ratiometric equivalent of -4 to +4 degrees. The ratio to degree mapping of LVDTs is specified in MRJ_SCU_STC_SRS_111. The ratio to degree mapping of RVDTs is specified in MRJ_SCU_STC_SRS_852 and MRJ_SCU_STC_SRS_84. Co_Pilot SHDL Fitted is specified in MRJ_SCU_STC_SRS_79. In-Range Criterion Check initialised to PASS NVM Read Failure is defined in MRJ_SCU_STC_SRS_711. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1041 | The STC Program shall perform the following actions depending on the In-Range Criterion check result: IF the In-Range Criterion check is PASS AND NVM Read Failure is FALSE AND NVM Empty Flag is FALSE THEN Store the 8 ratiometric offset parameters into Ratiometric_Position_Offset[channel] Set RiggingNotPerformed to FALSE Set Bit 29 of ARINC Label 106 to indicate “Aircraft Rigged”. ELSE store 0 into Ratiometric_Position_Offset[channel] Set RiggingNotPerformed to TRUE Clear Bit 29 of ARINC Label 106 to indicate “Aircraft Not Rigged”. ENDIF Where channel is in {Pilot_RVDT1, Pilot_RVDT2, Co-Pilot_RVDT1, Co-Pilot_RVDT2, RH_LVDT1, RH_LVDT2, LH_LVDT1, LH_LVDT2} In-Range Criterion check is defined in MRJ_SCU_STC_SRS_1039 NVM Read Failure is defined in MRJ_SCU_STC_SRS_711 NVM Empty Flag is defined in MRJ_SCU_STC_SRS_1038 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

### RAM Initialisation Tasks

| MRJ_SCU_STC_SRS_613 | Prior to entering runtime mode the STC Program shall clear (0) the RAM area used for storing variables. Justification: Good practice to avoid random un-initialised data. |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_611 | During start-up the STC Program shall copy time-critical functions (linker section 'ramfuncs') from flash into RAM in order to increase the runtime performance. Justification: Performance increase. |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

### Power-up Timing Constraints

| MRJ_SCU_STC_SRS_526 | The STC Program shall provide control and indication outputs within 940ms of the local power supply becoming available and the Control Lane reset signal to the Control Lane DSP being released. Note: Total start-up time for providing control and indication is 1000ms, of which 60ms are allocated to HW. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1147 | The STC Program shall enable its PWM outputs within 50ms after the release of software reset. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1182 | The STC Program shall initiate P-BIT within 440ms of the local power supply becoming available and the Control Lane reset signal to the Control Lane DSP being released. Note: Total start-up time for initiating PBIT is 500ms, of which 60ms are allocated to HW. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1256 | The STC Program shall provide Interlane messages within 160ms of the Control Lane reset signal to the Control Lane DSP being released in order to avoid the Loss-of-Interlane fault during warm-starts. Note: 40ms are reserved for the HW warm-start. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

### Power-up BIT

The time period for PBIT is defined as time from program reset to output of 1st set of ARINC labels. The following PBIT functions are being performed in every control loop cycle prior to outputting ARINC labels, and hence are described in the CBIT section:

	Register Check as defined in MRJ_SCU_STC_SRS_507

	ROM Test as defined in MRJ_SCU_STC_SRS_479 and MRJ_SCU_STC_SRS_614

	RAM Integrity Test as defined in MRJ_SCU_STC_SRS_306 and MRJ_SCU_STC_SRS_512

	Co_Pilot SHDL Operation 1 Health Monitor as defined in MRJ_SCU_STC_SRS_596

	Co_Pilot SHDL Operation 2 Health Monitor as defined in MRJ_SCU_STC_SRS_597

	EHSV1 Monitor as defined in MRJ_SCU_STC_SRS_606

	EHSV2 Monitor as defined in MRJ_SCU_STC_SRS_607

Figure 6.1-1: PBIT Period

| MRJ_SCU_STC_SRS_1185 | The STC Program shall perform Watchdog Test, Arithmetic Checksum Test and RAM Integrity Test before performing the Co-Pilot S-HDL Fitted test. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### RAM Integrity Test

| MRJ_SCU_STC_SRS_306 | During start-up the STC Program shall perform a check of the Static Random Access Memory (SRAM) used for data variables to verify its ability to store data by checking that logic 1's and 0's can be written and read back correctly from each bit in the used RAM area. Note: the SRAM area used for RAM functions will be verified by comparison with the copy in flash memory, as described in the CBIT section. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_512 | In case of a RAM check failure, the STC Program shall not service the watchdog and let it expire, in order to reset the STC Program. Note: This behaviour also complies with the Failsafe Level 1 condition of de-activating the SOV solenoid because the SOV is not yet activated at time of RAM check. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Watchdog Test and Service

| MRJ_SCU_STC_SRS_691 | Prior to entering the C-environment, the STC Program shall disable the DSP internal watchdog. Note: Internal watchdog not used as STC utilises external watchdog device. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_308 | As part of the power up sequence, and prior to enabling periodic interrupts, the STC Program shall perform a watchdog test by deliberately allowing the watchdog to timeout thus causing a processor reset: IF Watchdog test during P-BIT causes reset THEN Watchdog Test = PASS ELSE (if Watchdog test times out after 7.1ms) Watchdog Test = FAIL (unrecoverable) ENDIF |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_472 | During start-up the Watchdog shall be serviced via GPIOB0 with a high pulse width of greater than 300ns and less than 300ns to avoid a Watchdog timeout, with the only exception being the watchdog test. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

#### Reset Counter

| MRJ_SCU_STC_SRS_1006 | The STC Program shall determine whether a start-up is due to a Power On Reset (PoR) or a warm start (e.g. watchdog reset) by evaluating the discrete input signal on GPIO E1 defined in the Hardware/Software Interface Specification (HSIS) Ref. B4. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_476 | The STC Program shall maintain a counter for successive warm starts and not enter runtime mode when the counter equals or exceeds a threshold value of 10 before the next power cycle. Counter is set to 0 when last reset was a PoR Note: This enforces the Fault as Failsafe Level 1 and Non-recoverable and stops ARINC429 Data transmission. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_477 | If the reset counter has a value of at least ‘1’ then the STC Program shall raise the Watchdog Life Lost Fault with Failsafe Level 3, this will indicate an unexpected warm start has occured. Note: This has a lower priority than MRJ_SCU_STC_476. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Deleted

#### Arithmetic Checksum Test

| MRJ_SCU_STC_SRS_1187 | As part of the power up sequence, and prior to enabling periodic interrupts, the STC Program shall perform a complete arithmetic checksum validation of the program image stored in FLASH, by comparing the generated value with the embedded checksum. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Analog Supplies Check

| MRJ_SCU_STC_SRS_1258 | As part of the power up sequence, and as last task prior to enabling periodic interrupts, the STC Program shall perform the following Analog Supplies Check: WHILE Debounced_ANALOG_SUPPLIES_OK = FALSE DO Service the watchdog as defined in MRJ_SCU_STC_SRS_309 END WHILE Where Debounced_ANALOG_SUPPLIES_OK is defined in MRJ_SCU_STC_SRS_124 Note: The objective of this requirement is to ensure the analogue supplies are present by the time the sensor settling timer starts. The PWM signal is being provided within the Interrupt Service Routine. Note: Steering will still be disabled in the While loop because EHSV and SOV outputs will stay disabled until end of PBIT; refer to MRJ_SCU_STC_SRS_631 and MRJ_SCU_STC_SRS_1183. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

### Discrete Input Processing at Power-up

| MRJ_SCU_STC_SRS_1112 | The STC Program shall sample each of the Discrete Input signals defined in MRJ_SCU_STC_SRS_124 at the power up using the discrete input interface in accordance with the HW/SW Interface Specification. Note: The software debounce cycles for each of the discrete inputs are provided in the Table 6.5-5. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

## Executive - Runtime Processing

| MRJ_SCU_STC_SRS_720 | The STC Program shall calculate the processor idle time within each Major Cycle and track the smallest for the current power cycle in order to gain an indication of the worst case processor load. Major cycle is defined in Table 6.2-1 Justification: UEC internal processor load figure of 70% used on other DAL-A projects. |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

### Task Scheduling

| MRJ_SCU_STC_SRS_310 | After successful completion of power up processing as defined in Section 6.1, the STC Program ADC sampling function shall be invoked by a single timer interrupt every 5µs. Justification: Required to reach 100kHz ADC sampling rate (2 x 5us because 8 ADC channels sampled at a time). |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

After successful completion of power up processing as defined in Section 6.1, the STC Program will implement a number of iteration rates, where faster iterations are achieved as multiples of slower iterations.

The STC Program will implement iterations rates as follows:

Table 6.2-1 Iteration Rates

| MRJ_SCU_STC_SRS_311 | After successful completion of power up processing as defined in Section 6.1, the STC Program shall perform the following functions within the ADC cycle at an iteration rate given in Table 6.2-1:  LVDT/RVDT signal sampling (ADC) defined in MRJ_SCU_STC_SRS_59. Note: Entering the scheduler at 200kHz allows for sampling of 16 ADC channels in 2 steps of 8 channels each at 100kHz sampling rate. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_474 | After successful completion of power up processing as defined in Section 6.1, the STC Program shall perform the following functions in the Partial DFT cycle at the iteration rate given in Table 6.2-1:  Each Partial DFT Calculation being the processing of 32 of the 256 samples defined in MRJ_SCU_STC_SRS_60  Toggling of Watchdog service output defined in MRJ_SCU_STC_SRS_309. Note: This enables the Partial DFT calculation to be performed on each set of 32 ADC samples for each Channel as they become available. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_789 | After successful completion of power up processing as defined in Section 6.1, the STC Program shall perform the following functions in the DFT cycle at the iteration rate given in Table 6.2-1:  ARINC Input Processing - polling of ARINC429 receive buffers defined in Section 6.5.3.  ARINC Output Processing defined in Section 6.7.3.  DFT Calculation defined in MRJ_SCU_STC_SRS_60.  Control the VSW (Va/Vb Switch)defined in MRJ_SCU_STC_SRS_62. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_475 | After successful completion of power up processing as defined in Section 6.1, the STC Program shall perform the following functions within the Major cycle at an iteration rate given in Table 6.2-1:  Calculation of the magnitude of the 3.225 kHz frequency component of the LVDT and RVDT interfaces defined in MRJ_SCU_STC_SRS_60.  Ratiometric position calculation defined in MRJ_SCU_STC_SRS_65.  Total magnitude calculation defined in MRJ_SCU_STC_SRS_736.  SHDL Processing and Monitoring defined in Section 6.5.1.1.2.  SACT Logical Level Processing and Monitoring defined in Section 6.5.1.1.3.  Discrete Input Processing defined in Section 6.5.2.  Analogue Rudder Input Processing defined in 6.5.4  Steering Control Function defined in Section 6.6.  EHSV Output Control Function defined in Section 6.7.1.1.  SOV Output Control Function defined in Section 6.7.2.1.  LVDT/RVDT Enable Control defined in Section 6.7.2.2.  EHSV Enable Control defined in Section 6.7.2.3.  Built-In-Test defined in section 6.3.6 Health Monitoring.  ARINC Input Processing - decoding of received ARINC429 data (defined in Section 6.5.3).  Failsafe Level determination as defined in Section 6.3.1.Failsafe Levels  Validate Rigging data as defined in Section 6.3.7 Rigging record and data Check  SCS Rigging as defined in Section 6.15 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_314 | After successful completion of power up processing as defined in Section 6.1, the STC Program shall schedule the following functions from within the Major cycle at the Iteration rate given in Table 6.2-1:  Internal Interlane Interface Communication defined in Section 6.8.  Engineering Access Function defined in Section 8.10. Note: the complete functionality of these activities will be spread over several Major cycles. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

## Continuous Built-In-Test

Note: In the specification of the following Monitors

TRUE has been used to indicate a Failure condition identified by the Monitor indicating Unhealthy or Invalid status.

FALSE has been used to indicate there is no Failure condition identified by the Monitor indicating Healthy or Valid status.

| MRJ_SCU_STC_SRS_574 | The STC Program shall report the status of the Control Lane Monitors to the SMB Program via the Internal Interlane Interface as defined in MRJ_SCU_STC_SRS_641. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

### Failsafe Levels

Each BIT function is allocated a "Failsafe Level", the "Failsafe Level" have the following meanings:

- Failsafe Level 1 - The SCU de-activates the SOV solenoid to shut off steering control such that the SCS reverts to Free Castor

- Failsafe Level 2 - The SCU nullifies the failed part and continues steering control. The SCS will lose minor function but can still perform steering control

- Failsafe Level 3 - The SCU nullifies the failed part by using alternate data and continues steering control.  The SCS will lose redundancy, but can still perform steering control.

#### Failsafe Level 1

In case of Failsafe Level 1 faults as described below in this section, the STC Program will de-activate the SOV solenoid to shut off the steering control such that the SCS reverts to Free Castor (see MRJ_SCU_STC_SRS_194, MRJ_SCU_STC_SRS_195).

| MRJ_SCU_STC_SRS_585 | The STC Program shall enter and remain in Failsafe Level 1 when a failure is reported by any one of the following Monitors: LVDT Double Loss Monitor defined in MRJ_SCU_STC_SRS_1245, ASA Jump Monitor Status defined in MRJ_SCU_STC_SRS_1247, LVDT No Angle Found Fault defined in MRJ_SCU_STC_SRS_1240, EHSV1 and EHSV2 Monitor defined in MRJ_SCU_STC_SRS_605, Target Input Monitor is defined in MRJ_SCU_STC_SRS_608, Over Angle Monitor defined in MRJ_SCU_STC_SRS_1212, Steering Angle Monitor defined in MRJ_SCU_STC_SRS_1214, Watchdog Test Monitor defined in MRJ_SCU_STC_SRS_308, Watchdog Maximum Reset Count Monitor defined in MRJ_SCU_STC_SRS_476, ROM Test defined in MRJ_SCU_STC_SRS_478, MRJ_SCU_STC_SRS_479 and MRJ_SCU_STC_SRS_614 RAM Integrity test defined in MRJ_SCU_STC_SRS_306 and MRJ_SCU_STC_SRS_512 Rigging record and data Check defined in MRJ_SCU_STC_SRS_1043 Note1: Each of these individual Monitors is defined as non-recoverable, therefore ensuring the STC Program cannot leave Failsafe level 1 after any one of these has been reported. Note2:The ROM Test and Ram Test are classified as a recoverable Monitors in the Supplier Equipment Specification (Ref : B2) however this is only after a reset of the Control Lane. Note3: Some of the above fault conditions result in procesor resets which eventually end up with the processor stopping entry into a run-time mode. The SCU, as a whole, then behaves as if it is in Failsafe Level 1 because, firstly, the affected processor disables the SOV and secondly, the other processor will be in Failsafe Level 1 because of Loss of Interlane Communications. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_843 | The STC Program shall be in Failsafe Level 1 if an Airspeed cannot be determined in MRJ_SCU_STC_SRS_146. Note: This monitor is recoverable. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Failsafe Level 2

| MRJ_SCU_STC_SRS_809 | The STC Program shall enter Failsafe Level 2, if not in Failsafe Level 1, and remain at Failsafe Level 2 or higher, when a failure is reported by any one of the following Monitors: Pilot SHDL Operation 1 and 2 Health Monitor defined in MRJ_SCU_STC_SRS_595, Pilot SHDL Operation 1 and 2 Reasonableness Check Monitor defined in MRJ_SCU_STC_SRS_599, Co_Pilot SHDL Operation 1 and 2 Health Monitor defined in MRJ_SCU_STC_SRS_598, Co_Pilot SHDL Operation 1 and 2 Reasonableness Check Monitor defined in MRJ_SCU_STC_SRS_600 SOV Status Monitor defined in MRJ_SCU_STC_SRS_1197. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

Note: There are no failures in the STC Program that are recoverable Failsafe Level 2 faults.

#### Failsafe Level 3

| MRJ_SCU_STC_SRS_700 | The STC Program shall enter Failsafe Level 3, if not in Failsafe Level 1 or Failsafe Level 2, and remain at Failsafe Level 3 or higher, when a failure is reported by any one of the following Monitors: RH LVDT1 Health Monitor defined in MRJ_SCU_STC_SRS_67, RH LVDT2 Health Monitor defined in MRJ_SCU_STC_SRS_566, LH LVDT1 Health Monitor defined in MRJ_SCU_STC_SRS_567, LH LVDT2 Health Monitor defined in MRJ_SCU_STC_SRS_568, RH LVDT1 and RH LVDT2 Reasonableness Monitor defined in MRJ_SCU_STC_SRS_572, LH LVDT1 and LH LVDT2 Reasonableness Monitor defined in MRJ_SCU_STC_SRS_591, Pilot SHDL Operation 1 Health Monitor defined in MRJ_SCU_STC_SRS_592, Pilot SHDL Operation 2 Health Monitor defined in MRJ_SCU_STC_SRS_593, Co_Pilot SHDL Operation 1 Health Monitor defined in MRJ_SCU_STC_SRS_596, Co_Pilot SHDL Operation 2 Health Monitor defined in MRJ_SCU_STC_SRS_597, Pilot SHDL Switch Reasonableness Check Monitor defined in MRJ_SCU_STC_SRS_575, Co_Pilot SHDL Switch Reasonableness Check Monitor defined in MRJ_SCU_STC_SRS_601, Towing Switch Reasonableness Check Monitor defined in MRJ_SCU_STC_SRS_603 EHSV1 Monitor defined in MRJ_SCU_STC_SRS_606, EHSV2 Monitor defined in MRJ_SCU_STC_SRS_607, Watchdog Life Lost Fault defined in MRJ_SCU_STC_SRS_477, Cross Compare Check Monitor is defined in MRJ_SCU_STC_SRS_629, RiggingNotPerformed defined in MRJ_SCU_STC_SRS_1041, AircraftSerNumNoSync defined in MRJ_SCU_STC_SRS_1044. Notes: Each of these individual Monitors is defined as non-recoverable, therefore ensuring the STC Program cannot revert to Failsafe level lower than 3 after any one of these has been reported. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_699 | The STC Program shall be in Failsafe Level 3, if not in Failsafe Level 1 or Failsafe Level 2, while a failure is reported by any one of the following Monitors: NVM Read failure defined in and MRJ_SCU_STC_SRS_711 NVM Write failure defined in and MRJ_SCU_STC_SRS_1130 Loss of Interlane Communication is defined in MRJ_SCU_STC_SRS_915. Note: These Monitors are recoverable. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Failsafe Level None

The STC Program will determine and report the following Monitors, although they have no direct input into the Failsafe Level determination calculation.

ACE1B_Label 136 Monitor (Rudder Pedal Data) as defined in MRJ_SCU_STC_SRS_828,

ACE2B_Label 136 Monitor (Rudder Pedal Data) as defined in MRJ_SCU_STC_SRS_830,

ACE1B Reasonableness Check Monitor as defined in MRJ_SCU_STC_SRS_832,

ACE2B Reasonableness Check Monitor as defined in MRJ_SCU_STC_SRS_832,

ACE1B Label 136 and ACE2B Label 136 Reasonableness Check Monitor as defined in MRJ_SCU_STC_SRS_838.

Note: These Monitors are recoverable.

### Watchdog Service

| MRJ_SCU_STC_SRS_309 | The STC program shall service the Watchdog Timer (WDT) via GPIOB0 to avoid a Watchdog timeout during its finite life cycle with a high pulse width of greater than 300 ns and less than 3 ms. Note: Error conditions which require a watchdog reset as defined in MRJ_SCU_STC_SRS_313 overrule this requirement. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_313 | The STC Program shall disable interrupts and force a watchdog reset if any of the following error conditions occurred:  Thread is detected to have overrun as defined in MRJ_SCU_STC_SRS_312.  Register Configuration is detected to be corrupted as defined in MRJ_SCU_STC_SRS_507 and MRJ_SCU_STC_SRS_1189.  32-Bit CRC failure detected as defined in MRJ_SCU_STC_SRS_478.  Arithmetic Checksum failure detected as defined in MRJ_SCU_STC_SRS_479.  Rigging record and data Check defined in MRJ_SCU_STC_SRS_1043  RAM Code Check failure detected as defined in MRJ_SCU_STC_SRS_614. Internal Power Supply Monitor detected as defined in MRJ_SCU_STC_SRS_1168 Note: This action prevents servicing of the ARINC 429 devices and hence stops ARINC 429 bus transmissions. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_734 | Prior to forcing a watchdog reset, the STC Program shall set the GPIO outputs to default states as defined in the HW/SW interface specification [ref B4]. Justification: Setting the GPIOs to default states ensures system is in safe condition until the reset and re-initialisation of GPIOs occurs. |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

### Runtime Integrity Check

| MRJ_SCU_STC_SRS_312 | After successful completion of power up processing as defined in Section 6.1, the STC Program shall perform runtime checks to ensure that processing threads from the previous processing cycle have completed before being scheduled in. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : |
|  | Verification Method : |

### Register Check

| MRJ_SCU_STC_SRS_507 | The STC Program shall periodically compare the contents of its processor configuration registers with the values set at initialisation; and react to a mismatch as follows: IF for any register listed in Table “Register Check Mask Values” the following condition is met: (Initial Value bitwise-AND Mask) <> (Current Value bitwise-AND Mask) THEN Disable interrupts and force a watchdog reset ELSE No action ENDIF where: the period for reading the configuration registers is every major cycle (MRJ_SCU_STC_SRS_786); the Initial Value is set at initialisation as specified in Section "Register Initialisation" (MRJ_SCU_STC_SRS_463); Mask is as specified in Table “Register Check Mask Values”; Current Value is the value of the register read in the processing cycle in which the check is being performed. Table - Register Check Mask Values |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1189 | During Runtime processing, the STC Program shall verify the ARINC devices DEI-1085 control register configuration within every 100ms to confirm that the register configuration has not been corrupted. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

### ROM Test

#### 32-Bit CRC Failure

The STC Program will verify the program image stored in FLASH by checking the resulting 32-bit CRC and arithmetic checksum values against the values generated and stored in the image during the Configuration Management (CM) release process.

| MRJ_SCU_STC_SRS_478 | During Run Time processing, the STC Program shall perform a complete 32-bit CRC validation of the program image stored in FLASH at least once every 20 seconds by comparing the generated value with the embedded checksum value. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_802 | The STC Program shall satisfy MRJ_SCU_STC_SRS_478 by applying the following algorithm: The Seed value is 0x746D7361. The CRC is initialised to the seed. Set Starting Address to beginning of program image to be checked. Set Length to the number of 32-bit memory locations to perform the CRC algorithm. Perform the following loop Length times: Loop At Starting Address take in turn the 2nd byte, 1st byte, 4th byte and 3rd byte and perform the following: Read the current 8-bit data byte CRC = (CRC right shifted by 8 bits) XOR CRC_Table([CRC XOR current 8-bit data byte] AND 0x000000FF) Increment Starting Address to next 32-bit data in program image to be checked. End Loop Where The CRC_Table is given in MRJ_SCU_STC_SRS_688 Justification: Recommended 32-bit CRC algorithm as applied on other DAL-A projects. |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

#### Arithmetic Checksum

| MRJ_SCU_STC_SRS_479 | During Run Time processing, the STC Program shall perform a complete arithmetic checksum validation of the program image stored in FLASH at least every 30ms, by comparing the generated value with the embedded checksum. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### RAM Code Check

| MRJ_SCU_STC_SRS_614 | The STC Program shall compare any code copies located in RAM with their code image located in flash memory at least every 100ms to confirm that the software has not been corrupted. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

### Health Monitoring

| MRJ_SCU_STC_SRS_659 | The STC Program shall use the following Limit values in calculating Health Monitors (data format assumes straight signed 16-bit comparisons): Table 6.3-1 Monitoring Limits |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### RH LVDT1 Health Monitor

| MRJ_SCU_STC_SRS_67 | The STC Program shall determine RH LVDT1 Health Monitor status for the RH_LVDT1 channel defined in MRJ_SCU_STC_SRS_59 with a confirmation time of 150ms using the following logic: IF (RH LVDT1 Health Monitor Status = FALSE) THEN RH LVDT1 Health Monitor Status = ( Va_MAG [RH_LVDT1] < Lower_LVDT_Limit_1 ) OR ( Va_MAG [RH_LVDT1] > Upper_LVDT_Limit_1 ) OR ( Vb_MAG [RH_LVDT1] < Lower_LVDT_Limit_1 ) OR ( Vb_MAG [RH_LVDT1] > Upper_LVDT_Limit_1 ) OR ( (SUM_V [RH_LVDT1]) < Lower_LVDT_Sum_Limit_1 ) OR ( (SUM_V [RH_LVDT1]) > Upper_LVDT_Sum_Limit_1 ) ELSE do nothing (monitor status remains TRUE, making it Non-recoverable) ENDIF Where Va_MAG and Vb_MAG are the respective 3125 Hz component magnitudes calculated by MRJ_SCU_STC_SRS_736 for the RH LVDT1 interface and SUM_V is calculated by MRJ_SCU_STC_SRS_66 for the RH LVDT1 interface Lower_LVDT_Limit_1 is defined in MRJ_SCU_STC_SRS_659 Upper_LVDT_Limit_1 is defined in MRJ_SCU_STC_SRS_659 Lower_LVDT_Sum_Limit_1 is defined in MRJ_SCU_STC_SRS_659 Upper_LVDT_Sum_Limit_1 is defined in MRJ_SCU_STC_SRS_659 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_916 | During fault confirmation of the RH LVDT1 Health Monitor the STC Program shall use the last valid in range ratiometric value for RH_LVDT1 in SACT Logical Level Processing. With last valid in range ratiometric value = ratiometric value (most recent cycle - N cycles) N = 20 ie. time of (N * Major Cycle) = 102.4ms Initial value for last valid in range ratiometric values is -0.3202 (in Q0.15). |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### RH LVDT2 Health Monitor

| MRJ_SCU_STC_SRS_566 | The STC Program shall determine RH LVDT2 Health Monitor status for the RH_LVDT2 channel defined in MRJ_SCU_STC_SRS_59 with a confirmation time of 150ms using the following logic: IF (RH LVDT2 Health Monitor Status = FALSE) THEN RH LVDT2 Health Monitor Status = ( Va_MAG [RH_LVDT2] < Lower_LVDT_Limit_2 ) OR ( Va_MAG [RH_LVDT2] > Upper_LVDT_Limit_2 ) OR ( Vb_MAG [RH_LVDT2] < Lower_LVDT_Limit_2 ) OR ( Vb_MAG [RH_LVDT2] > Upper_LVDT_Limit_2 ) OR ( (SUM_V [RH_LVDT2]) < Lower_LVDT_Sum_Limit_2 ) OR ( (SUM_V [RH_LVDT2]) > Upper_LVDT_Sum_Limit_2 ) ELSE do nothing (monitor status remains TRUE, making it Non-recoverable) ENDIF Where Va_MAG and Vb_MAG are the respective 3225 Hz component magnitudes calculated by MRJ_SCU_STC_SRS_736 for the RH LVDT2 interface and SUM_V is calculated by MRJ_SCU_STC_SRS_66 for the RH LVDT2 interface Lower_LVDT_Limit_2 is defined in MRJ_SCU_STC_SRS_659 Upper_LVDT_Limit_2 is defined in MRJ_SCU_STC_SRS_659 Lower_LVDT_Sum_Limit_2 is defined in MRJ_SCU_STC_SRS_659 Upper_LVDT_Sum_Limit_2 is defined in MRJ_SCU_STC_SRS_659 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_917 | During fault confirmation of the RH LVDT2 Health Monitor the STC Program shall use the last valid in range ratiometric value for RH_LVDT2 in SACT Logical Level Processing. With last valid in range ratiometric value = ratiometric value (most recent cycle - N cycles) N = 20 ie. time of (N * Major Cycle) = 102.4ms Initial value for last valid in range ratiometric values is -0.3202 (in Q0.15). |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Deleted

#### RH LVDT1 and RH LVDT2 Reasonableness Check Monitor

| MRJ_SCU_STC_SRS_572 | The STC Program shall determine the status of the monitor using the following logic: IF (RH LVDT1 and RH LVDT2 Reasonableness Check Monitor Status = FALSE) THEN IF (RH LVDT1 Health Monitor status = TRUE) OR (RH LVDT2 Health Monitor status = TRUE) THEN ( The evaluation of the monitor is Inhibited ) RH LVDT1 and RH LVDT2 Reasonableness Check Monitor Status = FALSE ELSE with a confirmation time of 200ms RH LVDT1 and RH LVDT2 Reasonableness Check Monitor Status = Absolute( Ratiometric_Position[RH_LVDT1] - Ratiometric_Position[RH_LVDT2] ) > LVDT_Ratio_Diff_Limit ENDIF ELSE do nothing (monitor status remains TRUE, making it Non-recoverable) ENDIF Where Ratiometric_Position is defined in MRJ_SCU_STC_SRS_65 LVDT_Ratio_Diff_Limit is defined in MRJ_SCU_STC_SRS_659 RH_LVDT1 and RH_LVDT2 are channels defined in MRJ_SCU_STC_SRS_59 RH LVDT1 Health Monitor is defined in MRJ_SCU_STC_SRS_67 RH LVDT2 Health Monitor is defined in MRJ_SCU_STC_SRS_566 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_941 | During fault confirmation of the RH LVDT1 and RH LVDT2 Reasonableness Check Monitor the STC Program shall use the last valid in range ratiometric value in SACT Logical Level Processing. With last valid in range ratiometric value = ratiometric value (most recent cycle - N cycles) N = 20 ie. time of (N * Major Cycle) = 102.4ms Initial value for last valid in range ratiometric values is -0.3202 (in Q0.15). |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### LH LVDT1 Health Monitor

| MRJ_SCU_STC_SRS_567 | The STC Program shall determine LH LVDT1 Health Monitor status for the LH_LVDT1 channel defined in MRJ_SCU_STC_SRS_59 with a confirmation time of 150ms using the following logic: IF LH LVDT1 Health Monitor Status = FALSE THEN LH LVDT1 Health Monitor Status = ( Va_MAG [LH_LVDT1] < Lower_LVDT_Limit_1 ) OR ( Va_MAG [LH_LVDT1] > Upper_LVDT_Limit_1 ) OR ( Vb_MAG [LH_LVDT1] < Lower_LVDT_Limit_1 ) OR ( Vb_MAG[LH_LVDT1] > Upper_LVDT_Limit_1 ) OR ( (SUM_V [LH_LVDT1]) < Lower_LVDT_Sum_Limit_1 ) OR ( (SUM_V [LH_LVDT1]) > Upper_LVDT_Sum_Limit_1 ) ELSE do nothing (monitor status remains TRUE, making it Non-recoverable) ENDIF Where Va_MAG and Vb_MAG are the respective 3125 Hz component magnitudes calculated by MRJ_SCU_STC_SRS_736 for the LH LVDT1 interface and SUM_V is calculated by MRJ_SCU_STC_SRS_66 for the LH LVDT1 interface Lower_LVDT_Limit_1 is defined in MRJ_SCU_STC_SRS_659 Upper_LVDT_Limit_1 is defined in MRJ_SCU_STC_SRS_659 Lower_LVDT_Sum_Limit_1 is defined in MRJ_SCU_STC_SRS_659 Upper_LVDT_Sum_Limit_1 is defined in MRJ_SCU_STC_SRS_659 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_918 | During fault confirmation of the LH LVDT1 Health Monitor the STC Program shall use the last valid in range ratiometric value for LH_LVDT1 in SACT Logical Level Processing. With last valid in range ratiometric value = ratiometric value (most recent cycle - N cycles) N = 20 ie. time of (N * Major Cycle) = 102.4ms Initial value for last valid in range ratiometric values is -0.3202 (in Q0.15). |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### LH LVDT2 Health Monitor

| MRJ_SCU_STC_SRS_568 | The STC Program shall determine LH LVDT2 Health Monitor Status for the LH_LVDT2 channel defined in MRJ_SCU_STC_SRS_59 with a confirmation time of 150ms using the following logic: IF (LH LVDT2 Health Monitor Status = FALSE) THEN LH LVDT2 Health Monitor Status = ( Va_MAG [LH_LVDT2] < Lower_LVDT_Limit_2 ) OR ( Va_MAG [LH_LVDT2] > Upper_LVDT_Limit_2 ) OR ( Vb_MAG [LH_LVDT2] < Lower_LVDT_Limit_2 ) OR ( Vb_MAG [LH_LVDT2] > Upper_LVDT_Limit_2 ) OR ( (SUM_V [LH_LVDT2]) < Lower_LVDT_Sum_Limit_2 ) OR ( (SUM_V [LH_LVDT2]) > Upper_LVDT_Sum_Limit_2 ) ELSE do nothing (monitor status remains TRUE, making it Non-recoverable) ENDIF Where Va_MAG and Vb_MAG are the respective 3225 Hz component magnitudes calculated by MRJ_SCU_STC_SRS_736 for the LH LVDT2 interface and SUM_V is calculated by MRJ_SCU_STC_SRS_66 for the LH LVDT2 interface Lower_LVDT_Limit_2 is defined in MRJ_SCU_STC_SRS_659 Upper_LVDT_Limit_2 is defined in MRJ_SCU_STC_SRS_659 Lower_LVDT_Sum_Limit_2 is defined in MRJ_SCU_STC_SRS_659 Upper_LVDT_Sum_Limit_2 is defined in MRJ_SCU_STC_SRS_659 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_919 | During fault confirmation of the LH LVDT2 Health Monitor the STC Program shall use the last valid in range ratiometric value for LH_LVDT2 in SACT Logical Level Processing. With last valid in range ratiometric value = ratiometric value (most recent cycle - N cycles) N = 20 ie. time of (N * Major Cycle) = 102.4ms Initial value for last valid in range ratiometric values is -0.3202 (in Q0.15). |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Deleted

#### LH LVDT1 and LH LVDT2 Reasonableness Check Monitor

| MRJ_SCU_STC_SRS_591 | The STC Program shall determine the status of the monitor using the following logic: IF (LH LVDT1 and LH LVDT2 Reasonableness Check Monitor Status = FALSE) THEN IF (LH LVDT1 Health Monitor status = TRUE) OR (LH LVDT2 Health Monitor status = TRUE) THEN ( The evaluation of the monitor is Inhibited ) LH LVDT1 and LH LVDT2 Reasonableness Check Monitor Status = FALSE ELSE with a confirmation time of 200ms LH LVDT1 and LH LVDT2 Reasonableness Check Monitor Status = Absolute( Ratiometric_Position[LH_LVDT1] - Ratiometric_Position[LH_LVDT2] ) > LVDT_Ratio_Diff_Limit ENDIF ELSE do nothing (monitor status remains TRUE, making it Non-recoverable) ENDIF Where Ratiometric_Position is defined in MRJ_SCU_STC_SRS_65 LVDT_Ratio_Diff_Limit is defined in MRJ_SCU_STC_SRS_659 LH_LVDT1 and LH_LVDT2 are channels defined in MRJ_SCU_STC_SRS_59 LH LVDT1 Health Monitor is defined in MRJ_SCU_STC_SRS_567 LH LVDT2 Health Monitor is defined in MRJ_SCU_STC_SRS_568 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_942 | During fault confirmation of the LH LVDT1 and LH LVDT2 Reasonableness Check Monitor the STC Program shall use the last valid in range ratiometric value in SACT Logical Level Processing. With last valid in range ratiometric value = ratiometric value (most recent cycle - N cycles) N = 20 ie. time of (N * Major Cycle) = 102.4ms Initial value for last valid in range ratiometric values is -0.3202 (in Q0.15). |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Pilot SHDL Operation 1 Health Monitor

| MRJ_SCU_STC_SRS_592 | The STC Program shall determine Pilot SHDL Operation 1 Health Monitor Status for the Pilot_RVDT1 channel defined in MRJ_SCU_STC_SRS_59 with a confirmation time of 150ms using the following logic: IF (Pilot SHDL Operation 1 Health Monitor Status = FALSE) THEN Pilot SHDL Operation 1 Health Monitor Status = ( Va_MAG [Pilot_RVDT1] < Lower_RVDT_Limit_1 ) OR ( Va_MAG [Pilot_RVDT1] > Upper_RVDT_Limit_1 ) OR ( Vb_MAG [Pilot_RVDT1] < Lower_RVDT_Limit_1 ) OR ( Vb_MAG [Pilot_RVDT1] > Upper_RVDT_Limit_1 ) OR ( (SUM_V [Pilot_RVDT1]) < Lower_RVDT_Sum_Limit_1 ) OR ( (SUM_V [Pilot_RVDT1]) > Upper_RVDT_Sum_Limit_1 ) ELSE do nothing (monitor status remains TRUE, making it Non-recoverable) ENDIF Where Va_MAG and Vb_MAG are the respective 3125 Hz component magnitudes calculated by MRJ_SCU_STC_SRS_736 for the Pilot_RVDT1 interface and SUM_V is calculated by MRJ_SCU_STC_SRS_66 for the Pilot_RVDT1 interface Lower_RVDT_Limit_1 is defined in MRJ_SCU_STC_SRS_659 Upper_RVDT_Limit_1 is defined in MRJ_SCU_STC_SRS_659 Lower_RVDT_Sum_Limit_1 is defined in MRJ_SCU_STC_SRS_659 Upper_RVDT_Sum_Limit_1 is defined in MRJ_SCU_STC_SRS_659 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_920 | During fault confirmation of the Pilot SHDL Operation 1 Health Monitor the STC Program shall use the last valid in range ratiometric value for Pilot_RVDT1 in SHDL Logical Level Processing. With last valid in range ratiometric value = ratiometric value (most recent cycle - N cycles) N = 20 ie. time of (N * Major Cycle) = 102.4ms Initial value for last valid in range ratiometric values is 0. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Pilot SHDL Operation 2 Health Monitor

| MRJ_SCU_STC_SRS_593 | The STC Program shall determine Pilot SHDL Operation 2 Health Monitor Status for the Pilot_RVDT2 channel defined in MRJ_SCU_STC_SRS_59 with a confirmation time of 150ms using the following logic: IF (Pilot SHDL Operation 2 Health Monitor Status = FALSE) THEN Pilot SHDL Operation 2 Health Monitor Status = ( Va_MAG [Pilot_RVDT2] < Lower_RVDT_Limit_2 ) OR ( Va_MAG [Pilot_RVDT2] > Upper_RVDT_Limit_2 ) OR ( Vb_MAG [Pilot_RVDT2] < Lower_RVDT_Limit_2 ) OR ( Vb_MAG [Pilot_RVDT2] > Upper_RVDT_Limit_2 ) OR ( (SUM_V [Pilot_RVDT2]) < Lower_RVDT_Sum_Limit_2 ) OR ( (SUM_V [Pilot_RVDT2]) > Upper_RVDT_Sum_Limit_2 ) ELSE do nothing (monitor status remains TRUE, making it Non-recoverable) ENDIF Where Va_MAG and Vb_MAG are the respective 3225 Hz component magnitudes calculated by MRJ_SCU_STC_SRS_736 for the Pilot_RVDT2 interface and SUM_V is calculated by MRJ_SCU_STC_SRS_66 for the Pilot_RVDT2 interface Lower_RVDT_Limit_2 is defined in MRJ_SCU_STC_SRS_659 Upper_RVDT_Limit_2 is defined in MRJ_SCU_STC_SRS_659 Lower_RVDT_Sum_Limit_2 is defined in MRJ_SCU_STC_SRS_659 Upper_RVDT_Sum_Limit_2 is defined in MRJ_SCU_STC_SRS_659 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_921 | During fault confirmation of the Pilot SHDL Operation 2 Health Monitor the STC Program shall use the last valid in range ratiometric value for Pilot_RVDT2 in SHDL Logical Level Processing. With last valid in range ratiometric value = ratiometric value (most recent cycle - N cycles) N = 20 ie. time of (N * Major Cycle) = 102.4ms Initial value for last valid in range ratiometric values is 0. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Pilot SHDL Operation 1 and 2 Health Monitor

| MRJ_SCU_STC_SRS_595 | The STC Program shall determine Pilot S-HDL Operation 1 and 2 Health Monitor Status using the following logic: Pilot SHDL Operation 1 and 2 Health Monitor Status = (Pilot SHDL Operation 1 Health Monitor Status) AND (Pilot SHDL Operation 2 Health Monitor Status) Note: As the two individual Monitors used to determine this Monitor are Non-recoverable, this Monitor will be Non-recoverable. Where Pilot SHDL Operation 1 Health Monitor Status is defined in MRJ_SCU_STC_SRS_592 and Pilot SHDL Operation 2 Health Monitor Status is defined in MRJ_SCU_STC_SRS_593 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Pilot SHDL Operation 1 and 2 Reasonableness Check Monitor

| MRJ_SCU_STC_SRS_599 | The STC Program shall determine the status of the monitor using the following logic: IF (Pilot SHDL Operation 1 and 2 Reasonableness Check Monitor Status = FALSE) THEN IF (Pilot SHDL Operation 1 Health Monitor Status = TRUE) OR (Pilot SHDL Operation 2 Health Monitor Status = TRUE) THEN ( The evaluation of the monitor is Inhibited ) Pilot SHDL Operation 1 and 2 Reasonableness Check Monitor Status = FALSE ELSE with a confirmation time of 200ms Pilot SHDL Operation 1 and 2 Reasonableness Check Monitor Status = Absolute( Ratiometric_Position[Pilot_RVDT1] - Ratiometric_Position[Pilot_RVDT2] ) > RVDT_Ratio_Diff_Limit ENDIF ELSE do nothing (monitor status remains TRUE, making it Non-recoverable) ENDIF Where Ratiometric_Position is defined in MRJ_SCU_STC_SRS_65 RVDT_Ratio_Diff_Limit is defined in MRJ_SCU_STC_SRS_659 Pilot_RVDT1 and Pilot_RVDT2 are channels defined in MRJ_SCU_STC_SRS_59 Pilot SHDL Operation 1 Health Monitor Status is defined in MRJ_SCU_STC_SRS_592 Pilot SHDL Operation 2 Health Monitor Status is defined in MRJ_SCU_STC_SRS_593 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1020 | During fault confirmation of the Pilot SHDL Operation 1 and 2 Reasonableness Check Monitor the STC Program shall use the last in range ratiometric value for Pilot_RVDT1 and Pilot_RVDT2 in SHDL Logical Level Processing. With last valid in range ratiometric value = ratiometric value (most recent cycle - N cycles) N = 20 ie. time of (N * Major Cycle) = 102.4ms Initial value for last valid in range ratiometric values is 0. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Co_Pilot SHDL Operation 1 Health Monitor

| MRJ_SCU_STC_SRS_596 | The STC Program shall determine Co_Pilot SHDL Operation 1 Health Monitor Status for the Co_Pilot_RVDT1 channel defined in MRJ_SCU_STC_SRS_59 with a confirmation time of 150ms using the following logic: IF during PBIT OR (Co_Pilot_SHDL_Fitted) THEN IF (Co_Pilot SHDL Operation 1 Health Monitor Status = FALSE) THEN Co_Pilot SHDL Operation 1 Health Monitor Status = ( Va_MAG [Co_Pilot_RVDT1] < Lower_RVDT_Limit_1 ) OR ( Va_MAG [Co_Pilot_RVDT1] > Upper_RVDT_Limit_1 ) OR ( Vb_MAG [Co_Piot_RVDT1] < Lower_RVDT_Limit_1 ) OR ( Vb_MAG [Co_Pilot_RVDT1] > Upper_RVDT_Limit_1 ) OR ( (SUM_V [Co_Pilot_RVDT1]) < Lower_RVDT_Sum_Limit_1 ) OR ( (SUM_V [Co_Pilot_RVDT1]) > Upper_RVDT_Sum_Limit_1 ) ELSE do nothing (monitor status remains TRUE, making it Non-recoverable) ENDIF ELSE Co_Pilot SHDL Operation 1 Health Monitor Status = FALSE ENDIF Where Va_MAG and Vb_MAG are the respective 3125 Hz component magnitudes calculated by MRJ_SCU_STC_SRS_736 for the Co_Pilot_RVDT1 interface and SUM_V is calculated by MRJ_SCU_STC_SRS_66 for the Co_Pilot_RVDT1 interface Lower_RVDT_Limit_1 is defined in MRJ_SCU_STC_SRS_659 Upper_RVDT_Limit_1 is defined in MRJ_SCU_STC_SRS_659 Lower_RVDT_Sum_Limit_2 is defined in MRJ_SCU_STC_SRS_659 Upper_RVDT_Sum_Limit_2 is defined in MRJ_SCU_STC_SRS_659 Co_Pilot_SHDL_Fitted is defined in MRJ_SCU_STC_SRS_79 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_922 | During fault confirmation of the Co_Pilot SHDL Operation 1 Health Monitor the STC Program shall use the last valid in range ratiometric value for Co_Pilot_RVDT1 and Co_Pilot_RVDT1 in SHDL Logical Level Processing. With last valid in range ratiometric value = ratiometric value (most recent cycle - N cycles) N = 20 ie. time of (N * Major Cycle) = 102.4ms Initial value for last valid in range ratiometric values is 0. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Co_Pilot SHDL Operation 2 Health Monitor

| MRJ_SCU_STC_SRS_597 | The STC Program shall determine Co_Pilot SHDL Operation 2 Health Monitor Status for the Co_PIlot_RVDT2 channel defined in MRJ_SCU_STC_SRS_59 with a confirmation time of 150ms using the following logic: IF during PBIT OR (Co_Pilot_SHDL_Fitted) THEN IF (Co_Pilot SHDL Operation 2 Health Monitor Status = FALSE) THEN Co_Pilot SHDL Operation 2 Health Monitor Status = ( Va_MAG [Co_Pilot_RVDT2] < Lower_RVDT_Limit_2 ) OR ( Va_MAG [Co_Pilot_RVDT2] > Upper_RVDT_Limit_2 ) OR ( Vb_MAG [Co_Pilot_RVDT2] < Lower_RVDT_Limit_2 ) OR ( Vb_MAG [Co_PIlot_RVDT2] > Upper_RVDT_Limit_2 ) OR ( (SUM_V [Co_Pilot_RVDT2]) < Lower_RVDT_Sum_Limit_2 ) OR ( (SUM_V [Co_Pilot_RVDT2]) > Upper_RVDT_Sum_Limit_2 ) ELSE do nothing (monitor status remains TRUE, making it Non-recoverable) ENDIF ELSE Co_Pilot SHDL Operation 2 Health Monitor Status = FALSE ENDIF Where Va_MAG and Vb_MAG are the respective 3225 Hz component magnitudes calculated by MRJ_SCU_STC_SRS_736 for the Co_Pilot_RVDT2 interface and SUM_V is calculated by MRJ_SCU_STC_SRS_66 for the Co_Pilot_RVDT2 interface Lower_RVDT_Limit_2 is defined in MRJ_SCU_STC_SRS_659 Upper_RVDT_Limit_2 is defined in MRJ_SCU_STC_SRS_659 Lower_RVDT_Sum_Limit_2 is defined in MRJ_SCU_STC_SRS_659 Upper_RVDT_Sum_Limit_2 is defined in MRJ_SCU_STC_SRS_659 Co_Pilot_SHDL_Fitted is defined in MRJ_SCU_STC_SRS_79 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_923 | During fault confirmation of the Co_Pilot SHDL Operation 2 Health Monitor the STC Program shall use the last valid in range ratiometric value for Co_Pilot_RVDT2 and Co_Pilot_RVDT2 in SHDL Logical Level Processing. With last valid in range ratiometric value = ratiometric value (most recent cycle - N cycles) N = 20 ie. time of (N * Major Cycle) = 102.4ms Initial value for last valid in range ratiometric values is 0. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Co_Pilot SHDL Operation 1 and 2 Health Monitor

| MRJ_SCU_STC_SRS_598 | The STC Program shall determine Co_Pilot SHDL Operation 1 and 2 Health Monitor Status using the following logic: Co_Pilot SHDL Operation 1 and 2 Health Monitor Status = (Co_Pilot SHDL Operation 1 Health Monitor Status) AND (Co_Pilot SHDL Operation 2 Health Monitor Status) Note: As the two individual Monitors used to determine this Monitor are Non-recoverable, this Monitor will be Non-recoverable. Where Co_Pilot SHDL Operation 1 Health Monitor Status is calculated in MRJ_SCU_STC_SRS_596 and Co_Pilot SHDL Operation 2 Health Monitor Status is calculated in MRJ_SCU_STC_SRS_597 Note: Will evaluate to FALSE when Co_Pilot_SHDL_Fitted = FALSE |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Co_Pilot SHDL Operation 1 and 2 Reasonableness Check Monitor

| MRJ_SCU_STC_SRS_600 | The STC Program shall determine the status of the monitor using the following logic: IF NOT (Co_Pilot_SHDL_Fitted) THEN Co_Pilot SHDL Operation 1 and 2 Reasonableness Check Monitor Status = FALSE ELSE IF (Co_Pilot SHDL Operation 1 and 2 Reasonableness Check Monitor Status = FALSE) THEN IF (Co_Pilot SHDL Operation 1 Health Monitor status) OR (Co_Pilot SHDL Operation 2 Health Monitor status) THEN ( The evaluation of the monitor is Inhibited ) Co_Pilot SHDL Operation 1 and 2 Reasonableness Check Monitor Status = FALSE ELSE with a confirmation time of 200ms Co_Pilot SHDL Operation 1 and 2 Reasonableness Check Monitor Status = Absolute( Ratiometric_Position[Co_Pilot_RVDT1] - Ratiometric_Position[Co_Pilot_RVDT2] ) > RVDT_Ratio_Diff_Limit ENDIF ELSE do nothing (monitor status remains TRUE, making it Non-recoverable) ENDIF ENDIF Where Ratiometric_Position is defined in MRJ_SCU_STC_SRS_65 RVDT_Ratio_Diff_Limit is defined in MRJ_SCU_STC_SRS_659 Co_Pilot_RVDT1 and Co_Pilot_RVDT2 are channels defined in MRJ_SCU_STC_SRS_59 Co_Pilot SHDL Operation 1 Health Monitor status is defined in MRJ_SCU_STC_SRS_596 Co_Pilot SHDL Operation 2 Health Monitor status is defined in MRJ_SCU_STC_SRS_597 Co_Pilot_SHDL_Fitted is defined in MRJ_SCU_STC_SRS_79 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_943 | During fault confirmation of the Co_Pilot SHDL Operation 1 and 2 Reasonableness Check Monitor the STC Program shall use the last valid in range ratiometric value for Co_Pilot_RVDT1 and Co_Pilot_RVDT2 in SHDL Logical Level Processing. With last valid in range ratiometric value = ratiometric value (most recent cycle - N cycles) N = 20 ie. time of (N * Major Cycle) = 102.4ms Initial value for last valid in range ratiometric values is 0. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Pilot SHDL Switch Reasonableness Check Monitor

| MRJ_SCU_STC_SRS_575 | The STC Program shall determine Pilot SHDL Switch Reasonableness Check Monitor status with a confirmation time of 500ms according to the following logic: Table 6.3-2 Pilot SHDL Switch Truth Table Where Debounced_Pilot_Engage and Debounced_Pilot_Disengage are the values calculated in MRJ_SCU_STC_SRS_164 from Raw_Pilot_Engage and Raw_Pilot_Disengage defined in MRJ_SCU_STC_SRS_124 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Co_Pilot SHDL Switch Reasonableness Check Monitor

| MRJ_SCU_STC_SRS_601 | If Co_Pilot_SHDL_Fitted = TRUE then the STC Program shall determine the Co_Pilot SHDL Switch Reasonableness Check Monitor status with a confirmation time of 500ms according to the following logic, and otherwise not evaluate this monitor and set its status to FALSE: Table 6.3-3 Co_Pilot SHDL Switch Truth Table Where Debounced_Co_Pilot_Engage and Debounced_Co_Pilot_Disengage are the values calculated in MRJ_SCU_STC_SRS_164 from Raw_Co_Pilot_Engage and Raw_Co_Pilot_Disengage defined in MRJ_SCU_STC_SRS_124 Co_Pilot_SHDL_Fitted is defined in MRJ_SCU_STC_SRS_79 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Towing Switch Reasonableness Check Monitor

| MRJ_SCU_STC_SRS_603 | The STC Program shall determine the Towing Switch Reasonableness Check Monitor status with a confirmation time of 500ms according to the following logic: Table 6.3-4 Towing Switch Truth Table Where Debounced_Towing_Engage and Debounced_Towing_Disengage are the values calculated in MRJ_SCU_STC_SRS_164 from Raw_Towing_Engage and Raw_Towing_Disengage defined in MRJ_SCU_STC_SRS_124 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### A_DMC_9 LADC Monitor

| MRJ_SCU_STC_SRS_811 | The STC Program shall determine A_DMC_9 LADC Monitor Status according to the following logic: IF the following condition is true for 320ms with a tolerance of +10ms (( No valid A_DMC_9 LADC labels have been receive ) OR (A_DMC_9 LADC data have wrong (even) parity bit) OR (A_DMC_9 LADC have SSM data not set to "Normal Operation")) THEN A_DMC_9 LADC Monitor Status = TRUE ELSE IF the following condition is true for 320ms with a tolerance of +10ms ((A_DMC_9 LADC labels have been received at expected update rate with a tolerance of +10ms ) AND (A_DMC_9 LADC data have correct (odd) parity bit) AND (A_DMC_9 LADC have SSM data set to "Normal Operation") AND (A_DMC_9 LADC Airspeed < 35kts)) THEN A_DMC_9 LADC Monitor Status = FALSE ELSE No action (A_DMC_9 LADC Monitor Status remains unchanged) ENDIF ENDIF Where A_DMC_9 LADC ARINC Message is defined in MRJ_SCU_STC_SRS_135 Justification for timing tolerance: ARINC label reception requires tolerance for label polling and processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_946 | During fault confirmation of the A_DMC_9 LADC Monitor the STC Program shall use the last valid data for Aircraft Speed Processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### A_DMC_9 RADC Monitor

| MRJ_SCU_STC_SRS_1051 | The STC Program shall determine A_DMC_9 RADC Monitor Status according to the following logic: IF the following condition is true for 320ms with a tolerance of +10ms ( No valid A_DMC_9 RADC labels have been received) OR (A_DMC_9 RADC data have wrong (even) parity bit) OR (A_DMC_9 RADC have SSM data not set to "Normal Operation") THEN A_DMC_9 RADC Monitor Status = TRUE ELSE IF the following condition is true for 320ms with a tolerance of +10ms ((A_DMC_9 RADC labels have been received at expected update rate with a tolerance of +10ms ) AND (A_DMC_9 RADC data have correct (odd) parity bit) AND (A_DMC_9 RADC have SSM data set to "Normal Operation") AND (A_DMC_9 RADC Airspeed < 35kts)) THEN A_DMC_9 RADC Monitor Status = FALSE ELSE No action (A_DMC_9 RADC Monitor Status remains unchanged) ENDIF ENDIF Where A_DMC_9 RADC ARINC Message is defined in MRJ_SCU_STC_SRS_1087 Justification for timing tolerance: ARINC label reception requires tolerance for label polling and processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1052 | During fault confirmation of the A_DMC_9 RADC Monitor the STC Program shall use the last valid data for Aircraft Speed Processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### B_DMC_9 RADC Monitor

| MRJ_SCU_STC_SRS_816 | The STC Program shall determine B_DMC_9 RADC Monitor Status according to the following logic: IF the following condition is true for 320ms with a tolerance of +10ms (( No valid B_DMC_9 RADC labels have been received) OR (B_DMC_9 RADC data have wrong (even) parity bit) OR (B_DMC_9 RADC have SSM data not set to "Normal Operation")) THEN B_DMC_9 RADC Monitor Status = TRUE ELSE IF the following condition is true for 320ms with a tolerance of +10ms ((B_DMC_9 RADC labels have been received at expected update rate with a tolerance of +10ms ) AND (B_DMC_9 RADC data have correct (odd) parity bit) AND (B_DMC_9 RADC have SSM data set to "Normal Operation") AND (B_DMC_9 RADC Airspeed < 35kts)) THEN B_DMC_9 RADC Monitor Status = FALSE ELSE No action (B_DMC_9 RADC Monitor Status remains unchanged) ENDIF ENDIF Where B_DMC_9 RADC ARINC Message is defined in MRJ_SCU_STC_SRS_144 Justification for timing tolerance: ARINC label reception requires tolerance for label polling and processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_947 | During fault confirmation of the B_DMC_9 RADC Monitor the STC Program shall use the last valid data for Aircraft Speed Processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### B_DMC_9 LADC Monitor

| MRJ_SCU_STC_SRS_1054 | The STC Program shall determine B_DMC_9 LADC Monitor Status according to the following logic: IF the following condition is true for 320ms with a tolerance of +10ms ( No valid B_DMC_9 LADC labels have been received) OR (B_DMC_9 LADC data have wrong (even) parity bit) OR (B_DMC_9 LADC have SSM data not set to "Normal Operation") THEN B_DMC_9 LADC Monitor Status = TRUE ELSE IF the following condition is true for 320ms with a tolerance of +10ms ((B_DMC_9 LADC labels have been received at expected update rate with a tolerance of +10ms ) AND (B_DMC_9 LADC data have correct (odd) parity bit) AND (B_DMC_9 LADC have SSM data set to "Normal Operation") AND (B_DMC_9 LADC Airspeed < 35kts)) THEN B_DMC_9 LADC Monitor Status = FALSE ELSE No action (B_DMC_9 LADC Monitor Status remains unchanged) ENDIF ENDIF Where B_DMC_9 LADC ARINC Message is defined in MRJ_SCU_STC_SRS_1088 Justification for timing tolerance: ARINC label reception requires tolerance for label polling and processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1055 | During fault confirmation of the B_DMC_9 LADC Monitor the STC Program shall use the last valid data for Aircraft Speed Processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### A_DMC_9 ISI Monitor

| MRJ_SCU_STC_SRS_818 | The STC Program shall determine A_DMC_9 ISI Monitor Status according to the following logic: IF the following condition is true for 320ms with a tolerance of +10ms (( No valid A_DMC_9 ISI labels have been received) OR (A_DMC_9 ISI data have wrong (even) parity bit) OR (A_DMC_9 ISI have SSM data not set to "Normal Operation")) THEN A_DMC_9 ISI Monitor Status = TRUE ELSE IF the following condition is true for 320ms with a tolerance of +10ms ((A_DMC_9 ISI labels have been received at expected update rate with a tolerance of +10ms ) AND (A_DMC_9 ISI data have correct (odd) parity bit) AND (A_DMC_9 ISI have SSM data set to "Normal Operation") AND (A_DMC_9 ISI Airspeed < 35kts)) THEN A_DMC_9 ISI Monitor Status = FALSE ELSE No action (A_DMC_9 ISI Monitor Status remains unchanged) ENDIF ENDIF Where A_DMC_9 ISI ARINC Message is defined in MRJ_SCU_STC_SRS_848 Justification for timing tolerance: ARINC label reception requires tolerance for label polling and processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_944 | During fault confirmation of the A_DMC_9 ISI Monitor the STC Program shall use the last valid data for Aircraft Speed Processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### B_DMC_9 ISI Monitor

| MRJ_SCU_STC_SRS_813 | The STC Program shall determine B_DMC_9 ISI Monitor Status according to the following logic: IF the following condition is true for 320ms with a tolerance of +10ms (( No valid B_DMC_9 ISI labels have been received) OR (B_DMC_9 ISI data have wrong (even) parity bit) OR (B_DMC_9 ISI have SSM data not set to "Normal Operation")) THEN B_DMC_9 ISI Monitor Status = TRUE ELSE IF the following condition is true for 320ms with a tolerance of +10ms ((B_DMC_9 ISI labels have been received at expected update rate with a tolerance of +10ms ) AND (B_DMC_9 ISI data have correct (odd) parity bit) AND (B_DMC_9 ISI have SSM data set to "Normal Operation") AND (B_DMC_9 ISI Airspeed < 35kts)) THEN B_DMC_9 ISI Monitor Status = FALSE ELSE No action (B_DMC_9 ISI Monitor Status remains unchanged) ENDIF ENDIF Where B_DMC_9 ISI ARINC Message is defined in MRJ_SCU_STC_SRS_849 Justification for timing tolerance: ARINC label reception requires tolerance for label polling and processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_945 | During fault confirmation of the B_DMC_9 ISI Monitor the STC Program shall use the last valid data for Aircraft Speed Processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Deleted

#### Deleted

#### Deleted

#### Deleted

#### A_DMC_9 LADC and A_DMC_9 RADC Reasonableness Check Monitor

| MRJ_SCU_STC_SRS_1067 | If both A_DMC_9 LADC Monitor Status and A_DMC_9 RADC Monitor Status are not in failure, then the STC Program shall set A_DMC_9 LADC and A_DMC_9 RADC Reasonableness Check Monitor Status to TRUE if the absolute difference between A_DMC_9 LADC Airspeed and A_DMC_9 RADC Airspeed differs by more than the AirspeedTolerance defined by the following logic below for at least 420ms; otherwise leave A_DMC_9 LADC and A_DMC_9 RADC Reasonableness Check Monitor Status unchanged from an initial value of FALSE. AirspeedTolerance: IF ((A_DMC_9 LADC Airspeed + A_DMC_9 RADC Airspeed) / 2 < 35kts) THEN AirspeedTolerance = 35 kts ELSE AirspeedTolerance = 10 kts END IF Where A_DMC_9 LADC Monitor Status is defined in MRJ_SCU_STC_SRS_811 A_DMC_9 RADC Monitor Status is defined in MRJ_SCU_STC_SRS_1051 A_DMC_9 LADC Airspeed is defined in MRJ_SCU_STC_SRS_135 and A_DMC_9 RADC Airspeed is defined in MRJ_SCU_STC_SRS_1087 Note: The monitor is not recoverable. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1068 | During fault confirmation of the A_DMC_9 LADC and A_DMC_9 RADC Reasonableness Check Monitor the STC Program shall use the last valid data for Aircraft Speed Processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### B_DMC_9 LADC and B_DMC_9 RADC Reasonableness Check Monitor

| MRJ_SCU_STC_SRS_1070 | If both B_DMC_9 LADC Monitor Status and B_DMC_9 RADC Monitor Status are not in failure, then the STC Program shall set B_DMC_9 LADC and B_DMC_9 RADC Reasonableness Check Monitor Status to TRUE if the absolute difference between B_DMC_9 LADC Airspeed and B_DMC_9 RADC Airspeed differs by more than the AirspeedTolerance defined by the following logic below for at least 420ms; otherwise leave B_DMC_9 LADC and B_DMC_9 RADC Reasonableness Check Monitor Status unchanged from an initial value of FALSE. AirspeedTolerance: IF ((B_DMC_9 LADC Airspeed + B_DMC_9 RADC Airspeed) / 2 < 35kts) THEN AirspeedTolerance = 35 kts ELSE AirspeedTolerance = 10 kts END IF Where B_DMC_9 LADC Monitor Status is defined in MRJ_SCU_STC_SRS_1054 B_DMC_9 RADC Monitor Status is defined in MRJ_SCU_STC_SRS_816 B_DMC_9 LADC Airspeed is defined in MRJ_SCU_STC_SRS_1088 and B_DMC_9 RADC Airspeed is defined in MRJ_SCU_STC_SRS_144 Note: The monitor is not recoverable. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1071 | During fault confirmation of the B_DMC_9 LADC and B_DMC_9 RADC Reasonableness Check Monitor the STC Program shall use the last valid data for Aircraft Speed Processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### B_DMC_9 LADC  and B_DMC_9 ISI Reasonableness Check Monitor

| MRJ_SCU_STC_SRS_1073 | If both B_DMC_9 LADC Monitor Status and B_DMC_9 ISI Monitor Status are not in failure, then the STC Program shall set B_DMC_9 LADC and B_DMC_9 ISI Reasonableness Check Monitor Status to TRUE if the absolute difference between B_DMC_9 LADC Airspeed and B_DMC_9 ISI Airspeed differs by more than the AirspeedTolerance defined by the following logic below for at least 420ms; otherwise leave B_DMC_9 LADC and B_DMC_9 ISI Reasonableness Check Monitor Status unchanged from an initial value of FALSE. AirspeedTolerance: IF ((B_DMC_9 LADC Airspeed + B_DMC_9 ISI Airspeed) / 2 < 35kts) THEN AirspeedTolerance = 35 kts ELSE AirspeedTolerance = 10 kts END IF Where B_DMC_9 LADC Monitor Status is defined in MRJ_SCU_STC_SRS_1054 B_DMC_9 ISI Monitor Status is defined in MRJ_SCU_STC_SRS_813 B_DMC_9 LADC Airspeed is defined in MRJ_SCU_STC_SRS_1088 and B_DMC_9 ISI Airspeed is defined in MRJ_SCU_STC_SRS_849 Note: The monitor is not recoverable. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1074 | During fault confirmation of the B_DMC_9 LADC and B_DMC_9 ISI Reasonableness Check Monitor the STC Program shall use the last valid data for Aircraft Speed Processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### B_DMC_9 RADC  and B_DMC_9 ISI Reasonableness Check Monitor

| MRJ_SCU_STC_SRS_1076 | If both B_DMC_9 RADC Monitor Status and B_DMC_9 ISI Monitor Status are not in failure, then the STC Program shall set B_DMC_9 RADC and B_DMC_9 ISI Reasonableness Check Monitor Status to TRUE if the absolute difference between B_DMC_9 RADC Airspeed and B_DMC_9 ISI Airspeed differs by more than the AirspeedTolerance defined by the following logic below for at least 420ms; otherwise leave B_DMC_9 RADC and B_DMC_9 ISI Reasonableness Check Monitor Status unchanged from an initial value of FALSE. AirspeedTolerance: IF ((B_DMC_9 RADC Airspeed + B_DMC_9 ISI Airspeed) / 2 < 35kts) THEN AirspeedTolerance = 35 kts ELSE AirspeedTolerance = 10 kts END IF Where B_DMC_9 RADC Monitor Status is defined in MRJ_SCU_STC_SRS_816 B_DMC_9 ISI Monitor Status is defined in MRJ_SCU_STC_SRS_813 B_DMC_9 RADC Airspeed is defined in MRJ_SCU_STC_SRS_144 and B_DMC_9 ISI Airspeed is defined in MRJ_SCU_STC_SRS_849 Note: The monitor is not recoverable. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1077 | During fault confirmation of the B_DMC_9 RADC and B_DMC_9 ISI Reasonableness Check Monitor the STC Program shall use the last valid data for Aircraft Speed Processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Deleted

#### Deleted

#### B_DMC_9 LADC and B_DMC_9 RADC  and B_DMC_9 ISI Reasonableness Check Monitor

| MRJ_SCU_STC_SRS_1085 | If all B_DMC_9 LADC Monitor Status and B_DMC_9 RADC Monitor Status and B_DMC_9 ISI Monitor Status are not in failure, then the STC Program shall set B_DMC_9 LADC and B_DMC_9 RADC and B_DMC_9 ISI Reasonableness Check Monitor Status to TRUE if (Condition_1 is TRUE OR Condition_2 is TRUE OR Condition_3 is TRUE) for at least 420ms; otherwise leave B_DMC_9 LADC and B_DMC_9 RADC and B_DMC_9 ISI Reasonableness Check Monitor Status unchanged from an initial value of FALSE. Condition_1 is : The absolute difference between the B_DMC_9 LADC Airspeed and MidAirspeed differs by more than the AirspeedTolerance defined by the following logic below AND the absolute difference between the B_DMC_9 RADC Airspeed and MidAirspeed differs by more than the AirspeedTolerance defined by the following logic below. Condition_2 is : The absolute difference between the B_DMC_9 LADC Airspeed and MidAirspeed differs by more than the AirspeedTolerance defined by the following logic below AND the absolute difference between the B_DMC_9 ISI Airspeed and MidAirspeed differs by more than the AirspeedTolerance defined by the following logic below. Condition_3 is : The absolute difference between the B_DMC_9 RADC Airspeed and MidAirspeed differs by more than the AirspeedTolerance defined by the following logic below AND the absolute difference between the B_DMC_9 ISI Airspeed and MidAirspeed differs by more than the AirspeedTolerance defined by the following logic below. Where MidAirspeed is the median of B_DMC_9 LADC Airspeed and B_DMC_9 RADC Airspeed and B_DMC_9 ISI Airspeed. AirspeedTolerance: IF MidAirspeed < 35kts THEN AirspeedTolerance = 35 kts ELSE AirspeedTolerance = 10 kts END IF Where B_DMC_9 LADC Monitor Status is defined in MRJ_SCU_STC_SRS_1054 B_DMC_9 RADC Monitor Status is defined in MRJ_SCU_STC_SRS_816 B_DMC_9 ISI Monitor Status is defined in MRJ_SCU_STC_SRS_813 B_DMC_9 LADC Airspeed is defined in MRJ_SCU_STC_SRS_1088 B_DMC_9 RADC Airspeed is defined in MRJ_SCU_STC_SRS_144 and B_DMC_9 ISI Airspeed is defined in MRJ_SCU_STC_SRS_849 Note: The monitor is not recoverable. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1086 | During fault confirmation of the B_DMC_9 LADC and B_DMC_9 RADC and B_DMC_9 ISI Reasonableness Check Monitor the STC Program shall use the last valid data for Aircraft Speed Processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### ACE1B_Label 136 Monitor

| MRJ_SCU_STC_SRS_828 | The STC Program shall determine ACE1B Label 136 Monitor Status according to the following logic: IF the following condition is true for 200ms with a tolerance of +10ms (( No valid ACE1B Label 136 have been received) OR ( ACE1B Label 136 data have wrong (even) parity bit) OR ( ACE1B Label 136 have SSM data not set to "Normal Operation")) THEN ACE1B Label 136 Monitor Status = TRUE ELSE IF the following condition is true for 200ms with a tolerance of +10ms ((ACE1B Label 136 have been received at expected updated rate with a tolerance of +10ms) AND (ACE1B Label 136 data have correct (odd) parity bit) AND (ACE1B Label 136 have SSM data set to "Normal Operation")) THEN ACE1B Label 136 Monitor Status = FALSE ELSE do nothing (monitor status remains unchanged) ENDIF Where ACE1B Label 136 ARINC Message is defined in MRJ_SCU_STC_SRS_844 Justification for timing tolerance: ARINC label reception requires tolerance for label polling and processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_951 | During fault confirmation of the ACE1B Label 136 Monitor the STC Program shall use the last valid data for Rudder Pedal Operation Processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### ACE2B_Label 136 Monitor

| MRJ_SCU_STC_SRS_830 | The STC Program shall determine ACE2B Label 136 Monitor Status according to the following logic: IF the following condition is true for 200ms with a time tolerance of +10ms (( No valid ACE2B Label 136 have been received) OR ( ACE2B Label 136 data have wrong (even) parity bit) OR ( ACE2B Label 136 have SSM data not set to "Normal Operation")) THEN ACE2BLabel 136 Monitor Status = TRUE ELSE IF the following condition is true for 200ms with a tolerance of +10ms ((ACE2B Label 136 have been received at expected updated rate with a tolerance of +10ms) AND (ACE2B Label 136 data have correct (odd) parity bit) AND (ACE2B Label 136 have SSM data set to "Normal Operation")) THEN ACE2B Label 136 Monitor Status = FALSE ELSE do nothing (monitor status remains unchanged) ENDIF Where ACE2B Label 136 ARINC Message is defined in MRJ_SCU_STC_SRS_845 Justification for timing tolerance: ARINC label reception requires tolerance for label polling and processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_952 | During fault confirmation of the ACE2B Label 136 Monitor the STC Program shall use the last valid data for Rudder Pedal Operation Processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### ACE1B Reasonableness Check Monitor

| MRJ_SCU_STC_SRS_832 | The STC Program shall set ACE1B Reasonableness Check Monitor Status according to the following logic: IF (ACE1B Label 136 Monitor Status) THEN ACE1B Reasonableness Check Monitor Status = FALSE ELSEIF (Absolute (Rudder Position 1 - ACE1B label 136 Rudder Position data)) > Threshold for the last 200ms with a time tolerance of +10ms THEN ACE1B Reasonableness Check Monitor Status = TRUE ELSEIF (Absolute (Rudder Position 1 - ACE1B label 136 Rudder Position data)) <= Threshold for the last 200ms with a time tolerance of +10ms THEN ACE1B Reasonableness Check Monitor Status = FALSE ELSE (Do nothing) ACE1B Reasonableness Check Monitor Status stays at previous value, initialised to FALSE ENDIF where Threshold = 7.62mm (= 10% of 76.2mm) Rudder Position 1 is defined in MRJ_SCU_STC_SRS_854 ACE1B Label 136 Monitor Status is defined in MRJ_SCU_STC_SRS_828 and ACE1B Label 136 ARINC Message is defined in MRJ_SCU_STC_SRS_844 Justification for timing tolerance: ARINC label reception requires tolerance for label polling and processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_953 | During fault confirmation of the ACE1B Reasonableness Check Monitor the STC Program shall use the last valid data for Rudder Pedal Operation Processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### ACE2B Reasonableness Check Monitor

| MRJ_SCU_STC_SRS_836 | The STC Program shall set ACE2B Reasonableness Check Monitor Status according to the following logic: IF (ACE2B Label 136 Monitor Status) THEN ACE2B Reasonableness Check Monitor Status = FALSE ELSEIF (Absolute (Rudder Position 2 - ACE2B label 136 Rudder Position data)) > Threshold for the last 200ms with a time tolerance of +10ms THEN ACE2B Reasonableness Check Monitor Status = TRUE ELSEIF (Absolute (Rudder Position 2 - ACE2B label 136 Rudder Position data)) <= Threshold for the last 200ms with a time tolerance of +10ms THEN ACE2B Reasonableness Check Monitor Status = FALSE ELSE (Do nothing) ACE2B Reasonableness Check Monitor Status stays at previous value, initialised to FALSE ENDIF where Threshold = 7.62mm (= 10% of 76.2mm) Rudder Position 2 is defined in MRJ_SCU_STC_SRS_854 ACE2B Label 136 Monitor Status is defined in MRJ_SCU_STC_SRS_830 and ACE2B Label 136 ARINC Message is defined in MRJ_SCU_STC_SRS_845 Justification for timing tolerance: ARINC label reception requires tolerance for label polling and processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_954 | During fault confirmation of the ACE2B Reasonableness Check Monitor the STC Program shall use the last valid data for Rudder Pedal Operation Processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### ACE1B Label 136 and ACE2B Label 136 Reasonableness Check Monitor

| MRJ_SCU_STC_SRS_838 | The STC Program shall set ACE1B Label 136 and ACE2B Label 136 Reasonableness Check Monitor Status according to the following logic: IF (ACE1B Label 136 Monitor Status OR ACE2B Label 136 Monitor Status) THEN ACE1B Label 136 and ACE2B Label 136 Reasonableness Check Monitor Status = FALSE ELSEIF (Absolute (ACE1B label 136 Rudder Position data - ACE2B label 136 Rudder Position data)) > Threshold for the last 200ms with a time tolerance of +10ms THEN ACE1B Label 136 and ACE2B Label 136 Reasonableness Check Monitor Status = TRUE ELSEIF (Absolute (ACE1B label 136 Rudder Position data - ACE2B label 136 Rudder Position data)) <= Threshold for the last 200ms with a time tolerance of +10ms THEN ACE1B Label 136 and ACE2B Label 136 Reasonableness Check Monitor Status = FALSE ELSE (Do nothing) ACE1B Label 136 and ACE2B Label 136 Reasonableness Check Monitor Status stays at previous value, initialised to FALSE ENDIF where Threshold = 7.62mm (= 10% of 76.2mm) ACE1B Label 136 Monitor Status is defined in MRJ_SCU_STC_SRS_828 ACE2B Label 136 Monitor Status is defined in MRJ_SCU_STC_SRS_830 ACE1B Label 136 ARINC Message is defined in MRJ_SCU_STC_SRS_844 and ACE2B Label 136 ARINC Message is defined in MRJ_SCU_STC_SRS_845 Justification for timing tolerance: ARINC label reception requires tolerance for label polling and processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_955 | During fault confirmation of the ACE1B Label 136 and ACE2B Label 136 Reasonableness Check Monitor the STC Program shall use the last valid data for Rudder Pedal Operation Processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### EHSV1 Monitor

| MRJ_SCU_STC_SRS_606 | The STC Program shall determine EHSV1 Monitor Status with a confirmation time of 200ms according to the following logic: IF (EHSV1 Monitor Status = FALSE) THEN EHSV1 Monitor Status = Debounced_EHSV1_Fail ELSE do nothing (monitor status remains TRUE, making it Non-recoverable) ENDIF Where Debounced_EHSV1_Fail is defined in MRJ_SCU_STC_SRS_164 from Raw_EHSV1_Fail defined in MRJ_SCU_STC_SRS_124 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### EHSV2 Monitor

| MRJ_SCU_STC_SRS_607 | The STC Program shall determine EHSV2 Monitor Status with a confirmation time of 200ms according to the following logic: IF (EHSV2 Monitor Status = FALSE) THEN EHSV2 Monitor Status = Debounced_EHSV2_Fail ELSE do nothing (monitor status remains TRUE, making it Non-recoverable) ENDIF Where Debounced_EHSV2_Fail is defined in MRJ_SCU_STC_SRS_164 from Raw_EHSV2_Fail defined in MRJ_SCU_STC_SRS_124 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### EHSV1 and EHSV2 Monitor

| MRJ_SCU_STC_SRS_605 | The STC Program shall determine EHSV1 and EHSV2 Monitor Status according to the following logic: IF (EHSV1 and EHSV2 Monitor Status = FALSE) THEN EHSV1 and EHSV2 Monitor Status = (EHSV1 Monitor Status) AND (EHSV2 Monitor Status) ELSE do nothing (monitor status remains TRUE, making it Non-recoverable) ENDIF Where EHSV1 Monitor Status is defined in MRJ_SCU_STC_SRS_606 and EHSV2 Monitor Status is defined in MRJ_SCU_STC_SRS_607 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Deleted

#### Deleted

#### Deleted

#### Deleted

#### Cross Compare Check Monitor

| MRJ_SCU_STC_SRS_629 | The STC Program shall determine Cross Compare Check Monitor Status with a confirmation time of 200ms according to the following logic: IF Loss Of Interlane Communications Monitor Status as specified in MRJ_SCU_STC_SRS_915 is FALSE AND the received mode in Inter lane data as specified in MRJ_SCU_STC_SRS_648 is not Standby THEN IF Cross Compare Check Monitor Status = FALSE THEN Cross Compare Check Monitor Status = (absolute(Limited_Steering_Demand - Target_Steering_Angle for Cross Compare) > Angle_Tolerance) OR (absolute(SACT_Position - Actual_Steering_Angle for Cross Compare) > Angle_Tolerance) ELSE do nothing (Monitor remains TRUE) END IF ELSE do nothing (monitor remains unchanged, initialised to FALSE) END IF Where Angle_Tolerance is 5 deg and Limited_Steering_Demand is defined in MRJ_SCU_STC_SRS_176 and Target_Steering_Angle for Cross Compare is received in ML-CL Message defined in MRJ_SCU_STC_SRS_641 and SACT_Position is defined in MRJ_SCU_STC_SRS_1240 and Actual_Steering_Angle for Cross Compare is received in ML-CL Message defined in MRJ_SCU_STC_SRS_641 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Target Input Monitor

| MRJ_SCU_STC_SRS_608 | The STC Program shall determine Target Input Monitor Status according to the following logic: IF Target Input Monitor Status == FALSE THEN IF ((ACE1B label 136 and ACE2B Label 136 Reasonableness Check Monitor Status) OR Rudder Pedal Op Not Valid) AND ((Pilot SHDL Operation 1 and 2 Health Monitor Status) OR (Pilot SHDL Operation 1 and 2 Reasonableness Check Monitor Status) ) AND ((Co_Pilot SHDL Operation 1 and 2 Health Monitor Status) OR (Co_Pilot SHDL Operation 1 and 2 Reasonableness Check Monitor Status) ) THEN Target Input Monitor Status = TRUE ELSE Do nothing (Monitor remains FALSE) ENDIF ELSE Do nothing (Monitor remains TRUE) ENDIF Where ACE1B label 136 and ACE2B Label 136 Reasonableness Check Monitor Status is defined in MRJ_SCU_STC_SRS_838 and Pilot SHDL Operation 1 and 2 Health Monitor Status is defined in MRJ_SCU_STC_SRS_595 and Pilot SHDL Operation 1 and 2 Reasonableness Check Monitor Status is defined in MRJ_SCU_STC_SRS_599 and Co_Pilot SHDL Operation 1 and 2 Health Monitor Status is defined in MRJ_SCU_STC_SRS_598 and Co_Pilot SHDL Operation 1 and 2 Reasonableness Check Monitor Status is defined in MRJ_SCU_STC_SRS_600 and Rudder Pedal Op Not Valid is defined in MRJ_SCU_STC_SRS_154 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Deleted

#### Loss Of Interlane Communications Monitor

| MRJ_SCU_STC_SRS_915 | The STC Program shall determine Loss Of Interlane Communications Monitor Status when no valid message has been received from the Monitor Lane for 200ms, with a tolerance of +5ms and recover when a valid message is received. The validation of Interlane Messages is defined in Section 6.8 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Internal Power Supply Monitor

| MRJ_SCU_STC_SRS_1168 | The STC Program shall determine Internal Power Supply Monitor Status with a confirmation time of 100ms and a recovery time of 0ms according to the following logic: IF Debounced_ANALOG_SUPPLIES_OK = FALSE THEN Internal Power Supply Monitor Status = TRUE ELSE Internal Power Supply Monitor Status = FALSE END IF Where Debounced_ANALOG_SUPPLIES_OK is defined in MRJ_SCU_STC_SRS_124 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : |
|  | Verification Method : |

#### A_DMC_9 LADC & A_DMC_9 RADC & A_DMC_9 ISI Airspeed Reasonableness Check Monitor

| MRJ_SCU_STC_SRS_1178 | If all A_DMC_9 LADC Monitor Status and A_DMC_9 RADC Monitor Status and A_DMC_9 ISI Monitor Status are not in failure, then the STC Program shall set A_DMC_9 LADC and A_DMC_9 RADC and A_DMC_9 ISI Reasonableness Check Monitor Status to TRUE if (Condition_1 is TRUE OR Condition_2 is TRUE OR Condition_3 is TRUE) for at least 420ms; otherwise leave A_DMC_9 LADC and A_DMC_9 RADC and A_DMC_9 ISI Reasonableness Check Monitor Status unchanged from an initial value of FALSE. Condition_1 is : The absolute difference between the A_DMC_9 LADC Airspeed and MidAirspeed differs by more than the AirspeedTolerance defined by the following logic below AND the absolute difference between the A_DMC_9 RADC Airspeed and MidAirspeed differs by more than the AirspeedTolerance defined by the following logic below. Condition_2 is : The absolute difference between the A_DMC_9 LADC Airspeed and MidAirspeed differs by more than the AirspeedTolerance defined by the following logic below AND the absolute difference between the A_DMC_9 ISI Airspeed and MidAirspeed differs by more than the AirspeedTolerance defined by the following logic below. Condition_3 is : The absolute difference between the A_DMC_9 RADC Airspeed and MidAirspeed differs by more than the AirspeedTolerance defined by the following logic below AND the absolute difference between the A_DMC_9 ISI Airspeed and MidAirspeed differs by more than the AirspeedTolerance defined by the following logic below. Where MidAirspeed is the median of A_DMC_9 LADC Airspeed and A_DMC_9 RADC Airspeed and A_DMC_9 ISI Airspeed. AirspeedTolerance: IF MidAirspeed < 35kts THEN AirspeedTolerance = 35 kts ELSE AirspeedTolerance = 10 kts END IF Where A_DMC_9 LADC Monitor Status is defined in MRJ_SCU_STC_SRS_811 A_DMC_9 RADC Monitor Status is defined in MRJ_SCU_STC_SRS_1051 A_DMC_9 ISI Monitor Status is defined in MRJ_SCU_STC_SRS_818 A_DMC_9 LADC Airspeed is defined in MRJ_SCU_STC_SRS_135 A_DMC_9 RADC Airspeed is defined in MRJ_SCU_STC_SRS_1087 and A_DMC_9 ISI Airspeed is defined in MRJ_SCU_STC_SRS_848 Note: The monitor is not recoverable. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1203 | During fault confirmation of the A_DMC_9 LADC & A_DMC_9 RADC & A_DMC_9 ISI Airspeed Reasonableness Check Monitor the STC Program shall use the last valid data for Aircraft Speed Processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### A_DMC_9 LADC and A_DMC_9 ISI Reasonableness Check Monitor

| MRJ_SCU_STC_SRS_1179 | If both A_DMC_9 LADC Monitor Status and A_DMC_9 ISI Monitor Status are not in failure, then the STC Program shall set A_DMC_9 LADC and A_DMC_9 ISI Reasonableness Check Monitor Status to TRUE if the absolute difference between A_DMC_9 LADC Airspeed and A_DMC_9 ISI Airspeed differs by more than the AirspeedTolerance defined by the following logic below for at least 420ms; otherwise leave A_DMC_9 LADC and A_DMC_9 ISI Reasonableness Check Monitor Status unchanged from an initial value of FALSE. AirspeedTolerance: IF ((A_DMC_9 LADC Airspeed + A_DMC_9 ISI Airspeed) / 2 < 35kts) THEN AirspeedTolerance = 35 kts ELSE AirspeedTolerance = 10 kts END IF Where A_DMC_9 LADC Monitor Status is defined in MRJ_SCU_STC_SRS_811 A_DMC_9 ISI Monitor Status is defined in MRJ_SCU_STC_SRS_818 A_DMC_9 LADC Airspeed is defined in MRJ_SCU_STC_SRS_135 and A_DMC_9 ISI Airspeed is defined in MRJ_SCU_STC_SRS_848 Note: The monitor is not recoverable. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1204 | During fault confirmation of the A_DMC_9 LADC and A_DMC_9 ISI Reasonableness Check Monitor the STC Program shall use the last valid data for Aircraft Speed Processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### A_DMC_9 RADC and A_DMC_9 ISI Reasonableness Check Monitor

| MRJ_SCU_STC_SRS_1180 | If both A_DMC_9 RADC Monitor Status and A_DMC_9 ISI Monitor Status are not in failure, then the STC Program shall set A_DMC_9 RADC and A_DMC_9 ISI Reasonableness Check Monitor Status to TRUE if the absolute difference between A_DMC_9 RADC Airspeed and A_DMC_9 ISI Airspeed differs by more than the AirspeedTolerance defined by the following logic below for at least 420ms; otherwise leave A_DMC_9 RADC and A_DMC_9 ISI Reasonableness Check Monitor Status unchanged from an initial value of FALSE. AirspeedTolerance: IF ((A_DMC_9 RADC Airspeed + A_DMC_9 ISI Airspeed) / 2 < 35kts) THEN AirspeedTolerance = 35 kts ELSE AirspeedTolerance = 10 kts END IF Where A_DMC_9 RADC Monitor Status is defined in MRJ_SCU_STC_SRS_1051 A_DMC_9 ISI Monitor Status is defined in MRJ_SCU_STC_SRS_818 A_DMC_9 RADC Airspeed is defined in MRJ_SCU_STC_SRS_1087 and A_DMC_9 ISI Airspeed is defined in MRJ_SCU_STC_SRS_848 Note: The monitor is not recoverable. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1205 | During fault confirmation of the A_DMC_9 RADC and A_DMC_9 ISI Reasonableness Check Monitor the STC Program shall use the last valid data for Aircraft Speed Processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### AirspeedA and AirspeedB Reasonableness Check Monitor

| MRJ_SCU_STC_SRS_1181 | The STC Program shall set AirspeedA and AirspeedB Reasonableness Check Monitor Status to TRUE if the absolute difference between AirspeedA and AirspeedB differs by more than the AirspeedTolerance defined by the following logic below for at least 420ms; otherwise leave AirspeedA and AirspeedB Reasonableness Check Monitor Status unchanged from an initial value of FALSE. AirspeedTolerance: IF ((AirspeedA + AirspeedB) / 2 < 35kts) THEN AirspeedTolerance = 35 kts ELSE AirspeedTolerance = 10 kts END IF Where AirspeedA is defined in MRJ_SCU_STC_SRS_1089 and AirspeedB is defined in MRJ_SCU_STC_SRS_1090 Note: The monitor is not recoverable. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1206 | During fault confirmation of the AirspeedA and AirspeedB Reasonableness Check Monitor the STC Program shall use the last valid data for Aircraft Speed Processing. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### SOV Status Monitor

| MRJ_SCU_STC_SRS_1197 | The STC Program shall determine SOV Status Monitor status when the following logic is true: IF (DMCA Label 174 SDI:10 data is available) AND (DMCB Label 174 SDI:10 data is available) AND (DMCA Label 174 SDI:10 shows over 2450psi) AND (DMCB Label 174 SDI:10 shows over 2450psi) THEN A monitor failure is determined if the cross comparison disagreement as indicated in the table below is present and continues for 200ms consecutive: ELSE Do nothing (monitor is non-recoverable) END IF Table 6.3-5 SOV Status Truth Table Where SOV_OUT is defined in MRJ_SCU_STC_SRS_926, Debounced_Hydraulic_Pressure is defined in MRJ_SCU_STC_SRS_164 from Raw_Hydraulic_Pressure in MRJ_SCU_STC_SRS_124, DMCA Label 174 is defined in MRJ_SCU_STC_SRS_1201, DMCB Label 174 is defined in MRJ_SCU_STC_SRS_1202, Loss of Interlane Communication is defined in MRJ_SRS_STC_915, and SOV_OUT_ML is defined in MRJ_SCU_STC_SRS_648 as Interlane Message Word 4 Bit 2, 0 = SOV ON, 1 = SOV OFF. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Over Angle Monitor

| MRJ_SCU_STC_SRS_1212 | The STC Program shall determine Over Angle Monitor Status according to the following logic: IF Over Angle Monitor Status = FALSE THEN IF (Airspeed >= 30kts and SOV_OUT = energised) THEN With a confirmation time of 200ms: Over Angle Monitor Status = Absolute (SACT_Position) > Over Angle Limit ELSE Over Angle Monitor Status = FALSE END IF ELSE do nothing (Monitor remains TRUE) END IF Where the Over Angle Limit = Steering_Demand_Final_Limit + 2 Where Airspeed is defined in MRJ_SCU_STC_SRS_146 and SOV_OUT is defined in MRJ_SCU_STC_SRS_926 and SACT_Position is defined in MRJ_SCU_STC_SRS_1240 and Steering_Demand_Final_Limit is defined in MRJ_SCU_STC_SRS_175. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Steering Angle Monitor

| MRJ_SCU_STC_SRS_1213 | The STC Program shall perform the Steering Angle Monitor Check when the Airspeed is greater or equal to 30kts and the SOV_OUT signal is Energised. Where SOV_OUT is defined in MRJ_SCU_STC_SRS_926 Airspeed is defined in MRJ_SCU_STC_SRS_146 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1214 | The STC Program shall determine the Steering Angle Monitor Status with a confirmation time of 200ms according to the following logic: IF ((A AND (B AND C)) OR ((B AND C) AND (D XOR E)) OR (C AND F AND G)) AND (H OR I) THEN Steering Angle Monitor Status = TRUE ELSE do nothing (Monitor remains true) END IF Where A = ¦differential(Target Steering Angle)¦ - ¦differential(Actual Steering Angle)¦ < 0 deg/ sec * dt-interval sec B = differential(¦Target Steering Angle - Actual Steering Angle¦) > 0 deg/ sec * dt-interval sec C = ¦(Target Steering Angle - Actual Steering Angle)¦ > 0.5 deg D = differential(Target Steering Angle) >= 0.5 deg/sec * dt-interval sec E = differential(Actual Steering Angle) > 0.5 deg/sec * dt-interval sec F = ¦differential(Target Steering Angle)¦ > 0.5 deg/sec * dt-interval sec G = ¦differential(Actual Steering Angle)¦ <= 0.5 deg/sec * dt-interval sec H = ¦ Actual Steering Angle¦ <= 68 deg I = ¦ Target Steering Angle¦ <= 68 deg differential(Actual Steering Angle) = Actual Steering Angle (most recent) - Actual Steering Angle (most recent - N) differential(Target Steering Angle) = Target Steering Angle (most recent) - Target Steering Angle (most recent - N) N = 20 ie. time of (N * Major Cycle) = 102.4ms dt-interval = (5.12 * 1e-3 * N) sec Target Steering Angle is defined in MRJ_SCU_STC_SRS_176 Actual Steering Angle is defined in MRJ_SCU_STC_SRS_1035 Major Cycle is defined in Table 6.2-1 Note: This monitor will only be performed after sufficient (20) ASA and TSA samples are buffered. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### LVDT Double Loss Monitor

| MRJ_SCU_STC_SRS_1245 | The STC Program shall determine LVDT Double Loss Monitor status from an initial status of FALSE using the following logic: IF (LVDT Double Loss Monitor Status = FALSE) THEN LVDT Double Loss Monitor Status = TRUE if 2 or more of the following monitor are TRUE: LH LVDT1 Health Monitor Status defined in MRJ_SCU_STC_SRS_567 LH LVDT2 Health Monitor Status defined in MRJ_SCU_STC_SRS_568 RH LVDT1 Health Monitor Status defined in MRJ_SCU_STC_SRS_67 RH LVDT2 Health Monitor Status defined in MRJ_SCU_STC_SRS_566 LH LVDT1 and LH LVDT2 Reasonableness Check Monitor Status defined in MRJ_SCU_STC_SRS_591 RH LVDT1 and RH LVDT2 Reasonableness Check Monitor Status defined in MRJ_SCU_STC_SRS_572 ELSE do nothing (monitor status remains TRUE, making it Non-recoverable) ENDIF |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### ASA Jump Monitor

| MRJ_SCU_STC_SRS_1247 | The STC Program shall determine ASA Jump Monitor status from an initial status of FALSE using the following logic: IF (ASA Jump Monitor Status = FALSE) THEN ASA Jump Monitor Status = ¦differential(Actual Steering Angle)¦ > ((50 deg/sec * dt-interval sec) as computed more than 3 times in the recent 90ms) ELSE do nothing (monitor status remains TRUE, making it Non-recoverable) ENDIF With differential(Actual Steering Angle) = Actual Steering Angle (most recent) - Actual Steering Angle (most recent - N) N = 20 ie. time of (N * Major Cycle) = 102.4ms dt-interval = (5.12 * 1e-3 * N) sec Actual Steering Angle is defined in MRJ_SCU_SMB_SRS_1195 Major Cycle is defined in Table 6.2-1 Note: This monitor will only be performed after sufficient (20) ASA samples are buffered. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

### Rigging record and data Check

| MRJ_SCU_STC_SRS_1043 | In order to mitigate against Single Event Upsets within the rigging offset variables, the STC Program shall perform the following check: IF NVM read Failure is FALSE AND RiggingNotPerformed is FALSE AND AircraftSerNumNoSync is FALSE THEN compare the NVM sector 0 data read into RAM with NVM sector 1 data read into RAM IF Comparison is successful THEN The STC Program shall compare the NVM sector 0 data read into RAM with the Ratiometric_Position_Offset[channel] IF comparison is PASSED THEN do nothing (data is not corrupted) ELSE disable interrupts and expire the Watchdog ENDIF ELSE disable interrupts and expire the Watchdog ENDIF ELSE Store 0 into Ratiometric_Position_Offset[channel] /* refresh values */ ENDIF Where RiggingNotPerformed is defined in MRJ_SCU_STC_SRS_1041 AircraftSerNumNoSync is defined in MRJ_SCU_STC_SRS_1044 NVM sector 0 data read is defined in MRJ_SCU_STC_SRS_702 NVM sector 1 data read is defined in MRJ_SCU_STC_SRS_709 NVM Read Failure is defined in MRJ_SCU_STC_SRS_711 channel is in {Pilot_RVDT1, Pilot_RVDT2, Co-Pilot_RVDT1, Co-Pilot_RVDT2, RH_LVDT1, RH_LVDT2, LH_LVDT1, LH_LVDT2} Justification: For SEU mitigation RAM copies of variables and code will be compared by the SCU software with their flash or NVM counterparts if not refreshed during runtime operation. |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1044 | The STC Program shall perform the following A/C Serial Number check: IF NVM read Failure is FALSE AND RiggingNotPerformed is FALSE AND AircraftSerNumNoSync is FALSE AND Valid Label 226 received THEN The STC Program shall compare the Aircraft serial number read from NVM sector 0 with the Aircraft serial number received in Arinc label 226. IF the comparison is FAILED THEN Store 0 into Ratiometric_Position_Offset[channel] Set AircraftSerNumNoSync to TRUE Where channel is in {Pilot_RVDT1, Pilot_RVDT2, Co-Pilot_RVDT1, Co-Pilot_RVDT2, RH_LVDT1, RH_LVDT2, LH_LVDT1, LH_LVDT2} ELSE Do nothing (AircraftSerNumNoSync unchanged) ENDIF ENDIF Where RiggingNotPerformed is defined in MRJ_SCU_STC_SRS_1041 NVM sector 0 data read is defined in MRJ_SCU_STC_SRS_702 Aircraft serial number is defined in MRJ_SCU_STC_SRS_1048 and MRJ_SCU_STC_SRS_1049 Note: Valid Label 226 means Label 226 received from any ARINC bus with any expected SDI as specified in MRJ_SCU_STC_SRS_1048 and MRJ_SCU_STC_SRS_1049. Initial value of AircraftSerNumNoSync is FALSE. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

## Single-Shot Built-in-Test

### Co Pilot Steering Handle Fitted

| MRJ_SCU_STC_SRS_858 | The Co_Pilot Steering Handle is optional. At startup the STC Program shall set Co_Pilot SHDL Fitted = FALSE until a value has been determined by the logic defined in MRJ_SCU_STC_SRS_79. Justification: Co-Pilot SHDL needs to be confirmed to be present by P-BIT, so default setting to FALSE is applicable. |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_79 | During the power-up sequence the STC Program shall run Co_Pilot SHDL Operation 1 Health Monitor and Co_Pilot SHDL Operation 2 Health Monitor for a period of at least 200ms within a 300ms window and then set a value for Co_Pilot_Fitted using the following logic at the end of this period: IF (Co_Pilot SHDL Operation 1 Health Monitor Status) AND (Co_Pilot SHDL Operation 2 Health Monitor Status) AND (Debounced_Co_Pilot_Engage = Debounced_Co_Pilot_Disengage) THEN Co_Pilot SHDL Fitted = FALSE Co_Pilot SHDL Operation 1 Health Monitor Status = FALSE Co_Pilot SHDL Operation 2 Health Monitor Status = FALSE ELSE Co_Pilot SHDL Fitted = TRUE ENDIF Where Co_Pilot SHDL Operation 1 Health Monitor Status is defined in MRJ_SCU_STC_SRS_596 Co_Pilot SHDL Operation 2 Health Monitor Status is defined in MRJ_SCU_STC_SRS_597 Debounced_Co_Pilot_Engage and Debounced_Co_Pilot_Disengage are the values defined in MRJ_SCU_STC_SRS_164 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

## Input Processing

### Analogue Input Processing via ADC Peripheral

#### LVDT/RVDT Input Processing

##### HW Level Processing

| MRJ_SCU_STC_SRS_59 | The STC Program shall sample the following LVDT/RVDT signals at a rate of 100 kHz using the internal ADC interface in accordance with the HW/SW Interface Specification Table 6.5-1 LVDT/RVDT Sampling Signals |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_62 | During every 5.12 ms (nominal) major cycle, the STC Program shall capture 256 samples of data from A/D channels 1 to 16 with the VSW (Va/Vb Switch) discrete output set to '1' followed by capturing 256 samples of data from A/D channels 1 to 16 with the VSW (Va/Vb Switch) discrete output set to '0'. Table 6.5-2 Voltage_Selection GPIO |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_957 | The switch over of VSW discrete output shall occur after the last and before or during the first sample in a block of captured data. Note: the number of samples for a block of captured data is defined in MRJ_SCU_STC_SRS_60. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_60 | The STC Program shall perform full DFT on data samples for each of the signals listed in MRJ_SCU_STC_SRS_59 as follows: DFT_RE = SUM ((ADC_SAMPLE[1..256] * SIN_TABLE[1][1..256]) / 2) DFT_IM = SUM ((ADC_SAMPLE[1..256] * COS_TABLE[1][1..256]) / 2 ) DFT_RE = DFT_RE / 2^16 DFT_IM = DFT_IM / 2^16 where SIN_TABLE and COS_TABLE are given in section 6.12.5 using data formats: ADC_SAMPLE : Q0.11 SIN_TABLE : Q0.15 COS_TABLE : Q0.15 DFT_RE : (Q0.11*Q0.15 >> 1) *256 ) >>16 = Q8.9 DFT_IM : (Q0.11*Q0.15 >> 1) *256 ) >>16 = Q8.9 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_958 | The STC Program shall calculate the sum of squares terms for each of the signals listed in MRJ_SCU_STC_SRS_59 as follows: SUM_OF_SQUARES = (DFT_RE ^ 2) + (DFT_IM ^2) using data formats: DFT_RE : Q6.9 DFT_IM : Q6.9 SUM_OF_SQUARES = (Q6.9*Q6.9) + (Q6.9* Q6.9) = Q13.18 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_736 | The STC Program shall calculate the Va and Vb magnitudes for each of the signals listed in MRJ_SCU_STC_SRS_59 every major cycle: Va_MAG = sqrt ( SUM_OF_SQUARES ) (for data captured with VSW = 1) Vb_MAG = sqrt ( SUM_OF_SQUARES) (for data captured with VSW = 0) using data formats: SUM_OF_SQUARES : Q13.18 Va_MAG: Q6.9 (16 bit result with the MSB always set to 0) Vb_MAG: Q6.9 (16 bit result with the MSB always set to 0) |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1194 | Every major cycle the STC Program shall apply a 1st order FIR filter on each of the calculated Va and Vb magnitudes with the coefficients b0 = b1 = 0.5: Va_MAG = ( Va_MAG[n] + Va_MAG[n-1] ) * 0.5; Vb_MAG = ( Vb_MAG[n] + Vb_MAG[n-1] ) * 0.5; where [n] and [n-1] denote the current and previous (major cycle) values of the unfiltered Va_MAG or Vb_MAG respectively and using data formats: Va_MAG: Q6.9 (16 bit result with the MSB always set to 0) Vb_MAG: Q6.9 (16 bit result with the MSB always set to 0) |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_66 | Every major cycle theSTC Program shall calculate the VA+VB sum voltage for each of the RVDT/LVDT channels listed in MRJ_SCU_STC_SRS_59 as follows: SUM_V : Q7.8 = (( Va_MAG + Vb_MAG ) : Q6.9) shift right by one to convert to Q7.8 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_959 | Every major cycle theSTC Program shall calculate the VA-VB difference voltage for each of the RVDT/LVDT channels listed in MRJ_SCU_STC_SRS_59 as follows: DIFF_V : Q6.9 = (( Va_MAG : Q6.9 - Vb_MAG : Q6.9) |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_65 | Every major cycle, the STC Program shall calculate the ratiometric position information for each of the RVDT/LVDT channels listed in MRJ_SCU_STC_SRS_59 using the following algorithm: if DIFF_V term is determined to be zero Ratio = 0, otherwise set the Ratio to: RATIO =((DIFF_V : Q6.25) / (SUM_V :Q7.8) ) : Q-2.17 shift right by 2 to convert to Q0.15 Ratiometric_Position = RATIO - Ratiometric_Position_Offset[channel] channel in {Pilot_RVDT1, Pilot_RVDT2, Co-Pilot_RVDT1, Co-Pilot_RVDT2, RH_LVDT1, RH_LVDT2, LH_LVDT1, LH_LVDT2} Where Ratiometric_Position_Offset[channel] is defined in MRJ_SCU_STC_SRS_1041 and MRJ_SCU_STC_SRS_1044 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_112 | The STC Program shall calculate the ratiometric position information for each of the RVDT/LVDT channels listed in MRJ_SCU_STC_SRS_59 with a numerical accuracy of better than ± 1% of full scale over the full temperature range of the SCU, excluding variations in the RVDT/ LVDT. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

##### SHDL Logical Level Processing

| MRJ_SCU_STC_SRS_852 | The STC Program shall determine Pilot_SHDL_Operation_1 according to the following equation: Pilot_SHDL_Operation_1 = Ratiometric_Position[Pilot_RVDT1] / 0.009616071 Where Ratiometric_Position[Pilot_RVDT1] is defined in MRJ_SCU_STC_SRS_65 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_84 | The STC Program shall determine Pilot_SHDL_Operation_2 according to the following equation: Pilot_SHDL_Operation_2 = Ratiometric_Position[Pilot_RVDT2] / 0.009616071 Where Ratiometric_Position[Pilot_RVDT2] is defined in MRJ_SCU_STC_SRS_65 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_68 | The STC Program shall determine the Pilot_SHDL_Operation according to the following logic IF (Pilot SHDL Operation 1 Health Monitor Status) THEN IF (Pilot SHDL Operation 2 Health Monitor Status) THEN Pilot_SHDL_Operation_Valid = FALSE Pilot_SHDL_Operation = 0 ELSE Pilot_SHDL_Operation = Pilot_SHDL_Operation_2 Pilot_SHDL_Operation_Valid = TRUE ENDIF ELSE IF (Pilot SHDL Operation 2 Health Monitor Status) THEN Pilot_SHDL_Operation = Pilot_SHDL_Operation_1 Pilot_SHDL_Operation_Valid = TRUE ELSE IF (Pilot SHDL Operation 1 and 2 Reasonableness Check Monitor Status) THEN Pilot_SHDL_Operation_Valid = FALSE Pilot_SHDL_Operation = 0 ELSE Pilot_SHDL_Operation = mean of (Pilot_SHDL_Operation_1, Pilot_SHDL_Operation_2) Pilot_SHDL_Operation_Valid = TRUE ENDIF ENDIF ENDIF Where Pilot SHDL Operation 1 Health Monitor Status is defined in MRJ_SCU_STC_SRS_592 Pilot SHDL Operation 2 Health Monitor Status is defined in MRJ_SCU_STC_SRS_593 Pilot SHDL Operation 1 and 2 Reasonableness Check Monitor Status is defined in MRJ_SCU_STC_SRS_599 Pilot_SHDL_Operation_1 is defined in MRJ_SCU_STC_SRS_852 Pilot_SHDL_Operation_2 is defined in MRJ_SCU_STC_SRS_84 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_853 | The STC Program shall determine Co_Pilot_SHDL_Operation_1 according to the following equation: Co_Pilot_SHDL_Operation_1 = Ratiometric_Position[Co_Pilot_RVDT1] / 0.009616071 Where Ratiometric_Position[Co_Pilot_RVDT1] is defined in MRJ_SCU_STC_SRS_65 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_85 | The STC Program shall determine Co_Pilot_SHDL_Operation_2 according to the following equation: Co_Pilot_SHDL_Operation_2 = Ratiometric_Position[Co_Pilot_RVDT2] / 0.009616071 Where Ratiometric_Position[Co_Pilot_RVDT2] is defined in MRJ_SCU_STC_SRS_65 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_86 | The STC Program shall determine the Co_Pilot_SHDL_Operation according to the following logic IF (Co_Pilot SHDL Fitted = FALSE) Co-Pilot_SHDL_Operation_Valid = FALSE Co_Pilot_SHDL_Operation = 0 ELSE IF (Co_Pilot SHDL Operation 1 Health Monitor Status) THEN IF (Co_Pilot SHDL Operation 2 Health Monitor Status) THEN Co-Pilot_SHDL_Operation_Valid = FALSE Co_Pilot_SHDL_Operation = 0 ELSE Co_Pilot_SHDL_Operation = Co_Pilot_SHDL_Operation_2 Co-Pilot_SHDL_Operation_Valid = TRUE ENDIF ELSE IF (Co_Pilot SHDL Operation 2 Health Monitor Status) THEN Co_Pilot_SHDL_Operation = Co_Pilot_SHDL_Operation_1 Co-Pilot_SHDL_Operation_Valid = TRUE ELSE IF (Co_Pilot SHDL Operation 1 and 2 Reasonableness Check Monitor Status) THEN Co-Pilot_SHDL_Operation_Valid = FALSE Co_Pilot_SHDL_Operation = 0 ELSE Co_Pilot_SHDL_Operation = mean of (Co_Pilot_SHDL_Operation_1, Co_Pilot_SHDL_Operation_2) Co-Pilot_SHDL_Operation_Valid = TRUE ENDIF ENDIF ENDIF ENDIF Where Co_Pilot SHDL Operation 1 Health Monitor Status is defined in MRJ_SCU_STC_SRS_596 Co_Pilot SHDL Operation 2 Health Monitor Status is defined in MRJ_SCU_STC_SRS_597 Co_Pilot SHDL Operation 1 and 2 Reasonableness Check Monitor Status is defined in MRJ_SCU_STC_SRS_600 Co_Pilot_SHDL_Operation_1 is defined in MRJ_SCU_STC_SRS_853 Co_Pilot_SHDL_Operation_2 is defined in MRJ_SCU_STC_SRS_85 Co_Pilot SHDL Fitted is defined in MRJ_SCU_STC_SRS_79 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

##### SACT Logical Level Processing

Overview of SACT Determination

There are four LVDTs: two left-hand and two right-hand.  A voltage ratio is calculated for each LVDT interface as previously described.

Essentially there are two angles that correspond to any measured ratio, referred to here as the lower and upper angles which are on either side of the minimum ratio in Figure 6.5-1. The true angle is calculated on a scheme based on the eight candidate angles obtained from the four ratios in a way that does not contain discontinuities close to the zero degree point.  This scheme is depicted in Figure 6.5-1 below:

Figure 6.5-1 - Translation from Ratio to Angle

For a left-hand LVDT, the lower angle is obtained by linear interpolation using the section of the appropriate look-up table from the lower limit to the minimum, the “Left lower” curve in Figure 6.5-1. The upper angle is computed by reflection in the y = 27 line.

For a right-hand LVDT, the upper angle is obtained by polynomial interpolation using the section of the appropriate look-up table from the minimum to the upper limit, the “Right upper” curve in Figure 6.5-1  The lower angle is computed by reflection in the y = -27 line.

The interpolation is done using a ratio-to-angle lookup created by polynomial interpolation between the existing data points.  The integer part of the Q0.14 ratio gives the lower index into the look-up and the fractional part is used to derive the weights of the two table entries that are interpolated.  The angles are in Q8.7 to cover the full ±140° range.

Triples of angles are selected to determine angle validity.  There are 12 possible triples that can be selected from the eight angles using only one lower or upper angle from each LVDT.

Figure 6.5-2 - Angle Weighting Function

The weight for each angle is calculated by linear interpolation of the weighting function.  The weighting function, shown in Figure 6.5-2, is symmetric about the minimum ratio so it is only necessary to calculate the weight for the lower angle for an LVDT and duplicate it for the upper angle.  Since the weighting table is symmetric only the left-hand LVDT weighting function is needed.

The angle weights for invalid angles are set to zero e.g. by multiplying the weight by the validity of the angle they are associated with.  The resultant “valid weights” vector is summed.  If this sum is zero then the No Angle Found fault will be set and the previous angle will be used.  Note: this should never happen.

Assuming the “valid weights” sum is non-zero then the angles are weighted by the elements of the “valid weights” array.  The angle estimate is formed by dividing the weighted sum of the angles by the sum of the valid weights.

In summary the following processing is performed:

1)	The ratio is used to determine a pair of input angles via interpolation that could have produced that ratio: a lower and upper angle.

2)	 Each angle is weighted according to a non-linear weighting function.

3)	Triples of angles are selected using only one lower or upper angle from each LVDT.  The angle triples are checked for consistency and the angles in consistent triples are denoted as valid.

4)	The validity for each angle is complemented by LVDT Health Status and LVDT Reasonableness Status information.

5)	The angles, weights and validities are combined to calculate an overall angle estimate.

| MRJ_SCU_STC_SRS_1228 | The STC Program shall convert the Ratiometric_Position[LH_LVDT1] into a LH physical nose-wheel position value LH LVDT1 Lower Angle, referred to from now on as Angle_1, by interpolating a result from the Left_Lvdt_Angle_Lookup table held in the STC Program configuration data. Where Ratiometric_Position[LH_LVDT1] is the value defined in MRJ_SCU_STC_SRS_65. The angle interpolation algorithm is defined in MRJ_SCU_STC_SRS_1242. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1229 | The STC Program shall determine the LH LVDT1 Upper Angle, referred to from now on as Angle_2, by subtracting Angle_1 from 54 (in Q8.7). |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1230 | The STC Program shall convert the Ratiometric_Position[LH_LVDT2] into a LH physical nose-wheel position value LH LVDT2 Lower Angle, referred to from now on as Angle_3, by interpolating a result from the Left_Lvdt_Angle_Lookup table held in the STC Program configuration data. Where Ratiometric_Position[LH_LVDT2] is the value defined in MRJ_SCU_STC_SRS_65. The angle interpolation algorithm is defined in MRJ_SCU_STC_SRS_1242. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1231 | The STC Program shall determine the LH LVDT2 Upper Angle, referred to from now on as Angle_4, by subtracting Angle_3 from 54 (in Q8.7). |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1232 | The STC Program shall convert the Ratiometric_Position[RH_LVDT1] into a RH physical nose-wheel position value RH LVDT1 Upper Angle, referred to from now on as Angle_6, by interpolating a result from the Left_Lvdt_Angle_Lookup table held in the STC Program configuration data and multiplying the result by -1. Where Ratiometric_Position[RH_LVDT1] is the value defined in MRJ_SCU_STC_SRS_65. The angle interpolation algorithm is defined in MRJ_SCU_STC_SRS_1242. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1233 | The STC Program shall determine the RH LVDT1 Lower Angle, referred to from now on as Angle_5, by subtracting 54 (in Q8.7) from (Angle_6 * -1). |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1234 | The STC Program shall convert the Ratiometric_Position[RH_LVDT2] into a RH physical nose-wheel position value RH LVDT2 Upper Angle, referred to from now on as Angle_8, by interpolating a result from the Left_Lvdt_Angle_Lookup table held in the STC Program configuration data and multiplying the result by -1. Where Ratiometric_Position[RH_LVDT2] is the value defined in MRJ_SCU_STC_SRS_65. The angle interpolation algorithm is defined in MRJ_SCU_STC_SRS_1242. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1235 | The STC Program shall determine the RH LVDT2 Lower Angle, referred to from now on as Angle_7, by subtracting 54 (in Q8.7) from (Angle_8 * -1). |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1236 | The STC Program shall complement the 8 calculated angles with BITE information according to the following logic: IF (LH LVDT1 Health Monitor Status OR LH LVDT1 Reasonableness Fault) THEN Angle_1 = 200 deg (in Q8.7); (*set to out-of-range value*) Angle_2 = -200 deg (in Q8.7); END IF IF (LH LVDT2 Health Monitor Status OR LH LVDT2 Reasonableness Fault) THEN Angle_3 = 200 deg (in Q8.7); (*set to out-of-range value*) Angle_4 = -200 deg (in Q8.7); END IF IF (RH LVDT1 Health Monitor Status OR RH LVDT1 Reasonableness Fault) THEN Angle_5 = 200 deg (in Q8.7); (*set to out-of-range value*) Angle_6 = -200 deg (in Q7.8); END IF IF (RH LVDT2 Health Monitor Status OR RH LVDT2 Reasonableness Fault) THEN Angle_7 = 200 deg (in Q8.7); (*set to out-of-range value*) Angle_8 = -200 deg (in Q8.7); END IF Where LH LVDT1 Health Monitor Status is defined in MRJ_SCU_STC_SRS_567 LH LVDT2 Health Monitor Status is defined in MRJ_SCU_STC_SRS_568 RH LVDT1 Health Monitor Status is defined in MRJ_SCU_STC_SRS_67 RH LVDT2 Health Monitor Status is defined in MRJ_SCU_STC_SRS_566 LH LVDT1 Reasonableness Fault is defined in MRJ_SCU_STC_SRS_1241 LH LVDT2 Reasonableness Fault is defined in MRJ_SCU_STC_SRS_1241 RH LVDT1 Reasonableness Fault is defined in MRJ_SCU_STC_SRS_1241 RH LVDT2 Reasonableness Fault is defined in MRJ_SCU_STC_SRS_1241 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1237 | The STC Program shall determine the weighting factors Weight_n for the LH angles Angle_n according to the following algorithm: Weight_1 = interpolating a result from the Left_Lvdt_Weight_Lookup table held in the STC Program configuration data for Angle_1 Weight_2 = Weight_1 Weight_3 = interpolating a result from the Left_Lvdt_Weight_Lookup table held in the STC Program configuration data for Angle_3 Weight_4 = Weight_3 Where n = 1..4. The weighting interpolation algorithm is defined in MRJ_SCU_STC_SRS_1243. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1238 | The STC Program shall determine the weighting factors Weight_n for the RH angles Angle_n according to the following algorithm: Weight_5 = interpolating a result from the Left_Lvdt_Weight_Lookup table held in the STC Program configuration data for (Angle_5 * -1) Weight_6 = Weight_5 Weight_7 = interpolating a result from the Left_Lvdt_Weight_Lookup table held in the STC Program configuration data for (Angle_7 * -1) Weight_8 = Weight_7 Where n = 1..4. The weighting interpolation algorithm is defined in MRJ_SCU_STC_SRS_1243. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1239 | The STC Program shall determine the validity flags Validity_n for the 8 angles Angle_n according to the following algorithm: FOR each row of the 12 rows in the Triple table held in the STC Program configuration data DO Determine the difference between the minimum angle of the triplet and the maximum angle of the triplet IF difference is less than the difference threshold value THEN Set the 3 validity flags Validity_n for all 3 angles of the triplet to VALID (1) ELSE No action (Validity_n remains unchanged) END IF END FOR With an initial value for all 8 validity flags of INVALID (0). Where n = 1..8. Difference threshold value is 14 degree (in Q8.7), which corresponds to 5% of full scale -140..+140 deg. The Triple table is defined in MRJ_SCU_STC_SRS_1249 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1240 | The STC Program shall calculate the physical nose-wheel position SACT_Position according to the following algorithm: FOR all 8 angles Angle_n, Weight_n and Validity_n DO WeightAndValidity = Weight_n * Validity_n WeightSum = WeightSum + WeightAndValidity WeightedAngle = WeightedAngle + (WeightAndValidity * Angle_n) END FOR IF WeightSum > 0 THEN SACT_Position = WeightedAngle / WeightSum (in Q7.7) Transform SACT_Position into Q7.8 ELSE With a confirmation time of 100ms set LVDT No Angle Found Fault END IF With an initial value of 0 for WeightSum, WeightedAngle and SACT_Position. Where n = 1..8. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1241 | The STC Program shall identify the LVDT interfaces related to LVDT Reasonableness Faults according to the following logic: IF (LH LVDT1 and LH LVDT2 Reasonableness Check Monitor Status) THEN IF Validity_1 = INVALID and Validity_2 = INVALID THEN LH LVDT1 Reasonableness Fault = TRUE ELSE do nothing (fault status remains unchanged) IF Validity_3 = INVALID and Validity_4 = INVALID THEN LH LVDT2 Reasonableness Fault = TRUE ELSE do nothing (fault status remains unchanged) END IF IF (RH LVDT1 and RH LVDT2 Reasonableness Check Monitor Status) THEN IF Validity_5 = INVALID and Validity_6 = INVALID THEN RH LVDT1 Reasonableness Fault = TRUE ELSE do nothing (fault status remains unchanged) IF Validity_7 = INVALID and Validity_8 = INVALID THEN RH LVDT2 Reasonableness Fault = TRUE ELSE do nothing (fault status remains unchanged) END IF With an initial value of FALSE for all 4 Reasonableness Faults Where LH LVDT1 and LH LVDT2 Reasonableness Check Monitor Status is defined in MRJ_SCU_STC_SRS_591 RH LVDT1 and RH LVDT2 Reasonableness Check Monitor Status is defined in MRJ_SCU_STC_SRS_572 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1242 | As part of the SACT_Position determination, for any given ratio value (in Q0.15), the STC Program shall perform the angle interpolation by a lookup using the LH LVDT Ratio-to-Angle table via fractional indexing according to the following algorithm: Transform ratio from Q0.15 into Q0.14 IF ratio < Ratio_Low_Fault_Threshold OR ratio > Ratio_High_Fault_Threshold THEN Angle = -256 deg (in Q8.7) ELSE sratio = ratio - ratio_min_Q14 sratio = sratio SHIFT_LEFT by 15 index = (sratio + half_ratio_range_Q14) / ratio_range_Q14 iindex = index SHIFT_RIGHT by Angle_Index_Fbits findex = index - (iindex SHIFT_LEFT by Angle_Index_Fbits) IF iindex >= (Angle_Lookup_Len - 1) THEN iindex = Angle_Lookup_Len - 2 wLower = 0 wUpper = Angle_Index_Unity ELSE IF iindex <= 0 THEN iindex = 0 wLower = Angle_Index_Unity wUpper = 0 ELSE wLower = Angle_Index_Unity - findex wUpper = findex END IF theta1 = LH LVDT Ratio-to-Angle table(iindex) * wLower theta2 = LH LVDT Ratio-to-Angle table(iindex+1) * wUpper Angle = theta1 + theta2 (result is in Q18.12) Round Angle to Q8.7 END IF where ratio_min_Q14 = -0.4736 in Q1.14 half_ratio_range_Q14 = 0.66405 in Q1.14 ratio_range_Q14 = 1.3281 in Q1.14 Angle_Index_Ibits = 10 Angle_Index_Fbits = 15 - Angle_Index_Ibits Angle_Index_Unity = 1 SHIFT_LEFT by Angle_Index_Fbits Angle_Lookup_Len = 1024 Ratio_Low_Fault_Threshold = 5% of full expected range below minimum ratio in Q0.14 = -8552 Ratio_High_Fault_Threshold = 5% of full expected range above maximum ratio in Q0.14 = 8886 The LH LVDT Ratio-to-Angle table is defined in MRJ_SCU_STC_SRS_111 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1243 | As part of the SACT_Position determination, for any given angle value (in Q8.7), the STC Program shall perform the weight linear interpolation by a lookup using the LH LVDT Angle Weighting table according to the following algorithm: index = ((Angle + (2 SHIFT_LEFT by 6)) SHIFT_RIGHT by 7) + Weight_Lookup_Zero_Index index = maximum(index, 0) index = minimum(index, Weight_Lookup_Len - 1) Weight = LH LVDT Angle Weighting table (index) where Weight_Lookup_Zero_Index = 140 (Lookup index for zero degrees) Weight_Lookup_Len = 281 The LH LVDT Angle Weighting table is defined in MRJ_SCU_STC_SRS_1251 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

### Discrete Input Processing

| MRJ_SCU_STC_SRS_124 | The STC Program shall sample each of the following Discrete Input signals during each Major Cycle using the discrete input interface in accordance with the HW/SW Interface Specification: Table 6.5-5 Discrete Input GPIOs Where Major cycle is defined in Table 6.2-1 Note: D5, D6, E0 and F4 are active-low signals |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_964 | The STC Program shall only sample the EHSV1_Fail and EHSV2_Fail inputs in a window of 4.5 to 5.12ms after the EHSV_PWM duty cycle is updated. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_164 | The STC Program shall debounce the raw values read from each of the discrete inputs using the following algorithm: IF (Raw_Input_Signal = TRUE for Debounce_Cycles successive samples ) THEN set Debounced_Input_Signal = TRUE ELSE IF ( Raw_Input_Signal = FALSE for Debounce_Cycles successive samples ) THEN set Debounced_Input_Signal = FALSE ELSE Debounced_Input_Signal remains unchanged ENDIF ENDIF Note: The successive samples is a moving window of the latest sampled values advanced each Major cycle adding a new value and dropping the oldest. Where Raw_Input Signals are those listed in MRJ_SCU_STC_SRS_124 and Debounce_Cycles are defined for each Input_Signal in MRJ_SCU_STC_SRS_124 and Major cycle is defined in Table 6.2-1 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_186 | The STC Program shall set Pilot_Engage to the previous state of Pilot_Engage in case fault confirmation is in progress for the Pilot SHDL Switch Reasonableness Check Monitor, and otherwise determine Pilot_Engage according to the following logic: Table 6.5-6 Pilot Engage Selection Where Debounced_Pilot_Engage and Debounced_Pilot_Disengage are the values defined in MRJ_SCU_STC_SRS_164 from Raw_Pilot_Engage and Raw_Pilot_Disengage defined in MRJ_SCU_STC_SRS_124 and the Pilot SHDL Switch Reasonableness Check Monitor is defined in MRJ_SCU_STC_SRS_575 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_187 | The STC Program shall set the Co_Pilot_Engage status to FALSE if Co_Pilot_SHLD_Fitted is FALSE, else set Co_Pilot_Engage to the previous state of Co_Pilot_Engage in case fault confirmation is in progress for the Co_Pilot SHDL Switch Reasonableness Check Monitor, and otherwise determine Co_Pilot_Engage according to the following logic: Table 6.5-7 Co_Pilot Engage Selection Where Debounced_Co_Pilot_Engage and Debounced_Co_Pilot_Disengage are the values defined in MRJ_SCU_STC_SRS_164 from Raw_Co_Pilot_Engage and Raw_Co_Pilot_Disengage defined in MRJ_SCU_STC_SRS_124 and the Co_Pilot SHDL Switch Reasonableness Check Monitor status is defined in MRJ_SCU_STC_SRS_601 Co_Pilot_SHDL_Fitted is defined in MRJ_SCU_STC_SRS_79 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_508 | The STC Program shall set Towing_Disengage to the previous state of Towing_Disengage in case fault confirmation is in progress for the Towing Switch Reasonableness Check Monitor, and otherwise determine Towing_Disengage according to the following logic: Table 6.5-8 Towing Disengage Selection Where Debounced_Towing_Engage and Debounced_Towing_Disengage are the values defined in MRJ_SCU_STC_SRS_164 from Raw_Towing_Engage and Raw_Towing_Disengage defined in MRJ_SCU_STC_SRS_124 and the Towing Switch Reasonableness Check Monitor status is defined in MRJ_SCU_STC_SRS_603 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_188 | The STC Program shall initialise the discrete input variables to the values shown in table under MRJ_SCU_STC_SRS_124. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

### ARINC 429 Input Processing

Note: the following requirement is covered by the DSP Register initialisation:

The STC Program will provide a 1 MHz clock signal to the DEI-1085 ARINC device via GPIOA7.

| MRJ_SCU_STC_SRS_616 | The STC Program shall access the ARINC 429 transceivers via the External Interface (XINTF) by applying the following register mapping from memory Zone 1 to the DEI-1085 ARINC devices and accessing only non-reserved addresses: Table 6.5-9 DEI-1085 Address Mapping Note that “CR1” in the above table refers to bit 1 of the DEI-1085 control register for the device being accessed. This bit can be set to ‘1’ or ‘0’ for a DEI-1085 device by writing to the corresponding Control Register address (0x04001, 0x04011 or 0x04021). The actual register accessed within a DEI-1085 device may depend on the setting of the corresponding ‘CR1’ bit. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_617 | The STC Program shall apply the following mapping from ARINC Transceiver to external ARINC 429 Bus: Table 6.5-10 DEI-1085 Bus Mapping |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_137 | The STC Program shall poll the ARINC receivers for the labels listed in the following table more frequently than the transmission frequency: Table 6.5-11 ARINC Rx Rate |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-15-166 |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1209 | If ARINC data is used in a function regardless from which ARINC bus and SDI source it originated from, then the STC Program shall use the data from the label received most recently. This applies to the following label data: - Date data in Label 260 - Serial Number data in Label 226 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : |
|  | Verification Method : |

#### DMC ARINC 429 Input Processing

##### Label 022 (WOW Data) Input Processing

The format of Label 022 (WOW Data) will be as follows

Table 6.5-12 Label 022

| MRJ_SCU_STC_SRS_871 | The STC Program shall receive A_DMC_9 Label 022 messages from DMCA in the format of Label 022 specified in Table 6.5-12. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-15-166 |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_872 | The STC Program shall receive B_DMC_9 Label 022 messages from DMCB in the format of Label 022 specified in Table 6.5-12. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-15-166 |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_139 | The STC Program shall monitor the WOW Data ARINC messages from DMCA for correct update and valid data according to the following logic:- IF( WOW Data ARINC message received) THEN IF ( Nose_WOW_Valid ) AND ( ARINC SSM = Normal Operation ) AND ( SDI = DMC Ch-A ) AND ( Parity is ODD ) THEN NLG_WOW1 = Nose WOW State (bit 15 of message) NLG_WOW1_Valid = TRUE ELSE NLG_WOW1_Valid = FALSE ENDIF ELSE NLG_WOW1_Valid = FALSE ENDIF Where Nose_WOW_Valid is defined as Bit 16 of ARINC Label 022 defined in Table 6.5-12 ARINC_SSM is defined as Bits 31..30 of ARINC Label 022 defined in Table 6.5-12 SDI is defined as Bits 10..9 of ARINC Label 022 defined in Table 6.5-12 Parity is ODD or EVEN defined as the sum of all the bits in ARINC Label 022 defined in Table 6.5-12 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_142 | The STC Program shall monitor the WOW Data ARINC messages from DMCB for correct update and valid data according to the following logic:- IF( WOW Data ARINC message received) THEN IF ( Nose WOW Valid ) AND ( ARINC SSM = Normal Operation ) AND ( SDI = DMC Ch-B) AND ( Parity is ODD) THEN NLG_WOW2 = Nose WOW State (bit 15 of message) NLG_WOW2_Valid = TRUE ELSE NLG_WOW2_Valid = FALSE ENDIF ELSE NLG_WOW2_Valid = FALSE ENDIF Where Nose_WOW_Valid is defined as Bit 16 of ARINC Label 022 defined in Table 6.5-12 ARINC_SSM is defined as Bits 31..30 of ARINC Label 022 defined in Table 6.5-12 SDI is defined as Bits 10..9 of ARINC Label 022 defined in Table 6.5-12 Parity is ODD or EVEN defined as the sum of all the bits in ARINC Label 022 defined in Table 6.5-12 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_158 | The STC Program shall set the following power-up defaults for NLG WOW data: - NLG_WOW1 = TRUE - NLG_WOW1_Valid = FALSE - NLG_WOW2 = TRUE - NLG_WOW2_Valid = FALSE - Nose WOW State = TRUE |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

##### Label 206 (ISI) Input Processing

The format of Label 206 (ISI) containing Computed Airspeed will be as follows :-

Table 6.5-13 Label 206 (ISI)

| MRJ_SCU_STC_SRS_848 | The STC Program shall receive A_DMC_9 Label 206 (ISI) messages from DMCA in the format of Label 206 (ISI) specified in Table 6.5-13. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-15-166 |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_849 | The STC Program shall receive B_DMC_9 Label 206 (ISI) messages from DMCB in the format of Label 206 (ISI) specified in Table 6.5-13. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-15-166 |
|  | Verification Method : |

##### Label 206 (ADC) Input Processing

The format of Label 206 (ADC) containing Aircraft Speed calculated by Air Data Computer will be as follows :-

Table 6.5-14 Label 206 (ADC)

| MRJ_SCU_STC_SRS_135 | The STC Program shall receive A_DMC_9 L ADC Airspeed in the Computed Airspeed field in messages from DMCA in the format of Label 206 (ADC) specified in Table 6.5-14 with a Source/Destination Identifier set to Left ADC |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-15-166 |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1087 | The STC Program shall receive A_DMC_9 R ADC messages from DMCA in the format of Label 206 (ADC) specified in Table 6.5-14 with a Source/Destination Identifier set to Right ADC |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-15-166 |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1088 | The STC Program shall receive B_DMC_9 L ADC messages from DMCB in the format of Label 206 (ADC) specified in Table 6.5-14 with a Source/Destination Identifier set to Left ADC |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-15-166 |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_144 | The STC Program shall receive B_DMC_9 R ADC Airspeed in the Computed Airspeed field in messages from DMCB in the format of Label 206 (ADC) specified in Table 6.5-14 with a Source/Destination Identifier set to Right ADC |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-15-166 |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_160 | The STC Program shall set the following power-up defaults for Airspeed data: - Airspeed = 0 kts - AirspeedA_Not_Valid = TRUE - AirspeedB_Not_Valid = TRUE - Airspeed_Not_Valid = TRUE |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1089 | The STC Program shall determine AirspeedA according to the following logic: Set AirspeedA = 0 IF 2 or more out of [A_DMC_9 ISI Monitor Status, A_DMC_9 LADC Monitor Status, A_DMC_9 RADC Monitor Status] = TRUE THEN Set AirspeedA_Not_Valid to TRUE ELSE IF 1 out of [A_DMC_9 ISI Monitor Status, A_DMC_9 LADC Monitor Status, A_DMC_9 RADC Monitor Status] = TRUE THEN IF A_DMC_9 ISI Monitor Status = TRUE THEN IF A_DMC_9 LADC and A_DMC_9 RADC Reasonableness Check Monitor Status = FALSE THEN Set AirspeedA to average of (A_DMC_9 LADC Airspeed, A_DMC_9 RADC Airspeed) Set AirspeedA_Not_Valid to FALSE ELSE Set AirspeedA_Not_Valid to TRUE END IF ELSE IF A_DMC_9 RADC Monitor Status = TRUE THEN IF A_DMC_9 LADC and A_DMC_9 ISI Reasonableness Check Monitor Status = FALSE THEN Set AirspeedA to average of (A_DMC_9 LADC Airspeed, A_DMC_9 ISI Airspeed) Set AirspeedA_Not_Valid to FALSE ELSE Set AirspeedA_Not_Valid to TRUE END IF // Note: A_DMC_9 LADC Monitor Status = TRUE ELSE IF A_DMC_9 RADC and A_DMC_9 ISI Reasonableness Check Monitor Status = FALSE THEN Set AirspeedA to average of (A_DMC_9 RADC Airspeed, A_DMC_9 ISI Airspeed) Set AirspeedA_Not_Valid to FALSE ELSE Set AirspeedA_Not_Valid to TRUE END IF // Note: none out of [A_DMC_9 ISI Monitor Status, // A_DMC_9 LADC Monitor Status, // A_DMC_9 RADC Monitor Status] = TRUE ELSE IF A_DMC_9 LADC and A_DMC_9 RADC and A_DMC_9 ISI Reasonableness Check Monitor Status = FALSE THEN Set AirspeedA to average of (A_DMC_9 LADC Airspeed, A_DMC_9 RADC Airspeed, A_DMC_9 ISI Airspeed) Set AirspeedA_Not_Valid to FALSE ELSE IF A_DMC_9 LADC and A_DMC_9 RADC Reasonableness Check Monitor Status = FALSE THEN Set AirspeedA to average of (A_DMC_9 LADC Airspeed, A_DMC_9 RADC Airspeed) Set AirspeedA_Not_Valid to FALSE ELSE IF A_DMC_9 LADC and A_DMC_9 ISI Reasonableness Check Monitor Status = FALSE THEN Set AirspeedA to average of (A_DMC_9 LADC Airspeed, A_DMC_9 ISI Airspeed) Set AirspeedA_Not_Valid to FALSE ELSE IF A_DMC_9 RADC and A_DMC_9 ISI Reasonableness Check Monitor Status = FALSE THEN Set AirspeedA to average of (A_DMC_9 RADC Airspeed, A_DMC_9 ISI Airspeed) Set AirspeedA_Not_Valid to FALSE ELSE Set AirspeedA_Not_Valid to TRUE END IF Where A_DMC_9 LADC Monitor Status is defined in MRJ_SCU_STC_SRS_811 A_DMC_9 RADC Monitor Status is defined in MRJ_SCU_STC_SRS_1051 A_DMC_9 ISI Monitor Status is defined in MRJ_SCU_STC_SRS_818 A_DMC_9 LADC and A_DMC_9 RADC Reasonableness Check Monitor Status is defined in MRJ_SCU_STC_SRS_1067 A_DMC_9 LADC and A_DMC_9 ISI Reasonableness Check Monitor Status is defined in MRJ_SCU_STC_SRS_1179 A_DMC_9 RADC and A_DMC_9 ISI Reasonableness Check Monitor Status is defined in MRJ_SCU_STC_SRS_1180 A_DMC_9 LADC and A_DMC_9 RADC and A_DMC_9 ISI Reasonableness Check Monitor Status is defined in MRJ_SCU_STC_SRS_1178 A_DMC_9 LADC Airspeed is defined in MRJ_SCU_STC_SRS_135 A_DMC_9 RADC Airspeed is defined in MRJ_SCU_STC_SRS_1087 A_DMC_9 ISI Airspeed is defined in MRJ_SCU_STC_SRS_848 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1090 | The STC Program shall determine AirspeedB according to the following logic: Set AirspeedB = 0 IF 2 or more out of [B_DMC_9 ISI Monitor Status, B_DMC_9 LADC Monitor Status, B_DMC_9 RADC Monitor Status] = TRUE THEN Set AirspeedB_Not_Valid to TRUE ELSE IF 1 out of [B_DMC_9 ISI Monitor Status, B_DMC_9 LADC Monitor Status, B_DMC_9 RADC Monitor Status] = TRUE THEN IF B_DMC_9 ISI Monitor Status = TRUE THEN IF B_DMC_9 LADC and B_DMC_9 RADC Reasonableness Check Monitor Status = FALSE THEN Set AirspeedB to average of (B_DMC_9 LADC Airspeed, B_DMC_9 RADC Airspeed) Set AirspeedB_Not_Valid to FALSE ELSE Set AirspeedB_Not_Valid to TRUE END IF ELSE IF B_DMC_9 RADC Monitor Status = TRUE THEN IF B_DMC_9 LADC and B_DMC_9 ISI Reasonableness Check Monitor Status = FALSE THEN Set AirspeedB to average of (B_DMC_9 LADC Airspeed, B_DMC_9 ISI Airspeed) Set AirspeedB_Not_Valid to FALSE ELSE Set AirspeedB_Not_Valid to TRUE END IF ELSE // Note: B_DMC_9 LADC Monitor Status = TRUE IF B_DMC_9 RADC and B_DMC_9 ISI Reasonableness Check Monitor Status = FALSE THEN Set AirspeedB to average of (B_DMC_9 RADC Airspeed, B_DMC_9 ISI Airspeed) Set AirspeedB_Not_Valid to FALSE ELSE Set AirspeedB_Not_Valid to FALSE END IF END IF // Note: none out of [B_DMC_9 ISI Monitor Status, // B_DMC_9 LADC Monitor Status, // B_DMC_9 RADC Monitor Status] = TRUE ELSE IF B_DMC_9 LADC and B_DMC_9 RADC and B_DMC_9 ISI Reasonableness Check Monitor Status =FALSE THEN Set AirspeedB to average of (B_DMC_9 LADC Airspeed, B_DMC_9 RADC Airspeed, B_DMC_9 ISI Airspeed) Set AirspeedB_Not_Valid to FALSE ELSE IF B_DMC_9 LADC and B_DMC_9 RADC Reasonableness Check Monitor Status = FALSE THEN Set AirspeedB to average of (B_DMC_9 LADC Airspeed, B_DMC_9 RADC Airspeed) Set AirspeedB_Not_Valid to FALSE ELSE IF B_DMC_9 LADC and B_DMC_9 ISI Reasonableness Check Monitor Status = FALSE THEN Set AirspeedB to average of (B_DMC_9 LADC Airspeed, B_DMC_9 ISI Airspeed) Set AirspeedB_Not_Valid to FALSE ELSE IF B_DMC_9 RADC and B_DMC_9 ISI Reasonableness Check Monitor Status = FALSE THEN Set AirspeedB to average of (B_DMC_9 RADC Airspeed, B_DMC_9 ISI Airspeed) Set AirspeedB_Not_Valid to FALSE ELSE Set AirspeedB_Not_Valid to TRUE END IF Where B_DMC_9 LADC Monitor Status is defined in MRJ_SCU_STC_SRS_1054 B_DMC_9 RADC Monitor Status is defined in MRJ_SCU_STC_SRS_816 B_DMC_9 ISI Monitor Status is defined in MRJ_SCU_STC_SRS_813 B_DMC_9 LADC and B_DMC_9 RADC Reasonableness Check Monitor Status is defined in MRJ_SCU_STC_SRS_ 1070 B_DMC_9 LADC and B_DMC_9 ISI Reasonableness Check Monitor Status is defined in MRJ_SCU_STC_SRS_1073 B_DMC_9 RADC and B_DMC_9 ISI Reasonableness Check Monitor Status is defined in MRJ_SCU_STC_SRS_1076 B_DMC_9 LADC and B_DMC_9 RADC and B_DMC_9 ISI Reasonableness Check Monitor Status is defined in MRJ_SCU_STC_SRS_1085 B_DMC_9 LADC Airspeed is defined in MRJ_SCU_STC_SRS_1088 B_DMC_9 RADC Airspeed is defined in MRJ_SCU_STC_SRS_144 B_DMC_9 ISI Airspeed is defined in MRJ_SCU_STC_SRS_849 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_146 | The STC Program shall determine Airspeed and Airspeed_Valid according to the following logic: Set Airspeed = 0 IF AirspeedA_Not_Valid = TRUE and AirspeedB_Not_Valid = TRUE THEN Enter Failsafe level 1 Set Airspeed_Not_Valid = TRUE ELSE IF AirspeedA_Not_Valid = TRUE THEN Set Airspeed = AirSpeedB Set Airspeed_Not_Valid = FALSE ELSE IF AirspeedB_Not_Valid = TRUE THEN Set Airspeed = AirSpeedA Set Airspeed_Not_Valid = FALSE ELSE IF AirSpeedA and AirSpeedB Reasonableness Check Monitor Status = FALSE THEN Set Airspeed to average of (AirSpeedA, AirSpeedB) Set Airspeed_Not_Valid = FALSE ELSE Enter Failsafe level 1 Set Airspeed_Not_Valid = TRUE END IF Where AirspeedA_Not_Valid is defined in MRJ_SCU_STC_SRS_1089 AirspeedB_Not_Valid is defined in MRJ_SCU_STC_SRS_1090 AirspeedA is defined in MRJ_SCU_STC_SRS_1089 AirspeedB is defined in MRJ_SCU_STC_SRS_1090 AirspeedA and AirspeedB Reasonableness Check Monitor Status is defined in MRJ_SCU_STC_SRS_1181 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

##### Label 260 (Aircraft Date) Input Processing

The format of Label 260 (Aircraft Date) containing current Aircraft Date provided by the Air Data Computer will be as follows :-

Table 6.5-15 Label 260 (Aircraft Date)

| MRJ_SCU_STC_SRS_873 | The STC Program shall receive A_DMC_9 Label 260 (Aircraft Date) messages from DMCA in the format of Label 260 specified in Table 6.5-15. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-15-166 |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_874 | The STC Program shall receive B_DMC_9 Label 260 (Aircraft Date) messages from DMCB in the format of Label 260 specified in Table 6.5-15. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-15-166 |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_869 | The STC Program shall determine, once in a power cycle EHSV Selection from the lowest bit of "Aircraft Date Value: One’s digit of Day" field in the latest read Label 260, using '0' to indicate EVEN and '1' to indicate ODD. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_870 | The STC Program shall use the default value of EHSV Selection set to ODD until a valid Label 260 has been read. Justification: Arbitrary decision to use 'Odd', accepted by system / safety assessment. |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

##### Label 226 Aircraft Serial Number Input Processing

The format of Label 226 Aircraft Serial Number will be as follows :-

Table 6.5-16 Label 226 (Aircraft Serial Number)

| MRJ_SCU_STC_SRS_1048 | The STC Program shall receive A_DMC_9 Label 226 Aircraft Serial Number messages from DMCA in the format of Label 226 Aircraft Serial Number specified in section 6.5.3.1.5. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1049 | The STC Program shall receive B_DMC_9 Label 226 Aircraft Serial Number messages from DMCB in the format of Label 226 Aircraft Serial Number specified in section 6.5.3.1.5. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : |
|  | Verification Method : |

##### Label 107 SCS Rigging Command Input Processing

The format of Label 107 SCS Rigging Command will be as follows :-

| MRJ_SCU_STC_SRS_1171 | The STC Program shall receive A_DMC_9 Label 107 SCS Rigging Command messages from DMCA in the format of Label 107 SCS Rigging Command specified in section 6.5.3.1.6. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-15-166 |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1172 | The STC Program shall maintain a counter of Conduct Rigging commands received (bit 11 = 1) for up to a maximum of 3, and reset the counter if Label 107 is received with bit 11 cleared (0) or not received within 5000ms +/- 10ms. Rationale: 5 sec is considered appropriate time period for OMS to have timed-out. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

##### Label 174 Hydraulic System Pressure

The format of Label 174 (Hydraulic System Pressure) will be as follows:

| MRJ_SCU_STC_SRS_1201 | The STC Program shall receive A_DMC_9 Label 174 Hydraulic System Pressure messages from DMCA in the format of Label 174 Hydraulic System Pressure specified in section 6.5.3.1.7 with a Source/Destination Identifier set to ‘Right’. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1202 | The STC Program shall receive B_DMC_9 Label 174 Hydraulic System Pressure messages from DMCB in the format of Label 174 Hydraulic System Pressure specified in section 6.5.3.1.7 with a Source/Destination Identifier set to ‘Right’. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### ACE ARINC 429 Input Processing

##### Label 136 (Rudder Pedal Position) Input Processing

The format of Label 136 (Rudder Pedal Position) will be as follows :-

Table 6.5-17 Label 136

| MRJ_SCU_STC_SRS_844 | The STC Program shall receive ACE1B Label 136 messages from ACE1 in the format specified in Table 6.5-17 with a Source/Destination Identifier set to ACE_1B as Source |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-10-036 |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_845 | The STC Program shall receive ACE2B Label 136 messages from ACE2 in the format specified in Table 6.5-17 with a Source/Destination Identifier set to ACE_2B as Source |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-10-036 |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_154 | The STC Program shall determine Rudder Pedal Operation according to the following logic:- IF (ACE1B Label 136 Monitor status) OR (ACE1B Reasonableness Check Monitor Status) THEN IF (ACE2B Label 136 Monitor status) OR (ACE2B Reasonableness Check Monitor Status) THEN Rudder Pedal Operation = 0 Rudder Pedal Op Not Valid = TRUE ELSE Use Rudder Pedal Position from ACE2B Label 136 message Rudder Pedal Op Not Valid = FALSE ENDIF ELSE IF (ACE2B Label 136 Monitor status) OR (ACE2B Reasonableness Check Monitor Status) THEN Use Rudder Pedal Position from ACE1B Label 136 message Rudder Pedal Op Not Valid = FALSE ELSE IF (ACE1B Label 136 and ACE2B Label 136 Reasonableness Check Monitor Status) Rudder Pedal Operation = 0 Rudder Pedal Op Not Valid = TRUE ELSE Use mean of ( Rudder Pedal Position from ACE1B Label 136 message And Rudder Pedal Position from ACE2B Label 136 message) Rudder Pedal Op Not Valid = FALSE ENDIF ENDIF ENDIF Where ACE1B Label 136 Monitor status is defined in MRJ_SCU_STC_SRS_828 ACE2B Label 136 Monitor status is defined in MRJ_SCU_STC_SRS_830 ACE1B Reasonableness Check Monitor Status is defined in MRJ_SCU_STC_SRS_832 ACE2B Reasonableness Check Monitor Status is defined in MRJ_SCU_STC_SRS_836 ACE1B Label 136 and ACE2B Label 136 Reasonableness Check Monitor Status is defined in MRJ_SCU_STC_SRS_838 ACE1B Label 136 message is defined in MRJ_SCU_STC_SRS_844 ACE2B Label 136 message is defined in MRJ_SCU_STC_SRS_845 Note: 0 for Rudder_Pedal_Operation is represented by 0x0800 if Offset and Scaling Factor yet to be applied |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

### Analogue Input Processing via SPI Bus Peripheral

In order to read the ADC values from the two external AD7887 devices, the STC Program will upon start-up configure the SPI control registers for SPI Master Mode Operation, 16-bit SPI Data Transmission/Reception and 2MHz SPI Clock output.

Note: This is defined by MRJ_SCU_STC_SRS_525.

| MRJ_SCU_STC_SRS_714 | The STC Program shall read the Analogue_Rudder_Pedal_Position_1 data from the external ADC device 1 (Ref. C15) via McBSP SPI Bus at least every 30ms. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_803 | The STC Program shall satisfy MRJ_SCU_STC_SRS_714 by performing a defined sequence as follows: 1) Assert Rudder1_SPI_En and McBSP_En (Chip Select Low) via GPIOs A4 and F10 for AD7887 device 1 2) Transmit 16-bit command with value 0x2100 to SPI Tx FIFO (Power Management Mode 2, On-chip reference disabled) 3) Wait until DSP transmits data 4) Deselect Rudder1_SPI_En and McBSP_En (Chip Select High) 5) Read ADC value from SPI Rx FIFO |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_716 | The STC Program shall read the Analogue_Rudder_Pedal_Position_2 data from the external ADC device 2 (Ref. C15) via McBSP SPI Bus at least every 30ms |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_804 | The STC Program shall satisfy MRJ_SCU_STC_SRS_716 by performing a defined sequence as follows: 1) Assert Rudder2_SPI_En and McBSP_En (Chip Select Low) via GPIOs A5 and F10 for AD7887 device 2 2) Transmit 16-bit command with value 0x2100 to SPI Tx FIFO (Power Management Mode 2, On-chip reference disabled) 3) Wait until DSP transmits data 4) Deselect Rudder2_SPI_En and McBSP_En (Chip Select High) 5) Read ADC value from SPI Rx FIFO |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_854 | The STC Program shall convert the sampled values Analogue_Rudder_Pedal_Position_1 and Analogue_Rudder_Pedal_Position_2 to mm by using the following formula: Rudder Position (mm) = (Analogue_Rudder_Pedal_Position - 2048) * (2.5/4096) * 10.2 * (76.2/8) Further explanation of these terms can be found in HSIS (Ref B4) MRJ_SCU_HSIS_539 and MRJ_SCU_HSIS_552 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1254 | Using the sampled Rudder Position ADC values, the STC Program shall calculate the Rudder Position with a numerical error of less than 0.15% over the full scale (-76.2mm to +76.2mm). |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

## Steering Control

### Steering Demand Calculation

#### Pilot SHDL Steering Demand

| MRJ_SCU_STC_SRS_99 | The STC Program shall convert the Pilot_SHDL_Operation into a steering angle demand Unlimited_Pilot_SHDL_Command in every Major cycle using the following logic: IF( Pilot_SHDL_Operation >= 54.7deg ) THEN Unlimited_Pilot_SHDL_Command = (26.4 * Pilot_SHDL_Operation - 840.5deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= 44.3deg ) THEN Unlimited_Pilot_SHDL_Command = (16.6 * Pilot_SHDL_Operation - 298.58deg ) * (1/10.4) ELSE IF( Pilot_SHDL_Operation >= 34deg ) THEN Unlimited_Pilot_SHDL_Command = (14.4 * Pilot_SHDL_Operation - 205.32deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= 23.7deg ) THEN Unlimited_Pilot_SHDL_Command = (11.6 * Pilot_SHDL_Operation - 110.12deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= 13.3deg ) THEN Unlimited_Pilot_SHDL_Command = (9.2 * Pilot_SHDL_Operation - 51.64deg ) * (1/10.4) ELSE IF( Pilot_SHDL_Operation >= 3deg ) THEN Unlimited_Pilot_SHDL_Command = (Pilot_SHDL_Operation - 3deg ) * (6.8 / 10.3) ELSE IF( Pilot_SHDL_Operation >= -3deg ) THEN Unlimited_Pilot_SHDL_Command = 0deg ELSE IF( Pilot_SHDL_Operation >= -13.3deg ) THEN Unlimited_Pilot_SHDL_Command = (Pilot_SHDL_Operation + 3deg ) * (6.8 / 10.3) ELSE IF( Pilot_SHDL_Operation >= -23.7deg ) THEN Unlimited_Pilot_SHDL_Command = (9.2 * Pilot_SHDL_Operation + 51.64deg ) * (1/10.4) ELSE IF( Pilot_SHDL_Operation >= -34deg ) THEN Unlimited_Pilot_SHDL_Command = (11.6 * Pilot_SHDL_Operation + 110.12deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= -44.3deg ) THEN Unlimited_Pilot_SHDL_Command = (14.4 * Pilot_SHDL_Operation + 205.32deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= -54.7deg ) THEN Unlimited_Pilot_SHDL_Command = (16.6 * Pilot_SHDL_Operation + 298.58deg ) * (1/10.4) ELSE Unlimited_Pilot_SHDL_Command = (26.4 * Pilot_SHDL_Operation + 840.5deg ) * (1/10.3) END IF Where Pilot_SHDL_Operation is the value calculated in MRJ_SCU_STC_SRS_68 and Major cycle is defined in Table 6.2-1 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Co_Pilot SHDL Steering Demand

| MRJ_SCU_STC_SRS_102 | The STC Program shall convert the Co_Pilot_SHDL_Operation into a steering angle demand Unlimited_Co_Pilot_SHDL_Command in every Major cycle using the following logic: IF( Co_Pilot_SHDL_Operation >= 54.7deg ) THEN Unlimited_Co_Pilot_SHDL_Command = (26.4 * Co_Pilot_SHDL_Operation - 840.5deg ) * (1/10.3) ELSE IF( Co_Pilot_SHDL_Operation >= 44.3deg ) THEN Unlimited_Co_Pilot_SHDL_Command = (16.6 * Co_Pilot_SHDL_Operation - 298.58deg ) * (1/10.4) ELSE IF( Co_Pilot_SHDL_Operation >= 34deg ) THEN Unlimited_Co_Pilot_SHDL_Command = (14.4 * Co_Pilot_SHDL_Operation - 205.32deg ) * (1/10.3) ELSE IF( Co_Pilot_SHDL_Operation >= 23.7deg ) THEN Unlimited_Co_Pilot_SHDL_Command = (11.6 * Co_Pilot_SHDL_Operation - 110.12deg ) * (1/10.3) ELSE IF( Co_Pilot_SHDL_Operation >= 13.3deg ) THEN Unlimited_Co_Pilot_SHDL_Command = (9.2 * Co_Pilot_SHDL_Operation - 51.64deg ) * (1/10.4) ELSE IF( Co_Pilot_SHDL_Operation >= 3deg ) THEN Unlimited_Co_Pilot_SHDL_Command = (Co_Pilot_SHDL_Operation - 3deg ) * (6.8 / 10.3) ELSE IF( Co_Pilot_SHDL_Operation >= -3deg ) THEN Unlimited_Co_Pilot_SHDL_Command = 0deg ELSE IF( Co_Pilot_SHDL_Operation >= -13.3deg ) THEN Unlimited_Co_Pilot_SHDL_Command = (Co_Pilot_SHDL_Operation + 3deg ) * (6.8 / 10.3) ELSE IF( Co_Pilot_SHDL_Operation >= -23.7deg ) THEN Unlimited_Co_Pilot_SHDL_Command = (9.2 * Co_Pilot_SHDL_Operation + 51.64deg ) * (1/10.4) ELSE IF( Co_Pilot_SHDL_Operation >= -34deg ) THEN Unlimited_Co_Pilot_SHDL_Command = (11.6 * Co_Pilot_SHDL_Operation + 110.12deg ) * (1/10.3) ELSE IF( Co_Pilot_SHDL_Operation >= -44.3deg ) THEN Unlimited_Co_Pilot_SHDL_Command = (14.4 * Co_Pilot_SHDL_Operation + 205.32deg ) * (1/10.3) ELSE IF( Co_Pilot_SHDL_Operation >= -54.7deg ) THEN Unlimited_Co_Pilot_SHDL_Command = (16.6 * Co_Pilot_SHDL_Operation + 298.58deg ) * (1/10.4) ELSE Unlimited_Co_Pilot_SHDL_Command = (26.4 * Co_Pilot_SHDL_Operation + 840.5deg ) * (1/10.3) END IF Where Co_Pilot_SHDL_Operation is defined in MRJ_SCU_STC_SRS_86 and Major cycle is defined in Table 6.2-1 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### SHDL Command Calculation and Limiting

| MRJ_SCU_STC_SRS_165 | The STC Program shall calculate a maximum allowed value SHDL_Command_Limit for the Pilot and Co_Pilot SHDL Steering Demand as a function of Airspeed in every Major cycle using the following logic: IF( Airspeed >= 80 kts ) THEN SHDL_Command_Limit = 0 ELSEIF( Airspeed >= 20 kts ) SHDL_Command_Limit = (80 - Airspeed) * 17 / 12 ELSE SHDL_Command_Limit = 85 deg ENDIF Where Airspeed is defined in MRJ_SCU_STC_SRS_146 and Major Cycle is defined in Table 6.2-1 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_101 | The STC Program shall generate a limited Pilot SHDL Steering Demand in every Major cycle using the following logic: IF( Unlimited_Pilot_SHDL_Command > SHDL_Command_Limit ) THEN Pilot_SHDL_Steering_Demand = SHDL_Command_Limit ELSIF ( Unlimited_Pilot_SHDL_Command < (-1) * SHDL_Command_Limit ) Pilot_SHDL_Steering_Demand = (-1) * SHDL_Command_Limit ELSE Pilot_SHDL_Steering_Demand = Unlimited_Pilot_SHDL_Command ENDIF Where Unlimited_Pilot_SHDL_Command is defined in MRJ_SCU_STC_SRS_99 SHDL_Command_Limit is defined in MRJ_SCU_STC_SRS_165 and Major Cycle is defined in Table 6.2-1 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_104 | The STC Program shall generate a limited Co_Pilot SHDL Steering Demand in every Major cycle using the following logic IF( Co_Pilot_SHDL_Fitted ) THEN IF( Unlimited_Co_Pilot_SHDL_Command > SHDL_Command_Limit ) THEN Co_Pilot_SHDL_Steering_Demand = SHDL_Command_Limit ELSIF ( Unlimited_Co_Pilot_SHDL_Command < (-1) * SHDL_Command_Limit ) Co_Pilot_SHDL_Steering_Demand = (-1) * SHDL_Command_Limit ELSE Co_Pilot_SHDL_Steering_Demand = Unlimited_Co_Pilot_SHDL_Command ENDIF ELSE Co_Pilot_SHDL_Steering_Demand = 0 deg ENDIF Where Unlimited_Co_Pilot_SHDL_Command is the value defined in MRJ_SCU_STC_SRS_102 SHDL_Command_Limit is defined in MRJ_SCU_STC_SRS_165 Co_Pilot_SHDL_Fitted is defined in MRJ_SCU_STC_SRS_79 and Major Cycle is defined in Table 6.2-1 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Rudder Command Calculation

The STC Program  will convert the Rudder position and Airspeed into a steering angle demand Rudder_Steering_Demand in every Major cycle, to provide a maximum contribution to steering demand angle of 8deg.

Where Airspeed is defined in MRJ_SCU_STC_SRS_146

and Rudder_Pedal_Operation is defined in MRJ_SCU_STC_SRS_154

and Major cycle is defined in Table 6.2-1

| MRJ_SCU_STC_SRS_791 | If the Rudder Pedal Operation is within 5% of centre the STC Program shall convert the Rudder position into a steering angle demand Rudder_Steering_Demand of 0deg. Where 100% Rudder Pedal Operation is at 76.18mm deflection. 95mm deflection equates to indicated position of 2048 (after zero-position offset has been removed). So: - 100% deflection equates to 76.18 / 95 x 2048 = 1642.280 - 5% deflection equates to 0.05 x 1642.280 = 82.1 and Rudder Pedal Operation is defined in MRJ_SCU_STC_SRS_154 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_792 | If the Airspeed is >= 105kts and the Rudder Pedal Operation is not within 5% of centre the STC Program shall convert the Rudder position into a steering angle demand Rudder_Steering_Demand using the following logic: Rudder Pedal Operation := Rudder Pedal Operation - 2048 (* To centralize around zero *) Rudder Pedal Operation := Rudder Pedal Operation * -1 (* To account for ARINC signal Rudder right/left pedal*) IF (Rudder Pedal Operation > 82) (*Rudder Pedal Operation > 5% *) THEN Rudder_Steering_Demand := ( Rudder Pedal Operation - 82.1) / 1560.2 ELSE (* Rudder Pedal Operation < -5% *) Rudder_Steering_Demand := ( Rudder Pedal Operation + 82.1) / 1560.2 END IF Where 100% Rudder Pedal Operation is at 76.18mm deflection. 95mm deflection equates to indicated position of 2048 (after zero-position offset has been removed). So: - 100% deflection equates to 76.18 / 95 x 2048 = 1642.280 - 5% deflection equates to 0.05 x 1642.280 = 82.1 - 95% deflection equates to 0.95 x 1642.280 = 1560.2 and Rudder Pedal Operation is defined in MRJ_SCU_STC_SRS_154 and Airspeed is defined in MRJ_SCU_STC_SRS_146 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_793 | If the Airspeed is >= 60kts and Airspeed < 105kts and the Rudder Pedal Operation is not within 5% of centre the STC Program shall convert the Rudder position into a steering angle demand Rudder_Steering_Demand using the following logic: Rudder Pedal Operation := Rudder Pedal Operation - 2048 (* To centralize around zero*) Rudder Pedal Operation := Rudder Pedal Operation * -1 (* To account for ARINC signal Rudder right/left pedal*) IF (Rudder Pedal Operation > 82) (*Rudder Pedal Operation > 5% *) THEN Rudder_Steering_Demand := (1/1560.2) * {[(105kts - Airspeed) * (7 * Rudder Pedal Operation - 574.7) / 45)] + (Rudder Pedal Operation - 82.1)} ELSE (* Rudder Pedal Operation < -5% *) Rudder_Steering_Demand := (1/1560.2) * {[(105kts - Airspeed) * (7 * Rudder Pedal Operation + 574.7) / 45] + (Rudder Pedal Operation + 82.1)} END IF Where 100% Rudder Pedal Operation is at 76.18mm deflection. 95mm deflection equates to indicated position of 2048 (after zero-position offset has been removed). So: - 100% deflection equates to 76.18 / 95 x 2048 = 1642.280 - 5% deflection equates to 0.05 x 1642.280 = 82.1 - 35% deflection equates to 0.35 x 1642.280 = 574.7 - 95% deflection equates to 0.95 x 1642.280 = 1560.2 and Rudder Pedal Operation is defined in MRJ_SCU_STC_SRS_154 and Airspeed is defined in MRJ_SCU_STC_SRS_146 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_794 | If the Airspeed < 60kts and the Rudder Pedal Operation is not within 5% of centre the STC Program shall convert the Rudder position into a steering angle demand Rudder_Steering_Demand using the following logic: Rudder Pedal Operation := Rudder Pedal Operation - 2048 (* To centralize around zero *) Rudder Pedal Operation := Rudder Pedal Operation * -1 (* To account for ARINC signal Rudder right/left pedal*) IF (Rudder Pedal Operation > 82) //(*Rudder Pedal Operation > 5% *) THEN Rudder_Steering_Demand := 8deg * ( Rudder Pedal Operation - 82.1) / 1560.2 ELSE //(* Rudder Pedal Operation < -5% *) Rudder_Steering_Demand := 8deg * ( Rudder Pedal Operation + 82.1) / 1560.2 END IF Where 100% Rudder Pedal Operation is at 76.18mm deflection. 95mm deflection equates to indicated position of 2048 (after zero-position offset has been removed). So: - 100% deflection equates to 76.18 / 95 x 2048 = 1642.280 - 5% deflection equates to 0.05 x 1642.280 = 82.1 - 95% deflection equates to 0.95 x 1642.280 = 1560.2 and Rudder Pedal Operation is defined in MRJ_SCU_STC_SRS_154 and Airspeed is defined in MRJ_SCU_STC_SRS_146 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Steering Demand Summation

| MRJ_SCU_STC_SRS_174 | The STC Program shall combine the Pilot_SHDL_Steering_Demand, the Co_Pilot_SHDL_Steering_Demand and Rudder_Steering_Demand in every Major cycle according to the following logic IF( Pilot_Engaged AND Co_Pilot_Engaged) THEN Steering_Demand = Pilot_SHDL_Steering_Demand + Co_Pilot_SHDL_Steering_Demand + Rudder_Steering_Demand ELSE Steering_Demand = Pilot_SHDL_Steering_Demand + Co_Pilot_SHDL_Steering_Demand ENDIF Where Pilot_SHDL_Steering_Demand is defined in MRJ_SCU_STC_SRS_101 Co_Pilot_SHDL_Steering_Demand is defined in MRJ_SCU_STC_SRS_104 Rudder_Steering_Demand is defined in section 6.6.1.4 Rudder Command Calculation. Pilot_Engage is defined in MRJ_SCU_STC_SRS_186 Co_Pilot_Engage is defined in MRJ_SCU_STC_SRS_187. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Steering Demand Limiting

| MRJ_SCU_STC_SRS_175 | The STC Program shall calculate a maximum allowed value Steering_Demand_Final_Limit for the total steering demand as a function of Airspeed in every Major cycle using the following logic: IF( Airspeed >= 105 kts ) THEN Steering_Demand_Final_Limit = 1 deg ELSEIF( Airspeed >= 80 kts ) Steering_Demand_Final_Limit = ( -0.1556 * Airspeed ) + 17.338 ELSEIF( Airspeed >= 20 kts ) Steering_Demand_Final_Limit = ( -1.33517 * Airspeed ) + 111.70333 ELSE Steering_Demand_Final_Limit = 85 deg ENDIF Where Airspeed is defined in MRJ_SCU_STC_SRS_146 and Major cycle is defined in Table 6.2-1 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_176 | The STC Program shall generate a limited Steering Demand in every Major cycle using the following logic IF( Steering_Demand > Steering_Demand_Final_Limit ) THEN Limited_Steering_Demand = Steering_Demand_Final_Limit ELSIF ( Steering_Demand < (-1) * Steering_Demand_Final_Limit ) Limited_Steering_Demand = (-1) * Steering_Demand_Final_Limit ELSE Limited_Steering_Demand = Steering_Demand ENDIF Where Steering_Demand is defined in MRJ_SCU_STC_SRS_174 Steering_Demand_Final_Limit is defined in MRJ_SCU_STC_SRS_175 and Major cycle is defined in Table 6.2-1 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_228 | The STC Program shall generate a validity flag for the limited steering demand in every Major cycle using the following logic IF Airspeed_Valid AND (Rudder_Valid OR Pilot_SHDL_Operation_Valid OR Co-Pilot_SHDL_Operation_Valid) THEN Limited_Steering_Demand_Valid = TRUE ELSE Limited_Steering_Demand_Valid = FALSE ENDIF Where Airspeed_Valid is defined in MRJ_SCU_STC_SRS_146 Rudder_Valid is defined in MRJ_SCU_STC_SRS_154 Pilot_SHDL_Operation_Valid is defined in MRJ_SCU_STC_SRS_68 Co-Pilot_SHDL_Operation_Valid is defined in MRJ_SCU_STC_SRS_86 and Major cycle is defined in Table 6.2-1 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-15-166 |
|  | Verification Method : |

### Touchdown Protection

#### WOW Debouncing

| MRJ_SCU_STC_SRS_180 | The STC Program shall debounce the NLG_WOW1 signal using the following algorithm to generate a filtered version of the signal IF (NLG_WOW1 has been TRUE for the last 1.5s) THEN Debounced_NLG_WOW1 = TRUE ELSE IF (NLG_WOW1 has been FALSE for the last 200ms) THEN Debounced_NLG_WOW1 = FALSE ELSE Debounced_NLG_WOW1 remains unchanged, with an initial value of FALSE ENDIF Where NLG_WOW1 is the value calculated in MRJ_SCU_STC_SRS_139 quickly |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_181 | The STC Program shall debounce the NLG_WOW2 signal using the following algorithm to generate a filtered version of the signal IF (NLG_WOW2 has been TRUE for the last 1.5s) THEN Debounced_NLG_WOW2 = TRUE ELSE IF (NLG_WOW2 has been FALSE for the last 200ms) THEN Debounced_NLG_WOW2 = FALSE ELSE Debounced_NLG_WOW2 remains unchanged, with an initial value of FALSE ENDIF Where NLG_WOW2 is the value calculated in MRJ_SCU_STC_SRS_142 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_632 | The STC Program shall combine NLG_WOW1 and NLG_WOW2 signal using the following algorithm: IF ( Debounced_NLG_WOW1 ) AND (Debounced_NLG_WOW2 ) THEN NLG_WOW = TRUE ELSE NLG_WOW = FALSE ENDIF Where Debounced_NLG_WOW1 is the value calculated in MRJ_SCU_STC_SRS_180 and Debounced_NLG_WOW2 is the value calculated in MRJ_SCU_STC_SRS_181 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Touchdown Protection Selection

| MRJ_SCU_STC_SRS_182 | The STC Program shall recalculate the Touchdown Protection Active state in every Major cycle according to the following logic IF ( Touchdown_Protection_Active ) THEN IF ( NLG_WOW AND( Airspeed < 109 kts ) ) OR ( Airspeed < 94 kts ) THEN Touchdown_Protection_Active = FALSE ENDIF ELSE IF ( Airspeed > 119 kts ) THEN Touchdown_Protection_Active = TRUE ENDIF ENDIF Where NLG_WOW is the value calculated in MRJ_SCU_STC_SRS_632 Airspeed is the value calculated in MRJ_SCU_STC_SRS_146 and Major cycle is defined in Table 6.2-1 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_183 | The STC Program shall initialise the Touchdown_Protection_Active state to FALSE at startup of the SCU |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

### Actual Steering Angle Validation Logic

| MRJ_SCU_STC_SRS_232 | The STC Program shall update a SACT_Position_valid status in every Major cycle based on the internal SW status as detailed in the following logic:- IF((LVDT Double Loss Monitor status) OR (LVDT No Angle Found Fault)) THEN SACT_Position_Valid = FALSE ELSE SACT_Position_Valid = TRUE ENDIF Where LVDT Double Loss Monitor status is defined in MRJ_SCU_STC_SRS_1245 LVDT No Angle Found Fault is defined in MRJ_SCU_STC_SRS_1240 Major cycle is defined in Table 6.2-1 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-15-166 |
|  | Verification Method : |

### Loss Of Steering Detection

| MRJ_SCU_STC_SRS_184 | The STC Program shall update a Loss_Of_Steering status based on the internal SW status as detailed in the following logic:- Loss_Of_Steering = The STC Program is in Failsafe Level 1 Where Failsafe Level 1 is defined in MRJ_SCU_STC_SRS_585 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

### Steering Control Enable

| MRJ_SCU_STC_SRS_185 | The STC Program shall generate a Steering_Control_Enabled status as detailed in the following logic :- FSLevel_4 = Touchdown_Protection_Active AND SOV Status Monitor IF( Towing_Disengage OR Touchdown_Protection_Active OR Loss_Of_Steering ) THEN Steering_Control_Enabled = FALSE ELSE Steering_Control_Enabled = TRUE ENDIF Where Towing_Disengage is the value defined in MRJ_SCU_STC_SRS_508 Touchdown_Protection_Active is the value defined in MRJ_SCU_STC_SRS_182 SOV Status Monitor is defined in MRJ_SCU_STC_SRS_1197 and Loss_Of_Steering is the value defined in MRJ_SCU_STC_SRS_184 Note: FSLevel_4 is used for ARINC Label 106 bit 28. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

### Steering Control Loop

| MRJ_SCU_STC_SRS_192 | The STC Program shall calculate an error signal Steering_Error using the following algorithm Steering_Error = ( Limited_Steering_Demand - SACT_Position ) Where Limited_Steering_Demand is the value calculated in MRJ_SCU_STC_SRS_176 and SACT_Position is the value calculated in MRJ_SCU_STC_SRS_1240 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1114 | The STC Program shall limit the Steering_Error calculated in MRJ_SCU_STC_SRS_192 to +/- 8.25mA. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_196 | The STC Program shall reset the PID controller internal variables and set the EHSV_Demand to 0mA if the following condition is TRUE: Towing_Disengage OR Loss_of_Steering OR (Touchdown_Protection_Active AND NOT SOV Status Monitor) Where Towing_Disengage is the value defined in MRJ_SCU_STC_SRS_508 Touchdown_Protection_Active is the value defined in MRJ_SCU_STC_SRS_182 SOV Status Monitor is defined in MRJ_SCU_STC_SRS_1197 and Loss_Of_Steering is the value defined in MRJ_SCU_STC_SRS_184 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_193 | If the following condition is TRUE then the STC Program shall execute a PID controller with a 5.12ms refresh rate that shall act to minimise the Steering_Error calculated in MRJ_SCU_STC_SRS_192 by generating an output EHSV_Demand: NOT Towing_Disengage AND NOT Loss_of_Steering AND (NOT Touchdown_Protection_Active OR SOV Status Monitor) |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_197 | The STC Program shall generate an EHSV demand signal using the following PID Controller algorithm: Integral = (Integral + Steering_Error) range limited to +-KIntegral_Limit Differential = (Steering_Error - previous Steering_Error) range limited to +-KDifferential_Limit EHSV_Demand = (Kp * Steering_Error + Ki * Integral + Kd * Differential) range limited to +- K_EHSV_Demand_Limit Where: KIntegral_Limit = 0 deg KDifferential_Limit = 0 deg K_EHSV_Demand_Limit = 8.25 mA Ki = 0 mA/deg Kd = 0 mA/deg Kp = 2.67 mA/deg |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

## Output Processing

### Analogue Output Processing

#### EHSV Output Control

| MRJ_SCU_STC_SRS_274 | The STC Program shall convert the EHSV_Demand (as defined in MRJ_SCU_STC_SRS_193 ) into a PWM mark/space ratio using the formula: PWM Duty Cycle = 0.5 + EHSV_Demand [Amps] * 46.9317 [1/Amps] with minimum resolution of 12 bits. (Note that the EHSV_Demand is range limited and hence the PWM Duty Cycle cannot be outside the range 0 to 1) |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_269 | The STC Program shall update the mark/space ratio of the EHSV control PWM output GPIOB3 by writing (5000 * PWM Duty Cycle) to EV B Register CMPR5. (5000 count at 150MHz => 30kHz signal, which is set up by the initialisation of EV B register T3PR) |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_963 | The STC Program shall initialise the PWM Duty Cycle and the EHSV control PWM output GPIOB3 to 50% by writing the value 2500 to EV B Register CMPR5 . |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

#### LVDT / RVDT Excitation Control

| MRJ_SCU_STC_SRS_270 | The STC Program shall update the mark/space ratio of the LVDT/RVDT excitation PWM output GPIOA0 at a rate of 200KHz (5us intervals) using the waveform data held in a lookup table as defined in MRJ_SCU_STC_SRS_272 and in accordance with the HW/SW interface specification [ref. B4] |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_940 | The STC Program shall not use sensor “VA” or “VB” measurement results for monitoring and steering purposes until at least 150ms after the release of software reset. This allows for the other lane to start generating its excitation signal and sensor settling time. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

### Discrete Output Processing

#### SOV Hi-Side Switch Control

| MRJ_SCU_STC_SRS_926 | The STC Program shall control the Hi-Side SOV in accordance with the following hardware conditioning: Table 6.7-1 SOV GPIO |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_631 | The STC Program shall set SOV_Out to Not Energised (GPIO = 1) at least until the end of P-BIT during start-up. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_194 | When Steering_Control_Enabled is FALSE, the STC Program shall de-energise the high side of the Shutoff valve to remove hydraulic pressure to allow the system to enter free-castor mode. Where Steering_Control_Enabled is the value calculated in MRJ_SCU_STC_SRS_185 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_195 | When Steering_Control_Enabled is TRUE, the STC Program shall energise the high side of the Shutoff valve to apply hydraulic pressure to allow the EHSV to move the steering actuator. Where Steering_Control_Enabled is the value calculated in MRJ_SCU_STC_SRS_185 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Deleted

#### EHSV Enable Control

| MRJ_SCU_STC_SRS_275 | Before initialising the EHSV_PWM output, the STC Program shall initialise the EHSV1_SD Output Control and EHSV2_SD Output Control to Disabled in accordance with the following hardware conditioning, and maintain them in this state for at least 10ms after the EHSV_PWM has been initialised and is running: Table 6.7-3 EHSV GPIOs |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_784 | The STC Program shall disable the EHSV1_SD Output Control and EHSV2_SD Output Control and set EHSV_Disabled to TRUE if Towing_Disengage OR Loss_of_Steering OR (Touchdown_Protection_Active AND NOT SOV Status Monitor) and otherwise apply the following logic table: Table 6.7-4 Selection of EHSV GPIOs Where Towing_Disengage is the value defined in MRJ_SCU_STC_SRS_508 Touchdown_Protection_Active is the value defined in MRJ_SCU_STC_SRS_182 SOV Status Monitor is defined in MRJ_SCU_STC_SRS_1197 and Loss_Of_Steering is the value defined in MRJ_SCU_STC_SRS_184 EHSV1_Monitor_Status is defined in MRJ_SCU_STC_SRS_606 EHSV2_Monitor_Status is defined in MRJ_SCU_STC_SRS_607 EHSV_Selection is defined in MRJ_SCU_STC_SRS_869 and MRJ_SCU_STC_SRS_870 EHSV1_SD Output Control and EHSV2_SD Output Control are defined in MRJ_SCU_STC_SRS_275 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_962 | The STC Program shall ensure that the EHSV1_SD Output Control and EHSV2_SD Output Control are set to coherent states within 1ms of each other (i.e. a switchover needs to be completed within 1ms). Note: The latching of EHSV1_Monitor_Status and EHSV2_Monitor_Status under fault conditions prevents repeated switchovers of the EHSV Drivers. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1183 | During start-up (including P-BIT), the STC Program shall disable i.e. turn off the EHSV drive. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

### ARINC 429 Output Processing

| MRJ_SCU_STC_SRS_215 | The STC Program shall transmit the ARINC 429 labels listed in the following table at their given update rate: Table 6.7-5 ARINC Tx Rate Note: The register mapping from the External Interface Zone 1 to the DEI-1085 devices is defined in MRJ_SCU_STC_SRS_616. The mapping from ARINC Transceivers to ARINC 429 Bus is defined in 6.5-10 DEI-1085 Bus Mapping. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-15-166 |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1190 | For ARINC label transmission the STC Program shall set the EN_TX signal (GPIO B10) low (0) when loading ARINC data to be transmitted to the DEI-1085 transmit buffer via SPI bus and set the EN_TX signal high (1) for ARINC data to be transmitted by the DEI-1085 device, in accordance with Ref. B4 and Ref. C11. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_956 | The SSM of ARINC 429 data transmitted by the SCU shall be set to 'Normal Operation' Justification: 'Functional Test' setting is not being used because there are no requirements for functional tests to be performed. ‘No Computed Data’ setting is not being used because no labels are being transmitted before calculating valid data. ‘Failure Warning’ setting is not being used because are no requirements for Failure Warning condition to be performed |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1184 | During start-up (including P-BIT), the STC Program shall not transmit ARINC labels. Justification: To avoid spurious fault indications during start-up. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### Label 104 (Failure Information 1) Output Processing

| MRJ_SCU_STC_SRS_772 | The STC Program shall generate Label 104 (Failure Information 1) as follows: Table 6.7-6 Label 104 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-15-166 |
|  | Verification Method : |

#### Label 105 (Failure Information 2) Output Processing

| MRJ_SCU_STC_SRS_220 | The STC Program shall generate Label 105 (Failure Information 2) as follows: Table 6.7-7 Label 105 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-15-166 |
|  | Verification Method : |

#### Label 106 (Control Status Information) Output Processing

| MRJ_SCU_STC_SRS_221 | The STC Program shall generate Label 106 (Control Status Information) as follows Table 6.7-8 Label 106 (Part 1) |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-15-166 |
|  | Verification Method : |

Table 6.7-8 Label 106 (Part 2)

#### Label 313 (Actual Steering Angle) Output Processing

| MRJ_SCU_STC_SRS_222 | The STC Program shall generate Label 313 (Actual Steering Angle) as follows Table 6.7-9 Label 313 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-15-166 |
|  | Verification Method : |

#### Label 334 (Rudder Pedal Operation) Output Processing

| MRJ_SCU_STC_SRS_774 | The STC Program shall generate Label 334 (Rudder Pedal Operation) as follows Table 6.7-10 Label 334 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-15-166 |
|  | Verification Method : |

#### Label 335 (Target Steering Angle) Output Processing

| MRJ_SCU_STC_SRS_223 | The STC Program shall generate Label 335 (Target Steering Angle) as follows Table 6.7-11 Label 335 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-15-166 |
|  | Verification Method : |

#### Label 336 (Pilot SHDL Operation Angle) Output Processing

| MRJ_SCU_STC_SRS_779 | The STC Program shall generate Label 336 (Pilot SHDL Operation Angle) as follows Table 6.7-12 Label 336 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-15-166 |
|  | Verification Method : |

#### Label 337 (Co_Pilot SHDL Operation Angle) Output Processing

| MRJ_SCU_STC_SRS_781 | The STC Program shall generate Label 337 (Co_Pilot SHDL Operation Angle) as follows Table 6.7-13 Label 337 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-15-166 |
|  | Verification Method : |

### McBSP SPI Output Processing

| MRJ_SCU_STC_SRS_1152 | For interfacing with the NVM device, the STC Program shall ensure NVM_SPI_nEN GPIOF9 and MCBSP_nCS GPIOF10 are low at least 20 ns before the first transition of the SPI Clock. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1153 | For interfacing with the NVM device, the STC Program shall ensure NVM_SPI_nEN GPIOF9 and MCBSP_nCS GPIOF10 are high no sooner than 500 ns after the last transition of the SPI Clock. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1154 | For interfacing with the NVM device, the STC Program shall ensure NVM_SPI_nEN GPIOF9 and MCBSP_nCS GPIOF10 are high for at least 100 ns between each SPI transfer cycle. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1155 | For interfacing with the NVM device, the STC Program shall ensure RUDDER1_ SPI_nEN GPIOA4 and RUDDER2_SPI_nEN GPIOA5 signals are set to high at least 20 ns before either NVM_SPI_nEN or MCBSP_nCS are set to low. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1157 | For interfacing with the Rudder Analogue devices, the STC Program shall ensure MCBSP_nCS and RUDDER1_ SPI_nEN GPIOA4 and RUDDER2_SPI_nEN GPIOA5 are low respectively at least 20 ns before the first transition of the SPI Clock. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1158 | For interfacing with the Rudder Analogue devices, the STC Program shall ensure ensure RUDDER1_ SPI_nEN GPIOA4 or RUDDER2_SPI_nEN GPIOA5 signal respectively and MCBSP_nCS are high no sooner than 500 ns after the last transition of the SPI Clock. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1159 | For interfacing with the Rudder Analogue devices, the STC Program shall ensure RUDDER1_ SPI_nEN GPIOA4 and RUDDER2_SPI_nEN GPIOA5 signals respectively and the MCBSP_nCS GPIOF10 are high for at least 200 ns between each SPI transfer cycle. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1160 | For interfacing with the Rudder Analogue devices, the STC Program shall ensure NVM_SPI_nEN GPIOF9 signal and the other RUDDERx_SPI_nEN signal GPIOA4 or GPIOA5 respectively are set to high at least 20 ns before either RUDDERx_SPI_nEN or MCBSP_nCS are set to low. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

## Internal Interlane Interface Communication

| MRJ_SCU_STC_SRS_643 | The STC Program shall utilize SPI-A Bus for communication with the Monitor Lane. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

The SPI Bus speed will be 1.5 MHz.

Note: This is defined by the Register Initialisation requirements.

Interlane messages can, in principle, be of any length. The constraints are that the SPI servicing be done once every Major Cycle and that the buffer size is 16 (16-bit) words. This means that messages need to be fragmented such that each fragment has a maximum size of 8 words (to ensure that data is not lost). Also, the Control Lane is configured to be the Master and the Monitor Lane is configured to be the Slave. Since the data received from the Slave is clocked by data transmitted from the Master, the length of a Control Lane message has to be greater than or equal to the length of a Monitor Lane message to ensure that the Monitor Lane transmit buffer does not overflow.

| MRJ_SCU_STC_SRS_651 | The STC Program shall apply the following message layout for data being transmitted and received: Figure 6.8-1 Internal Interlane Interface Message Layout With SoM: Start-of-Message = 0xFF F: First Fragment flag of a complete Interlane Message (set to 1 for first fragment, else set to 0) Frag: Fragment Counter (last fragment counter value = 0) Cntrl: Control Byte (XOR-byte) for data stream and checksum Len: Length of data payload in bytes Chksum_High/Low: 32-bit CRC over data payload Data1 .. Data4: Data Payload (max. 4 Words) Justification: Start-of-Message, First Fragment flag, Fragment Counter and Length increase data communication integrity (Control Byte and Checksum trace to SES). |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_653 | The 32-bit CRC checksum shall be generated over the raw Data Payload by applying the following algorithm: The Seed value is 0x746D7361. The CRC is initialised to the seed. Set Starting Address to beginning of Data Payload (Data1). Extend Data Payload buffer with dummy 16-bit word(s) set to 0 until 32-bit boundary is reached. Set Length to the number of 32-bit memory locations to perform the CRC algorithm. Perform the following loop Length times: Loop At Starting Address take in turn the 2nd byte, 1st byte, 4th byte and 3rd byte and perform the following: Read the current 8-bit data byte CRC = (CRC right shifted by 8 bits) XOR CRC_Table([CRC XOR current 8-bit data byte] AND 0x000000FF) Increment Starting Address to next 32-bit word in Data Payload. End Loop Note: The CRC_Table is given in MRJ_SCU_STC_SRS_688 of this document. Justification: The same CRC algorithm is being used for the 32-bit CRC ROM Failure Monitor. |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_652 | For data transmissions the Control Byte shall be generated by applying the following algorithm: a) Find a byte value which is not used in the Chksum field and Data Payload area. b) Perform an Exclusive-OR operation on this byte value. Note: this step is required to rule out the generation of a 0xFF value in XOR-operation on the data stream. c) Store the result of the previous operation as Control Byte. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_924 | The STC Program shall transmit an Internal Interlane Interface message fragment every Major cycle (padded out to 8 words with '0's to ensure that Monitor Lane messages cannot be lost). |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_934 | The STC Program shall initiate Inter-Lane transmissions by writing transmit data to the register SPITXBUF. Note: The STC is configured as SPI master during start-up. This means that the STC initiates data transmission. Justification: STC is 'Master' and SMB is 'Slave'; SPI Tx buffer has to be used to send data via SPI bus. |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_935 | The act of writing to SPITXBUF causes data from the Monitor Lane to be clocked into SPIRXBUF. The STC Program shall read data received from the Monitor Lane from SPIRXBUF. Note: The STC is configured as SPI master during start-up. This means that whenever the STC transmits data then it also receives data from the SMB, which is configured as SPI slave. Justification: SPI Rx buffer has to be used to receive data from SPI bus. |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_866 | For data transmissions the Fragment Counter shall be decremented for each fragment transmission, ending with the value 0 for the last fragment of a complete Interlane Message. Justification: Maintaining a fragment counter increases the data communication integrity. |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_654 | For data transmissions the Chksum field and all Data Payload bytes shall be Exclusive-ORed with the Control Byte. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_655 | For data reception the Chksum field and the Data Payload bytes shall be Exclusive-ORed with the Control Byte. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_656 | For data reception the 32-Bit CRC checksum shall be verified by generating the 32-bit CRC over the raw Data Payload as outlined in MRJ_SCU_STC_SRS_653. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_657 | For data reception the dataset shall be processed only in case of a checksum match. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_641 | The STC Program shall send the following message, as fragments, to the Monitor Lane via the Internal Interlane Interface: Table 6.8-1 CL-ML Interlane Message |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_932 | Complete received messages shall be reconstructed from message fragments (where applicable) and validated by ensuring that the correct number of fragments with valid fragment count are present, discarding any invalid messages. Justification: Maintaining a fragment counter increases the data communication integrity. |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_648 | The STC Program shall receive the following message from the Monitor Lane via the Internal Interlane Interface Table 6.8-2 ML-CL Interlane Message |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1148 | For interfacing with the interlane bus, the STC Program shall ensure ILC_nCS GPIO F3 is low at least 230 ns before the first transition of the SPI Clock. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1149 | For interfacing with the interlane bus, the STC Program shall ensure ILC_nCS GPIO F3 is high no sooner than 200 ns after the last transition of the SPI Clock. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1150 | For interfacing with the interlane bus, the STC Program shall ensure ILC_nCS GPIO F3 is high for at least 300 ns between each SPI transfer cycle. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

## Deleted

Note: This section was previously used for ARINC 604 Rigging requirements.

## Engineering Access

### STC CAN Bus Protocol (SBP)

The STC Program will utilise the CAN peripheral on the DSP for transmitting data following the CAN 2.0B standard.

The STC Program will configure the CAN bus with a speed of 500 kHz.

Note: this is being defined by the initialisation values for the CAN peripheral in MRJ_SCU_STC_SRS_517.

The STC Program will transmit CAN dataset bytes 0 to bytes 7 based on the following convention.

Note: the byte order is partially defined by the initialisation values for the CAN peripheral in MRJ_SCU_STC_SRS_517.

Figure 6.10-1 Engineering Interface Dataset

| MRJ_SCU_STC_SRS_667 | The STC Program shall apply the CAN Message ID definitions from the table below for message transmission. Table 6.10-1 CAN Message IDs |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : CAN ICD |
|  | Verification Method : |

If contention exists between the CPU and the eCAN controller for access to certain eCAN

register areas, a CPU read may erroneously read all zeros (0x00000000), and a CPU write

may erroneously fail to execute (TI document CAN Errata SPRZ193L).

| MRJ_SCU_STC_SRS_937 | For all CPU reads from the eCAM mailbox RAM area, if the read returns all zeros, the software shall discard the first read and use the result of a second read. Interrupts must be disabled during the consecutive CPU reads. Justification: Recommended errata work-around from TI. |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_938 | For all CPU writes to the LAM/MOTO/MOTS register area, the software shall write the data twice with a minimum of 4 CPU cycles in between writes. Interrupts must be disabled during the consecutive CPU writes. Justification: Recommended errata work-around from TI. |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

### CAN Bus Message Framing

| MRJ_SCU_STC_SRS_1097 | To support engineering development and testing the STC Program shall send the SBP_STATUS message according to the schedule provided in Table 6.10-1. Table 6.10-1: CAN Transmission Schedule |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : |
|  | Verification Method : |

### CAN Bus Message Definitions

#### SBP_STATUS ID 102 (Steering Status)

| MRJ_SCU_STC_SRS_1100 | The update rate for the Steering Status message from STC Program shall be 10ms ± 1ms. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1101 | The Steering Status message from STC Program shall comply with the format in the Table 6.10-2. Table 6.10-2: Steering Status |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### SBP_STATUS ID 103 (Rudder and Discrete)

| MRJ_SCU_STC_SRS_1104 | The update rate for the Rudder and Discrete Status message from STC Program shall be 10ms ± 1ms. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1105 | The Rudder and Discrete Status message from STC Program shall comply with the format in the Table 6.10-3. Table 6.10-3: Rudder and Discrete Status |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

#### SBP_STATUS ID 110 to 119 (Sensor #n Status)

| MRJ_SCU_STC_SRS_1107 | The update rate for the Sensor #n Status message for Sensor #n from STC Program shall be 51.2ms ± 1ms. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1103 | The Sensor #n Status message from STC Program shall comply with the format in the Table 6.10-4. Table 6.10-4: Sensor #n Status |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

## Implementation Constraints

### Software Development and Verification

The STC Program will be developed and verified to meet the objectives of DO-178B for Level A software.

The STC Program will be implemented in C and Assembler coding language for the SM320F2812PGFMEP DSP.
The assembler source code will be written in accordance with the conventions of the C language compiler for the target system, e.g. stack implementation, register usage and global variable naming.

### Memory

| MRJ_SCU_STC_SRS_498 | The STC Program should not utilise more than 70% of the available FLASH memory for the initial certification of the product. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_499 | The STC Program should not utilise more than 70% of the available RAM for the initial certification of the product. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1162 | The STC Program shall not use more than 70% of its allocated stack space. Justification: Stack margin of less than 70% ensures no stack overrun during runtime. |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

### Processor Throughput

| MRJ_SCU_STC_SRS_501 | The STC Program should not utilise more than 70% of the available processing throughput for the initial certification of the product. BIT processing during otherwise idle time will not be included in total processing time. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

### Additional Safety Related Requirements

| MRJ_SCU_STC_SRS_503 | The STC Program should be implemented in a way which protects data and code items located in RAM against Single Event Upsets by means of: - either refreshing data values continuously in a periodic manner - or verifying that the data value has not changed. Justification: For SEU mitigation RAM copies of variables and code will be compared by the SCU software with their flash or NVM counterparts if not refreshed during runtime operation. |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_729 | The STC Program shall use the -me compiler option for all code files, which run from flash and call functions (by use of BF or SBF) containing instructions that use program-memory addressing, where the BF or SBF instruction is within 8 words of the instruction using program-memory addressing. Instructions that use program-memory addressing include MAC/XMAC, DMAC/XMACD, QMACL, IMACL, PREAD/XPREAD and PWRITE/XPWRITE. This requirement only applies to code where the BF or SBF instruction is within 8 words of the instruction using program-memory addressing. Note: This requirement stems from a known DSP errata. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_723 | The STC Program shall not locate code within 8 words of the end of valid memory (flash and RAM). The valid code sections will be defined in the linker command file. Note: This requirement is related to the linker command file and is part of the protection mechanism against a known DSP errata. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_939 | The STC Program shall not use Flash/ROM memory area 0x3F7FF0 through 0x3F7FF7. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_730 | The STC Program shall be implemented in a way so that the McBSP FIFO is not being written to while reading from it. Note: This requirement stems from a known DSP errata. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_731 | The STC Program shall not read the following address locations: 0x7001, 0x7401, 0x7C01. Note: This requirement stems from a known DSP errata. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_733 | The ADC interface timing of the STC Program shall take into account one extra ADC clock period when reading/polling the ADC result register. Note: This requirement stems from a known DSP errata. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1144 | During accesses to the eCAN registers, the STC Program shall use the double read/double write functions as proposed by the TI Errata sheet [ref. C16]. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1208 | The STC Program shall not include an interrupt enable instruction immediately after a PIEACK clear instruction. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

## STC Constants Data

### SACT LVDT Ratio to Steering Actuator Position Lookup Table

| MRJ_SCU_STC_SRS_111 | The STC Program shall have a lookup table derived from the following LH LVDT Ratio-to-Angle Lookup Table to convert between LVDT Ratiometric Position and Actual Steering Angle ('Angle' [deg]). The ratio-to-angle lookup table is created by polynomial interpolation between the existing data points. The integer part of the Q0.14 ratio gives the lower index into the look-up and the fractional part is used to derive the weights of the two table entries that are linearly interpolated. The angles are in Q8.7 to cover the full ±140° range. left_lvdt_angle_lookup [1024] = { 3456, 3168, 3044, 2951, 2875, 2802, 2741, 2680, 2629, 2577, 2529, 2482, 2436, 2395, 2355, 2314, 2276, 2239, 2203, 2166, 2133, 2099, 2065, 2032, 2002, 1971, 1940, 1910, 1881, 1852, 1822, 1793, 1766, 1739, 1711, 1684, 1657, 1632, 1606, 1581, 1555, 1530, 1505, 1481, 1457, 1432, 1408, 1385, 1362, 1338, 1315, 1292, 1270, 1247, 1225, 1203, 1181, 1159, 1137, 1115, 1094, 1072, 1051, 1029, 1008, 988, 967, 947, 926, 906, 886, 866, 846, 825, 805, 785, 766, 746, 727, 708, 688, 669, 650, 630, 612, 593, 574, 555, 536, 517, 499, 480, 462, 444, 426, 407, 389, 371, 353, 335, 318, 300, 282, 264, 246, 229, 211, 194, 176, 159, 141, 124, 107, 89, 72, 55, 37, 20, 3, -14, -31, -48, -64, -81, -98, -115, -132, -148, -165, -181, -198, -215, -231, -248, -264, -281, -297, -313, -330, -346, -362, -378, -395, -411, -427, -443, -459, -475, -491, -507, -524, -539, -555, -571, -587, -603, -619, -635, -651, -667, -682, -698, -714, -730, -745, -761, -776, -792, -808, -823, -839, -854, -870, -885, -901, -916, -931, -947, -962, -978, -993, -1008, -1024, -1039, -1054, -1069, -1084, -1099, -1114, -1130, -1145, -1160, -1175, -1190, -1205, -1220, -1235, -1250, -1265, -1281, -1296, -1311, -1325, -1340, -1355, -1370, -1385, -1400, -1415, -1430, -1445, -1460, -1475, -1490, -1505, -1520, -1535, -1550, -1565, -1579, -1594, -1609, -1623, -1638, -1653, -1667, -1682, -1697, -1712, -1726, -1741, -1756, -1770, -1785, -1800, -1814, -1829, -1843, -1858, -1873, -1887, -1902, -1916, -1931, -1945, -1960, -1975, -1989, -2004, -2018, -2033, -2048, -2062, -2076, -2091, -2105, -2120, -2134, -2149, -2163, -2178, -2192, -2206, -2221, -2235, -2250, -2264, -2279, -2293, -2308, -2322, -2337, -2351, -2365, -2380, -2394, -2409, -2423, -2438, -2452, -2466, -2481, -2495, -2509, -2524, -2538, -2552, -2567, -2581, -2595, -2610, -2624, -2638, -2653, -2667, -2681, -2695, -2710, -2724, -2738, -2753, -2767, -2781, -2796, -2810, -2824, -2839, -2853, -2867, -2881, -2895, -2910, -2924, -2938, -2952, -2966, -2981, -2995, -3009, -3023, -3037, -3052, -3066, -3080, -3094, -3109, -3123, -3137, -3152, -3166, -3180, -3195, -3209, -3223, -3237, -3252, -3266, -3280, -3294, -3308, -3323, -3337, -3351, -3365, -3379, -3394, -3408, -3422, -3436, -3450, -3465, -3479, -3493, -3507, -3521, -3536, -3550, -3564, -3578, -3592, -3607, -3621, -3635, -3649, -3663, -3678, -3692, -3706, -3720, -3735, -3749, -3763, -3778, -3792, -3806, -3821, -3835, -3849, -3863, -3878, -3892, -3906, -3920, -3934, -3949, -3963, -3977, -3991, -4006, -4020, -4034, -4049, -4063, -4077, -4092, -4106, -4120, -4134, -4149, -4163, -4177, -4191, -4205, -4220, -4234, -4248, -4262, -4277, -4291, -4305, -4320, -4334, -4348, -4363, -4377, -4391, -4406, -4420, -4434, -4449, -4463, -4477, -4492, -4506, -4521, -4535, -4550, -4564, -4578, -4593, -4607, -4622, -4636, -4650, -4665, -4679, -4693, -4708, -4722, -4736, -4751, -4765, -4780, -4794, -4809, -4823, -4837, -4852, -4866, -4881, -4896, -4910, -4925, -4939, -4954, -4968, -4983, -4998, -5012, -5026, -5041, -5055, -5070, -5084, -5099, -5113, -5128, -5142, -5157, -5172, -5187, -5201, -5216, -5231, -5245, -5260, -5275, -5289, -5304, -5318, -5333, -5347, -5362, -5377, -5391, -5406, -5421, -5435, -5450, -5465, -5480, -5494, -5509, -5524, -5538, -5553, -5568, -5582, -5597, -5612, -5627, -5641, -5656, -5671, -5686, -5701, -5716, -5730, -5745, -5760, -5775, -5790, -5805, -5819, -5834, -5849, -5864, -5879, -5894, -5909, -5924, -5939, -5954, -5968, -5983, -5998, -6013, -6028, -6043, -6058, -6073, -6088, -6103, -6118, -6133, -6148, -6163, -6178, -6193, -6209, -6224, -6239, -6254, -6269, -6284, -6299, -6314, -6329, -6345, -6360, -6375, -6390, -6405, -6420, -6436, -6451, -6466, -6481, -6497, -6512, -6527, -6542, -6558, -6573, -6589, -6604, -6619, -6635, -6650, -6665, -6681, -6696, -6712, -6727, -6742, -6758, -6773, -6789, -6804, -6820, -6835, -6851, -6866, -6882, -6897, -6913, -6928, -6944, -6959, -6975, -6990, -7006, -7022, -7037, -7053, -7068, -7084, -7100, -7115, -7131, -7147, -7162, -7178, -7194, -7210, -7226, -7242, -7257, -7273, -7289, -7305, -7321, -7337, -7353, -7369, -7385, -7401, -7417, -7433, -7449, -7465, -7481, -7497, -7513, -7529, -7545, -7561, -7577, -7593, -7609, -7625, -7641, -7657, -7674, -7690, -7706, -7722, -7738, -7754, -7770, -7787, -7803, -7819, -7835, -7852, -7868, -7885, -7901, -7918, -7934, -7951, -7967, -7984, -8000, -8016, -8033, -8049, -8066, -8082, -8099, -8115, -8132, -8148, -8165, -8181, -8198, -8214, -8231, -8248, -8265, -8281, -8298, -8315, -8332, -8349, -8366, -8383, -8400, -8417, -8434, -8451, -8468, -8485, -8502, -8518, -8535, -8552, -8569, -8586, -8604, -8621, -8638, -8655, -8672, -8689, -8706, -8724, -8741, -8758, -8776, -8793, -8810, -8828, -8845, -8862, -8879, -8897, -8914, -8931, -8949, -8966, -8984, -9001, -9019, -9037, -9055, -9072, -9090, -9108, -9125, -9143, -9161, -9178, -9196, -9214, -9232, -9250, -9268, -9286, -9304, -9322, -9340, -9358, -9376, -9394, -9412, -9430, -9448, -9466, -9485, -9503, -9521, -9539, -9558, -9576, -9594, -9613, -9631, -9650, -9668, -9686, -9705, -9723, -9742, -9761, -9779, -9798, -9817, -9835, -9854, -9873, -9892, -9911, -9930, -9948, -9967, -9986, -10005, -10024, -10044, -10063, -10082, -10101, -10120, -10139, -10158, -10177, -10196, -10215, -10235, -10254, -10274, -10293, -10313, -10332, -10352, -10371, -10391, -10411, -10431, -10450, -10470, -10490, -10510, -10530, -10550, -10570, -10590, -10610, -10630, -10650, -10670, -10690, -10710, -10730, -10750, -10771, -10791, -10812, -10832, -10853, -10873, -10894, -10914, -10935, -10955, -10976, -10996, -11017, -11038, -11059, -11080, -11101, -11122, -11143, -11165, -11186, -11207, -11229, -11250, -11271, -11293, -11314, -11335, -11356, -11378, -11399, -11421, -11443, -11465, -11487, -11509, -11531, -11553, -11575, -11597, -11619, -11641, -11664, -11686, -11709, -11731, -11754, -11776, -11799, -11822, -11844, -11867, -11890, -11913, -11936, -11959, -11982, -12005, -12028, -12051, -12075, -12098, -12122, -12145, -12169, -12192, -12216, -12240, -12264, -12287, -12311, -12335, -12360, -12384, -12408, -12432, -12456, -12481, -12505, -12530, -12554, -12579, -12604, -12629, -12654, -12679, -12704, -12730, -12755, -12781, -12806, -12832, -12858, -12884, -12910, -12936, -12962, -12988, -13014, -13040, -13066, -13093, -13120, -13147, -13174, -13201, -13228, -13255, -13282, -13310, -13337, -13365, -13393, -13420, -13448, -13476, -13505, -13533, -13561, -13590, -13619, -13648, -13677, -13706, -13736, -13765, -13794, -13823, -13853, -13883, -13914, -13944, -13975, -14005, -14036, -14067, -14098, -14129, -14161, -14192, -14224, -14256, -14288, -14320, -14352, -14385, -14419, -14452, -14485, -14518, -14551, -14585, -14619, -14654, -14688, -14723, -14758, -14794, -14829, -14865, -14901, -14937, -14973, -15011, -15049, -15086, -15125, -15163, -15202, -15240, -15279, -15318, -15356, -15397, -15437, -15478, -15520, -15563, -15605, -15648, -15690, -15733, -15777, -15822, -15867, -15913, -15959, -16006, -16053, -16101, -16149, -16197, -16246, -16298, -16350, -16402, -16456, -16509, -16565, -16620, -16678, -16737, -16797, -16856, -16917, -16981, -17045, -17112, -17181, -17253, -17325, -17397, -17476, -17557, -17644, -17732, -17822, -17920 } |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

### PID Controller Tuning Constants

See MRJ_SCU_STC_SRS_197.

### LVDT/RVDT Excitation Lookup Table

| MRJ_SCU_STC_SRS_272 | The STC shall use the following circular table of 16 bit unsigned constants for updating the compare register associated with the PWM1 output, using the next value in the table with each compare register update. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

### 32-Bit CRC Table

| MRJ_SCU_STC_SRS_688 | The STC Program shall utilise the following CRC Table for any of its 32-Bit CRC calculations. Justification: This is the table utilised by the CRC utility used to embed the checksums during build process CRCTable[256] = {0x00000000UL, 0x77073096UL, 0xEE0E612CUL, 0x990951BAUL, 0x076DC419UL, 0x706AF48FUL, 0xE963A535UL, 0x9E6495A3UL, 0x0EDB8832UL, 0x79DCB8A4UL, 0xE0D5E91EUL, 0x97D2D988UL, 0x09B64C2BUL, 0x7EB17CBDUL, 0xE7B82D07UL, 0x90BF1D91UL, 0x1DB71064UL, 0x6AB020F2UL, 0xF3B97148UL, 0x84BE41DEUL, 0x1ADAD47DUL, 0x6DDDE4EBUL, 0xF4D4B551UL, 0x83D385C7UL, 0x136C9856UL, 0x646BA8C0UL, 0xFD62F97AUL, 0x8A65C9ECUL, 0x14015C4FUL, 0x63066CD9UL, 0xFA0F3D63UL, 0x8D080DF5UL, 0x3B6E20C8UL, 0x4C69105EUL, 0xD56041E4UL, 0xA2677172UL, 0x3C03E4D1UL, 0x4B04D447UL, 0xD20D85FDUL, 0xA50AB56BUL, 0x35B5A8FAUL, 0x42B2986CUL, 0xDBBBC9D6UL, 0xACBCF940UL, 0x32D86CE3UL, 0x45DF5C75UL, 0xDCD60DCFUL, 0xABD13D59UL, 0x26D930ACUL, 0x51DE003AUL, 0xC8D75180UL, 0xBFD06116UL, 0x21B4F4B5UL, 0x56B3C423UL, 0xCFBA9599UL, 0xB8BDA50FUL, 0x2802B89EUL, 0x5F058808UL, 0xC60CD9B2UL, 0xB10BE924UL, 0x2F6F7C87UL, 0x58684C11UL, 0xC1611DABUL, 0xB6662D3DUL, 0x76DC4190UL, 0x01DB7106UL, 0x98D220BCUL, 0xEFD5102AUL, 0x71B18589UL, 0x06B6B51FUL, 0x9FBFE4A5UL, 0xE8B8D433UL, 0x7807C9A2UL, 0x0F00F934UL, 0x9609A88EUL, 0xE10E9818UL, 0x7F6A0DBBUL, 0x086D3D2DUL, 0x91646C97UL, 0xE6635C01UL, 0x6B6B51F4UL, 0x1C6C6162UL, 0x856530D8UL, 0xF262004EUL, 0x6C0695EDUL, 0x1B01A57BUL, 0x8208F4C1UL, 0xF50FC457UL, 0x65B0D9C6UL, 0x12B7E950UL, 0x8BBEB8EAUL, 0xFCB9887CUL, 0x62DD1DDFUL, 0x15DA2D49UL, 0x8CD37CF3UL, 0xFBD44C65UL, 0x4DB26158UL, 0x3AB551CEUL, 0xA3BC0074UL, 0xD4BB30E2UL, 0x4ADFA541UL, 0x3DD895D7UL, 0xA4D1C46DUL, 0xD3D6F4FBUL, 0x4369E96AUL, 0x346ED9FCUL, 0xAD678846UL, 0xDA60B8D0UL, 0x44042D73UL, 0x33031DE5UL, 0xAA0A4C5FUL, 0xDD0D7CC9UL, 0x5005713CUL, 0x270241AAUL, 0xBE0B1010UL, 0xC90C2086UL, 0x5768B525UL, 0x206F85B3UL, 0xB966D409UL, 0xCE61E49FUL, 0x5EDEF90EUL, 0x29D9C998UL, 0xB0D09822UL, 0xC7D7A8B4UL, 0x59B33D17UL, 0x2EB40D81UL, 0xB7BD5C3BUL, 0xC0BA6CADUL, 0xEDB88320UL, 0x9ABFB3B6UL, 0x03B6E20CUL, 0x74B1D29AUL, 0xEAD54739UL, 0x9DD277AFUL, 0x04DB2615UL, 0x73DC1683UL, 0xE3630B12UL, 0x94643B84UL, 0x0D6D6A3EUL, 0x7A6A5AA8UL, 0xE40ECF0BUL, 0x9309FF9DUL, 0x0A00AE27UL, 0x7D079EB1UL, 0xF00F9344UL, 0x8708A3D2UL, 0x1E01F268UL, 0x6906C2FEUL, 0xF762575DUL, 0x806567CBUL, 0x196C3671UL, 0x6E6B06E7UL, 0xFED41B76UL, 0x89D32BE0UL, 0x10DA7A5AUL, 0x67DD4ACCUL, 0xF9B9DF6FUL, 0x8EBEEFF9UL, 0x17B7BE43UL, 0x60B08ED5UL, 0xD6D6A3E8UL, 0xA1D1937EUL, 0x38D8C2C4UL, 0x4FDFF252UL, 0xD1BB67F1UL, 0xA6BC5767UL, 0x3FB506DDUL, 0x48B2364BUL, 0xD80D2BDAUL, 0xAF0A1B4CUL, 0x36034AF6UL, 0x41047A60UL, 0xDF60EFC3UL, 0xA867DF55UL, 0x316E8EEFUL, 0x4669BE79UL, 0xCB61B38CUL, 0xBC66831AUL, 0x256FD2A0UL, 0x5268E236UL, 0xCC0C7795UL, 0xBB0B4703UL, 0x220216B9UL, 0x5505262FUL, 0xC5BA3BBEUL, 0xB2BD0B28UL, 0x2BB45A92UL, 0x5CB36A04UL, 0xC2D7FFA7UL, 0xB5D0CF31UL, 0x2CD99E8BUL, 0x5BDEAE1DUL, 0x9B64C2B0UL, 0xEC63F226UL, 0x756AA39CUL, 0x026D930AUL, 0x9C0906A9UL, 0xEB0E363FUL, 0x72076785UL, 0x05005713UL, 0x95BF4A82UL, 0xE2B87A14UL, 0x7BB12BAEUL, 0x0CB61B38UL, 0x92D28E9BUL, 0xE5D5BE0DUL, 0x7CDCEFB7UL, 0x0BDBDF21UL, 0x86D3D2D4UL, 0xF1D4E242UL, 0x68DDB3F8UL, 0x1FDA836EUL, 0x81BE16CDUL, 0xF6B9265BUL, 0x6FB077E1UL, 0x18B74777UL, 0x88085AE6UL, 0xFF0F6A70UL, 0x66063BCAUL, 0x11010B5CUL, 0x8F659EFFUL, 0xF862AE69UL, 0x616BFFD3UL, 0x166CCF45UL, 0xA00AE278UL, 0xD70DD2EEUL, 0x4E048354UL, 0x3903B3C2UL, 0xA7672661UL, 0xD06016F7UL, 0x4969474DUL, 0x3E6E77DBUL, 0xAED16A4AUL, 0xD9D65ADCUL, 0x40DF0B66UL, 0x37D83BF0UL, 0xA9BCAE53UL, 0xDEBB9EC5UL, 0x47B2CF7FUL, 0x30B5FFE9UL, 0xBDBDF21CUL, 0xCABAC28AUL, 0x53B39330UL, 0x24B4A3A6UL, 0xBAD03605UL, 0xCDD70693UL, 0x54DE5729UL, 0x23D967BFUL, 0xB3667A2EUL, 0xC4614AB8UL, 0x5D681B02UL, 0x2A6F2B94UL, 0xB40BBE37UL, 0xC30C8EA1UL, 0x5A05DF1BUL, 0x2D02EF8DUL}; |
| --- | --- |
|  | Object Type : Derived |
|  | Reqt Source : |
|  | Verification Method : |

### Sine and Cosine Lookup tables

| MRJ_SCU_STC_SRS_892 | The STC Program shall use the following table of 16 bit signed constants as K_CosTable[1] applicable to DFT calculation of A/D channels 1 to 8: |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_960 | The STC Program shall use the following table of 16 bit signed constants as K_CosTable[2] applicable to DFT calculation of A/D channels 9 to 16: |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_893 | The STC Program shall use the following table of 16 bit signed constants as K_SinTable[1] applicable to DFT calculation of A/D channels 1 to 8: |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_961 | The STC Program shall use the following table of 16 bit signed constants as K_SinTable[2] applicable to DFT calculation of A/D channels 9 to 16: |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : HWSWIF |
|  | Verification Method : |

### LVDT Triple Table

| MRJ_SCU_STC_SRS_1249 | The STC Program shall use the following LVDT Triple Table within the determination logic of the SACT_Position. Note: this table is 0-based indexed, where triplet angles 0 to 7 correspond to Angle_1 to Angle_8. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

### LH LVDT Angle Weighting Table

| MRJ_SCU_STC_SRS_1251 | The STC Program shall have a lookup table derived from the following LH LVDT Angle-to-Weight Table to convert between Actual Steering Angle ('Angle' [deg]) and Weighting factor. The weight for each angle is calculated by linear interpolation of the weighting function. left_lvdt_weight_lookup [281] = { 595, 630, 665, 700, 770, 805, 840, 893, 945, 980, 1015, 1068, 1103, 1155, 1208, 1243, 1278, 1330, 1365, 1400, 1470, 1505, 1523, 1575, 1628, 1663, 1715, 1750, 1785, 1838, 1873, 1908, 1960, 1995, 2030, 2083, 2118, 2153, 2206, 2241, 2258, 2311, 2363, 2398, 2433, 2468, 2503, 2538, 2573, 2608, 2643, 2696, 2731, 2748, 2801, 2836, 2871, 2906, 2923, 2958, 3011, 3046, 3063, 3098, 3133, 3168, 3203, 3221, 3256, 3291, 3326, 3361, 3378, 3413, 3431, 3448, 3501, 3536, 3536, 3571, 3606, 3623, 3641, 3658, 3693, 3728, 3746, 3763, 3781, 3798, 3833, 3851, 3868, 3886, 3903, 3921, 3938, 3956, 3973, 3973, 3991, 4008, 4008, 4043, 4043, 4043, 4061, 4078, 4078, 4078, 4078, 4078, 4096, 4096, 4096, 4078, 4078, 4096, 4078, 4061, 4061, 4043, 4026, 4026, 4008, 3991, 3973, 3956, 3921, 3886, 3868, 3851, 3816, 3763, 3728, 3676, 3623, 3588, 3536, 3483, 3413, 3343, 3291, 3221, 3133, 3046, 2958, 2871, 2766, 2661, 2573, 2451, 2328, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 2328, 2451, 2573, 2661, 2766, 2871, 2958, 3046, 3133, 3221, 3291, 3343, 3413, 3483, 3536, 3588, 3623, 3676, 3728, 3763, 3816, 3851, 3868, 3886, 3921, 3956, 3973, 3991, 4008, 4026, 4026, 4043, 4061, 4061, 4078, 4096, 4078, 4078, 4096, 4096, 4096, 4078, 4078, 4078, 4078, 4078, 4061, 4043, 4043, 4043, 4008, 4008, 3991, 3973, 3973, 3956, 3938, 3921, 3903, 3886, 3868, 3851, 3833, 3798, 3781, 3763, 3746, 3728, 3693, 3658, 3641, 3623, 3606, 3571, 3536, 3536, 3501, 3448, 3431, 3413, 3378, 3361, 3326, 3291, 3256, 3221, 3203, 3168, 3133, 3098, 3063, 3046, 3011, 2958, 2923, 2906, 2871, 2836, 2836 } |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

## BIT Codes

| MRJ_SCU_STC_SRS_1253 | The STC program shall apply the BITE Code numbers for the BITE Monitors as defined in this section. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SATA-15-166 |
|  | Verification Method : |

The following table contains the BIT Codes to be used by the Control Lane:

* BIT Code 2 and 3 are determined as Failure status if any of the specified conditions are in Failure

Table 6.13-1 Control Lane BIT Codes

## Event Codes

| MRJ_SCU_STC_SRS_857 | The STC Program shall detect events (any change of state) and transmit associated information to the ML. The following table contains the Event Codes to be used by the Control Lane: Table 6.14-1 Control Lane Event Codes |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

## SCS Rigging

### Rigging Mode Enable Condition

| MRJ_SCU_STC_SRS_1119 | The STC Program shall enter into Rigging Mode and set the trigger to write the rigging data into NVM ‘WriteRiggingData’ to TRUE, when ARINC Label 107 has been received 3 times with bit 11 set to ‘1’ consecutively and the “Rigging Mode Enable” condition is TRUE. Where “Rigging Mode Enable” condition is (NLG WOW1=In Air) AND (NLG WOW2=In Air) AND (Aircraft speed <= 21kts) AND (RH LVDT1 Health Monitor = No Failure) AND (RH LVDT2 Health Monitor = No Failure) AND (RH LVDT1 & RH LVDT2 Reasonableness Check Monitor = No Failure) AND (LH LVDT1 Health Monitor = No Failure) AND (LH LVDT2 Health Monitor = No Failure) AND (LH LVDT1 & LH LVDT2 Reasonableness Check Monitor = No Failure) AND (Pilot S-HDL Operation #1 Health Monitor = No Failure) AND (Pilot S-HDL Operation #2 Health Monitor = No Failure) AND (Pilot S-HDL Operation #1 & #2 Reasonableness Check Monitor = No Failure) AND (Co-Pilot S-HDL not fitted OR (Co-Pilot S-HDL fitted AND (-4deg <= Co-Pilot S-HDL Operation #1 <= +4deg) (Co-Pilot S-HDL Operation #1 Health Monitor = No Failure) AND (Co-Pilot S-HDL Operation #2 Health Monitor = No Failure) AND (Co-Pilot S-HDL Operation #1 & #2 Reasonableness Check Monitor = No Failure) AND (Towing Switch Reasonableness Check Monitor = No Failure) AND (Towing Disengage = TRUE) AND (-4deg <= Pilot S-HDL Operation #1 <= +4deg) AND (-0.3577 [corresponds to -4deg] <= Ratiometric_Position[RH_LVDT1] <= -0.2798 [corresponds to +4deg]) AND (-0.3577 [corresponds to -4deg] <= Ratiometric_Position[RH_LVDT2] <= -0.2798 [corresponds to +4deg]) AND (-0.2798 [corresponds to -4deg] <= Ratiometric_Position[LH_LVDT1] <= -0.3577 [corresponds to +4deg]) AND (-0.2798 [corresponds to -4deg] <= Ratiometric_Position[LH_LVDT2] <= -0.3577 [corresponds to +4deg]) |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1120 | After entry to Rigging Mode, the STC Program shall remain in Rigging Mode as long as Rigging Mode Enable is True, else exit Rigging Mode. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1121 | If the “Rigging Mode Enable” is TRUE OR “Remain in Rigging Mode” is TRUE Then, The STC Program shall set “Mode” as Rigging Mode [01] on Bits 17-16 of ARINC Label 106, Else set “Mode” as Control Mode [00] on Bits 17-16 of ARINC Label 106. Where “Rigging Mode Enable” is defined in MRJ_SCU_STC_SRS_1119, “Remain in Rigging Mode” is defined in MRJ_SCU_STC_SRS_1120, ARINC Label 106 is defined in MRJ_SCU_STC_SRS_221 and MRJ_SCU_STC_SRS_855. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

### Rigging Data Calculation

| MRJ_SCU_STC_SRS_1123 | The STC Program shall calculate the average ratiometric values (yt) for all the 8 LRVDT sensors are defined in MRJ_SCU_STC_SRS_59 using exponential average (EA) of the LRVDT ratio samples as follows: rt is the generic VDT ratio sample calculated at time t α is the smoothness factor (adimensional) and for this implementation shall be equal to 0.01 yt is the generic rigging offset i.e. average at time t yt-1 is the generic rigging offset i.e. average from the previous calculation Note: The EA is continuously calculated from the power on every SW major cycle in conjunction with new ratios calculations. Initial value of y shall be 0.0 for RVDTs and -0.3202 for LVDTs. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

### Rigging Operation and Storting Rigging Data

| MRJ_SCU_STC_SRS_1125 | When the STC Program is in Rigging Mode and the trigger to write the rigging data into NVM ‘WriteRiggingData’ is set to TRUE then the STC Program shall perform the below steps: 1) Set the trigger ‘WriteRiggingData’ to FALSE. 2) Erase the NVM Sector 0 and Sector 1. 3) Clear Bit 29 of ARINC Label 106 to indicate “Aircraft Not Rigged”. 4) Compute the Rigging Data as defined in MRJ_SCU_STC_SRS_1126. 5) Write the Rigging data into NVM Sector 0 and Sector 1 as defined in MRJ_SCU_STC_SRS_1127. 6) Read back the Rigging Data as defined in MRJ_SCU_STC_SRS_1128. 7) Perform comparison of written and read-back Rigging Data as defined in MRJ_SCU_STC_SRS_1129 and MRJ_SCU_STC_SRS_1130. 8) On successful comparison set Bit 29 of ARINC Label 106 to indicate “Aircraft Rigged”. Where “WriteRiggingData’” is defined in MRJ_SCU_STC_SRS_1119 |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1126 | The STC Program shall calculate the rigging offset for all the 8 LRVDT sensors as follows: For Pilot_RVDT1_Ratiometric_Position_Offset, Pilot_RVDT2_Ratiometric_Position_Offset, RH_LVDT1_Ratiometric_Position_Offset, RH_LVDT2_Ratiometric_Position_Offset, LH_LVDT1_Ratiometric_Position_Offset, LH_LVDT2_Ratiometric_Position_Offset: RiggingOffset = average ratiometric value - 0 deg ratio value. For Co-Pilot_RVDT1_Ratiometric_Position_Offset, Co-Pilot_RVDT2_Ratiometric_Position_Offset: IF Co_Pilot SHDL Fitted THEN Rigging Offset = average ratiometric value - 0 deg ratio value ELSE Rigging Offset = ratiometric value equivalent to +10 degrees END IF Note: The 0 degree ratio value for RVDT’s is 0 and for LVDT’s is -0.3202 Where Average ratiometric value is defined in MRJ_SCU_STC_SRS_1123 and Co_Pilot SHDL Fitted is defined in MRJ_SCU_STC_SRS_79; Co_Pilot SHDL Fitted result needs to be available for performing this calculation. Justification: +10 is choosen as it is outside the range. That is outside +/-4deg. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1127 | The STC Program shall store 2 copies of the following configuration parameters into the NVM device via McBSP SPI Bus (1 copy in sector 0, the other copy in '1's complement format in sector 1): Table 6.15-1 Rigging Configuration Parameters in NVM Sector 0 Table 6.15-2 Rigging Configuration Parameters in NVM Sector 1 Note: Ratiometric offset is calculated as defined in MRJ_SCU_STC_SRS_1126. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1128 | The STC Program shall read back the configuration parameters stored in NVM sector 0 and the copy of the configuration parameters stored in NVM sector 1 defined in MRJ_SCU_STC_SRS_1127. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1129 | The STC Program shall check the values of the configuration parameters and the copy written to NVM match with the read back values, and in case of mismatch repeat the write and read backs up to two times (i.e. up to three total write/read attempts, including the original write/read operation) until there is a match. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

| MRJ_SCU_STC_SRS_1130 | In case of a failure after the maximum amount of retries the STC Program shall set NVM write failure to TRUE and enter failsafe level 3. |
| --- | --- |
|  | Object Type : Traceable |
|  | Reqt Source : SES |
|  | Verification Method : |

### Deleted