| FROM_ID | FROM_TEXT | TO_ID |
| --- | --- | --- |
| SCU_STC_SRS_585 | The STC Program shall enter and remain in Failsafe Level 2 when a failure is reported by any one of the following Monitors: LVDT Double Loss Monitor, ASA Jump Monitor Status, LVDT No Angle Found Fault, EHSV1 and EHSV2 Monitor, Target Input Monitor, Over Angle Monitor, Watchdog Test Monitor, Watchdog Maximum Reset Count, ROM Test, RAM Integrity test, Rigging record and data Check, Aircraft Information check Note1: Each of these individual Monitors is defined as non-recoverable, therefore ensuring the STC Program cannot leave Failsafe level 2 after any one of these has been reported. Note2: The ROM Test and Ram Test are classified as a recoverable Monitors in the Supplier Equipment Specification (Ref : B2) however this is only after a reset of the Control Lane. Note3: Some of the above fault conditions result in processor resets which eventually end up with the processor stopping entry into a run-time mode. The SCU then behaves as if it is in Failsafe Level 2 because, firstly, the affected processor disables the SOV and secondly, the other processor will be in Failsafe Level 2 because of Loss of Interlane Communications. Object Type: Traceable Reqt Source: SES Verification Method: | SCU_SYS_100 |
| SCU_STC_SRS_811 | The STC Program shall determine A_DMC_9 LADC Monitor Status according to the following logic: IF the following condition is true for 220ms with a tolerance of +10ms (( No valid A_DMC_9 LADC labels have been received ) OR (A_DMC_9 LADC data have wrong (even) parity bit) OR (A_DMC_9 LADC have SSM data set to "Normal Operation")) THEN A_DMC_9 LADC Monitor Status = TRUE ELSE IF the following condition is true for 320ms with a tolerance of +10ms ((A_DMC_9 LADC labels have been received at expected update rate with a tolerance of +10ms ) AND (A_DMC_9 LADC data have correct (odd) parity bit) AND (A_DMC_9 LADC have SSM data set to "Normal Operation") AND (A_DMC_9 LADC Airspeed < 35kts)) THEN A_DMC_9 LADC Monitor Status = FALSE ELSE No action (A_DMC_9 LADC Monitor Status remains unchanged) ENDIF ENDIF Where A_DMC_9 LADC ARINC Message is defined in SCU_STC_SRS_135 Justification for timing tolerance: ARINC label reception requires tolerance for label polling and processing. Object Type: Traceable Reqt Source: SES Verification Method: | SCU_SYS_101 |
| SCU_STC_SRS_99 | The STC Program shall convert the Pilot_SHDL_Operation into a steering angle demand Unlimited_Pilot_SHDL_Command in every Major cycle using the following logic: IF( Pilot_SHDL_Operation >= 54.7deg ) THEN Unlimited_Pilot_SHDL_Command = (26.4 * Pilot_SHDL_Operation - 840.5deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= 44.3deg ) THEN Unlimited_Pilot_SHDL_Command = (16.6 * Pilot_SHDL_Operation - 298.58deg ) * (1/10.4) ELSE IF( Pilot_SHDL_Operation >= 34.7deg ) THEN Unlimited_Pilot_SHDL_Command = (14.4 * Pilot_SHDL_Operation - 205.32deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= 23.7deg ) THEN Unlimited_Pilot_SHDL_Command = (11.6 * Pilot_SHDL_Operation - 110.12deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= 13.3deg ) THEN Unlimited_Pilot_SHDL_Command = (9.2 * Pilot_SHDL_Operation - 51.64deg ) * (1/10.4) ELSE IF( Pilot_SHDL_Operation >= 3deg ) THEN Unlimited_Pilot_SHDL_Command = (Pilot_SHDL_Operation - 3deg ) * (6.8 / 10.3) ELSE IF( Pilot_SHDL_Operation >= -3deg ) THEN Unlimited_Pilot_SHDL_Command = 0deg ELSE IF( Pilot_SHDL_Operation >= -13.3deg ) THEN Unlimited_Pilot_SHDL_Command = (Pilot_SHDL_Operation + 3deg ) * (6.8 / 10.3) ELSE IF( Pilot_SHDL_Operation >= -23.7deg ) THEN Unlimited_Pilot_SHDL_Command = (9.2 * Pilot_SHDL_Operation + 51.64deg ) * (1/10.4) ELSE IF( Pilot_SHDL_Operation >= -34deg ) THEN Unlimited_Pilot_SHDL_Command = (11.6 * Pilot_SHDL_Operation + 110.12deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= -44.3deg ) THEN Unlimited_Pilot_SHDL_Command = (14.4 * Pilot_SHDL_Operation + 205.32deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= -54.7deg ) THEN Unlimited_Pilot_SHDL_Command = (16.6 * Pilot_SHDL_Operation + 298.58deg ) * (1/10.4) END IF Where Pilot_SHDL_Operation is the value calculated in SCU_STC_SRS_68 and Major cycle is defined in Table 6.2-1 Object Type: Traceable Reqt Source: SES Verification Method: | SCU_SYS_102 |
| SCU_STC_SRS_180 | The STC Program shall debounce the NLG_WOW1 signal using the following algorithm to generate a filtered version of the signal IF (NLG_WOW1 has been TRUE for the last 1.5s) THEN Debounced_NLG_WOW1 = FALSE ELSE IF (NLG_WOW1 has been FALSE for the last 100ms) THEN Debounced_NLG_WOW1 = TRUE ELSE Debounced_NLG_WOW1 remains unchanged, with an initial value of FALSE ENDIF Where NLG_WOW1 is the value calculated in SCU_STC_SRS_139 Object Type: Traceable Reqt Source: SES Verification Method: | SCU_SYS_103 |
| SCU_STC_SRS_137 | The STC Program shall poll the ARINC receivers for the labels listed in the following table more frequently than the transmission frequency: Table: ARINC Rx Rate Object Type: Traceable Reqt Source: SATA-15-166 Verification Method: ARINC Receiver; Label (octal); Transmission Interval (ms). DMCA1; 022; 40.96. DMCA1; 226; 1400. DMCA1; 174 (Right); 100. DMCA1; 206 (ISI); 67. DMCA1; 206 (L ADC); 200. DMCA1; 206 (R ADC); 100. DMCA1; 260; 1000. DMCA1; 107; Aperiodic (polling rate 1000). DMCB1; 022; 40.96. DMCB1; 226; 1400. DMCB1; 174 (Right); 100. DMCB1; 206 (ISI); 67. DMCB1; 206 (L ADC); 100. DMCB1; 206 (R ADC); 200. DMCB1; 260; 1000. ACE1B1; 136; 100. ACE1B2; 136; 10. | SCU_SYS_110 |
| SCU_STC_SRS_871 |  | SCU_SYS_110 |
| SCU_STC_SRS_135 | The STC Program shall receive A_DMC_9 L ADC Airspeed in the Computed Airspeed field in messages from DMCA in the format of Label 206 (ADC) specified in below Table with a Source/Destination Identifier set to Left ADC Object Type: Traceable Reqt Source: SATA-15-166 Verification Method: Bit Position; Field; Description; Definition. 8-1; Label; Octal Label – ARINC429 Message ID; Label = 206 (octal) Bit 1=msb, Bit 8=lsb. 10-9; SDI; Source/Destination Identifier; [00] = Not Used [01] = Left ADC [10] = Right ADC [11] = Not Used. 28-12; Computed Airspeed; Aircraft Speed as calculated by the Air Data Computer (ADC); Range: 0 to 1023.99609375 Units: Knots LSB Resolution: 0.00390625 kts MSB Value: 512 kts Bit 28 = msb; Bit 11 = lsb. 29; Sign; Sign; [0] = Positive [1]= Negative. 31-30; SSM; Sign / Status Matrix Type: BNR (Numeric); [00] = Failure Warning [01] = No Computed Data [10] = Functional Test [11] = Normal Operation. 32; Parity; Parity Bit (Odd); Set to ‘Zero’ or ‘One’ in order to make the total number of Ones in the message an Odd value.. | SCU_SYS_110 |
| SCU_STC_SRS_99 | The STC Program shall convert the Pilot_SHDL_Operation into a steering angle demand Unlimited_Pilot_SHDL_Command in every Major cycle using the following logic: IF( Pilot_SHDL_Operation >= 54.7deg ) THEN Unlimited_Pilot_SHDL_Command = (26.4 * Pilot_SHDL_Operation - 840.5deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= 44.3deg ) THEN Unlimited_Pilot_SHDL_Command = (16.6 * Pilot_SHDL_Operation - 298.58deg ) * (1/10.4) ELSE IF( Pilot_SHDL_Operation >= 34.7deg ) THEN Unlimited_Pilot_SHDL_Command = (14.4 * Pilot_SHDL_Operation - 205.32deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= 23.7deg ) THEN Unlimited_Pilot_SHDL_Command = (11.6 * Pilot_SHDL_Operation - 110.12deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= 13.3deg ) THEN Unlimited_Pilot_SHDL_Command = (9.2 * Pilot_SHDL_Operation - 51.64deg ) * (1/10.4) ELSE IF( Pilot_SHDL_Operation >= 3deg ) THEN Unlimited_Pilot_SHDL_Command = (Pilot_SHDL_Operation - 3deg ) * (6.8 / 10.3) ELSE IF( Pilot_SHDL_Operation >= -3deg ) THEN Unlimited_Pilot_SHDL_Command = 0deg ELSE IF( Pilot_SHDL_Operation >= -13.3deg ) THEN Unlimited_Pilot_SHDL_Command = (Pilot_SHDL_Operation + 3deg ) * (6.8 / 10.3) ELSE IF( Pilot_SHDL_Operation >= -23.7deg ) THEN Unlimited_Pilot_SHDL_Command = (9.2 * Pilot_SHDL_Operation + 51.64deg ) * (1/10.4) ELSE IF( Pilot_SHDL_Operation >= -34deg ) THEN Unlimited_Pilot_SHDL_Command = (11.6 * Pilot_SHDL_Operation + 110.12deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= -44.3deg ) THEN Unlimited_Pilot_SHDL_Command = (14.4 * Pilot_SHDL_Operation + 205.32deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= -54.7deg ) THEN Unlimited_Pilot_SHDL_Command = (16.6 * Pilot_SHDL_Operation + 298.58deg ) * (1/10.4) END IF Where Pilot_SHDL_Operation is the value calculated in SCU_STC_SRS_68 and Major cycle is defined in Table 6.2-1 Object Type: Traceable Reqt Source: SES Verification Method: | SCU_SYS_104 |
| SCU_STC_SRS_1 | The SCU software shall implement the following state logic: INITIAL_STATE = NORMAL IF failure_detected THEN STATE := FAILSAFE_LEVEL_1 END IF IF critical_failure_detected THEN STATE := FAILSAFE_LEVEL_2 END IF | SCU_SYS_5 |
| SCU_STC_SRS_2 | The SCU software shall implement the following state logic: INITIAL_STATE = NORMAL IF failure_detected THEN STATE := FAILSAFE_LEVEL_1 END IF | SCU_SYS_6 |
| SCU_STC_SRS_3 | The SCU software shall implement the following state logic: INITIAL_STATE = NORMAL IF failure_detected THEN STATE := FAILSAFE END IF IF maintenance_mode_requested THEN STATE := MAINTENANCE END IF | SCU_SYS_7 |
| SCU_STC_SRS_4 | The SCU software shall follow the state transitions defined below: FROM; Event; To. Normal; Failure_detected; FAILSAFE_LEVEL_1. | SCU_SYS_8 |
| SCU_STC_SRS_999 |  | SCU_SYS_99 |
| SCU_STC_SRS_504 |  | SCU_SYS_99 |
| SCU_STC_SRS_124 |  | SCU_SYS_163 |