| ID | DESC |
| --- | --- |
| SCU_SYS_100 | The SCU system shall enter and remain in Failsafe Level 1 when a failure is reported by any one of the following Monitors: LVDT Double Loss Monitor, ASA Jump Monitor Status, LVDT No Angle Found Fault, EHSV1 and EHSV2 Monitor, Target Input Monitor, Over Angle Monitor, Steering Angle Monitor, Watchdog Test Monitor, Watchdog Maximum Reset Count, ROM Test, RAM Integrity test, Rigging record and data Check, Note1: Each of these individual Monitors is defined as non-recoverable, therefore ensuring the STC Program cannot leave Failsafe level 1 after any one of these has been reported. Note2: The ROM Test and Ram Test are classified as a recoverable Monitors in the Supplier Equipment Specification (Ref: B2) however this is only after a reset of the Control Lane. Note3: Some of the above fault conditions result in processor resets which eventually end up with the processor stopping entry into a run-time mode. The SCU then behaves as if it is in Failsafe Level 1 because, firstly, the affected processor disables the SOV and secondly, the other processor will be in Failsafe Level 2 because of Loss of Interlane Communications. |
| SCU_SYS_101 | The SCU system shall determine A_DMC_9 LADC Monitor Status according to the following logic: IF the following condition is true for 320ms (( No valid A_DMC_9 LADC labels have been received ) OR (A_DMC_9 LADC data have wrong (even) parity bit) OR (A_DMC_9 LADC have SSM data not set to "Normal Operation")) THEN A_DMC_9 LADC Monitor Status = TRUE ELSE IF the following condition is true for 320ms ((A_DMC_9 LADC labels have been received at expected update rate with a tolerance of +10ms ) AND (A_DMC_9 LADC data have correct (odd) parity bit) AND (A_DMC_9 LADC have SSM data set to "Normal Operation") AND (A_DMC_9 LADC Airspeed < 35kts)) THEN A_DMC_9 LADC Monitor Status = FALSE ELSE No action (A_DMC_9 LADC Monitor Status remains unchanged) ENDIF ENDIF |
| SCU_SYS_102 | The SCU system shall convert the Pilot_SHDL_Operation into a steering angle demand Unlimited_Pilot_SHDL_Command using the following logic: IF( Pilot_SHDL_Operation >= 54.7deg ) THEN Unlimited_Pilot_SHDL_Command = (26.4 * Pilot_SHDL_Operation - 840.5deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= 44.3deg ) THEN Unlimited_Pilot_SHDL_Command = (16.6 * Pilot_SHDL_Operation - 298.58deg ) * (1/10.4) ELSE IF( Pilot_SHDL_Operation >= 34deg ) THEN Unlimited_Pilot_SHDL_Command = (14.4 * Pilot_SHDL_Operation - 205.32deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= 23.7deg ) THEN Unlimited_Pilot_SHDL_Command = (11.6 * Pilot_SHDL_Operation - 110.12deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= 13.3deg ) THEN Unlimited_Pilot_SHDL_Command = (9.2 * Pilot_SHDL_Operation - 51.64deg ) * (1/10.4) ELSE IF( Pilot_SHDL_Operation >= 3deg ) THEN Unlimited_Pilot_SHDL_Command = (Pilot_SHDL_Operation - 3deg ) * (6.8 / 10.3) ELSE IF( Pilot_SHDL_Operation >= -3deg ) THEN Unlimited_Pilot_SHDL_Command = 0deg ELSE IF( Pilot_SHDL_Operation >= -13.3deg ) THEN Unlimited_Pilot_SHDL_Command = (Pilot_SHDL_Operation + 3deg ) * (6.8 / 10.3) ELSE IF( Pilot_SHDL_Operation >= -23.7deg ) THEN Unlimited_Pilot_SHDL_Command = (9.2 * Pilot_SHDL_Operation + 51.64deg ) * (1/10.4) ELSE IF( Pilot_SHDL_Operation >= -34deg ) THEN Unlimited_Pilot_SHDL_Command = (11.6 * Pilot_SHDL_Operation + 110.12deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= -44.3deg ) THEN Unlimited_Pilot_SHDL_Command = (14.4 * Pilot_SHDL_Operation + 205.32deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= -54.7deg ) THEN Unlimited_Pilot_SHDL_Command = (16.6 * Pilot_SHDL_Operation + 298.58deg ) * (1/10.4) ELSE Unlimited_Pilot_SHDL_Command = (26.4 * Pilot_SHDL_Operation + 840.5deg ) * (1/10.3) END IF |
| SCU_SYS_103 | The SCU system shall debounce the NLG_WOW1 signal using the following algorithm IF (NLG_WOW1 has been TRUE for the last 1.5s) THEN Debounced_NLG_WOW1 = TRUE ELSE IF (NLG_WOW1 has been FALSE for the last 200ms) THEN Debounced_NLG_WOW1 = FALSE ELSE Debounced_NLG_WOW1 remains unchanged, with an initial value of TRUE ENDIF |
| SCU_SYS_110 | The SCU system shall receive the below ARINC labels: Label (octal). 022. 226. 174 (Right). 206 (ISI). 206 (L ADC). 206 (R ADC). 260. 107. 022. 226. 174 (Right). 206 (ISI). 206 (L ADC). 206 (R ADC). 260. 136. 136. |
| Scu_Sys_5 | The SCU shall operate according to the following state behavior: INITIAL_STATE = NORMAL IF a failure is detected THEN STATE := FAILSAFE_LEVEL_1 END IF IF a critical failure is detected THEN STATE := FAILSAFE_LEVEL_2 END IF |
| SCU_SYS_6 | The SCU shall operate according to the following state behavior: INITIAL_STATE = NORMAL IF a failure is detected THEN STATE := FAILSAFE_LEVEL_1 END IF IF a critical failure is detected THEN STATE := FAILSAFE_LEVEL_2 END IF |
| SCU_SYS_7 | The SCU shall operate according to the following state behavior: INITIAL_STATE = NORMAL IF failure is detected THEN STATE := FAILSAFE END IF |
| SCU_SYS_9 | The SCU software shall implement the following state logic: INITIAL_STATE = NORMAL IF maintenance_mode_requested THEN STATE := MAINTENANCE END IF |
| SCU_SYS_8 | The SCU software shall follow the state transitions defined below: FROM; Event; To. Normal; Failure_detected; FAILSAFE_LEVEL_1. Normal; critical_failure_detected; FAILSAFE_LEVEL_2. |
| SCU_SYS_99 | SCU shall compute output as follows: IF (X >= 34.7deg) THEN A ELSE IF (X >= 3deg) THEN B ELSE C |