# pip install python-docx ARINC LABELS

The SCU system shall poll the ARINC receivers for the labels listed in the following table more frequently than the transmission frequency:

| ARINC Receiver | Label (octal) | Transmission Interval (ms) |
| --- | --- | --- |
| DMCA1 | 022 | 40.96 |
| DMCA1 | 226 | 1400 |
| DMCA1 | 174 (Right) | 100 |
| DMCA1 | 206 (ISI) | 67 |
| DMCA1 | 206 (L ADC) | 100 |
| DMCA1 | 206 (R ADC) | 100 |
| DMCA1 | 260 | 1000 |
| DMCA1 | 107 | Aperiodic (polling rate 1000) |
| DMCB1 | 022 | 40.96 |
| DMCB1 | 226 | 1400 |
| DMCB1 | 174 (Right) | 100 |
| DMCB1 | 206 (ISI) | 67 |
| DMCB1 | 206 (L ADC) | 100 |
| DMCB1 | 206 (R ADC) | 100 |
| DMCB1 | 260 | 1000 |
| ACE1B1 | 136 | 10 |
| ACE1B2 | 136 | 10 |

Table: ARINC Rx Rate

# Label 022 (WOW Data)

The SCU system shall receive A_DMC_9 Label 022 messages from DMCA in the format of Label 022 specified in below Table.

The format of Label 022 (WOW Data) will be as follows:

Table Label 022

# Label 206 (L ADC)

The STC Program shall receive A_DMC_9 L ADC Airspeed in the Computed Airspeed field in messages from DMCA in the format of Label 206 (ADC) specified in below Table with a Source/Destination Identifier set to Left ADC

| Bit Position | Field | Description | Definition |
| --- | --- | --- | --- |
| 7-0 | Label | Octal Label – ARINC429 Message ID | Label = 206 (octal) Bit 1=msb, Bit 8=lsb |
| 9-8 | SDI | Source/Destination Identifier | [00] = Not Used [01] = Left ADC [10] = Right ADC [11] = Not Used |
| 27-10 | Computed Airspeed | Aircraft Speed as calculated by the Air Data Computer (ADC) | Range: 0 to 1023.99609375 Units: Knots LSB Resolution: 0.00390625 kts MSB Value: 512 kts Bit 28 = msb; Bit 11 = lsb |
| 28 | Sign | Sign | [0] = Positive (Always) |
| 30-29 | SSM | Sign / Status Matrix Type: BNR (Numeric) | [00] = Failure Warning [01] = No Computed Data [10] = Functional Test [11] = Normal Operation |
| 31 | Parity | Parity Bit (Odd) | Set to ‘Zero’ or ‘One’ in order to make the total number of Ones in the message an Odd value. |

Table Label 206