# General Description

## Steering Control System

The MRJ aircraft requires that one SCU will be installed in each aircraft. The SCU receives Control and Monitor lane information via the ARINC429 bus. The SCU transmits steering and flight status information to the Primary Flight Control Computer (PFCC) via (DMC-Avionics1 and DMC-Avionics2). The SCU also transmit errors, warnings and failure information to the Engine Indicating and Crew Alerting System (EICAS) within the Data Management Cabinet (DMC). The SCS turns to Free Castor Mode (Steering free mode) when a safety critical SCU failure has occurred. System troubleshooting can be accomplished by using failure message information from the DMC and the SCU in accordance with the Built In Test (BIT) result.

The SCS has a shutoff valve (SOV) which shuts off (de-Energize) the hydraulic pressure to the SCS per SCU commands. When the SOV Command is ‘ON’ or Energize, the SCV provides hydraulic pressure to the SCS, and the SCV detects it via the pressure switch. The SCU sends control commands to Electro Hydraulic Servo Valve (EHSV), which controls the hydraulic flow and direction to steering actuators. The SCS also has anti-shimmy functions, to prevent or minimize shimmy generated from the Nose Landing Gear (NLG) during taxi, takeoff, and landing. The SCS has a pressure sensor discrete to check actual pressure in the system. The SCS is configured for 1 Pilot steering handle with the option for an additional co-Pilot steering handle.

## SCU Architecture

### System Architecture

The SCU controls the Nose Wheel steering angle by using the following inputs:

- Rotary Variable Differential Transducer (RVDT) via the pilot steering handle; which provide indication of the steering angle requested by the Pilot

- Linear Variable Differential Transducer (LVDT) via the steering actuators; which provide indication of the actual steering angle (Actual Steering Angle)

- Data from ARINC429 interfaces and Analog signal which provided the Rudder operation information of the steering angle requested by the Pilot

- Data from ARINC429 interfaces indicating whether the aircraft is on the ground as well as the ground speed of the aircraft.

The SCU Control Lane provides the following control functions:

- Nose Wheel steering angle position control

- Nose Wheel steering de-activation control

The SCU Monitor Lane provides the following monitor/reporting functions:

- Nose Wheel steering angle position monitoring

- Steering Control System status monitoring

- EICAS Failure Indication for the Steering Control System

- BIT Reporting for the Steering Control System

### SCU Equipment Architecture

The SCU Equipment Architecture is based on a single Control Lane and a single Monitor Lane.

The Control Lane is responsible for the calculation of the target steering angle (TSA) via the DSP microcontroller. The Control Lane is also responsible for controlling the actuation of the Nose Wheel Steering via the Electro Hydraulic Servo Valve (EHSV).

The Monitor Lane is responsible for monitoring the Control Lane and, if necessary, will activate the Shutoff Valve (SOV) solenoid to disable steering control upon detection of a fault which may otherwise lead to hazardous or catastrophic behaviour. Additionally the Control Lane has the capability to disable steering control by de-energising the SOV if it detects faults within the Monitor Lane.

## SCU Inputs

The SCU receives the following inputs, which are routed to both the Control Lane and the Monitor Lane for independent processing:-

  - 2 x Pilot RVDT Steering Command Signal

  - 2 x Co-Pilot RVDT Steering Command Signal

  - 2 x LH LVDT Steering Position Signal

  - 2 x RH LVDT Steering Position Signal

  - 2 x Rudder Command from ACE1B via ARINC429 communication

  - 2 x Rudder Command from ACE1B via Analog Signal

  - 2 x Rudder Command from ACE2B via ARINC429 communication

  - 2 x Rudder Command from ACE2B via Analog Signal

  - 3 x Aircraft Speed from A-DMC via ARINC429 communication

  - 3 x Aircraft Speed from B-DMC via ARINC429 communication

  - Steering System Hydraulic Pressure Switch Discrete Input

  - Pilot Enable Discrete Input

  - co-Pilot Enable Discrete Input

  - 2 x Disengage Discrete Inputs

The Control Lane and the Monitor Lane uses a different ‘primary’ RVDT steering angle command input and LVDT actual steering angle input in order to provide complete independence from each other. Additionally, they also monitor the LVDT and RVDT inputs provided as the ‘primary’ signals for the opposite lane.

The Monitor Lane provides monitoring of the actual steering angle. If the actual steering angle exceeds safety limits or is significantly different to the commanded steering angle, the Monitor Lane will send a message to de-energize the SOV solenoids.  When the SOV solenoids are de-energised, the hydraulic pressure from the steering system is removed, thereby removing control of the Nose Wheel steering from the EHSV and allowing the Nose Wheel to free castor. This over-ride is required to guard against any excessive or inadvertent steering angle being commanded by the Control Lane. In addition the Monitor Lane provides BIT reporting for the Steering Control System (SCS) and EICAS indication.

Monitoring of the SCU circuits and the SCU interfaces is predominately performed in software. The software operation is monitored by a separate, independent hardware watchdog.

Communications with other aircraft systems is via ARINC 429 interfaces.

The Control Lane and the Monitor Lane communicate internally via a point-to-point serial interface

## SCU Outputs

The SCU provides the following outputs:

  - 4 x servo amplifier outputs for driving the high and low sides of the two solenoids coils within the EHSV.

  - 1 SOV solenoid output a high-side (via CL) and a low-side (via ML) solenoid driver for the SOV.

The Control Lane and Monitor Lane are configured to receive via 4 ARINC RX busses and provide 1 ARINC TX bus:

  - The primary function of the Control Lane ARINC TX bus is to provide failure indication, BIT data and information for steering control status.

  - The primary function of the Monitor Lane ARINC TX bus is to provide failure indication, BIT data, support for rigging and information for steering control status.

## Power Supply

The internal power supply receives 28V DC power from two independent busses, which is consolidated to an internal DC-DC power supply.

## Target & Actual Steering Angle

The Control Lane and the Monitor lane use a different 'primary' RVDT steering angle command inputs and LVDT actual steering angle inputs to provide independence from each other.

The steering angle feedback is provided to the SCU from a Left Hand LVDT (LH_LVDT) and a Right Hand LVDT (RH_LVDT) assembly. Each LVDT assembly comprises of two channels, channel A and channel B. The Control Lane provides the excitation signal for the primary coils for the channel A LVDTs, the secondary coils of the channel A LVDTs are monitored by both Control and Monitor lanes.  The Monitor Lane provides the excitation signal for the primary coils for the channel B LVDTs, the secondary coils of the channel B LVDTs are monitored by both Control and Monitor lanes.

- Channel A LVDTs are primary to Control Lane and secondary to Monitor Lane

- Channel B LVDTs are primary to Monitor Lane and secondary to Control Lane

This allows the 'secondary' LVDT to be used for the remainder of a flight where there is a detected failure of one of the 'primary' LVDT or RVDT inputs.

The SCU receives target steering angle information from a pilot steering handle and a co-pilot steering handle. Each handle comprises of two RVDTs, Channel A and Channel B. The Control Lane provides the excitation signal for the primary coils for the channel A RVDTs, the secondary coils of the channel A RVDTs are monitored by both Control and Monitor lanes.  The Monitor Lane provides the excitation signal for the primary coils for the channel B RVDTs, the secondary coils of the channel B RVDTs are monitored by both Control and Monitor lanes.

- Channel A RVDTs are primary to Control Lane and secondary to Monitor Lane

- Channel B RVDTs are primary to Monitor Lane and secondary to Control Lane

The SCU also receives target steering angle from two inputs from Rudder 1 and two inputs from Rudder 2. The Monitor Lane and the Control Lane will each monitor all four rudder command inputs.

The actual steering command is a function of aircraft speed. The aircraft speed is provided to the SCU via the ARINC 429 receiver channels.

## Shut Off Valve (SOV)

Both the Control and Monitor Lanes provide monitoring of the actual steering angle and if this exceeds safe limits or is significantly different to the commanded steering angle they will independently de-energise the SOV solenoid.

When the SOV is de-energised, the hydraulic pressure from the steering system is removed, thereby removing control of the Nose Wheel steering from the EHSV and allowing the Nose Wheel to free castor. This over-ride is required to guard against excessive steering angle being commanded by the control lane.

## Monitor Lane Function

In addition the Monitor Lane provides BIT reporting for the Steering Control System and EICAS indication.

Monitoring of the SCU circuits and the SCU interfaces is predominately performed in software. Software operation is monitored by a hardware watchdog.

The monitor lane performs a check on the integrity of the SOV system once every flight.

Communications with other aircraft systems is provided via ARINC 429 interfaces.

Background checks will be continuously performed on the integrity of SCU sub-systems.

## ARINC 429 Data

Both the Control Lane and Monitor Lane each receive data on four (4) ARINC 429 Receive busses and each output data on one (1) ARINC 429 Transmit bus. Data on each ARINC bus is identified in the Electrical Interface Control Document [A2].

ARINC inputs provide aircraft speed, Weight On Wheel information and Rudder Commands.

## Discrete Inputs (Switches)

The SCU receives several discrete inputs from the steering control system. These include Pilot/Co-Pilot steering handle enable inputs, Hydraulic Pressure indication, and disengage inputs from the ground maintenance panel. The discrete input interface is designed to interface with these inputs.

Figure 4.10-1: SCU Architecture

## SCU BITE

The following summarise the Built-In Test features provided by the SCU.

### Control Lane

- DSP Integrity

- Watchdog test

- RAM integrity Test

- ROM Test

- Inputs

- LVDT - open, short and reasonableness checks.  Reasonableness checks are carried out via cross-comparison.

- LH_LVDT1

- LH_LVDT2

- RH_LVDT1

- RH_LVDT2

- RVDT - open, short and reasonableness checks.  Reasonableness checks are carried out via cross-comparison. In the event of failure the secondary RVDT from the other lane will be used. Note: In the event that during PBIT the primary and secondary RVDTs for the co-pilot are deemed to be open or short circuit then it will be assumed that the system is operating in single pilot mode.

- Pilot_RVDT1

- Pilot_RVDT2

- Co_Pilot_RVDT1 (optional)

- Co_Pilot_RVDT2 (optional)

- Discrete Inputs -   Errors in discrete inputs will be deduced via cross comparison of signals.

- Pilot enable Switch

- Co_Pilot enable Switch  (optional)

- Towing Disengage Switch#1

- Hydraulic Pressure Switch  - Errors will be deduced by SOV Test

- ARINC 429 Rx Bus - Loss of data labels, loss of data bus, and reasonableness checks.  Reasonableness checks are carried out via cross comparison of data i.e. for WOW, airspeed and rudder Commands.

- ARINC_RX_DMCA1

- ARINC_RX_DMCB1

- ARINC_RX_ACE1B1

- ARINC_RX_ACE2B1

- Outputs

- EHSV Coil - coil open/short will be deduced via monitors on the EHSV drivers. These will be checked during CBIT.  In the event of failure the alternate EHSV Coil will be used.

- EHSV CoilA

- EHSV CoilB

- EHSV Drivers - driver stuck ON or stuck OFF will be deduced via monitors on the EHSV drivers. These will be checked during CBIT. In the event of failure the alternate EHSV Drivers will be used.

- EHSV Hi-side Driver #1

- EHSV Hi-side Driver #2

### Monitor Lane

- DSP Integrity

- Watchdog test

- RAM Integrity Test

- ROM Integrity Test

- Inputs

- LVDT - open, short and reasonableness checks. Reasonableness checks are carried out via cross-comparison. In the event of failure the secondary LVDT from the other lane will be used.

- LH_LVDT1

- LH_LVDT2

- RH_LVDT1

- RH_LVDT2

- RVDT - open, short and reasonableness checks. Reasonableness checks are carried out via cross-comparison. In the event of failure the secondary RVDT from the other lane will be used. Note: In the event that the primary and secondary RVDTs for the co-pilot are deemed to be open circuit then it will be assumed that the system is operating in single pilot mode.

- Pilot_RVDT1

- Pilot_RVDT2

- Co_Pilot_RVDT1 (optional)

- Co_Pilot_RVDT2 (optional)

- Discrete Inputs -  Errors in discrete inputs will be deduced via cross comparison of signals.

- Pilot enable Switch

- Co_Pilot enable Switch (optional)

- Towing Disengage Switch

- Hydraulic Pressure Switch  - Errors will be deduced by SOV Test

- ARINC 429 Rx Bus - Loss of data labels, loss of data bus, and reasonableness checks. Reasonableness checks are carried out via cross comparison of data i.e. for WOW, airspeed and rudder commands.

- ARINC_RX_DMCA2

- ARINC_RX_DMCB2

- ARINC_RX_ACE1B2

- ARINC_RX_ACE2B2

- Outputs

- SOV Coil - coil open/short will be deduced via monitors on the SOV driver. This will be checked during PBIT.

- SOV Coil

- SOV Driver - driver stuck ON or stuck OFF will be deduced via monitors on the SOV driver. These will be checked during PBIT.

- SOV Driver

- SOV enable test - performed at power-up when aircraft is on-ground and airspeed is less than 10knots. Also BIT performed during normal operation.

- SOV disable test - performed at power-up alongside SOV enable test. Also BIT performed when hydraulics are not enabled i.e. when steering has been disabled by Monitor Lane.

- Power Supply

- 28V aircraft inputs - the +28V aircraft power supplies will be monitored for availability. The SCU will operate if one or other or both +28V supplies are available.

- +28V #1

- +28V #2

- Internal Power Supply - the health of the internal supply is checked via hardware monitors on the internal supply rail.

- PSU Monitors

- Other

- Steering fault indication - The monitor lane will report a fault and will de-activate the SOV when the steering angle exceeds the maximum allowed angle for a given speed.

- Steering fault indication - The monitor lane will report a fault and will de-activate the SOV if the commanded steering angle is not consistent with the value determined by the Monitor Lane.

# CSCI Description

## CSCI Software Overview

The SCU Steering Control (STC) program has the following main functional requirements:

- Executive

o	Power-up Processing:

-	Create C-Environment

-	Initialise DSP Registers

-	Perform PBIT

-	Load Configuration Parameters from NVM

o	Runtime Processing:

-	Level-1 tasks

-	Level-2 tasks

-	Level-3 tasks

- CBIT

o	Perform CBIT on DSP resources

-	Kick Watchdog

-	Calculate Arithmetic Flash Checksum and 32-Bit CRC

-	Check Register Values

o	Perform CBIT on SCU components

-	EHSV Feedback BIT

-	LVDT/RVDT BIT

-	Disagreement BIT

-	ARINC Bus BIT

- Input Processing

o	Analogue Input Processing

-	LVDT/RVDT data

-	SHDL Logical Level Processing & Monitoring

-	SACT Logical Level Processing & Monitoring

-	Rudder Position Inputs (via external ADC)

o	Discrete Input Processing

-	Hydraulic Pressure Switch

-	Pilot SHDL Engagement Switch On and Off

-	Co_Pilot SHDL Engagement Switch On and Off

-	Towing Switch Engage

-	Towing Switch Disengage

o	ARINC 429 Input Processing

-	WOW data from DMCA and DMCB

-	Airspeed data from DMCA and DMCB

-	Rudder Pedal Command from ACE1 and ACE2

- Steering Control

o	Steering Demand Calculation

-	Pilot SHDL Steering Demand

-	Co_Pilot SHDL Steering Demand

-	SHDL Command Calculation and Limiting

-	Rudder Command Calculation

-	Steering Demand Summation

-	Steering Demand Limiting

o	Touchdown Protection

-	WOW Debouncing

-	Touchdown Protection Selection

o	Loss of Steering Detection

o	Steering Control Enable

o	Steering Control Loop

- Output Processing

o	EHSV Output Processing

o	Discrete Output Processing

-	SOV Hi-side Output Processing

o	ARINC 429 Output Processing

-	BIT Information

-	Control Status

-	Actual Steering Angle

-	Target Steering Angle

- Cross Lane Communication

o	Transmit STC messages

o	Receive SMB messages

- Rigging Function

- Engineering Access Interface

o	SCU Status Messages

### Executive

During start-up the STC Program will create the C-environment, initialise the DSP Registers and perform PBIT functions such as Watchdog test and SRAM check. Configuration parameters will then be loaded from NVM.

At runtime the scheduler will execute the required tasks for each level of processing period. ADC channels have to be sampled at the highest processing rate (8 channels every 5us). The second level of processing cycles contains the partial DFT calculations. On the third level the DFT calculations are completed. The fourth level (every 5ms) performs the ratiometric calculations for the LVDT and RVDT interfaces.

CBIT monitors and functions are being scheduled at their required cycle rate.

The scheduler will include runtime integrity checks to detect if a processing cycle overruns and has not finished by the time it is supposed to be scheduled in.

### CBIT

The flash content will be checked continuously by calculating an arithmetic checksum and a 32-bit CRC and comparing these checksums with the values stored during unit production.

DSP register settings will be checked continuously with their initialisation values.

The STC program will monitor system components such as LVDTs, RVDTs, SHDL inputs, EHSVs, switches and power supply for faults .

Received ARINC data will be monitored for validity and transmitted ARINC data will be checked by looping the data externally back into the SCU for comparison with the transmitted data.

### Input Processing

The STC Program will sample analogue input values from the LVDT and RVDT interfaces. The resulting DFT calculations will be used to determine the SHDL positions and the actual LVDT / RVDT positions SACT.

Discrete input signals will be sampled and debounced for further processing.

ARINC 429 input labels for WOW status, Airspeed and Rudder Pedal Position will be checked for validity.

### Steering Control

The STC Program will perform the Pilot and Co-Pilot S-HDL Steering command calculation and limiting and the Rudder Command calculation. This will be followed up by the Steering Demand summation and limiting function.

The steering control logic function will determine the steering error, which will be fed into a PID controller.

A Touchdown Protection function will be implemented in order to prevent steering at high speed. This function will also include WOW sensor debouncing.

### Output Processing

The EHSV will be controlled via an analogue PWM excitation signal at a frequency of ca. 50kHz. The mark/space ratio will be updated at a rate of 200Hz.

The LVDT/RVDTs require an excitation frequency of ca. 3kHz. The mark/space ratio will be updated at a rate of 200kHz.

The high-side of the SOV will be controlled via a discrete output.

The ARINC 429 outputs consist of labels for Failure Information, Control Information, Actual Steering Angle and Target Steering Angle.

### Internal Interlane Interface Communication

The Control and Monitor Lanes will exchange system status information via an SPI Bus internal to the SCU. The Control Lane will report its BIT status and Steering Angle information to the Monitor Lane.

### Rigging Function

The OMS can enter a rigging dialogue with the SCU monitor lane, which allows the Steering Handles and Nose Wheel angle inputs to be rigged and allows checking that extreme settings of the handles and actual steering angle are within tolerance. The Monitor Lane requests the Control Lane to calibrate the RVDTs and LVDTs and to store results in NVM. The zero-setting ratios are then applied as a correction to the RVDT & LVDT ratiometric values.

### Engineering Access Interface

Transmission of SCU Status Messages

## CSCI External Interfaces

Figure 5.2-1 shows the External Interfaces for the STC Program.  For a more complete description of each hardware interface, refer to Section 5.2.3.

Figure 5.2-1: External Interfaces

### Input Signals

A summary of the inputs to the STC Program is given in Table 5.2-1.

Table 5.2-1 Input Signals (Part 1)

Table 5.2-1 Input Signals (Part 2)

Table 5.2-1 Input Signals (Part 3)

Table 5.2-1 Input Signals (Part 4)

Table 5.2-1 Input Signals (Part 5)

Table 5.2-1 Input Signals (Part 6)

### Output Signals

A summary of the outputs from the STC Program is given in Table 5.2-2.

Table 5.2-2: Output Signals (part 1)

Table 5.2-2: Output Signals (part 2)

Table 5.2-2: Output Signals (part 3)

Table 5.2-2: Output Signals (part 4)

### Hardware/Software Interface and Assumptions

The STC Program utilises the SM320F2812-EP Digital Signal Processor (DSP) with a 150 MHz clock (30 MHz oscillator frequency).  Internal control registers are used to initialize the DSP functional settings and communications interface.  Periodically, the control registers will be read for feedback on the status of various hardware operations. 
Refer to Ref. C5 for additional information relating to this Interface.

#### PWM Outputs

The STC Program utilises three PWM channels from the Event A peripheral. One channel will be used for generating the LVDT/RVDT excitation signals. The second channel will be used for generating the EHSV excitation signal. The third channel will be used to provide the clock signal for the external ARINC device.

- LVDT/RVDT excitation signal: 	PWM1 from EV-A peripheral

- EHSV excitation signal: 	PWM10 from EV-B peripheral

- ARINC clock signal: 		T2PWM from EV-A peripheral

Refer to Ref. C8 for additional information relating to this Interface.

#### LVDT/RVDT Sensor Analogue Inputs

The steering angle feedback is provided to the SCU from a Left Hand LVDT (LH_LVDT) and a Right Hand LVDT (RH_LVDT) assembly. Each LVDT assembly comprises of two channels, channel A and channel B. The Control Lane provides the excitation signal for the primary coils for the channel A LVDTs, the secondary coils of the channel A LVDTs are monitored by both Control and Monitor lanes.  The Monitor Lane provides the excitation signal for the primary coils for the channel B LVDTs, the secondary coils of the channel B LVDTs are monitored by both Control and Monitor lanes.

The SCU receives target steering angle information from a pilot steering handle and a co-pilot steering handle. Each handle comprises of two RVDTs, Channel A and Channel B. The Control Lane provides the excitation signal for the primary coils for the channel A RVDTs, the secondary coils of the channel A RVDTs are monitored by both Control and Monitor lanes.  The Monitor Lane provides the excitation signal for the primary coils for the channel B RVDTs, the secondary coils of the channel B RVDTs are monitored by both Control and Monitor lanes.

#### DSP External Interface

The external interface of the DSP (XINTF) allows for mapping of external devices onto the DSP’s address space. The ARINC Bus provides the capability to communicate with other aircraft systems. The ARINC protocol which will be used is ARINC429 High Speed (Ref. C3). The SCU utilises 3 devices of type DEI-1085 ARINC 429 Enhanced Transceiver  (Ref. C11). The ARINC devices will be mapped onto the DSP’s External Interface (XINTF) Zone 1.

Refer to Ref. C7 for additional information relating to this Interface.

#### Discrete Inputs

The STC Program receives the following Open-Circuit/Ground type discrete signals via GPIO inputs:

- Pilot SHDL Engage

- Pilot SHDL Disengage

- Co_Pilot SHDL Engage

- Co_Pilot SHDL Disengage

- Towing Engage

- Towing Disengage

- EHSV1 Fail

- EHSV2 Fail

- Hydraulic Pressure

- Analog_Supplies_Ok

#### Discrete Outputs

The STC Program outputs the following Open-Circuit/Ground type discrete signals via GPIO outputs:

- LVDT/RVDT VA-VB Switch

- EHSV1 Enable/Disable

- EHSV2 Enable/Disable

- NVM SPI Enable/Disable

- Rudder1 SPI Enable/Disable

- Rudder2 SPI Enable/Disable

- McBSP SPI Enable/Disable

- Watchdog Trigger

The STC Program outputs the following 28V/Open-Circuit type discrete signals via GPIO outputs:

- SOV Energise/De-energise

#### CAN Bus Inputs/Outputs

The DSP CAN Bus will be utilised to provide an engineering interface. The CAN protocol which will be used is CAN 2.0B (Ref. C10). The STC Program will transmit messages containing the system control status data.

Refer to Ref. C9 for additional information relating to this Interface.

#### External Non-Volatile Memory Inputs/Outputs

The STC Program will utilise a Numonyx Forte Serial Flash Memory M25P16 in order to store SCU Configuration Parameters (Ref. C13). The NVM device will be connected to the DSP via McBSP SPI Bus.

Refer to Ref. C14 for additional information relating to this Interface.

#### External ADC Inputs

The STC Program will utilise two Analog Devices AD7887 devices in order to sample the analogue Rudder Pedal Position signals (Ref. C15). The ADC devices will be connected to the DSP via McBSP SPI Bus.

Refer to Ref. C14 for additional information relating to this Interface.

#### Internal Interlane Interface Inputs/Outputs

The STC Program will utilise SPI-A Bus for communicating with the Monitor Lane.

Refer to Ref. C12 for additional information relating to this Interface.

### Software/Software Interface (Internal)

The STC Program will have no internal CSCI / CSCI Interfaces.

# CSCI Software Requirements

| SCU_STC_SRS_585 | The STC Program shall enter and remain in Failsafe Level 2 when a failure is reported by any one of the following Monitors: LVDT Double Loss Monitor, ASA Jump Monitor Status, LVDT No Angle Found Fault, EHSV1 and EHSV2 Monitor, Target Input Monitor, Over Angle Monitor, Watchdog Test Monitor, Watchdog Maximum Reset Count, ROM Test, RAM Integrity test, Rigging record and data Check, Aircraft Information check Note1: Each of these individual Monitors is defined as non-recoverable, therefore ensuring the STC Program cannot leave Failsafe level 2 after any one of these has been reported. Note2: The ROM Test and Ram Test are classified as a recoverable Monitors in the Supplier Equipment Specification (Ref : B2) however this is only after a reset of the Control Lane. Note3: Some of the above fault conditions result in processor resets which eventually end up with the processor stopping entry into a run-time mode. The SCU then behaves as if it is in Failsafe Level 2 because, firstly, the affected processor disables the SOV and secondly, the other processor will be in Failsafe Level 2 because of Loss of Interlane Communications. Object Type: Traceable Reqt Source: SES Verification Method: |
| --- | --- |
| SCU_STC_SRS_811 | The STC Program shall determine A_DMC_9 LADC Monitor Status according to the following logic: IF the following condition is true for 220ms with a tolerance of +10ms (( No valid A_DMC_9 LADC labels have been received ) OR (A_DMC_9 LADC data have wrong (even) parity bit) OR (A_DMC_9 LADC have SSM data set to "Normal Operation")) THEN A_DMC_9 LADC Monitor Status = TRUE ELSE IF the following condition is true for 320ms with a tolerance of +10ms ((A_DMC_9 LADC labels have been received at expected update rate with a tolerance of +10ms ) AND (A_DMC_9 LADC data have correct (odd) parity bit) AND (A_DMC_9 LADC have SSM data set to "Normal Operation") AND (A_DMC_9 LADC Airspeed < 35kts)) THEN A_DMC_9 LADC Monitor Status = FALSE ELSE No action (A_DMC_9 LADC Monitor Status remains unchanged) ENDIF ENDIF Where A_DMC_9 LADC ARINC Message is defined in SCU_STC_SRS_135 Justification for timing tolerance: ARINC label reception requires tolerance for label polling and processing. Object Type: Traceable Reqt Source: SES Verification Method: |
| SCU_STC_SRS_99 | The STC Program shall convert the Pilot_SHDL_Operation into a steering angle demand Unlimited_Pilot_SHDL_Command in every Major cycle using the following logic: IF( Pilot_SHDL_Operation >= 54.7deg ) THEN Unlimited_Pilot_SHDL_Command = (26.4 * Pilot_SHDL_Operation - 840.5deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= 44.3deg ) THEN Unlimited_Pilot_SHDL_Command = (16.6 * Pilot_SHDL_Operation - 298.58deg ) * (1/10.4) ELSE IF( Pilot_SHDL_Operation >= 34.7deg ) THEN Unlimited_Pilot_SHDL_Command = (14.4 * Pilot_SHDL_Operation - 205.32deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= 23.7deg ) THEN Unlimited_Pilot_SHDL_Command = (11.6 * Pilot_SHDL_Operation - 110.12deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= 13.3deg ) THEN Unlimited_Pilot_SHDL_Command = (9.2 * Pilot_SHDL_Operation - 51.64deg ) * (1/10.4) ELSE IF( Pilot_SHDL_Operation >= 3deg ) THEN Unlimited_Pilot_SHDL_Command = (Pilot_SHDL_Operation - 3deg ) * (6.8 / 10.3) ELSE IF( Pilot_SHDL_Operation >= -3deg ) THEN Unlimited_Pilot_SHDL_Command = 0deg ELSE IF( Pilot_SHDL_Operation >= -13.3deg ) THEN Unlimited_Pilot_SHDL_Command = (Pilot_SHDL_Operation + 3deg ) * (6.8 / 10.3) ELSE IF( Pilot_SHDL_Operation >= -23.7deg ) THEN Unlimited_Pilot_SHDL_Command = (9.2 * Pilot_SHDL_Operation + 51.64deg ) * (1/10.4) ELSE IF( Pilot_SHDL_Operation >= -34deg ) THEN Unlimited_Pilot_SHDL_Command = (11.6 * Pilot_SHDL_Operation + 110.12deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= -44.3deg ) THEN Unlimited_Pilot_SHDL_Command = (14.4 * Pilot_SHDL_Operation + 205.32deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= -54.7deg ) THEN Unlimited_Pilot_SHDL_Command = (16.6 * Pilot_SHDL_Operation + 298.58deg ) * (1/10.4) END IF Where Pilot_SHDL_Operation is the value calculated in SCU_STC_SRS_68 and Major cycle is defined in Table 6.2-1 Object Type: Traceable Reqt Source: SES Verification Method: |
| SCU_STC_SRS_180 | The STC Program shall debounce the NLG_WOW1 signal using the following algorithm to generate a filtered version of the signal IF (NLG_WOW1 has been TRUE for the last 1.5s) THEN Debounced_NLG_WOW1 = FALSE ELSE IF (NLG_WOW1 has been FALSE for the last 100ms) THEN Debounced_NLG_WOW1 = TRUE ELSE Debounced_NLG_WOW1 remains unchanged, with an initial value of FALSE ENDIF Where NLG_WOW1 is the value calculated in SCU_STC_SRS_139 Object Type: Traceable Reqt Source: SES Verification Method: |
| SCU_STC_SRS_137 | The STC Program shall poll the ARINC receivers for the labels listed in the following table more frequently than the transmission frequency: ARINC Receiver; Label (octal); Transmission Interval (ms). DMCA1; 022; 40.96. DMCA1; 226; 1400. DMCA1; 174 (Right); 100. DMCA1; 206 (ISI); 67. DMCA1; 206 (L ADC); 200. DMCA1; 206 (R ADC); 100. DMCA1; 260; 1000. DMCA1; 107; Aperiodic (polling rate 1000). DMCB1; 022; 40.96. DMCB1; 226; 1400. DMCB1; 174 (Right); 100. DMCB1; 206 (ISI); 67. DMCB1; 206 (L ADC); 100. DMCB1; 206 (R ADC); 200. DMCB1; 260; 1000. ACE1B1; 136; 100. ACE1B2; 136; 10. Table: ARINC Rx Rate Object Type: Traceable Reqt Source: SATA-15-166 Verification Method: |
| SCU_STC_SRS_135 | The STC Program shall receive A_DMC_9 L ADC Airspeed in the Computed Airspeed field in messages from DMCA in the format of Label 206 (ADC) specified in below Table with a Source/Destination Identifier set to Left ADC Bit Position; Field; Description; Definition. 8-1; Label; Octal Label – ARINC429 Message ID; Label = 206 (octal) Bit 1=msb, Bit 8=lsb. 10-9; SDI; Source/Destination Identifier; [00] = Not Used [01] = Left ADC [10] = Right ADC [11] = Not Used. 28-12; Computed Airspeed; Aircraft Speed as calculated by the Air Data Computer (ADC); Range: 0 to 1023.99609375 Units: Knots LSB Resolution: 0.00390625 kts MSB Value: 512 kts Bit 28 = msb; Bit 11 = lsb. 29; Sign; Sign; [0] = Positive [1]= Negative. 31-30; SSM; Sign / Status Matrix Type: BNR (Numeric); [00] = Failure Warning [01] = No Computed Data [10] = Functional Test [11] = Normal Operation. 32; Parity; Parity Bit (Odd); Set to ‘Zero’ or ‘One’ in order to make the total number of Ones in the message an Odd value.. Object Type: Traceable Reqt Source: SATA-15-166 Verification Method: |
| SCU_STC_SRS_1038 | If the read value of first 2 bytes of NVM Sector 0 and NVM Sector 1 are 0xFF then the STC Program shall set the NVM Empty Flag to TRUE, and otherwise set the NVM Empty Flag to FALSE and validate the Rigging data as defined in requirements SCU_STC_SRS_710, SCU_STC_SRS_711, SCU_STC_SRS_1039, SCU_STC_SRS_1041. Justification: An NVM cell value of 0xFF at beginning of the sectors would indicate an erased NVM device. Hence SCU is not rigged, and a comparison of the two sectors would not be applicable as they don’t contain rigging values. Object Type: Derived Reqt Source: Verification Method: |