#### Features

## TMS320F281x Digital Signal Processors

TMS320F2810, TMS320F2810-Q1, TMS320F2811, TMS320F2811-Q1

TMS320F2812, TMS320F2812-Q1

www.ti.com	SPRS174V – APRIL 2001 – REVISED FEBRUARY 2021

128-bit security key/lock

–  Protects flash/OTP and L0/L1 SARAM

TMS320F2810, TMS320F2810-Q1, TMS320F2811, TMS320F2811-Q1

TMS320F2812, TMS320F2812-Q1

SPRS174V – APRIL 2001 – REVISED FEBRUARY 2021

An IMPORTANT NOTICE at the end of this data sheet addresses availability, warranty, changes, use in safety-critical applications, intellectual property matters and other important disclaimers. PRODUCTION DATA.

High-performance static CMOS technology

  - 150 MHz (6.67-ns cycle time)

  - Low-power (1.8-V core at 135 MHz,

1.9-V core at 150 MHz, 3.3-V I/O) design

- JTAG boundary scan support

  - IEEE Standard 1149.1-1990 IEEE Standard Test Access Port and Boundary-Scan Architecture

- High-performance 32-bit CPU (TMS320C28x)

  - 16 × 16 and 32 × 32 MAC operations

  - 16 × 16 dual MAC

  - Harvard bus architecture

  - Atomic operations

  - Fast interrupt response and processing

  - Unified memory programming model

  - 4M linear program/data address reach

  - Code-efficient (in C/C++ and Assembly)

  - TMS320F24x/LF240x processor source code compatible

- On-chip memory

  - Up to 128K × 16 flash

(Four 8K × 16 and six 16K × 16 sectors)

  - 1K × 16 OTP ROM

  - L0 and L1: 2 blocks of 4K × 16 each Single- Access RAM (SARAM)

  - H0: 1 block of 8K × 16 SARAM

  - M0 and M1: 2 blocks of 1K × 16 each SARAM

- Boot ROM (4K × 16)

  - With software boot modes

  - Standard math tables

- External interface (F2812)

  - Over 1M × 16 total memory

  - Programmable wait states

  - Programmable read/write strobe timing

  - Three individual chip selects

- Endianness: Little endian

- Clock and system control

  - On-chip oscillator

  - Watchdog timer module

- Three external interrupts

- Peripheral Interrupt Expansion (PIE) block that supports 45 peripheral interrupts

- Three 32-bit CPU timers

  - Prevents firmware reverse-engineering

- Motor control peripherals

  - Two Event Managers (EVA, EVB)

  - Compatible to 240xA devices

- Serial port peripherals

  - Serial Peripheral Interface (SPI)

  - Two Serial Communications Interfaces (SCIs), standard UART

  - Enhanced Controller Area Network (eCAN)

  - Multichannel Buffered Serial Port (McBSP)

- 12-bit ADC, 16 channels

  - 2 × 8 channel input multiplexer

  - Two Sample-and-Hold

  - Single/simultaneous conversions

  - Fast conversion rate: 80 ns/12.5 MSPS

- Up to 56 General-Purpose I/O (GPIO) pins

- Advanced emulation features

  - Analysis and breakpoint functions

  - Real-time debug via hardware

- Development tools include

  - ANSI C/C++ compiler/assembler/linker

  - Code Composer Studio™ IDE

  - DSP/BIOS™

  - JTAG scan controllers

    - IEEE Standard 1149.1-1990 IEEE Standard Test Access Port and Boundary-Scan Architecture

- Low-power modes and power savings

  - IDLE, STANDBY, HALT modes supported

  - Disable individual peripheral clocks

- Package options

  - 179-ball MicroStar BGA™ with external memory interface (GHH, ZHH) (F2812)

  - 176-pin Low-Profile Quad Flatpack (LQFP) with external memory interface (PGF) (F2812)

  - 128-pin LQFP without external memory interface (PBK) (F2810, F2811)

- Temperature options

  - A: –40°C to 85°C (GHH, ZHH, PGF, PBK)

  - S: –40°C to 125°C (GHH, ZHH, PGF, PBK)

  - Q: –40°C to 125°C (PGF, PBK)

Copyright © 2021 Texas Instruments Incorporated	Submit Document Feedback	1

Product Folder Links: TMS320F2810 TMS320F2810-Q1 TMS320F2811 TMS320F2811-Q1 TMS320F2812 TMS320F2812-Q1

(AEC-Q100 qualification for automotive applications)

#### Applications

- Advanced Driver Assistance Systems (ADAS)

- Building automation

- Electronic point of sale

- Electric Vehicle/Hybrid Electric Vehicle (EV/HEV) powertrain

- Factory automation

#### Description

- Grid infrastructure

- Industrial transport

- Medical, healthcare, and fitness

- Motor drives

- Power delivery

- Telecom infrastructure

- Test and measurement

The TMS320F2810, TMS320F2811, and TMS320F2812-Q1 devices, members of the TMS320C28x DSP generation, are highly integrated, high-performance solutions for demanding control applications.

Throughout this document, TMS320F2810, TMS320F2811, and TMS320F2812-Q1 are abbreviated as F2810, F2811, and F2812-Q1, respectively. F281x denotes all three devices.

Device Information

| PART NUMBER(1) | PACKAGE | BODY SIZE |
| --- | --- | --- |
| TMS320F2812ZHH | MicroStar BGA (179) | 12.0 mm × 12.0 mm |
| TMS320F2812GHH | MicroStar BGA (179) | 12.0 mm × 12.0 mm |
| TMS320F2812PGF | LQFP (176) | 24.0 mm × 24.0 mm |
| TMS320F2811PBK | LQFP (128) | 14.0 mm × 14.0 mm |
| TMS320F2810PBK | LQFP (128) | 14.0 mm × 14.0 mm |

(1)	For more information on these devices, see Mechanical, Packaging, and Orderable Information.

#### Functional Block Diagram

Memory Bus

TINT0

CPU-Timer 0

CPU-Timer 1

Real-Time JTAG

M1 SARAM 1K x 16

M0 SARAM 1K x 16

TINT1

CPU-Timer 2 TINT2

PIE

(A)

(96 Interrupts)

INT14

INT[12:1]

External Interface

(B)

(XINTF)

Control Address (19)

Data (16)

XINT13

XNMI

External Interrupt Control (XINT1/2/13, XNMI)

INT13 NMI

G P I

GPIO Pins   O

M U X

SCIA/SCIB SPI McBSP

eCAN

FIFO FIFO FIFO

L0 SARAM 4K x 16

L1 SARAM 4K x 16

C28x CPU

Flash

128K x 16 (F2812)

128K x 16 (F2811)

64K x 16 (F2810)

EVA/EVB

12-Bit ADC

16 Channels

XRS X1/XCLKIN

X2 XF_XPLLDIS

System Control (Oscillator and PLL

+

Peripheral Clocking

+

Low-Power Modes

+

Watchdog)

RS CLKIN

Boot ROM 4K x 16

H0 SARAM 8K x 16

OTP 1K x 16

Memory Bus

Peripheral Bus

Protected by the code-security module.

- 45 of the possible 96 interrupts are used on the devices.

- XINTF is available on the F2812 device only.

Figure 4-1. Functional Block Diagram

#### Table of Contents

  - Features	1

  - Applications	2

  - Description	2

  - Functional Block Diagram	3

  - Revision History	4

  - Device Comparison	5

    - Related Products	6

  - Terminal Configuration and Functions	7

    - Pin Diagrams	7

    - Signal Descriptions	10

  - Specifications	19

    - Absolute Maximum Ratings	19

    - ESD Ratings – Commercial	20

    - ESD Ratings – Automotive	20

    - Recommended Operating Conditions	21

    - Power Consumption Summary	22

    - Electrical Characteristics	25

    - Thermal Resistance Characteristics for 179-Ball

GHH Package.	25

    - Thermal Resistance Characteristics for 179-Ball

ZHH Package.	25

    - Thermal Resistance Characteristics for 176-Pin

PGF Package.	26

    - Thermal Resistance Characteristics for 128-Pin

PBK Package.	26

    - Thermal Design Considerations	26

    - Timing and Switching Characteristics	27

  - Detailed Description	87

    - Brief Descriptions	87

    - Peripherals	94

    - Memory Maps.	126

    - Register Map	131

    - Device Emulation Registers	133

    - External Interface, XINTF (F2812 Only).	134

    - Interrupts.	136

    - System Control.	140

    - OSC and PLL Block	142

    - PLL-Based Clock Module	144

    - External Reference Oscillator Clock Option	144

    - Watchdog Block	145

    - Low-Power Modes Block	146

  - Applications, Implementation, and Layout	147

    - TI Reference Design	147

  - Device and Documentation Support	148

    - Getting Started.	148

    - Device and Development Support Tool Nomenclature	148

    - Tools and Software	149

    - Documentation Support	150

    - Support Resources	151

    - Trademarks.	152

    - Electrostatic Discharge Caution	152

    - Glossary.	152

  - Mechanical, Packaging, and Orderable Information	153

    - Packaging Information	153

#### Revision History

Changes from July 12, 2019 to January 14, 2021 (from Revision U (July 2019) to Revision V

(January 2021))	Page

- Global: Updated the numbering format for tables, figures and cross-references throughout the document.	1

- Device Comparison: Updated part numbers	5

#### Device Comparison

Table 6-1 provides a summary of each device’s features.

Table 6-1. Device Comparison

| FEATURE(1) |  | TYPE(2) | F2810 F2810-Q1 | F2811 F2811-Q1 | F2812 F2812-Q1 |
| --- | --- | --- | --- | --- | --- |
| Instruction Cycle (at 150 MHz) |  | – | 6.67 ns | 6.67 ns | 6.67 ns |
| Single-Access RAM (SARAM) (16-bit word) |  | – | 18K | 18K | 18K |
| 3.3-V On-Chip Flash (16-bit word) |  | – | 64K | 128K | 128K |
| Code Security for On-Chip Flash/SARAM/OTP |  | – | Yes | Yes | Yes |
| Boot ROM |  | – | Yes | Yes | Yes |
| OTP ROM (1K x 16) |  | – | Yes | Yes | Yes |
| External Memory Interface |  | 0 | – | – | Yes |
| Event Managers A and B (EVA and EVB) |  | – | EVA, EVB | EVA, EVB | EVA, EVB |
| - General-Purpose (GP) Timers |  | – | 4 | 4 | 4 |
| - Compare (CMP)/PWM |  | 0 | 16 | 16 | 16 |
| - Capture (CAP)/QEP Channels |  | 0 | 6/2 | 6/2 | 6/2 |
| Watchdog Timer |  | – | Yes | Yes | Yes |
| 12-Bit ADC |  | 0 | Yes | Yes | Yes |
| - Channels |  |  | 16 | 16 | 16 |
| 32-Bit CPU Timers |  | – | 3 | 3 | 3 |
| Serial Peripheral Interface (SPI) |  | 0 | Yes | Yes | Yes |
| Serial Communications Interfaces A and B (SCIA and SCIB) |  | 0 | SCIA, SCIB | SCIA, SCIB | SCIA, SCIB |
| Controller Area Network (CAN) |  | 0 | Yes | Yes | Yes |
| Multichannel Buffered Serial Port (McBSP) |  | 0 | Yes | Yes | Yes |
| Digital I/O Pins (Shared) |  | – | 56 | 56 | 56 |
| External Interrupts |  | – | 3 | 3 | 3 |
| Supply Voltage |  | – | 1.8-V Core (135 MHz), 1.9-V Core (150 MHz), 3.3-V I/O |  |  |
| Packaging | 128-pin PBK | – | Yes | Yes | – |
|  | 176-pin PGF |  | – | – | Yes |
|  | 179-ball GHH |  | – | – | Yes |
|  | 179-ball ZHH |  | – | – | Yes |
| Temperature Options | A: –40°C to 85°C | – | Yes | Yes | Yes |
|  | S: –40°C to 125°C | – | Yes | Yes | Yes |
|  | Q: –40°C to 125°C (AEC-Q100 Qualification) | – | Yes | Yes | PGF only |

- The TMS320F281x DSPs Silicon Errata has been posted on the Texas Instruments (TI) website. It will be updated as needed.

- A type change represents a major functional feature difference in a peripheral module. Within a peripheral type, there may be minor differences between devices that do not affect the basic functionality of the module. These device-specific differences are listed in the C2000 Real-Time Control Peripherals Reference Guide and in the peripheral reference guides.

###### Related Products

For information about other devices in this family of products, see the following links:

Original Delfino™ series:

TMS320F2833x Delfino™ Microcontrollers

The F2833x series is the original Delfino MCU. It is the first C2000™ MCU that is offered with a floating-point unit (FPU). It has the first-generation ePWM timers that are used throughout the rest of the Delfino and Piccolo™ families. The 12.5-MSPS, 12-bit ADC is still class-leading for an integrated analog-to-digital converter. The F2833x has a 150-MHz CPU and up to 512KB of on-chip Flash. It is available in a 176-pin QFP or 179-ball BGA package.

Newest Delfino™ series:

TMS320F2837xD Delfino™ Microcontrollers

The F2837xD series sets a new standard for performance with dual subsystems. Each subsystem consists of a C28x CPU and a parallel control law accelerator (CLA), each running at 200 MHz. Enhancing performance are TMU and VCU accelerators. New capabilities include multiple 16-bit/12-bit mode ADCs, DAC, Sigma-Delta filters, USB, configurable logic block (CLB), on-chip oscillators, and enhanced versions of all peripherals. The F2837xD is available with up to 1MB of Flash. It is available in a 176-pin QFP or 337-pin BGA package.

TMS320F2837xS Delfino™ Microcontrollers

The F2837xS series is a pin-to-pin compatible version of F2837xD but with only one C28x-CPU-and-CLA subsystem enabled. It is also available in a 100-pin QFP to enable compatibility with the Piccolo™ TMS320F2807x series.

#### Terminal Configuration and Functions

###### Pin Diagrams

Figure 7-1 shows the ball locations for the 179-ball GHH and ZHH ball grid array (BGA) packages. Figure 7-2 shows the pin assignments for the 176-pin PGF low-profile quad flatpack (LQFP) and Figure 7-3 shows the pin assignments for the 128-pin PBK LQFP. Table 7-1 describes the function(s) of each pin.

VDD

XZCS0AND1

PWM10

CAP6_

QEPI2

XD[8]

T3CTRIP_ T4CTRIP/

PDPINTB EVBSOC

XZCS2  SCITXDB

XR/W

T4PWM_

T4CMP

C4TRIP	TEST2	VDD3VFL

XWE	CANTXA CANRXA

XRD	PWM12  CAP4_

QEP3

CAP5_

QEP4

TEST1

PWM11

C5TRIP	VDDIO

XD[10]

XD[12]

SPICLKA

SPISTEA T3PWM_

T3CMP

C6TRIP TCLKINB

XHOLDA

MCLKXA MFSRA

XD[13]

T1PWM_

T1CMP

MCLKRA

MFSXA

CAP1_	CAP2_	CAP3_

QEP1	QEP2	QEPI1

T1CTRIP_

PDPINTA

T2CTRIP/

EVASOC

XMP/MC

C3TRIP XCLKOUT

AVSSREFBG

ADCINA5

XHOLD

XA[13]  C2TRIP

C1TRIP

ADCINB6 ADCINB5 ADCINB4 ADCINA1 ADCINA6	XRS	XA[18]

XINT2_

ADCSOC

XINT1_

XBIO

ADCINB3 ADCINB0 ADCINB1 ADCINA2

SCITXDA

XA[12]	XA[10]

ADCLO ADCINA3 ADCINA7 XREADY  XA[17]

XA[15]

XD[14]	TRST XZCS6AND7

ADCINA0 ADCINA4

SCIRXDA  XA[16]	XD[15]	XA[14]  XF_

XPLLDIS

VSS

VDD

VSS

VDD

PWM8

P

VDDIO

XD[11]

XA[2]

PWM9

SPISOMIA

PWM7

N

PWM2

PWM1

SCIRXDB

VSS

XD[9]

X2

XA[3]

XA[1]

SPISIMOA

M

PWM3

PWM4

VDDIO

TDIRB

VSS

XD[6]

XD[7]

VSS

VDD

L

PWM6

VDD

VSS

X1/ XCLKIN

PWM5

VSS

XD[4]

VSS

K

VSS

XA[4]

XA[5]

T2PWM_ T2CMP

XD[3]

XD[1]

VDDIO

XD[2]

XD[5]

VDD

J	 H

XA[6]

VDD

VSS

VDDIO

XD[0]

VSS

XA[0]

MDRA

MDXA

G

TDIRA

XA[7]

TCLKINA

VSSA1

VDDA1

ADCINB7

ADC- RESEXT

F

VSS

XA[8]

XNMI_ XINT13

VDDIO

ADC- BGREFIN

ADC- REFM

ADC- REFP

AVDDREFBG

E

XA[9]

VDD

TDI

TDO

TMS

VSS

VSS

VSS

VDD

EMU1

VSS

VDD

EMU0

VSS1

VSSA2

ADCINB2

VDDAIO

D	 C	 B

XA[11]

TCK

TESTSEL

VDD1

VDDA2

VSSAIO

A

1	2	3	4	5	6

7	8	9

10	11

12	13	14

Figure 7-1. TMS320F2812 179-Ball GHH/ZHH MicroStar BGA™ (Bottom View)

132

89

133

88

134

135

136

137

138

139

140

141

142

143

144

145

146

147

148

149

150

151

152

153

154

155

156

157

158

159

160

161

162

163

164

165

166

167

168

169

170

171

172

173

174

175

87

86

85

84

83

82

81

80

79

78

77

76

75

74

73

72

71

70

69

68

67

66

65

64

63

62

61

60

59

58

57

56

55

54

53

52

51

50

49

48

47

46

176

45

1

44

XA[11] TDI XA[10]

Vss Vdd TDO

TMS XA[9]

C3TRIP C2TRIP C1TRIP XA[8] VSS

XCLKOUT XA[7] TCLKINA TDIR

T2CTRIP/EVASOC VDDIO

VSS

VDD XA[6]

T1CTRIP_PDPINTA CAP3_QEPI1 XA[5]

CAP2_QEP2 CAP1_QEP1 VSS T2PWM_T2CMP XA[4]

T1PWM_T1CMP PWM6

VDD VSS

PWM5

XD[13]

XD[12] PWM4 PWM3 PWM2 PWM1 SCIRXDB SCITXDB CANRXA

131

130

129

128

127

126

125

124

123

122

121

120

119

118

117

116

115

114

113

112

111

110

109

108

107

106

105

104

103

102

101

100

99

98

97

96

95

94

93

92

91

90

XZCS6AND7

TESTSEL

TRST TCK EMU0 XA[12]

XD[14] XF_XPLLDIS

XA[13]

VSS VDD XA[14] VDDIO EMU1 XD[15]

XA[15] XINT1_XBIO XNMI_XINT13 XINT2_ADCSOC

XA[16] VSS VDD

SCITXDA XA[17]

SCIRXDA XA[18]

XHOLD XRS XREADY

VDD1 VSS1 ADCBGREFIN

Vssa2 Vdda2 ADCINA7

ADCINA6 ADCINA5 ADCINA4 ADCINA3 ADCINA2 ADCINA1 ADCINA0 ADCLO

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

VSSAIO

XZCS2

CANTXA VSS XA[3]

XWE T4CTRIP/EVBSOC XHOLDA

VDDIO

XA[2]

T3CTRIP_PDPINTB VSS

X1/XCLKIN X2

VDD XD[11]

XD[10] TCLKINB TDIRB

VSS

Vdd3vfl XD[9]

TEST1 TEST2 XD[8] VDDIO C6TRIP C5TRIP C4TRIP

CAP6_QEPI2

CAP5_QEP4 VSS CAP4_QEP3 VDD T4PWM_T4CMP XD[7] T3PWM_T3CMP VSS

XR/W PWM12 PWM11 PWM10 PWM9 PWM8 PWM7

VDDAIO ADCINB0 ADCINB1 ADCINB2 ADCINB3 ADCINB4 ADCINB5 ADCINB6 ADCINB7 ADCREFM ADCREFP AVSSREFBG AVDDREFBG

VDDA1 VSSA1

ADCRESEXT

XMP/MC XA[0] VSS MDRA XD[0]

MDXA VDD XD[1]

MCLKRA MFSXA XD[2] MCLKXA

MFSRA XD[3] VDDIO VSS XD[4]

SPICLKA SPISTEA

XD[5]

VDD VSS XD[6]

SPISIMOA SPISOMIA

XRD XA[1]

XZCS0AND1

Figure 7-2. TMS320F2812 176-Pin PGF LQFP (Top View)

96

65

97

64

98

99

100

101

102

103

104

105

106

107

108

109

110

111

112

113

114

115

116

117

118

119

120

121

122

123

124

125

126

127

63

62

61

60

59

58

57

56

55

54

53

52

51

50

49

48

47

46

45

44

43

42

41

40

39

38

37

36

35

34

128

33

1

32

TDI VSS VDD TDO TMS

C3TRIP C2TRIP C1TRIP VSS XCLKOUT

TCLKINA TDIRA

T2CTRIP/EVASOC VDDIO

VDD

T1CTRIP_PDPINTA CAP3_QEPI1 CAP2_QEP2 CAP1_QEP1 T2PWM_T2CMP T1PWM_T1CMP PWM6

Vdd Vss PWM5

PWM4 PWM3 PWM2 PWM1 SCIRXDB SCITXDB CANRXA

95

94

93

92

91

90

89

88

87

86

85

84

83

82

81

80

79

78

77

76

75

74

73

72

71

70

69

68

67

66

TESTSEL

TRST TCK

EMU0 XF_XPLLDIS

Vdd Vss Vddio EMU1

XINT1_XBIO XNMI_XINT13

XINT2_ADCSOC

VSS VDD SCITXDA SCIRXDA

XRS Vdd1 Vss1

ADCBGREFIN

VSSA2 VDDA2 ADCINA7 ADCINA6

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

ADCINA5 ADCINA4 ADCINA3 ADCINA2 ADCINA1 ADCINA0 ADCLO VSSAIO

CANTXA VDD

VSS

T4CTRIP/EVBSOC  T3CTRIP_PDPINTB VSS

X1/XCLKIN

X2 VDD

TCLKINB

TDIRB VSS VDD3VFL TEST1 TEST2 VDDIO  C6TRIP C5TRIP

C4TRIP CAP6_QEPI2 CAP5_QEP4

CAP4_QEP3 VDD T4PWM_T4CMP

T3PWM_T3CMP VSS

PWM12

PWM11 PWM10 PWM9 PWM8

PWM7

VDDAIO ADCINB0 ADCINB1 ADCINB2 ADCINB3 ADCINB4 ADCINB5 ADCINB6 ADCINB7 ADCREFM ADCREFP AVSSREFBG AVDDREFBG

VDDA1 VSSA1 ADCRESEXT

VSS

MDRA MDXA VDD MCLKRA MFSXA MCLKXA MFSRA VDDIO

VSS

SPICLKA SPISTEA

VDD VSS SPISIMOA SPISOMIA

Figure 7-3. TMS320F2810 and TMS320F2811 128-Pin PBK LQFP (Top View)

###### Signal Descriptions

Table 7-1 specifies the signals on the F281x devices. All digital inputs are TTL-compatible. All outputs are 3.3 V with CMOS levels. Inputs are not 5-V tolerant. A 100-µA (or 20-µA) pullup/pulldown is used.

Table 7-1. Signal Descriptions

| NAME | PIN NO. (1) |  |  | I/O/Z(2) | PU/PD(3) | DESCRIPTION |
| --- | --- | --- | --- | --- | --- | --- |
|  | 179-BALL GHH/ZHH | 176-PIN PGF | 128-PIN PBK |  |  |  |
| XINTF SIGNALS (F2812 ONLY) |  |  |  |  |  |  |
| XA[18] | D7 | 158 | – | O/Z | – | 19-bit XINTF Address Bus |
| XA[17] | B7 | 156 | – | O/Z | – |  |
| XA[16] | A8 | 152 | – | O/Z | – |  |
| XA[15] | B9 | 148 | – | O/Z | – |  |
| XA[14] | A10 | 144 | – | O/Z | – |  |
| XA[13] | E10 | 141 | – | O/Z | – |  |
| XA[12] | C11 | 138 | – | O/Z | – |  |
| XA[11] | A14 | 132 | – | O/Z | – |  |
| XA[10] | C12 | 130 | – | O/Z | – |  |
| XA[9] | D14 | 125 | – | O/Z | – |  |
| XA[8] | E12 | 121 | – | O/Z | – |  |
| XA[7] | F12 | 118 | – | O/Z | – |  |
| XA[6] | G14 | 111 | – | O/Z | – |  |
| XA[5] | H13 | 108 | – | O/Z | – |  |
| XA[4] | J12 | 103 | – | O/Z | – |  |
| XA[3] | M11 | 85 | – | O/Z | – |  |
| XA[2] | N10 | 80 | – | O/Z | – |  |
| XA[1] | M2 | 43 | – | O/Z | – |  |
| XA[0] | G5 | 18 | – | O/Z | – |  |
| XD[15] | A9 | 147 | – | I/O/Z | PU | 16-bit XINTF Data Bus |
| XD[14] | B11 | 139 | – | I/O/Z | PU |  |
| XD[13] | J10 | 97 | – | I/O/Z | PU |  |
| XD[12] | L14 | 96 | – | I/O/Z | PU |  |
| XD[11] | N9 | 74 | – | I/O/Z | PU |  |
| XD[10] | L9 | 73 | – | I/O/Z | PU |  |
| XD[9] | M8 | 68 | – | I/O/Z | PU |  |
| XD[8] | P7 | 65 | – | I/O/Z | PU |  |
| XD[7] | L5 | 54 | – | I/O/Z | PU |  |
| XD[6] | L3 | 39 | – | I/O/Z | PU |  |
| XD[5] | J5 | 36 | – | I/O/Z | PU |  |
| XD[4] | K3 | 33 | – | I/O/Z | PU |  |
| XD[3] | J3 | 30 | – | I/O/Z | PU |  |
| XD[2] | H5 | 27 | – | I/O/Z | PU |  |
| XD[1] | H3 | 24 | – | I/O/Z | PU |  |
| XD[0] | G3 | 21 | – | I/O/Z | PU |  |

XMP/ MC

XHOLD

XHOLDA

NAME

179-BALL GHH/ZHH

F1

E7

K10

PIN NO. (1)

176-PIN PGF

17

159

82

128-PIN PBK

–

–

–

I/O/Z(2)

I

I

O/Z

PU/PD(3)

PD

PU

–

DESCRIPTION

Microprocessor/Microcomputer Mode Select. Switches between microprocessor and microcomputer mode. When high, Zone 7 is enabled on the external interface. When low, Zone 7 is disabled from the external interface, and on-chip boot ROM may be accessed instead. This signal is latched into the XINTCNF2 register on a reset and the user can modify this bit in software. The state of the XMP/ MC pin is ignored after reset.

External Hold Request. XHOLD, when active (low), requests the XINTF to release the external bus and place all buses and strobes into a high-impedance state. The XINTF will release the bus when any current access is complete and there are no pending accesses on the XINTF.

External Hold Acknowledge. XHOLDA is driven active (low) when the XINTF has granted a XHOLD request. All XINTF buses and strobe signals will be in a high-impedance state.

XHOLDA is released when the XHOLD signal is released. External devices should only drive the external bus when XHOLDA is active (low).

XINTF Zone 0 and Zone 1 Chip Select.

XZCS0AND1

XZCS2

XZCS6AND7

XWE

XRD

XR/ W

XREADY

P1

P13

B13

N11

M3

N4

B6

44	–

88	–

133	–

84	–

42	–

51	–

161	–

O/Z

O/Z

O/Z

O/Z

O/Z

O/Z

I

- XZCS0AND1 is active (low) when an access to the XINTF Zone 0 or Zone 1 is performed.

XINTF Zone 2 Chip Select. XZCS2 is active

- (low) when an access to the XINTF Zone 2 is performed.

XINTF Zone 6 and Zone 7 Chip Select.

- XZCS6AND7 is active (low) when an access to the XINTF Zone 6 or Zone 7 is performed.

Write Enable. Active-low write strobe. The write

- strobe waveform is specified, per zone basis, by the Lead, Active, and Trail periods in the

XTIMINGx registers.

Read Enable. Active-low read strobe. The read strobe waveform is specified, per zone basis,

- by the Lead, Active, and Trail periods in the XTIMINGx registers. NOTE: The XRD and XWE signals are mutually exclusive.

Read Not Write Strobe. Normally held high. When low, XR/ W indicates write cycle is active;

- when high, XR/ W indicates read cycle is

active.

Ready Signal. Indicates peripheral is ready to complete the access when asserted to 1.

PU	XREADY can be configured to be a synchronous or an asynchronous input. See the timing diagrams for more details.

| NAME |  | PIN NO. (1) |  |  | I/O/Z(2) | PU/PD(3) | DESCRIPTION |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 179-BALL GHH/ZHH | 176-PIN PGF | 128-PIN PBK |  |  |  |
| JTAG AND MISCELLANEOUS SIGNALS |  |  |  |  |  |  |  |
| X1/XCLKIN |  | K9 | 77 | 58 | I | – | Oscillator Input – input to the internal oscillator. This pin is also used to feed an external clock. The 28x can be operated with an external clock source, provided that the proper voltage levels be driven on the X1/XCLKIN pin. It should be noted that the X1/XCLKIN pin is referenced to the 1.8-V (or 1.9-V) core digital power supply (VDD), rather than the 3.3-V I/O supply (VDDIO). A clamping diode may be used to clamp a buffered clock signal to ensure that the logic- high level does not exceed VDD (1.8 V or 1.9 V) or a 1.8-V oscillator may be used. |
| X2 |  | M9 | 76 | 57 | O | – | Oscillator Output |
| XCLKOUT |  | F11 | 119 | 87 | O | – | Output clock derived from SYSCLKOUT to be used for external wait-state generation and as a general-purpose clock source. XCLKOUT is either the same frequency, 1/2 the frequency, or 1/4 the frequency of SYSCLKOUT. At reset, XCLKOUT = SYSCLKOUT/4. The XCLKOUT signal can be turned off by setting bit 3 (CLKOFF) of the XINTCNF2 register to 1. Unlike other GPIO pins, the XCLKOUT pin is not placed in a high-impedance state during reset. |
| TESTSEL |  | A13 | 134 | 97 | I | PD | Test Pin. Reserved for TI. Must be connected to ground. |
|  |  | D6 | 160 | 113 | I/O | PU | Device Reset (in) and Watchdog Reset (out). Device reset. XRS causes the device to terminate execution. The PC will point to the address contained at the location 0x3FFFC0. When XRS is brought to a high level, execution begins at the location pointed to by the PC. This pin is driven low by the DSP when a watchdog reset occurs. During watchdog reset, the XRS pin will be driven low for the watchdog reset duration of 512 XCLKIN cycles. The output buffer of this pin is an open-drain with an internal pullup (100 µA, typical). If this pin is driven by an external device, it should be done using an open-drain device. |
| XRS |  |  |  |  |  |  |  |
| TEST1 |  | M7 | 67 | 51 | I/O | – | Test Pin. Reserved for TI. On F281x devices, TEST1 must be left unconnected. |
| TEST2 |  | N7 | 66 | 50 | I/O | – | Test Pin. Reserved for TI. On F281x devices, TEST2 must be left unconnected. |

| NAME |  | PIN NO. (1) |  |  | I/O/Z(2) | PU/PD(3) | DESCRIPTION |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 179-BALL GHH/ZHH | 176-PIN PGF | 128-PIN PBK |  |  |  |
| JTAG |  |  |  |  |  |  |  |
|  |  | B12 | 135 | 98 | I | PD | JTAG test reset with internal pulldown. TRST, when driven high, gives the scan system control of the operations of the device. If this signal is not connected or driven low, the device operates in its functional mode, and the test reset signals are ignored. NOTE: Do not use pullup resistors on TRST; it has an internal pulldown device. TRST is an active-high test pin and must be maintained low at all times during normal device operation. In a low-noise environment, TRST may be left floating. In other instances, an external pulldown resistor is highly recommended. The value of this resistor should be based on drive strength of the debugger pods applicable to the design. A 2.2- kΩ resistor generally offers adequate protection. Since this is application-specific, it is recommended that each target board be validated for proper operation of the debugger and the application. |
| TRST |  |  |  |  |  |  |  |
| TCK |  | A12 | 136 | 99 | I | PU | JTAG test clock with internal pullup |
| TMS |  | D13 | 126 | 92 | I | PU | JTAG test-mode select (TMS) with internal pullup. This serial control input is clocked into the TAP controller on the rising edge of TCK. |
| TDI |  | C13 | 131 | 96 | I | PU | JTAG test data input (TDI) with internal pullup. TDI is clocked into the selected register (instruction or data) on a rising edge of TCK. |
| TDO |  | D12 | 127 | 93 | O/Z | – | JTAG scan out, test data output (TDO). The contents of the selected register (instruction or data) is shifted out of TDO on the falling edge of TCK. |
| EMU0 |  | D11 | 137 | 100 | I/O/Z | PU | Emulator pin 0. When TRST is driven high, this pin is used as an interrupt to or from the emulator system and is defined as input/output through the JTAG scan. This pin is also used to put the device into boundary-scan mode. With the EMU0 pin at a logic-high state and the EMU1 pin at a logic-low state, a rising edge on the TRST pin would latch the device into boundary-scan mode. NOTE: An external pullup resistor is recommended on this pin. The value of this resistor should be based on the drive strength of the debugger pods applicable to the design. A 2.2-kΩ to 4.7-kΩ resistor is generally adequate. Since this is application- specific, it is recommended that each target board be validated for proper operation of the debugger and the application. |

| NAME | PIN NO. (1) |  |  | I/O/Z(2) | PU/PD(3) | DESCRIPTION |
| --- | --- | --- | --- | --- | --- | --- |
|  | 179-BALL GHH/ZHH | 176-PIN PGF | 128-PIN PBK |  |  |  |
| EMU1 | C9 | 146 | 105 | I/O/Z | PU | Emulator pin 1. When TRST is driven high, this pin is used as an interrupt to or from the emulator system and is defined as input/output through the JTAG scan. This pin is also used to put the device into boundary-scan mode. With the EMU0 pin at a logic-high state and the EMU1 pin at a logic-low state, a rising edge on the TRST pin would latch the device into boundary-scan mode. NOTE: An external pullup resistor is recommended on this pin. The value of this resistor should be based on the drive strength of the debugger pods applicable to the design. A 2.2-kΩ to 4.7-kΩ resistor is generally adequate. Since this is application- specific, it is recommended that each target board be validated for proper operation of the debugger and the application. |
| ADC ANALOG INPUT SIGNALS |  |  |  |  |  |  |
| ADCINA7 | B5 | 167 | 119 | I | – | 8-channel analog inputs for Sample-and-Hold A. The ADC pins should not be driven before the VDDA1, VDDA2, and VDDAIO pins have been fully powered up. |
| ADCINA6 | D5 | 168 | 120 | I | – |  |
| ADCINA5 | E5 | 169 | 121 | I | – |  |
| ADCINA4 | A4 | 170 | 122 | I | – |  |
| ADCINA3 | B4 | 171 | 123 | I | – |  |
| ADCINA2 | C4 | 172 | 124 | I | – |  |
| ADCINA1 | D4 | 173 | 125 | I | – |  |
| ADCINA0 | A3 | 174 | 126 | I | – |  |
| ADCINB7 | F5 | 9 | 9 | I | – | 8-channel analog inputs for Sample-and-Hold B. The ADC pins should not be driven before the VDDA1, VDDA2, and VDDAIO pins have been fully powered up. |
| ADCINB6 | D1 | 8 | 8 | I | – |  |
| ADCINB5 | D2 | 7 | 7 | I | – |  |
| ADCINB4 | D3 | 6 | 6 | I | – |  |
| ADCINB3 | C1 | 5 | 5 | I | – |  |
| ADCINB2 | B1 | 4 | 4 | I | – |  |
| ADCINB1 | C3 | 3 | 3 | I | – |  |
| ADCINB0 | C2 | 2 | 2 | I | – |  |
| ADCREFP | E2 | 11 | 11 | I/O | – | ADC Voltage Reference Output (2 V). Requires a low ESR (under 1.5 Ω) ceramic bypass capacitor of 10 µF to analog ground. [Can accept external reference input (2 V) if the software bit is enabled for this mode. 1–10 µF low ESR capacitor can be used in the external reference mode.] NOTE: Use the ADC Clock rate to derive the ESR specification from the capacitor data sheet that is used in the system. |
| ADCREFM | E4 | 10 | 10 | I/O | – | ADC Voltage Reference Output (1 V). Requires a low ESR (under 1.5 Ω) ceramic bypass capacitor of 10 µF to analog ground. [Can accept external reference input (1 V) if the software bit is enabled for this mode. 1–10 µF low ESR capacitor can be used in the external reference mode.] NOTE: Use the ADC Clock rate to derive the ESR specification from the capacitor data sheet that is used in the system. |

| NAME | PIN NO. (1) |  |  | I/O/Z(2) | PU/PD(3) | DESCRIPTION |
| --- | --- | --- | --- | --- | --- | --- |
|  | 179-BALL GHH/ZHH | 176-PIN PGF | 128-PIN PBK |  |  |  |
| ADCRESEXT | F2 | 16 | 16 | O | – | ADC External Current Bias Resistor. Use 24.9 kΩ ± 5% for ADC clock range 1–18.75 MHz; use 20 kΩ ± 5% for ADC clock range 18.75 MHz–25 MHz. |
| ADCBGREFIN | E6 | 164 | 116 | – | – | Test Pin. Reserved for TI. Must be left unconnected. |
| AVSSREFBG | E3 | 12 | 12 | – | – | ADC Analog GND |
| AVDDREFBG | E1 | 13 | 13 | – | – | ADC Analog Power (3.3-V) |
| ADCLO | B3 | 175 | 127 | – | – | Common Low Side Analog Input. Connect to analog ground. |
| VSSA1 | F3 | 15 | 15 | – | – | ADC Analog GND |
| VSSA2 | C5 | 165 | 117 | – | – | ADC Analog GND |
| VDDA1 | F4 | 14 | 14 | – | – | ADC Analog 3.3-V Supply |
| VDDA2 | A5 | 166 | 118 | – | – | ADC Analog 3.3-V Supply |
| VSS1 | C6 | 163 | 115 | – | – | ADC Digital GND |
| VDD1 | A6 | 162 | 114 | – | – | ADC Digital 1.8-V (or 1.9-V) Supply |
| VDDAIO | B2 | 1 | 1 | – | – | 3.3-V Analog I/O Power Pin |
| VSSAIO | A2 | 176 | 128 | – | – | Analog I/O Ground Pin |
| POWER SIGNALS |  |  |  |  |  |  |
| VDD | H1 | 23 | 20 | – | – | 1.8-V or 1.9-V Core Digital Power Pins. See Section 8.4, Recommended Operating Conditions, for voltage requirements. |
| VDD | L1 | 37 | 29 | – | – |  |
| VDD | P5 | 56 | 42 | – | – |  |
| VDD | P9 | 75 | 56 | – | – |  |
| VDD | P12 | – | 63 | – | – |  |
| VDD | K12 | 100 | 74 | – | – |  |
| VDD | G12 | 112 | 82 | – | – |  |
| VDD | C14 | 128 | 94 | – | – |  |
| VDD | B10 | 143 | 102 | – | – |  |
| VDD | C8 | 154 | 110 | – | – |  |
| VSS | G4 | 19 | 17 | – | – | Core and Digital I/O Ground Pins |
| VSS | K1 | 32 | 26 | – | – |  |
| VSS | L2 | 38 | 30 | – | – |  |
| VSS | P4 | 52 | 39 | – | – |  |
| VSS | K6 | 58 | – | – | – |  |
| VSS | P8 | 70 | 53 | – | – |  |
| VSS | M10 | 78 | 59 | – | – |  |
| VSS | L11 | 86 | 62 | – | – |  |
| VSS | K13 | 99 | 73 | – | – |  |
| VSS | J14 | 105 | – | – | – |  |
| VSS | G13 | 113 | – | – | – |  |
| VSS | E14 | 120 | 88 | – | – |  |
| VSS | B14 | 129 | 95 | – | – |  |
| VSS | D10 | 142 | – | – | – |  |
| VSS | C10 | – | 103 | – | – |  |
| VSS | B8 | 153 | 109 | – | – |  |

| NAME | PIN NO. (1) |  |  | I/O/Z(2) | PU/PD(3) | DESCRIPTION |
| --- | --- | --- | --- | --- | --- | --- |
|  | 179-BALL GHH/ZHH | 176-PIN PGF | 128-PIN PBK |  |  |  |
| VDDIO | J4 | 31 | 25 | – | – | 3 3-V I/O Digital Power Pins |
| VDDIO | L7 | 64 | 49 | – | – |  |
| VDDIO | L10 | 81 | – | – | – |  |
| VDDIO | N14 | – | – | – | – |  |
| VDDIO | G11 | 114 | 83 | – | – |  |
| VDDIO | E9 | 145 | 104 | – | – |  |
| VDD3VFL | N8 | 69 | 52 | – | – | 3.3-V Flash Core Power Pin. This pin should be connected to 3.3 V at all times after power-up sequence requirements have been met. |
| GPIO OR PERIPHERAL SIGNALS |  |  |  |  |  |  |
| GPIOA OR EVA SIGNALS |  |  |  |  |  |  |
| GPIOA0 - PWM1 (O) | M12 | 92 | 68 | I/O | PU | GPIO or PWM Output Pin #1 |
| GPIOA1 - PWM2 (O) | M14 | 93 | 69 | I/O | PU | GPIO or PWM Output Pin #2 |
| GPIOA2 - PWM3 (O) | L12 | 94 | 70 | I/O | PU | GPIO or PWM Output Pin #3 |
| GPIOA3 - PWM4 (O) | L13 | 95 | 71 | I/O | PU | GPIO or PWM Output Pin #4 |
| GPIOA4 - PWM5 (O) | K11 | 98 | 72 | I/O | PU | GPIO or PWM Output Pin #5 |
| GPIOA5 - PWM6 (O) | K14 | 101 | 75 | I/O | PU | GPIO or PWM Output Pin #6 |
| GPIOA6 - T1PWM_T1CMP (I) | J11 | 102 | 76 | I/O | PU | GPIO or Timer 1 Output |
| GPIOA7 - T2PWM_T2CMP (I) | J13 | 104 | 77 | I/O | PU | GPIO or Timer 2 Output |
| GPIOA8 - CAP1_QEP1 (I) | H10 | 106 | 78 | I/O | PU | GPIO or Capture Input #1 |
| GPIOA9 - CAP2_QEP2 (I) | H11 | 107 | 79 | I/O | PU | GPIO or Capture Input #2 |
| GPIOA10 - CAP3_QEPI1 (I) | H12 | 109 | 80 | I/O | PU | GPIO or Capture Input #3 |
| GPIOA11 - TDIRA (I) | F14 | 116 | 85 | I/O | PU | GPIO or Timer Direction |
| GPIOA12 - TCLKINA (I) | F13 | 117 | 86 | I/O | PU | GPIO or Timer Clock Input |
| GPIOA13 - C1TRIP (I) | E13 | 122 | 89 | I/O | PU | GPIO or Compare 1 Output Trip |
| GPIOA14 - C2TRIP (I) | E11 | 123 | 90 | I/O | PU | GPIO or Compare 2 Output Trip |
| GPIOA15 - C3TRIP (I) | F10 | 124 | 91 | I/O | PU | GPIO or Compare 3 Output Trip |
| GPIOB OR EVB SIGNALS |  |  |  |  |  |  |
| GPIOB0 - PWM7 (O) | N2 | 45 | 33 | I/O | PU | GPIO or PWM Output Pin #7 |
| GPIOB1 - PWM8 (O) | P2 | 46 | 34 | I/O | PU | GPIO or PWM Output Pin #8 |
| GPIOB2 - PWM9 (O) | N3 | 47 | 35 | I/O | PU | GPIO or PWM Output Pin #9 |
| GPIOB3 - PWM10 (O) | P3 | 48 | 36 | I/O | PU | GPIO or PWM Output Pin #10 |
| GPIOB4 - PWM11 (O) | L4 | 49 | 37 | I/O | PU | GPIO or PWM Output Pin #11 |
| GPIOB5 - PWM12 (O) | M4 | 50 | 38 | I/O | PU | GPIO or PWM Output Pin #12 |
| GPIOB6 - T3PWM_T3CMP (I) | K5 | 53 | 40 | I/O | PU | GPIO or Timer 3 Output |
| GPIOB7 - T4PWM_T4CMP (I) | N5 | 55 | 41 | I/O | PU | GPIO or Timer 4 Output |
| GPIOB8 - CAP4_QEP3 (I) | M5 | 57 | 43 | I/O | PU | GPIO or Capture Input #4 |
| GPIOB9 - CAP5_QEP4 (I) | M6 | 59 | 44 | I/O | PU | GPIO or Capture Input #5 |
| GPIOB10 - CAP6_QEPI2 (I) | P6 | 60 | 45 | I/O | PU | GPIO or Capture Input #6 |
| GPIOB11 - TDIRB (I) | L8 | 71 | 54 | I/O | PU | GPIO or Timer Direction |
| GPIOB12 - TCLKINB (I) | K8 | 72 | 55 | I/O | PU | GPIO or Timer Clock Input |
| GPIOB13 - C4TRIP (I) | N6 | 61 | 46 | I/O | PU | GPIO or Compare 4 Output Trip |

| NAME |  |  | PIN NO. (1) |  |  | I/O/Z(2) | PU/PD(3) | DESCRIPTION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 179-BALL GHH/ZHH | 176-PIN PGF | 128-PIN PBK |  |  |  |
| GPIOB14 - C5TRIP (I) |  |  | L6 | 62 | 47 | I/O | PU | GPIO or Compare 5 Output Trip |
| GPIOB15 - C6TRIP (I) |  |  | K7 | 63 | 48 | I/O | PU | GPIO or Compare 6 Output Trip |
| GPIOD OR EVA SIGNALS |  |  |  |  |  |  |  |  |
| GPIOD0 - |  |  | H14 | 110 | 81 | I/O | PU | GPIO or Timer 1 Compare Output Trip |
| T1CTRIP_PDPINTA (I) |  |  |  |  |  |  |  |  |
| GPIOD1 | - T2CTRIP/ |  | G10 | 115 | 84 | I/O | PU | GPIO or Timer 2 Compare Output Trip or External ADC Start-of-Conversion EV-A |
| EVASOC (I) |  |  |  |  |  |  |  |  |
| GPIOD OR EVB SIGNALS |  |  |  |  |  |  |  |  |
| GPIOD5 - |  | (I) | P10 | 79 | 60 | I/O | PU | GPIO or Timer 3 Compare Output Trip |
| T3CTRIP_PDPINTB |  |  |  |  |  |  |  |  |
| GPIOD6 | - T4CTRIP/ (I) |  | P11 | 83 | 61 | I/O | PU | GPIO or Timer 4 Compare Output Trip or External ADC Start-of-Conversion EV-B |
| EVBSOC |  |  |  |  |  |  |  |  |
| GPIOE OR INTERRUPT SIGNALS |  |  |  |  |  |  |  |  |
| GPIOE0 - XINT1_ XBIO (I) |  |  | D9 | 149 | 106 | I/O/Z | – | GPIO or XINT1 or XBIO input |
| GPIOE1 - XINT2_ADCSOC (I) |  |  | D8 | 151 | 108 | I/O/Z | – | GPIO or XINT2 or ADC start-of-conversion |
| GPIOE2 - XNMI_XINT13 (I) |  |  | E8 | 150 | 107 | I/O | PU | GPIO or XNMI or XINT13 |
| GPIOF OR SPI SIGNALS |  |  |  |  |  |  |  |  |
| GPIOF0 - SPISIMOA (O) |  |  | M1 | 40 | 31 | I/O/Z | – | GPIO or SPI slave in, master out |
| GPIOF1 - SPISOMIA (I) |  |  | N1 | 41 | 32 | I/O/Z | – | GPIO or SPI slave out, master in |
| GPIOF2 - SPICLKA (I/O) |  |  | K2 | 34 | 27 | I/O/Z | – | GPIO or SPI clock |
| GPIOF3 - SPISTEA (I/O) |  |  | K4 | 35 | 28 | I/O/Z | – | GPIO or SPI slave transmit enable |
| GPIOF OR SCI-A SIGNALS |  |  |  |  |  |  |  |  |
| GPIOF4 - SCITXDA (O) |  |  | C7 | 155 | 111 | I/O | PU | GPIO or SCI asynchronous serial port TX data |
| GPIOF5 - SCIRXDA (I) |  |  | A7 | 157 | 112 | I/O | PU | GPIO or SCI asynchronous serial port RX data |
| GPIOF OR CAN SIGNALS |  |  |  |  |  |  |  |  |
| GPIOF6 - CANTXA (O) |  |  | N12 | 87 | 64 | I/O | PU | GPIO or eCAN transmit data |
| GPIOF7 - CANRXA (I) |  |  | N13 | 89 | 65 | I/O | PU | GPIO or eCAN receive data |
| GPIOF OR McBSP SIGNALS |  |  |  |  |  |  |  |  |
| GPIOF8 - MCLKXA (I/O) |  |  | J1 | 28 | 23 | I/O | PU | GPIO or McBSP transmit clock |
| GPIOF9 - MCLKRA (I/O) |  |  | H2 | 25 | 21 | I/O | PU | GPIO or McBSP receive clock |
| GPIOF10 - MFSXA (I/O) |  |  | H4 | 26 | 22 | I/O | PU | GPIO or McBSP transmit frame synch |
| GPIOF11 - MFSRA (I/O) |  |  | J2 | 29 | 24 | I/O | PU | GPIO or McBSP receive frame synch |
| GPIOF12 - MDXA (O) |  |  | G1 | 22 | 19 | I/O | – | GPIO or McBSP transmitted serial data |
| GPIOF13 - MDRA (I) |  |  | G2 | 20 | 18 | I/O | PU | GPIO or McBSP received serial data |

| NAME |  | PIN NO. (1) |  |  | I/O/Z(2) | PU/PD(3) | DESCRIPTION |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 179-BALL GHH/ZHH | 176-PIN PGF | 128-PIN PBK |  |  |  |
| GPIOF OR XF CPU OUTPUT SIGNAL |  |  |  |  |  |  |  |
| GPIOF14 | - XF_ (O) | A11 | 140 | 101 | I/O | PU | This pin has three functions: - XF – General-purpose output pin. - XPLLDIS – This pin is sampled during reset to check whether the PLL must be disabled. The PLL will be disabled if this pin is sensed low. HALT and STANDBY modes cannot be used when the PLL is disabled. - GPIO – GPIO function |
| XPLLDIS |  |  |  |  |  |  |  |
| GPIOG OR SCI-B SIGNALS |  |  |  |  |  |  |  |
| GPIOG4 - SCITXDB (O) |  | P14 | 90 | 66 | I/O/Z | – | GPIO or SCI asynchronous serial port transmit data |
| GPIOG5 - SCIRXDB (I) |  | M13 | 91 | 67 | I/O/Z | – | GPIO or SCI asynchronous serial port receive data |

- Typical drive strength of the output buffer for all pins is 4 mA except for TDO, XCLKOUT, XF, XINTF, EMU0, and EMU1 pins, which are 8 mA.

- I = Input, O = Output, Z = High impedance

- PU = pin has internal pullup; PD = pin has internal pulldown. Pullup/pulldown strength is given in Section 8.6, Electrical Characteristics Over Recommended Operating Conditions. The pullups/pulldowns are enabled in boundary scan mode.

Note

Other than the power supply pins, no pin should be driven before the 3.3-V rail has reached recommended operating conditions. However, it is acceptable for an I/O pin to ramp along with the 3.3-V supply.

#### Specifications

###### Absolute Maximum Ratings

over operating temperature ranges (unless otherwise noted)(1)

|  |  | MIN | MAX | UNIT |
| --- | --- | --- | --- | --- |
| Supply voltage | VDDIO | –0.3 | 4.6 | V |
|  | VDD3VFL | –0.3 | 4.6 |  |
|  | VDDA1 | –0.3 | 4.6 |  |
|  | VDDA2 | –0.3 | 4.6 |  |
|  | VDDAIO | –0.3 | 4.6 |  |
|  | AVDDREFBG | –0.3 | 4.6 |  |
| Supply voltage | VDD | –0.5 | 2.5 | V |
|  | VDD1 | –0.5 | 2.5 |  |
| Input voltage | VIN | –0.3 | 4.6 | V |
| Output voltage | VO | –0.3 | 4.6 | V |
| Input clamp current | IIK (VIN < 0 or VIN > VDDIO)(2) | –20 | 20 | mA |
| Output clamp current | IOK (VO < 0 or VO > VDDIO) | –20 | 20 | mA |
| Operating ambient temperature, TA | A version (GHH, ZHH, PGF, PBK)(3) | –40 | 85 | °C |
|  | S version (GHH, ZHH, PGF, PBK)(3) | –40 | 125 |  |
|  | Q version (PGF, PBK)(3) | –40 | 125 |  |
| Junction temperature | TJ | –40 | 150 | °C |
| Storage temperature | Tstg (3) | –65 | 150 | °C |

- Stresses beyond those listed under Absolute Maximum Ratings may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated under Section 8.4 is not implied. Exposure to absolute-maximum-rated conditions for extended periods may affect device reliability. All voltage values are with respect to VSS.

- Continuous clamp current per pin is ±2 mA

- Long-term high-temperature storage and/or extended use at maximum temperature conditions may result in a reduction of overall device life. For additional information, see the Semiconductor and IC Package Thermal Metrics Application Report.

###### ESD Ratings – Commercial

|  |  | VALUE | UNIT |
| --- | --- | --- | --- |
| TMS320F2812 in 179-ball ZHH package |  |  |  |
| V(ESD) Electrostatic discharge (ESD) | Human-body model (HBM), per ANSI/ESDA/JEDEC JS-001(1) | ±2000 | V |
|  | Charged-device model (CDM), per JEDEC specification JESD22- C101(2) | ±500 |  |
| TMS320F2812 in 179-ball GHH package |  |  |  |
| V(ESD) Electrostatic discharge (ESD) | Human-body model (HBM), per ANSI/ESDA/JEDEC JS-001(1) | ±2000 | V |
|  | Charged-device model (CDM), per JEDEC specification JESD22- C101(2) | ±500 |  |

- JEDEC document JEP155 states that 500-V HBM allows safe manufacturing with a standard ESD control process.

- JEDEC document JEP157 states that 250-V CDM allows safe manufacturing with a standard ESD control process.

###### ESD Ratings – Automotive

|  |  |  | VALUE | UNIT |
| --- | --- | --- | --- | --- |
| TMS320F2812 in 176-pin PGF package |  |  |  |  |
| V(ESD) Electrostatic discharge | Human body model (HBM), per AEC Q100-002(1) | All pins | ±2000 | V |
|  | Charged device model (CDM), per AEC Q100-011 | All pins | ±500 |  |
|  |  | Corner pins on 176-pin PGF: 1, 44, 45, 88, 89, 132, 133, 176 | ±750 |  |
| TMS320F2810 and TMS320F2811 in 128-pin PBK package |  |  |  |  |
| V(ESD) Electrostatic discharge | Human body model (HBM), per AEC Q100-002(1) | All pins | ±2000 | V |
|  | Charged device model (CDM), per AEC Q100-011 | All pins | ±500 |  |
|  |  | Corner pins on 128-pin PBK: 1, 32, 33, 64, 65, 96, 97, 128 | ±750 |  |

(1)	AEC Q100-002 indicates HBM stressing is done in accordance with the ANSI/ESDA/JEDEC JS-001 specification.

###### Recommended Operating Conditions

|  |  | MIN(1) | NOM | MAX | UNIT |
| --- | --- | --- | --- | --- | --- |
| Device supply voltage, I/O, VDDIO |  | 3.14 | 3.3 | 3.47 | V |
| Device supply voltage, CPU, VDD, VDD1 | 1.8 V (135 MHz) | 1.71 | 1.8 | 1.89 | V |
|  | 1.9 V (150 MHz) | 1.81 | 1.9 | 2 |  |
| Supply ground, VSS |  | 0 |  |  | V |
| ADC supply voltage, VDDA1, VDDA2, AVDDREFBG, VDDAIO |  | 3.14 | 3.3 | 3.47 | V |
| Flash programming supply voltage, VDD3VFL |  | 3.14 | 3.3 | 3.47 | V |
| Device clock frequency (system clock), fSYSCLKOUT | VDD = 1.9 V ± 5% | 2 |  | 150 | MHz |
|  | VDD = 1.8 V ± 5% | 2 |  | 135 |  |
| High-level input voltage, VIH | All inputs except X1/XCLKIN | 2 |  | VDDIO | V |
|  | X1/XCLKIN (@ 50 µA max) | 0.7VDD |  | VDD |  |
| Low-level input voltage, VIL | All inputs except X1/XCLKIN | 0.8 |  |  | V |
|  | X1/XCLKIN (@ 50 µA max) | 0.3VDD |  |  |  |
| High-level output source current, VOH = 2.4 V, IOH | All I/Os except Group 2 | –4 |  |  | mA |
|  | Group 2(2) | –8 |  |  |  |
| Low-level output sink current, VOL = VOL MAX, IOL | All I/Os except Group 2 | 4 |  |  | mA |
|  | Group 2(2) | 8 |  |  |  |
| Ambient temperature, TA | A version | –40 |  | 85 | °C |
|  | S version | –40 |  | 125 |  |
|  | Q version | –40 |  | 125 |  |

- See Section 8.12.2 for power sequencing of VDDIO, VDDAIO, VDD, VDDA1/VDDA2/AVDDREFBG, and VDD3VFL.

- Group 2 pins are as follows: XINTF pins, T1CTRIP_PDPINTA, TDO, XCLKOUT, XF, EMU0, and EMU1.

###### Power Consumption Summary

    - TMS320F281x Current Consumption by Power-Supply Pins Over Recommended Operating Conditions During Low-Power Modes at 150-MHz SYSCLKOUT

| MODE | TEST CONDITIONS | IDD |  | IDDIO (1) |  | IDD3VFL |  | IDDA (2) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TYP | MAX(3) | TYP | MAX(3) | TYP | MAX(3) | TYP | MAX(3) |
| Operational | All peripheral clocks are enabled. All PWM pins are toggled at 100 kHz. Data is continuously transmitted out of the SCIA, SCIB, and CAN ports. The hardware multiplier is exercised. Code is running out of flash with 5 wait-states. | 195 mA(4) | 230 mA | 15 mA | 30 mA | 40 mA | 45 mA | 40 mA | 50 mA |
| IDLE | - Flash is powered down - XCLKOUT is turned off - All peripheral clocks are on, except ADC | 125 mA | 150 mA | 5 mA | 10 mA | 2 µA | 4 µA | 1 µA | 20 µA |
| STANDBY | - Flash is powered down - Peripheral clocks are turned off - Pins without an internal PU/PD are tied high/low | 5 mA | 10 mA | 5 µA | 20 µA | 2 µA | 4 µA | 1 µA | 20 µA |
| HALT | - Flash is powered down - Peripheral clocks are turned off - Pins without an internal PU/PD are tied high/low - Input clock is disabled | 70 µA |  | 5 µA | 20 µA | 2 µA | 4 µA | 1 µA | 20 µA |

- IDDIO current is dependent on the electrical loading on the I/O pins.

- IDDA includes current into VDDA1, VDDA2, AVDDREFBG, and VDDAIO pins.

- MAX numbers are at 125°C, and MAX voltage (VDD = 1.89 V; VDDIO, VDD3VFL, VDDA = 3.47 V).

- IDD represents the total current drawn from the 1.8-V rail (VDD). It includes a small amount of current (<1 mA) drawn by VDD1.

Note

HALT and STANDBY modes cannot be used when the PLL is disabled.

    - Current Consumption Graphs

250

200

Current (mA)

150

100

50

0

0	20	40	60	80	100	120	140	160

SYSCLKOUT (MHz)

IDD	 IDDIO	 IDD3VFL	 IDDA	 Total 3.3-V current

- Test conditions are as defined in Section 8.5.1 for operational currents.

- IDD represents the total current drawn from the 1.8-V rail (VDD). It includes a small amount of current (<1 mA) drawn by VDD1.

- IDDA represents the current drawn by VDDA1 and VDDA2 rails.

- Total 3.3-V current is the sum of IDDIO, IDD3VFL, and IDDA. It includes a small amount of current (<1 mA) drawn by VDDAIO.

Figure 8-1. Typical Current Consumption Over Frequency

700

600

500

Power (mW)

400

300

200

100

0

0	20	40	60	80	100	120	140	160

SYSCLKOUT (MHz)

Total Power

Figure 8-2. Typical Power Consumption Over Frequency

    - Reducing Current Consumption

28x DSPs incorporate a unique method to reduce the device current consumption. A reduction in current consumption can be achieved by turning off the clock to any peripheral module which is not used in a given application. Table 8-1 indicates the typical reduction in current consumption achieved by turning off the clocks to various peripherals.

Table 8-1. Typical Current Consumption by Various Peripherals (at 150 MHz)

| PERIPHERAL MODULE(1) (3) | IDD CURRENT REDUCTION (mA) |
| --- | --- |
| eCAN | 12 |
| EVA | 6 |
| EVB | 6 |
| ADC | 8(2) |
| SCI | 4 |
| SPI | 5 |
| McBSP | 13 |

- All peripheral clocks are disabled upon reset. Writing to/reading from peripheral registers is possible only after the peripheral clocks are turned on.

- This number represents the current drawn by the digital portion of the ADC module. Turning off the clock to the ADC module results in the elimination of the current drawn by the analog portion of the ADC (IDDA) as well.

- Power savings can be achieved by powering down the flash. This must be done by code running off RAM (not flash).

###### Electrical Characteristics

over recommended operating conditions (unless otherwise noted)

| PARAMETER |  |  | TEST CONDITIONS |  | MIN | TYP | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VOH | High-level output voltage |  | IOH = IOH MAX |  | 2.4 |  |  | V |
|  |  |  | IOH = 50 µA |  | VDDIO – 0.2 |  |  |  |
| VOL | Low-level output voltage |  | IOL = IOL MAX |  | 0.4 |  |  | V |
| IIL | Input current | With pullup | VDDIO = 3.3 V, | All I/Os(1) (including | –80 | –140 | –190 | µA |
|  | (low level) |  | VIN = 0 V |  |  |  |  |  |
|  |  |  |  | XRS) except EVB |  |  |  |  |
|  |  |  |  | GPIOB/EVB | –13 | –25 | –35 |  |
|  |  | With pulldown | VDDIO = 3.3 V, VIN = 0 V |  | ±2 |  |  |  |
| IIH | Input current (high level) | With pullup | VDDIO = 3.3 V, VIN = VDD |  | ±2 |  |  | µA |
|  |  | With pulldown(2) | VDDIO = 3.3 V, VIN = VDD |  | 28 | 50 | 80 |  |
| IOZ | Leakage current (for pins without internal PU/ PD), high-impedance state (off-state) |  | VO = VDDIO or 0 V |  | ±2 |  |  | µA |
| Ci | Input capacitance |  |  |  | 2 |  |  | pF |
| Co | Output capacitance |  |  |  | 3 |  |  | pF |

- The following pins have no internal PU/PD: GPIOE0, GPIOE1, GPIOF0, GPIOF1, GPIOF2, GPIOF3, GPIOF12, GPIOG4, and GPIOG5.

- The following pins have an internal pulldown: XMP/ MC, TESTSEL, and TRST.

###### Thermal Resistance Characteristics for 179-Ball GHH Package

|  |  | °C/W(1) |
| --- | --- | --- |
| RΘJC | Junction-to-case thermal resistance | 16.08 |
| RΘJA | Junction-to-free air thermal resistance | 42.57 |
| PsiJT | Junction-to-package top | 0.658 |

- These values are based on a JEDEC defined 2S2P system (with the exception of the Theta JC [RΘJC] value, which is based on a JEDEC defined 1S0P system) and will change based on environment as well as application. For more information, see these EIA/ JEDEC standards:

  - JESD51-2, Integrated Circuits Thermal Test Method Environmental Conditions - Natural Convection (Still Air)

  - JESD51-3, Low Effective Thermal Conductivity Test Board for Leaded Surface Mount Packages

  - JESD51-7, High Effective Thermal Conductivity Test Board for Leaded Surface Mount Packages

  - JESD51-9, Test Boards for Area Array Surface Mount Package Thermal Measurements

###### Thermal Resistance Characteristics for 179-Ball ZHH Package

|  |  | °C/W(1) |
| --- | --- | --- |
| RΘJC | Junction-to-case thermal resistance | 16.08 |
| RΘJA | Junction-to-free air thermal resistance | 42.57 |
| PsiJT | Junction-to-package top | 0.658 |

- These values are based on a JEDEC defined 2S2P system (with the exception of the Theta JC [RΘJC] value, which is based on a JEDEC defined 1S0P system) and will change based on environment as well as application. For more information, see these EIA/ JEDEC standards:

  - JESD51-2, Integrated Circuits Thermal Test Method Environmental Conditions - Natural Convection (Still Air)

  - JESD51-3, Low Effective Thermal Conductivity Test Board for Leaded Surface Mount Packages

  - JESD51-7, High Effective Thermal Conductivity Test Board for Leaded Surface Mount Packages

  - JESD51-9, Test Boards for Area Array Surface Mount Package Thermal Measurements

###### Thermal Resistance Characteristics for 176-Pin PGF Package

|  |  | °C/W(1) |
| --- | --- | --- |
| RΘJC | Junction-to-case thermal resistance | 9.73 |
| RΘJA | Junction-to-free air thermal resistance | 41.88 |
| PsiJT | Junction-to-package top | 0.247 |

- These values are based on a JEDEC defined 2S2P system (with the exception of the Theta JC [RΘJC] value, which is based on a JEDEC defined 1S0P system) and will change based on environment as well as application. For more information, see these EIA/ JEDEC standards:

  - JESD51-2, Integrated Circuits Thermal Test Method Environmental Conditions - Natural Convection (Still Air)

  - JESD51-3, Low Effective Thermal Conductivity Test Board for Leaded Surface Mount Packages

  - JESD51-7, High Effective Thermal Conductivity Test Board for Leaded Surface Mount Packages

  - JESD51-9, Test Boards for Area Array Surface Mount Package Thermal Measurements

###### Thermal Resistance Characteristics for 128-Pin PBK Package

|  |  | °C/W(1) |
| --- | --- | --- |
| RΘJC | Junction-to-case thermal resistance | 10.76 |
| RΘJA | Junction-to-free air thermal resistance | 41.65 |
| PsiJT | Junction-to-package top | 0.271 |

- These values are based on a JEDEC defined 2S2P system (with the exception of the Theta JC [RΘJC] value, which is based on a JEDEC defined 1S0P system) and will change based on environment as well as application. For more information, see these EIA/ JEDEC standards:

  - JESD51-2, Integrated Circuits Thermal Test Method Environmental Conditions - Natural Convection (Still Air)

  - JESD51-3, Low Effective Thermal Conductivity Test Board for Leaded Surface Mount Packages

  - JESD51-7, High Effective Thermal Conductivity Test Board for Leaded Surface Mount Packages

  - JESD51-9, Test Boards for Area Array Surface Mount Package Thermal Measurements

###### Thermal Design Considerations

Based on the end application design and operational profile, the IDD and IDDIO currents could vary. Systems that exceed the recommended maximum power dissipation in the end product may require additional thermal enhancements. Ambient temperature (TA) varies with the end application and product design. The critical factor that affects reliability and functionality is TJ, the junction temperature, not the ambient temperature. Hence, care should be taken to keep TJ within the specified limits. Tcase should be measured to estimate the operating junction temperature TJ. Tcase is normally measured at the center of the package top-side surface. The thermal application report Semiconductor and IC Package Thermal Metrics helps to understand the thermal metrics and definitions.

###### Timing and Switching Characteristics

    - Timing Parameter Symbology

Timing parameter symbols used are created in accordance with JEDEC Standard 100. To shorten the symbols, some of the pin names and other related terminology have been abbreviated as follows:

Lowercase subscripts and their meanings:

Letters and symbols and their meanings:

a	access time	H	High

c	cycle time (period)	L	Low

d	delay time	V	Valid

f	fall time	X	Unknown, changing, or don't care level

h	hold time	Z	High impedance r	rise time

su	setup time

t	transition time

- valid time

- pulse duration (width)

      - General Notes on Timing Parameters

All output signals from the 28x devices (including XCLKOUT) are derived from an internal clock such that all output transitions for a given half-cycle occur with a minimum of skewing relative to each other.

The signal combinations shown in the following timing diagrams may not necessarily represent actual cycles. For actual cycle examples, see the appropriate cycle description section of this document.

      - Test Load Circuit

This test load circuit is used to measure all switching characteristics provided in this document.

4.0 pF

1.85 pF

Transmission Line

Tester Pin Electronics

Data Sheet Timing Reference Point

42 	3.5 nH



(A)

Z0 = 50

Output Under Test

Device Pin

(A)

A. The data sheet provides timing at the device pin. For output timing analysis, the tester pin electronics and its transmission line effects must be taken into account. A transmission line with a delay of 2 ns or longer can be used to produce the desired transmission line effect. The transmission line is intended as a load only. It is not necessary to add or subtract the transmission line delay (2 ns or longer) from the data sheet timing. Input requirements in this data sheet are tested with an input slew rate of < 4 Volts per nanosecond (4 V/ns) at the device pin.

Figure 8-3. 3.3-V Test Load Circuit

      - Signal Transition Levels

Note that some of the signals use different reference voltages, see the recommended operating conditions table. Output levels are driven to a minimum logic-high level of 2.4 V and to a maximum logic-low level of 0.4 V.

Figure 8-4 shows output levels.

2.4 V (VOH)

0.4 V (VOL)

Figure 8-4. Output Levels

Output transition times are specified as follows:

- For a high-to-low transition, the level at which the output is said to be no longer high is below VOH(MIN) and the level at which the output is said to be low is VOL(MAX) and lower.

- For a low-to-high transition, the level at which the output is said to be no longer low is above VOL(MAX) and the level at which the output is said to be high is VOH(MIN) and higher.

Figure 8-5 shows the input levels.

2.0 V (VIH)

0.8 V (VIL)

Figure 8-5. Input Levels

Input transition times are specified as follows:

- For a high-to-low transition on an input signal, the level at which the input is said to be no longer high is below VIH(MIN) and the level at which the input is said to be low is VIL(MAX) and lower.

- For a low-to-high transition on an input signal, the level at which the input is said to be no longer low is above VIL(MAX) and the level at which the input is said to be high is VIH(MIN) and higher.

Note

See the individual timing diagrams for levels used for testing timing parameters.

    - Power Supply Sequencing

TMS320F2812/F2811/F2810 silicon requires dual voltages (1.8-V or 1.9-V and 3.3-V) to power up the CPU, Flash, ADC, and the I/Os. To ensure the correct reset state for all modules during power up, there are some requirements to be met while powering up/powering down the device.

- Option 1:

In this approach, an external power sequencing circuit enables VDDIO first, then VDD and VDD1

(1.8 V or 1.9 V). After 1.8 V (or 1.9 V) ramps, the 3.3 V for Flash (VDD3VFL) and ADC (VDDA1/VDDA2/ AVDDREFBG) modules are ramped up. While option 1 is still valid, TI has simplified the requirement. Option 2 is the recommended approach.

- Option 2:

Enable power to all 3.3-V supply pins (VDDIO, VDD3VFL, VDDA1/VDDA2/VDDAIO/AVDDREFBG) and then ramp

1.8 V (or 1.9 V) (VDD/VDD1) supply pins.

1.8 V or 1.9 V (VDD/VDD1) should not reach 0.3 V until VDDIO has reached 2.5 V. This ensures the reset signal from the I/O pin has propagated through the I/O buffer to provide power-on reset to all the modules inside the device. See Figure 8-7 for power-on reset timing.

- Power-Down Sequencing:

During power-down, the device reset should be asserted low (8 µs, minimum) before the VDD supply reaches

1.5 V. This will help to keep on-chip flash logic in reset prior to the VDDIO/VDD power supplies ramping down. It is recommended that the device reset control from “Low-Dropout (LDO)” regulators or voltage supervisors be used to meet this constraint. LDO regulators that facilitate power-sequencing (with the aid of additional external components) may be used to meet the power sequencing requirement. See www.spectrumdigital.com for F2812 eZdsp™ schematics and updates.

Note

The GPIO pins are undefined until VDD = 1 V and VDDIO = 2.5 V.

VDD_3.3V

(A)

VDD_1.8V

(B)

XRS

(C)

2.5 V

3.3 V

<10 ms

1.8 V

(or 1.9 V)

(D)

>1 ms

(E)

3.3 V

1.8 V

(or 1.9 V)

1.5 V

(F)

>8 μs

XRS

Power-Up Sequence	Power-Down Sequence

- VDD_3.3V – VDDIO, VDD3VFL, VDDAIO, VDDA1, VDDA2, AVDDREFBG

- VDD_1.8V – VDD, VDD1

- 1.8-V (or 1.9-V) supply should ramp after the 3.3-V supply reaches at least 2.5 V.

- Reset ( XRS) should remain low until supplies and clocks are stable. See Figure 8-7, Power-on Reset in Microcomputer Mode (XMP/ MC

= 0), for minimum requirements.

- Voltage supervisor or LDO reset control will trip reset ( XRS) first when the 3.3-V supply is off regulation. Typically, this occurs a few milliseconds before the 1.8-V (or 1.9-V) supply reaches 1.5 V.

- Keeping reset low ( XRS) at least 8 µs prior to the 1.8-V (or 1.9-V) supply reaching 1.5 V will keep the flash module in complete reset before the supplies ramp down.

- Since the state of GPIO pins is undefined until the 1.8-V (or 1.9-V) supply reaches at least 1 V, this supply should be ramped as quickly as possible (after the 3.3-V supply reaches at least 2.5 V).

- Other than the power supply pins, no pin should be driven before the 3.3-V rail has been fully powered up.

Figure 8-6. F2812/F2811/F2810 Typical Power-Up and Power-Down Sequence – Option 2

    - Reset Timing

      - Reset ( XRS) Timing Requirements

|  |  |  | MIN(1) NOM MAX | UNIT |
| --- | --- | --- | --- | --- |
| tw(RSL1) | Pulse duration, stable XCLKIN to XRS high |  | 8tc(CI) | cycles |
| tw(RSL2) | Pulse duration, XRS low | Warm reset | 8tc(CI) | cycles |
| tw(WDRS) | Pulse duration, reset pulse generated by watchdog |  | 512tc(CI) | cycles |
| td(EX) | Delay time, address/data valid after XRS high |  | 32tc(CI) | cycles |
| tOSCST (2) | Oscillator start-up time |  | 1 10 | ms |
| tsu(XPLLDIS) | Setup time for XPLLDIS pin |  | 16tc(CI) | cycles |
| th(XPLLDIS) | Hold time for XPLLDIS pin |  | 16tc(CI) | cycles |
| th(XMP/MC) | Hold time for XMP/ MC pin |  | 16tc(CI) | cycles |
| th(boot-mode) | Hold time for boot-mode pins |  | 2520tc(CI) (3) | cycles |

- If external oscillator/clock source are used, reset time has to be low at least for 1 ms after VDD reaches 1.5 V.

- Dependent on crystal/resonator and board design.

- The boot ROM reads the password locations. Therefore, this timing requirement includes the wakeup time for flash. See the TMS320x281x DSP Boot ROM Reference Guide and the TMS320x281x DSP System Control and Interrupts Reference Guide for further information.

VDDIO, VDD3VFL

(A)

VDDAn  , VDDAIO

(3.3 V)

VDD, VDD1 [1.8 V (or 1.9 V)]

0.3 V

2.5 V

XCLKIN

X1

XCLKOUT

(B)

XCLKIN/8

tOSCST

User-Code Dependent

XRS

Address/

Data/ Control

tw(RSL1)

Address/Data Valid. Internal Boot-ROM Code Execution Phase

td(EX)

tsu(XPLLDIS)

XPLLDIS Sampling

User-Code Execution Phase

User-Code Dependent th(XPLLDIS)

XF/XPLLDIS

(Don’t Care)

GPIOF14

XMP/MC

th(XMP/MC)

(Don’t Care)

(C)

th(boot-mode)

User-Code Dependent

Boot-Mode Pins

(D)

GPIO Pins as Input

Boot-ROM Execution Starts	Peripheral/GPIO Function Based on Boot Code

I/O Pins	GPIO Pins as Input (State Depends on Internal PU/PD)

User-Code Dependent

- VDDAn – VDDA1/VDDA2 and AVDDREFBG

- Upon power up, SYSCLKOUT is XCLKIN/2 if the PLL is enabled. Since both the XTIMCLK and CLKMODE bits in the XINTCNF2 register come up with a reset state of 1, SYSCLKOUT is further divided by 4 before it appears at XCLKOUT. This explains why XCLKOUT = XCLKIN/8 during this phase.

- After reset, the Boot ROM code executes instructions for 1260 SYSCLKOUT cycles (SYSCLKOUT = XCLKIN/2) and then samples BOOT Mode pins. Based on the status of the Boot Mode pin, the boot code branches to destination memory or boot code function in ROM. The BOOT Mode pins should be held high/low for at least 2520 XCLKIN cycles from boot ROM execution time for proper selection of Boot modes. If Boot ROM code executes after power-on conditions (in debugger environment), the Boot code execution time is based on the current SYSCLKOUT speed. The SYSCLKOUT will be based on user environment and could be with or without PLL enabled.

- The state of the GPIO pins is undefined (that is, they could be input or output) until the 1.8-V (or 1.9-V) supply reaches at least 1 V and 3.3-V supply reaches 2.5 V.

Figure 8-7. Power-on Reset in Microcomputer Mode (XMP/ MC = 0) (See Note D)

VDDIO, VDD3VFL VDDAn, VDDAIO

(3.3 V)

VDD, VDD1 [1.8 V (or 1.9 V)]

2.5 V

0.3 V

XCLKIN

X1

XCLKOUT

tOSCST

XCLKIN/8(A)	User-Code Dependent

XRS

Address/

Data/ Control

(Don’t Care)

tw(RSL)

td(EX)

Address/Data/Control Valid Execution Begins From External Boot Address 0x3FFFC0

XPLLDIS Sampling

th(XPLLDIS)

XF/XPLLDIS

XMP/MC

(Don’t Care)		GPIOF14/XF (User-Code Dependent) tsu(XPLLDIS)

(Don’t Care)

th(XMP/MC)

I/O Pins	User-Code Dependent

(B)

Input Configuration (State Depends on Internal PU/PD)

- Upon power up, SYSCLKOUT is XCLKIN/2 if the PLL is enabled. Since both the XTIMCLK and CLKMODE bits in the XINTCNF2 register come up with a reset state of 1, SYSCLKOUT is further divided by 4 before it appears at XCLKOUT. This explains why XCLKOUT = XCLKIN/8 during this phase.

- The state of the GPIO pins is undefined (that is, they could be input or output) until the 1.8-V (or 1.9-V) supply reaches at least 1 V and 3.3-V supply reaches 2.5 V.

Figure 8-8. Power-on Reset in Microprocessor Mode (XMP/ MC = 1)

XCLKIN/8

(XCLKIN * 5)

User-Code Dependent

tw(RSL2)

t

User-Code Execution Phase

d(EX)

User-Code Execution

(Don’t Care)

tsu(XPLLDIS)	th(XPLLDIS)

GPIOF14/XF	(Don’t Care)	GPIOF14

(Don’t Care)

XPLLDIS Sampling

th(XMP/MC)

User-Code Dependent

(Don’t Care)

XCLKIN

X1

XCLKOUT

XRS

Address/

Data/ Control

XF/XPLLDIS

XMP/MC

Boot-ROM Execution Starts

Peripheral/GPIO Function

GPIO Pins as Input

th(boot-mode)

Peripheral/GPIO Function

User-Code Execution Starts

(A)

Boot-Mode Pins

User-Code Dependent

GPIO Pins as Input (State Depends on Internal PU/PD)

User-Code Dependent

I/O Pins

- After reset, the Boot ROM code executes instructions for 1260 SYSCLKOUT cycles (SYSCLKOUT = XCLKIN/2) and then samples BOOT Mode pins. Based on the status of the Boot Mode pin, the boot code branches to destination memory or boot code function in ROM. The BOOT Mode pins should be held high/low for at least 2520 XCLKIN cycles from boot ROM execution time for proper selection of Boot modes. If Boot ROM code executes after power-on conditions (in debugger environment), the Boot code execution time is based on the current SYSCLKOUT speed. The SYSCLKOUT will be based on user environment and could be with or without PLL enabled.

Figure 8-9. Warm Reset in Microcomputer Mode

Write to PLLCR

X1/XCLKIN

SYSCLKOUT

XCLKIN * 2

(Current CPU Frequency)

XCLKIN/2

[CPU Frequency While PLL is Stabilizing With the Desired Frequency.

This Period (PLL Lock-up Time, tp) is 131 072 XCLKIN Cycles Long.]

XCLKIN * 4

(Changed CPU Frequency)

Figure 8-10. Effect of Writing Into PLLCR Register

    - Clock Specifications

      - Device Clock Table

This section provides the timing requirements and switching characteristics for the various clock options available on the F281x DSPs. Section 8.12.4.1.1 lists the cycle times of various clocks.

        - Clock Table and Nomenclature

|  |  | MIN | NOM | MAX | UNIT |
| --- | --- | --- | --- | --- | --- |
| On chip oscillator clock | tc(OSC), Cycle time | 28.6 50 |  |  | ns |
|  | Frequency | 20 |  | 35 | MHz |
| XCLKIN | tc(CI), Cycle time | 6.67 |  | 250 | ns |
|  | Frequency | 4 |  | 150 | MHz |
| SYSCLKOUT | tc(SCO), Cycle time | 6.67 |  | 500 | ns |
|  | Frequency | 2 |  | 150 | MHz |
| XCLKOUT | tc(XCO), Cycle time | 6.67 |  | 2000 | ns |
|  | Frequency | 0.5 |  | 150 | MHz |
| HSPCLK | tc(HCO), Cycle time | 6.67 | 13.3(1) |  | ns |
|  | Frequency |  | 75(1) | 150 | MHz |
| LSPCLK | tc(LCO), Cycle time | 13.3 | 26.6(1) |  | ns |
|  | Frequency |  | 37.5(1) | 75 | MHz |
| ADC clock | tc(ADCCLK), Cycle time(2) | 40 |  |  | ns |
|  | Frequency | 25 |  |  | MHz |
| SPI clock | tc(SPC), Cycle time | 50 |  |  | ns |
|  | Frequency | 20 |  |  | MHz |
| McBSP | tc(CKG), Cycle time | 50 |  |  | ns |
|  | Frequency | 20 |  |  | MHz |
| XTIMCLK | tc(XTIM), Cycle time | 6.67 |  |  | ns |
|  | Frequency | 150 |  |  | MHz |

- This is the default reset value if SYSCLKOUT = 150 MHz.

- The maximum value for ADCCLK frequency is 25 MHz. For SYSCLKOUT values of 25 MHz or lower, ADCCLK has to be SYSCLKOUT/2 or lower. ADCCLK = SYSCLKOUT is not a valid mode for any value of SYSCLKOUT.

      - Clock Requirements and Characteristics

        - Input Clock Requirements

The clock provided at the XCLKIN pin generates the internal CPU clock cycle.

          - Input Clock Frequency

| PARAMETER |  |  |  | MIN | TYP | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| fx | Input clock frequency | Resonator |  | 20 | 35 |  | MHz |
|  |  | Crystal |  | 20 | 35 |  |  |
|  |  | XCLKIN | Without PLL | 4 | 150 |  |  |
|  |  |  | With PLL | 5 | 100 |  |  |
| fl | Limp mode clock frequency |  |  | 2 |  |  | MHz |

          - XCLKIN Timing Requirements – PLL Bypassed or Enabled

| NO. |  |  | MIN | MAX | UNIT |
| --- | --- | --- | --- | --- | --- |
| C8 | tc(CI) | Cycle time, XCLKIN | 6.67 | 250 | ns |
| C9 | tf(CI) | Fall time, XCLKIN | 6 |  | ns |
| C10 | tr(CI) | Rise time, XCLKIN | 6 |  | ns |
| C11 | tw(CIL) | Pulse duration, X1/XCLKIN low as a percentage of tc(CI) | 40 | 60 | % |
| C12 | tw(CIH) | Pulse duration, X1/XCLKIN high as a percentage of tc(CI) | 40 | 60 | % |

          - XCLKIN Timing Requirements – PLL Disabled

| NO. |  |  |  | MIN | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| C8 | tc(CI) | Cycle time, XCLKIN |  | 6.67 | 250 | ns |
| C9 | tf(CI) | Fall time, XCLKIN | Up to 30 MHz | 6 |  | ns |
|  |  |  | 30 MHz to 150 MHz | 2 |  |  |
| C10 | tr(CI) | Rise time, XCLKIN | Up to 30 MHz | 6 |  | ns |
|  |  |  | 30 MHz to 150 MHz | 2 |  |  |
| C11 | tw(CIL) | Pulse duration, X1/XCLKIN low as a percentage of tc(CI) | XCLKIN ≤ 120 MHz | 40 | 60 | % |
|  |  |  | 120 < XCLKIN ≤ 150 MHz | 45 | 55 |  |
| C12 | tw(CIH) | Pulse duration, X1/XCLKIN high as a percentage of tc(CI) | XCLKIN ≤ 120 MHz | 40 | 60 | % |
|  |  |  | 120 < XCLKIN ≤ 150 MHz | 45 | 55 |  |

Table 8-2. Possible PLL Configuration Modes

| PLL MODE | REMARKS | SYSCLKOUT |
| --- | --- | --- |
| PLL Disabled | Invoked by tying XPLLDIS pin low upon reset. PLL block is completely disabled. Clock input to the CPU (CLKIN) is directly derived from the clock signal present at the X1/XCLKIN pin. | XCLKIN |
| PLL Bypassed | Default PLL configuration upon power-up, if PLL is not disabled. The PLL itself is bypassed. However, the /2 module in the PLL block divides the clock input at the X1/XCLKIN pin by two before feeding it to the CPU. | XCLKIN/2 |
| PLL Enabled | Achieved by writing a non-zero value “n” into PLLCR register. The /2 module in the PLL block now divides the output of the PLL by two before feeding it to the CPU. | (XCLKIN * n) / 2 |

        - Output Clock Characteristics

          - XCLKOUT Switching Characteristics (PLL Bypassed or Enabled)

| NO.(1) (2) | PARAMETER |  | MIN | TYP | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| C1 | tc(XCO) | Cycle time, XCLKOUT | 6.67(3) |  |  | ns |
| C3 | tf(XCO) | Fall time, XCLKOUT | 2 |  |  | ns |
| C4 | tr(XCO) | Rise time, XCLKOUT | 2 |  |  | ns |
| C5 | tw(XCOL) | Pulse duration, XCLKOUT low | H – 2 | H + 2 |  | ns |
| C6 | tw(XCOH) | Pulse duration, XCLKOUT high | H – 2 | H + 2 |  | ns |
| C7 | tp | PLL lock time(4) | 131072tc(CI) |  |  | ns |

- A load of 40 pF is assumed for these parameters.

- H = 0.5tc(XCO)

- The PLL must be used for maximum frequency operation.

- This parameter has changed from 4096 XCLKIN cycles in the earlier revisions of the silicon.

C10

C8

C9

XCLKIN

(A)

C1

C3

C6

C4

C5

(A)(B)

XCLKOUT

- The relationship of XCLKIN to XCLKOUT depends on the divide factor chosen. The waveform relationship shown in Figure 8-11 is intended to illustrate the timing parameters only and may differ based on configuration.

- XCLKOUT configured to reflect SYSCLKOUT.

Figure 8-11. Clock Timing

    - Peripherals

      - General-Purpose Input/Output (GPIO) – Output Timing

        - General-Purpose Output Switching Characteristics

| PARAMETER |  |  | MIN MAX | UNIT |
| --- | --- | --- | --- | --- |
| td(XCOH-GPO) | Delay time, XCLKOUT high to GPIO low/high | All GPIOs | 1tc(SCO) | cycle |
| tr(GPO) | Rise time, GPIO switching low to high | All GPIOs | 10 | ns |
| tf(GPO) | Fall time, GPIO switching high to low | All GPIOs | 10 | ns |
| fGPO | Toggling frequency, GPO pins |  | 20 | MHz |

XCLKOUT

(A)

tf(GPO)

tr(GPO)

td(XCOH-GPO)

GPIO

A. XCLKOUT = SYSCLKOUT

Figure 8-12. General-Purpose Output Timing

      - General-Purpose Input/Output (GPIO) – Input Timing

GPIO

Signal

See Note (A)

1	1	0	0	0	0	0	0	0

1	0	0	0

1	1	1	1	1	1	1	1	1

Sampling period, determined by GPxQUAL

[QUALPRD]

Sampling Window

(SYSCLKOUT cycle x 2 x QUALPRD) x 5

QUALPRD = 1

(SYSCLKOUT/2)

SYSCLKOUT

Output From Qualifier

- This glitch is ignored by the input qualifier. The QUALPRD bit field specifies the qualification sampling period. It can vary from 00 to 0xFF. Input qualification is not applicable when QUALPRD = 00. For any other value “n”, the qualification sampling period is 2n SYSCLKOUT cycles (that is, at every 2n SYSCLKOUT cycle, the GPIO pin will be sampled). Six consecutive samples must be of the same value for a given input to be recognized.

- For the qualifier to detect the change, the input must be stable for 10 SYSCLKOUT cycles or greater. In other words, the inputs should be stable for (5 × QUALPRD × 2) SYSCLKOUT cycles. This would enable five sampling periods for detection to occur. Since external signals are driven asynchronously, a 13-SYSCLKOUT-wide pulse provides reliable recognition.

Figure 8-13. GPIO Input Qualifier – Example Diagram for QUALPRD = 1

        - General-Purpose Input Timing Requirements

|  |  |  | MIN MAX | UNIT |
| --- | --- | --- | --- | --- |
| tw(GPI) Pulse duration GPIO low/ high | All GPIOs | With no qualifier | 2tc(SCO) | cycles |
|  |  | With qualifier | 1tc(SCO) + IQT(1) |  |

(1)	Input Qualification Time (IQT) = [tc(SCO) × 2 × QUALPRD] × 5 + [tc(SCO) × 2 × QUALPRD].

SYSCLK

tw(GPI)

GPIOxn

Figure 8-14. General-Purpose Input Timing

Note

The pulse width requirement for general-purpose input is applicable for the XBIO and ADCSOC pins as well.

      - Event Manager Interface

        - PWM Timing

PWM refers to all PWM outputs on EVA and EVB.

          - PWM Switching Characteristics

| PARAMETER(1) (2) | TEST CONDITIONS | MIN MAX | UNIT |
| --- | --- | --- | --- |
| tw(PWM) (3) Pulse duration, PWMx output high/low |  | 25 | ns |
| td(PWM)XCO Delay time, XCLKOUT high to PWMx output switching | XCLKOUT = SYSCLKOUT/4 | 10 | ns |

- See the GPIO output timing for fall/rise times for PWM pins.

- PWM pin toggling frequency is limited by the GPIO output buffer switching frequency (20 MHz).

- PWM outputs may be 100%, 0%, or increments of tc(HCO) with respect to the PWM period.

          - Timer and Capture Unit Timing Requirements

|  |  |  | MIN | MAX(2) (1) | UNIT |
| --- | --- | --- | --- | --- | --- |
| tw(TDIR) | Pulse duration, TDIRx low/high | Without input qualifier | 2tc(SCO) |  | cycles |
|  |  | With input qualifier | 1tc(SCO) + IQT(3) |  |  |
| tw(CAP) | Pulse duration, CAPx input low/high | Without input qualifier | 2tc(SCO) |  | cycles |
|  |  | With input qualifier | 1tc(SCO) + IQT(3) |  |  |
| tw(TCLKINL) | Pulse duration, TCLKINx low as a percentage of TCLKINx cycle time |  | 40 | 60 | % |
| tw(TCLKINH) | Pulse duration, TCLKINx high as a percentage of TCLKINx cycle time |  | 40 | 60 | % |
| tc(TCLKIN) | Cycle time, TCLKINx |  | 4tc(HCO) |  | ns |

- The QUALPRD bit field value can range from 0 (no qualification) through 0xFF (510 SYSCLKOUT cycles). The qualification sampling period is 2n SYSCLKOUT cycles, where “n” is the value stored in the QUALPRD bit field. As an example, when QUALPRD = 1, the qualification sampling period is 1 × 2 = 2 SYSCLKOUT cycles (that is, the input is sampled every 2 SYSCLKOUT cycles). Six such samples will be taken over five sampling windows, each window being 2n SYSCLKOUT cycles. For QUALPRD = 1, the minimum width that is needed is 5 × 2 = 10 SYSCLKOUT cycles. However, since the external signal is driven asynchronously, a 11-SYSCLKOUT-wide pulse ensures reliable recognition.

- Maximum input frequency to the QEP = min[HSPCLK/2, 20 MHz]

- Input Qualification Time (IQT) = [tc(SCO) × 2 × QUALPRD] × 5 + [tc(SCO) × 2 × QUALPRD].

XCLKOUT

(A)

PWMx

td(PWM)XCO

tw(PWM)

A. XCLKOUT = SYSCLKOUT

Figure 8-15. PWM Output Timing

XCLKOUT

(A)

tw(TDIR)

TDIRx

A. XCLKOUT = SYSCLKOUT

Figure 8-16. TDIRx Timing

          - External ADC Start-of-Conversion – EVA – Switching Characteristics

| PARAMETER(1) | MIN MAX | UNIT |
| --- | --- | --- |
| td(XCOH-EVASOCL) Delay time, XCLKOUT high to EVASOC low | 1tc(SCO) | cycle |
| tw(EVASOCL) Pulse duration, EVASOC low | 32tc(HCO) | ns |

(1)	XCLKOUT = SYSCLKOUT

td(XCOH-EVASOCL)

tw(EVASOCL)

XCLKOUT

EVASOC

Figure 8-17. EVASOC Timing

          - External ADC Start-of-Conversion – EVB – Switching Characteristics

| PARAMETER(1) | MIN MAX | UNIT |
| --- | --- | --- |
| td(XCOH-EVBSOCL) Delay time, XCLKOUT high to EVBSOC low | 1tc(SCO) | cycle |
| tw(EVBSOCL) Pulse duration, EVBSOC low | 32tc(HCO) | ns |

(1)	XCLKOUT = SYSCLKOUT

td(XCOH-EVBSOCL)

tw(EVBSOCL)

XCLKOUT

EVBSOC

Figure 8-18. EVBSOC Timing

      - Low-Power Mode Wakeup Timing

        - IDLE Mode Timing Requirements

|  |  | MIN MAX | UNIT |
| --- | --- | --- | --- |
| tw(WAKE-INT) Pulse duration, external wake-up signal | Without input qualifier | 2tc(SCO) | cycles |
|  | With input qualifier | 1tc(SCO) + IQT(1) |  |

(1)	Input Qualification Time (IQT) = [tc(SCO) × 2 × QUALPRD] × 5 + [tc(SCO) × 2 × QUALPRD].

        - IDLE Mode Switching Characteristics

| PARAMETER |  | TEST CONDITIONS | MIN MAX | UNIT |
| --- | --- | --- | --- | --- |
| td(WAKE-IDLE) | Delay time, external wake signal to program execution resume(1) |  |  |  |
|  | - Wake-up from Flash – Flash module in active state | Without input qualifier | 8tc(SCO) | cycles |
|  |  | With input qualifier | 8tc(SCO) + IQT(2) |  |
|  | - Wake-up from Flash – Flash module in sleep state | Without input qualifier | 1050tc(SCO) | cycles |
|  |  | With input qualifier | 1050tc(SCO) + IQT(2) |  |
|  | - Wake-up from SARAM | Without input qualifier | 8tc(SCO) | cycles |
|  |  | With input qualifier | 8tc(SCO) + IQT(2) |  |

- This is the time taken to begin execution of the instruction that immediately follows the IDLE instruction. Execution of an ISR (triggered by the wake-up) signal involves additional latency.

- Input Qualification Time (IQT) = [tc(SCO) × 2 × QUALPRD] × 5 + [tc(SCO) × 2 × QUALPRD].

td(WAKE-IDLE)

tw(WAKE-INT)

A0-A15

XCLKOUT

(A)

(B)

WAKE INT

- XCLKOUT = SYSCLKOUT

- WAKE INT can be any enabled interrupt, WDINT, XNMI, or XRS.

Figure 8-19. IDLE Entry and Exit Timing

        - STANDBY Mode Timing Requirements

|  |  | MIN MAX | UNIT |
| --- | --- | --- | --- |
| tw(WAKE-INT) Pulse duration, external wake-up signal | Without input qualifier | 12tc(CI) | cycles |
|  | With input qualifier | (2 + QUALSTDBY) * tc(CI) (1) |  |

(1)	QUALSTDBY is a 6-bit field in the LPMCR0 register.

        - STANDBY Mode Switching Characteristics

| PARAMETER |  | TEST CONDITIONS | MIN | MAX | UNIT |
| --- | --- | --- | --- | --- | --- |
| td(IDLE-XCOH) | Delay time, IDLE instruction executed to XCLKOUT high |  | 32tc(SCO) | 45tc(SCO) | cycles |
| td(WAKE-STBY) | Delay time, external wake signal to program execution resume(1) |  |  |  |  |
|  | - Wake-up from Flash – Flash module in active state | Without input qualifier | 12tc(CI) |  | cycles |
|  |  | With input qualifier | 12tc(CI) + tw(WAKE-INT) |  |  |
|  | - Wake-up from Flash – Flash module in sleep state | Without input qualifier | 1125tc(SCO) |  | cycles |
|  |  | With input qualifier | 1125tc(SCO) + tw(WAKE-INT) |  |  |
|  | - Wake-up from SARAM | Without input qualifier | 12tc(CI) |  | cycles |
|  |  | With input qualifier | 12tc(CI) + tw(WAKE-INT) |  |  |

(1)	This is the time taken to begin execution of the instruction that immediately follows the IDLE instruction. Execution of an ISR (triggered by the wake-up) signal involves additional latency.

A

C

E

B	D	F

STANDBY

STANDBY

Normal Execution

Flushing Pipeline

tw(WAKE-INT)

td(WAKE-STBY)

td(IDLE-XCOH)

32 SYSCLKOUT Cycles

Device Status

Wake-up Signal

X1/XCLKIN

XCLKOUT

- IDLE instruction is executed to put the device into STANDBY mode.

- The PLL block responds to the STANDBY signal. SYSCLKOUT is held for the number of cycles indicated below before being turned off:

  - 16 cycles, when DIVSEL = 00 or 01

  - 32 cycles, when DIVSEL = 10

  - 64 cycles, when DIVSEL = 11

This delay enables the CPU pipeline and any other pending operations to flush properly. If an access to XINTF is in progress and its access time is longer than this number, then it will fail. It is recommended that STANDBY mode be entered from SARAM without an XINTF access in progress.

- Clocks to the peripherals are turned off. However, the PLL and watchdog are not shut down. The device is now in STANDBY mode.

- The external wake-up signal is driven active.

- After a latency period, the STANDBY mode is exited.

- Normal execution resumes. The device will respond to the interrupt (if enabled).

Figure 8-20. STANDBY Entry and Exit Timing

        - HALT Mode Timing Requirements

|  | MIN MAX | UNIT |
| --- | --- | --- |
| tw(WAKE-XNMI) Pulse duration, XNMI wakeup signal | 2tc(CI) | cycles |
| tw(WAKE-XRS) Pulse duration, XRS wakeup signal | 8tc(CI) | cycles |

        - HALT Mode Switching Characteristics

| PARAMETER |  | MIN | TYP | MAX | UNIT |
| --- | --- | --- | --- | --- | --- |
| td(IDLE-XCOH) | Delay time, IDLE instruction executed to XCLKOUT high | 32tc(SCO) | 45tc(SCO) |  | cycles |
| tp | PLL lock-up time | 131072tc(CI) |  |  | cycles |
| td(WAKE) | Delay time, PLL lock to program execution resume |  |  |  |  |
|  | - Wake up from flash – Flash module in sleep state | 1125tc(SCO) |  |  | cycles |
|  | - Wake up from SARAM | 35tc(SCO) |  |  | cycles |

A

C

E

G

B	D	F

HALT

HALT

Flushing Pipeline

PLL Lock-up Time

Wake-up Latency

Normal

Execution

tw(WAKE-XNMI)

t

td(wake)

p

td(IDLE-XCOH)

Oscillator Start-up Time

32 SYSCLKOUT Cycles

Device Status

XNMI

X1/XCLKIN

XCLKOUT

(H)

- IDLE instruction is executed to put the device into HALT mode.

- The PLL block responds to the HALT signal. SYSCLKOUT is held for another 32 cycles before the oscillator is turned off and the CLKIN to the core is stopped. This 32-cycle delay enables the CPU pipe and any other pending operations to flush properly.

- Clocks to the peripherals are turned off and the internal oscillator and PLL are shut down. The device is now in HALT mode and consumes absolute minimum power.

- When XNMI is driven active, the oscillator is turned on; but the PLL is not activated. The pulse duration of 2tc(CI) is applicable when an external oscillator is used. If the internal oscillator is used, the oscillator wake-up time should be added to this parameter.

- When XNMI is deactivated, it initiates the PLL lock sequence, which takes 131,072 X1/XCLKIN cycles.

- When CLKIN to the core is enabled, the device will respond to the interrupt (if enabled), after a latency. The HALT mode is now exited.

- Normal operation resumes.

- XCLKOUT = SYSCLKOUT

Figure 8-21. HALT Wakeup Using XNMI

      - Serial Peripheral Interface (SPI) Master Mode Timing

Section 8.12.5.5.1 lists the master mode timing (clock phase = 0) and Section 8.12.5.5.2 lists the master mode timing (clock phase = 1). Figure 8-22 and Figure 8-23 show the timing waveforms.

        - SPI Master Mode External Timing (Clock Phase = 0)

| NO.(1) (2) (3) (4) (5) | PARAMETER |  | BRR EVEN |  | BRR ODD |  | UNIT |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MIN | MAX | MIN | MAX |  |
| 1 | tc(SPC)M | Cycle time, SPICLK | 4tc(LSPCLK) | 128tc(LSPCLK) | 5tc(LSPCLK) | 127tc(LSPCLK) | ns |
| 2 | tw(SPC1)M | Pulse duration, SPICLK first pulse | 0.5tc(SPC)M – 10 | 0.5tc(SPC)M + 10 | 0.5tc(SPC)M + 0.5tc(LSPCLK) – 10 | 0.5tc(SPC)M + 0.5tc(LSPCLK) + 10 | ns |
| 3 | tw(SPC2)M | Pulse duration, SPICLK second pulse | 0.5tc(SPC)M – 10 | 0.5tc(SPC)M + 10 | 0.5tc(SPC)M – 0.5tc(LSPCLK) – 10 | 0.5tc(SPC)M – 0.5tc(LSPCLK) + 10 | ns |
| 4 | td(SIMO)M | Delay time, SPICLK to SPISIMO valid | 10 |  | 10 |  | ns |
| 5 | tv(SIMO)M | Valid time, SPISIMO valid after SPICLK | 0.5tc(SPC)M – 10 |  | 0.5tc(SPC)M – 0.5tc(LSPCLK) – 10 |  | ns |
| 8 | tsu(SOMI)M | Setup time, SPISOMI before SPICLK | 35 |  | 35 |  | ns |
| 9 | th(SOMI)M | Hold time, SPISOMI valid after SPICLK | 0 |  | 0 |  | ns |
| 23 | td(SPC)M | Delay time, SPISTE active to SPICLK | 1.5tc(SPC)M – 3tc(SYSCLK) – 10 |  | 1.5tc(SPC)M – 3tc(SYSCLK) – 10 |  | ns |
| 24 | td(STE)M | Delay time, SPICLK to SPISTE inactive | 0.5tc(SPC)M – 10 |  | 0.5tc(SPC)M – 0.5tc(LSPCLK) – 10 |  | ns |

- The MASTER / SLAVE bit (SPICTL.2) is set and the CLOCK PHASE bit (SPICTL.3) is cleared.

- tc(SPC) = SPI clock cycle time = LSPCLK/4 or LSPCLK/(SPIBRR +1)

- tc(LCO) = LSPCLK cycle time

- Internal clock prescalers must be adjusted such that the SPI clock speed is limited to the following SPI clock rate: Master mode transmit 25-MHz MAX, master mode receive 12.5-MHz MAX

Slave mode transmit 12.5-MAX, slave mode receive 12.5-MHz MAX.

- The active edge of the SPICLK signal referenced is controlled by the clock polarity bit (SPICCR.6).

1

2

3

4

5

Master Out Data Is Valid

8

9

Master In Data Must Be Valid

23

24

SPICLK

(clock polarity = 0)

SPICLK

(clock polarity = 1)

SPISIMO

SPISOMI

SPISTE

Figure 8-22. SPI Master Mode External Timing (Clock Phase = 0)

        - SPI Master Mode External Timing (Clock Phase = 1)

| NO.(1) (2) (3) (4) (5) | PARAMETER |  | BRR EVEN |  | BRR ODD |  | UNIT |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MIN | MAX | MIN | MAX |  |
| 1 | tc(SPC)M | Cycle time, SPICLK | 4tc(LSPCLK) | 128tc(LSPCLK) | 5tc(LSPCLK) | 127tc(LSPCLK) | ns |
| 2 | tw(SPC1)M | Pulse duration, SPICLK first pulse | 0.5tc(SPC)M – 10 | 0.5tc(SPC)M + 10 | 0.5tc(SPC)M – 0.5tc(LSPCLK) – 10 | 0.5tc(SPC)M – 0.5tc(LSPCLK) + 10 | ns |
| 3 | tw(SPC2)M | Pulse duration, SPICLK second pulse | 0.5tc(SPC)M – 10 | 0.5tc(SPC)M + 10 | 0.5tc(SPC)M + 0.5tc(LSPCLK) – 10 | 0.5tc(SPC)M + 0.5tc(LSPCLK) + 10 | ns |
| 6 | td(SIMO)M | Delay time, SPISIMO valid to SPICLK | 0.5tc(SPC)M – 10 |  | 0.5tc(SPC)M + 0.5tc(LSPCLK) – 10 |  | ns |
| 7 | tv(SIMO)M | Valid time, SPISIMO valid after SPICLK | 0.5tc(SPC)M – 10 |  | 0.5tc(SPC)M – 0.5tc(LSPCLK) – 10 |  | ns |
| 10 | tsu(SOMI)M | Setup time, SPISOMI before SPICLK | 35 |  | 35 |  | ns |
| 11 | th(SOMI)M | Hold time, SPISOMI valid after SPICLK | 0 |  | 0 |  | ns |
| 23 | td(SPC)M | Delay time, SPISTE active to SPICLK | 2tc(SPC)M – 3tc(SYSCLK) – 10 |  | 2tc(SPC)M – 3tc(SYSCLK) – 10 |  | ns |
| 24 | td(STE)M | Delay time, SPICLK to SPISTE inactive | 0.5tc(SPC) – 10 |  | 0.5tc(SPC) – 0.5tc(LSPCLK) – 10 |  | ns |

- The MASTER/SLAVE bit (SPICTL.2) is set and the CLOCK PHASE bit (SPICTL.3) is set.

- tc(SPC) = SPI clock cycle time = LSPCLK/4 or LSPCLK/(SPIBRR + 1)

- Internal clock prescalers must be adjusted such that the SPI clock speed is limited to the following SPI clock rate: Master mode transmit 25 MHz MAX, master mode receive 12.5 MHz MAX

Slave mode transmit 12.5 MHz MAX, slave mode receive 12.5 MHz MAX.

- tc(LCO) = LSPCLK cycle time

- The active edge of the SPICLK signal referenced is controlled by the CLOCK POLARITY bit (SPICCR.6).

1

2

6

7

Master Out Data Is Valid 10

11

Master In Data Must

Be Valid

23

24

3

SPICLK

(clock polarity = 0)

SPICLK

(clock polarity = 1)

SPISIMO

SPISOMI

SPISTE

Figure 8-23. SPI Master Mode External Timing (Clock Phase = 1)

      - Serial Peripheral Interface (SPI) Slave Mode Timing

Section 8.12.5.6.1 lists the slave mode timing (clock phase = 0) and Section 8.12.5.6.2 lists the slave mode timing (clock phase = 1). Figure 8-24 and Figure 8-25 show the timing waveforms.

        - SPI Slave Mode External Timing (Clock Phase = 0)

| NO. (1) (2) (4) (3) (5) | PARAMETER |  | MIN MAX | UNIT |
| --- | --- | --- | --- | --- |
| 12 | tc(SPC)S | Cycle time, SPICLK | 4tc(SYSCLK) | ns |
| 13 | tw(SPC1)S | Pulse duration, SPICLK first pulse | 2tc(SYSCLK) – 1 | ns |
| 14 | tw(SPC2)S | Pulse duration, SPICLK second pulse | 2tc(SYSCLK) – 1 | ns |
| 15 | td(SOMI)S | Delay time, SPICLK to SPISOMI valid | 35 | ns |
| 16 | tv(SOMI)S | Valid time, SPISOMI data valid after SPICLK | 0 | ns |
| 19 | tsu(SIMO)S | Setup time, SPISIMO valid before SPICLK | 1.5tc(SYSCLK) | ns |
| 20 | th(SIMO)S | Hold time, SPISIMO data valid after SPICLK | 1.5tc(SYSCLK) | ns |
| 25 | tsu(STE)S | Setup time, SPISTE active before SPICLK | 1.5tc(SYSCLK) | ns |
| 26 | th(STE)S | Hold time, SPISTE inactive after SPICLK | 1.5tc(SYSCLK) | ns |

- The MASTER / SLAVE bit (SPICTL.2) is cleared and the CLOCK PHASE bit (SPICTL.3) is cleared.

- tc(SPC) = SPI clock cycle time = LSPCLK/4 or LSPCLK/(SPIBRR + 1)

- tc(LCO) = LSPCLK cycle time

- Internal clock prescalers must be adjusted such that the SPI clock speed is limited to the following SPI clock rate: Master mode transmit 25-MHz MAX, master mode receive 12.5-MHz MAX

Slave mode transmit 12.5-MHz MAX, slave mode receive 12.5-MHz MAX.

- The active edge of the SPICLK signal referenced is controlled by the CLOCK POLARITY bit (SPICCR.6).

12

13

14

15

16

SPISOMI Data Is Valid

19

20

SPISIMO Data

Must Be Valid

25

26

SPICLK

(clock polarity = 0)

SPICLK

(clock polarity = 1)

SPISOMI

SPISIMO

SPISTE

Figure 8-24. SPI Slave Mode External Timing (Clock Phase = 0)

        - SPI Slave Mode External Timing (Clock Phase = 1)

| NO. (1) (2) (3) (4) | PARAMETER |  | MIN MAX | UNIT |
| --- | --- | --- | --- | --- |
| 12 | tc(SPC)S | Cycle time, SPICLK | 4tc(SYSCLK) | ns |
| 13 | tw(SPC1)S | Pulse duration, SPICLK first pulse | 2tc(SYSCLK) – 1 | ns |
| 14 | tw(SPC2)S | Pulse duration, SPICLK second pulse | 2tc(SYSCLK) – 1 | ns |
| 17 | td(SOMI)S | Delay time, SPICLK to SPISOMI valid | 35 | ns |
| 18 | tv(SOMI)S | Valid time, SPISOMI data valid after SPICLK | 0 | ns |
| 21 | tsu(SIMO)S | Setup time, SPISIMO valid before SPICLK | 1.5tc(SYSCLK) | ns |
| 22 | th(SIMO)S | Hold time, SPISIMO data valid after SPICLK | 1.5tc(SYSCLK) | ns |
| 25 | tsu(STE)S | Setup time, SPISTE active before SPICLK | 1.5tc(SYSCLK) | ns |
| 26 | th(STE)S | Hold time, SPISTE inactive after SPICLK | 1.5tc(SYSCLK) | ns |

- The MASTER / SLAVE bit (SPICTL.2) is cleared and the CLOCK PHASE bit (SPICTL.3) is cleared.

- tc(SPC) = SPI clock cycle time = LSPCLK/4 or LSPCLK/(SPIBRR + 1)

- Internal clock prescalers must be adjusted such that the SPI clock speed is limited to the following SPI clock rate: Master mode transmit 25-MHz MAX, master mode receive 12.5-MHz MAX

Slave mode transmit 12.5-MHz MAX, slave mode receive 12.5-MHz MAX.

- The active edge of the SPICLK signal referenced is controlled by the CLOCK POLARITY bit (SPICCR.6).

12

13

14

17

SPISOMI Data Is Valid

Data Valid

Data Valid

21

18

22

SPISIMO Data

Must Be Valid

25

26

SPICLK

(clock polarity = 0)

SPICLK

(clock polarity = 1)

SPISOMI

SPISIMO

SPISTE

Figure 8-25. SPI Slave Mode External Timing (Clock Phase = 1)

      - External Interface (XINTF) Timing

Each XINTF access consists of three parts: Lead, Active, and Trail. The user configures the Lead/Active/Trail wait states in the XTIMING registers. There is one XTIMING register for each XINTF zone. Table 8-3 shows the relationship between the parameters configured in the XTIMING register and the duration of the pulse in terms of XTIMCLK cycles.

Table 8-3. Relationship Between Parameters Configured in XTIMING and Duration of Pulse

| DESCRIPTION(1) (2) |  | DURATION (ns) |  |
| --- | --- | --- | --- |
|  |  | X2TIMING = 0 | X2TIMING = 1 |
| LR | Lead period, read access | XRDLEAD × tc(XTIM) | (XRDLEAD × 2) × tc(XTIM) |
| AR | Active period, read access | (XRDACTIVE + WS + 1) × tc(XTIM) | (XRDACTIVE × 2 + WS + 1) × tc(XTIM) |
| TR | Trail period, read access | XRDTRAIL × tc(XTIM) | (XRDTRAIL × 2) × tc(XTIM) |
| LW | Lead period, write access | XWRLEAD × tc(XTIM) | (XWRLEAD × 2) × tc(XTIM) |
| AW | Active period, write access | (XWRACTIVE + WS + 1) × tc(XTIM) | (XWRACTIVE × 2 + WS + 1) × tc(XTIM) |
| TW | Trail period, write access | XWRTRAIL × tc(XTIM) | (XWRTRAIL × 2) × tc(XTIM) |

- tc(XTIM) – Cycle time, XTIMCLK

- WS refers to the number of wait states inserted by hardware when using XREADY. If the zone is configured to ignore XREADY (USEREADY = 0), then WS = 0.

Minimum wait state requirements must be met when configuring each zone’s XTIMING register. These requirements are in addition to any timing requirements as specified by that device’s data sheet. No internal device hardware is included to detect illegal settings.

        - USEREADY = 0

If the XREADY signal is ignored (USEREADY = 0), then:

1.	Lead:	LR ≥ tc(XTIM) LW ≥ tc(XTIM)

These requirements result in the following XTIMING register configuration restrictions (no hardware to detect illegal XTIMING configurations):

| XRDLEAD | XRDACTIVE | XRDTRAIL | XWRLEAD | XWRACTIVE | XWRTRAIL | X2TIMING |
| --- | --- | --- | --- | --- | --- | --- |
| ≥ 1 | ≥ 0 | ≥ 0 | ≥ 1 | ≥ 0 | ≥ 0 | 0, 1 |

Examples of valid and invalid timing when not sampling XREADY (no hardware to detect illegal XTIMING configurations):

|  | XRDLEAD | XRDACTIVE | XRDTRAIL | XWRLEAD | XWRACTIVE | XWRTRAIL | X2TIMING |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Invalid | 0 | 0 | 0 | 0 | 0 | 0 | 0, 1 |
| Valid | 1 | 0 | 0 | 1 | 0 | 0 | 0, 1 |

        - Synchronous Mode (USEREADY = 1, READYMODE = 0)

If the XREADY signal is sampled in the synchronous mode (USEREADY = 1, READYMODE = 0), then:

- Lead:	LR ≥ tc(XTIM) LW ≥ tc(XTIM)

- Active:	AR ≥ 2 × tc(XTIM)

AW ≥ 2 × tc(XTIM)

NOTE: Restriction does not include external hardware wait states.

These requirements result in the following XTIMING register configuration restrictions (no hardware to detect illegal XTIMING configurations):

| XRDLEAD | XRDACTIVE | XRDTRAIL | XWRLEAD | XWRACTIVE | XWRTRAIL | X2TIMING |
| --- | --- | --- | --- | --- | --- | --- |
| ≥ 1 | ≥ 2 | ≥ 0 | ≥ 1 | ≥ 2 | ≥ 0 | 0, 1 |

Examples of valid and invalid timing when using synchronous XREADY (no hardware to detect illegal XTIMING configurations):

|  | XRDLEAD | XRDACTIVE | XRDTRAIL | XWRLEAD | XWRACTIVE | XWRTRAIL | X2TIMING |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Invalid | 0 | 0 | 0 | 0 | 0 | 0 | 0, 1 |
| Invalid | 1 | 0 | 0 | 1 | 0 | 0 | 0, 1 |
| Valid | 1 | 2 | 0 | 1 | 2 | 0 | 0, 1 |

        - Asynchronous Mode (USEREADY = 1, READYMODE = 1)

If the XREADY signal is sampled in the asynchronous mode (USEREADY = 1, READYMODE = 1), then:

- Lead:	LR ≥ tc(XTIM) LW ≥ tc(XTIM)

- Active:	AR ≥ 2 × tc(XTIM)

AW ≥ 2 × tc(XTIM)

NOTE: Restriction does not include external hardware wait states

- Lead + Active:	LR + AR ≥ 4 × tc(XTIM)

LW + AW ≥ 4 × tc(XTIM)

NOTE: Restriction does not include external hardware wait states

These requirements result in the following XTIMING register configuration restrictions (no hardware to detect illegal XTIMING configurations):

| XRDLEAD | XRDACTIVE | XRDTRAIL | XWRLEAD | XWRACTIVE | XWRTRAIL | X2TIMING |
| --- | --- | --- | --- | --- | --- | --- |
| ≥ 1 | ≥ 2 | 0 | ≥ 1 | ≥ 2 | 0 | 0, 1 |

or (no hardware to detect illegal XTIMING configurations):

| XRDLEAD | XRDACTIVE | XRDTRAIL | XWRLEAD | XWRACTIVE | XWRTRAIL | X2TIMING |
| --- | --- | --- | --- | --- | --- | --- |
| ≥ 2 | ≥ 2 | 0 | ≥ 2 | ≥ 2 | 0 | 0, 1 |

Examples of valid and invalid timing when using asynchronous XREADY (no hardware to detect illegal XTIMING configurations):

|  | XRDLEAD | XRDACTIVE | XRDTRAIL | XWRLEAD | XWRACTIVE | XWRTRAIL | X2TIMING |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Invalid | 0 | 0 | 0 | 0 | 0 | 0 | 0, 1 |
| Invalid | 1 | 0 | 0 | 1 | 0 | 0 | 0, 1 |
| Invalid | 1 | 1 | 0 | 1 | 1 | 0 | 0 |
| Valid | 1 | 2 | 0 | 1 | 2 | 0 | 1 |
| Valid | 1 | 2 | 0 | 1 | 2 | 0 | 0, 1 |
| Valid | 2 | 2 | 0 | 2 | 2 | 0 | 0, 1 |

Unless otherwise specified, all XINTF timing is applicable for the clock configurations shown in Table 8-4.

Table 8-4. XINTF Clock Configurations

| MODE | SYSCLKOUT | XTIMCLK | XCLKOUT |
| --- | --- | --- | --- |
| 1 Example: | 150 MHz | SYSCLKOUT 150 MHz | SYSCLKOUT 150 MHz |
| 2 Example: | 150 MHz | SYSCLKOUT 150 MHz | 1/2 SYSCLKOUT 75 MHz |
| 3 Example: | 150 MHz | 1/2 SYSCLKOUT 75 MHz | 1/2 SYSCLKOUT 75 MHz |
| 4 Example: | 150 MHz | 1/2 SYSCLKOUT 75 MHz | 1/4 SYSCLKOUT 37.5 MHz |

The relationship between SYSCLKOUT and XTIMCLK is shown in Figure 8-26.

(A)

XINTCNF2 (CLKOFF)

0

1

XBANK

XTIMING7

XTIMING2

XTIMING6

XTIMING1

XTIMING0

C28x CPU

/2

1

0

XINTCNF2 (XTIMCLK)

(A)

1

0

XINTCNF2 (CLKMODE)

/2

XCLKOUT

0

XTIMCLK

SYSCLKOUT

LEAD/ACTIVE/TRAIL

(A)

Default value after reset

Figure 8-26. Relationship Between XTIMCLK and SYSCLKOUT

      - XINTF Signal Alignment to XCLKOUT

For each XINTF access, the number of lead, active, and trail cycles is based on the internal clock XTIMCLK. Strobes such as XRD, XWE, and zone chip-select ( XZCS) change state in relationship to the rising edge of XTIMCLK. The external clock, XCLKOUT, can be configured to be either equal to or one-half the frequency of XTIMCLK.

For the case where XCLKOUT = XTIMCLK, all of the XINTF strobes will change state with respect to the rising edge of XCLKOUT. For the case where XCLKOUT = one-half XTIMCLK, some strobes will change state either on the rising edge of XCLKOUT or the falling edge of XCLKOUT. In the XINTF timing tables, the notation XCOHL is used to indicate that the parameter is with respect to either case; XCLKOUT rising edge (high) or XCLKOUT falling edge (low). If the parameter is always with respect to the rising edge of XCLKOUT, the notation XCOH is used.

For the case where XCLKOUT = one-half XTIMCLK, the XCLKOUT edge with which the change will be aligned can be determined based on the number of XTIMCLK cycles from the start of the access to the point at which the signal changes. If this number of XTIMCLK cycles is even, the alignment will be with respect to the rising edge of XCLKOUT. If this number is odd, then the signal will change with respect to the falling edge of XCLKOUT. Examples include the following:

- Strobes that change at the beginning of an access always align to the rising edge of XCLKOUT. This is because all XINTF accesses begin with respect to the rising edge of XCLKOUT.

Examples:	XZCSL	Zone chip-select active-low XRNWL	XR/ W active-low

- Strobes that change at the beginning of the active period will align to the rising edge of XCLKOUT if the total number of lead XTIMCLK cycles for the access is even. If the number of lead XTIMCLK cycles is odd, then the alignment will be with respect to the falling edge of XCLKOUT.

Examples:	XRDL	XRD active-low

XWEL	XWE active-low

- Strobes that change at the beginning of the trail period will align to the rising edge of XCLKOUT if the total number of lead + active XTIMCLK cycles (including hardware waitstates) for the access is even. If the number of lead + active XTIMCLK cycles (including hardware waitstates) is odd, then the alignment will be with respect to the falling edge of XCLKOUT.

Examples:	XRDH	XRD inactive-high

XWEH	XWE inactive-high

- Strobes that change at the end of the access will align to the rising edge of XCLKOUT if the total number of lead + active + trail XTIMCLK cycles (including hardware waitstates) is even. If the number of lead + active + trail XTIMCLK cycles (including hardware waitstates) is odd, then the alignment will be with respect to the falling edge of XCLKOUT.

Examples:	XZCSH	Zone chip-select inactive-high XRNWH	XR/ W inactive-high

      - External Interface Read Timing

        - External Memory Interface Read Switching Characteristics

| PARAMETER |  | MIN | MAX | UNIT |
| --- | --- | --- | --- | --- |
| td(XCOH-XZCSL) | Delay time, XCLKOUT high to zone chip-select active-low | 1 |  | ns |
| td(XCOHL-XZCSH) | Delay time, XCLKOUT high/low to zone chip-select inactive-high | –2 | 3 | ns |
| td(XCOH-XA) | Delay time, XCLKOUT high to address valid | 2 |  | ns |
| td(XCOHL-XRDL) | Delay time, XCLKOUT high/low to XRD active-low | 1 |  | ns |
| td(XCOHL-XRDH) | Delay time, XCLKOUT high/low to XRD inactive-high | –2 | 1 | ns |
| th(XA)XZCSH | Hold time, address valid after zone chip-select inactive-high | (1) |  | ns |
| th(XA)XRD | Hold time, address valid after XRD inactive-high | (1) |  | ns |

(1)	During inactive cycles, the XINTF address bus will always hold the last address put out on the bus. This includes alignment cycles.

        - External Memory Interface Read Timing Requirements

|  |  | MIN MAX | UNIT |
| --- | --- | --- | --- |
| ta(A) | Access time, read data from address valid | (LR + AR) – 14(1) | ns |
| ta(XRD) | Access time, read data valid from XRD active-low | AR – 12(1) | ns |
| tsu(XD)XRD | Setup time, read data valid before XRD strobe inactive-high | 12 | ns |
| th(XD)XRD | Hold time, read data valid after XRD inactive-high | 0 | ns |

(1)	LR = Lead period, read access. AR = Active period, read access. See Table 8-3.

Trail

Lead

Active

td(XCOH-XZCSL)

td(XCOHL-XZCSH)

td(XCOH-XA)

td(XCOHL-XRDL)

td(XCOHL-XRDH)

tsu(XD)XRD

ta(A)

ta(XRD)

th(XD)XRD

DIN

XCLKOUT = XTIMCLK XCLKOUT = 1/2 XTIMCLK

XZCS0AND1, XZCS2,

XZCS6AND7

XA[0:18]

XRD XWE

XR/W

XD[0:15] XREADY

- All XINTF accesses (lead period) begin on the rising edge of XCLKOUT. When necessary, the device will insert an alignment cycle before an access to meet this requirement.

- During alignment cycles, all signals will transition to their inactive state.

- For USEREADY = 0, the external XREADY input signal is ignored.

- XA[0:18] will hold the last address put on the bus during inactive cycles, including alignment cycles.

Figure 8-27. Example Read Access

XTIMING register parameters used for this example:

| XRDLEAD | XRDACTIVE | XRDTRAIL | USEREADY | X2TIMING | XWRLEAD | XWRACTIVE | XWRTRAIL | READYMODE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ≥ 1 | ≥ 0 | ≥ 0 | 0 | 0 | N/A(1) | N/A(1) | N/A(1) | N/A(1) |

(1)	N/A = “Don’t care” for this example

      - External Interface Write Timing

        - External Memory Interface Write Switching Characteristics

| PARAMETER |  | MIN | MAX | UNIT |
| --- | --- | --- | --- | --- |
| td(XCOH-XZCSL) | Delay time, XCLKOUT high to zone chip-select active-low | 1 |  | ns |
| td(XCOHL-XZCSH) | Delay time, XCLKOUT high or low to zone chip-select inactive-high | –2 | 3 | ns |
| td(XCOH-XA) | Delay time, XCLKOUT high to address valid | 2 |  | ns |
| td(XCOHL-XWEL) | Delay time, XCLKOUT high/low to XWE low | 2 |  | ns |
| td(XCOHL-XWEH) | Delay time, XCLKOUT high/low to XWE high | 2 |  | ns |
| td(XCOH-XRNWL) | Delay time, XCLKOUT high to XR/ W low | 1 |  | ns |
| td(XCOHL-XRNWH) | Delay time, XCLKOUT high/low to XR/ W high | –2 | 1 | ns |
| ten(XD)XWEL | Enable time, data bus driven from XWE low | 0 |  | ns |
| td(XWEL-XD) | Delay time, data valid after XWE active-low | 4 |  | ns |
| th(XA)XZCSH | Hold time, address valid after zone chip-select inactive-high | (1) |  | ns |
| th(XD)XWE | Hold time, write data valid after XWE inactive-high | TW – 2(2) |  | ns |
| tdis(XD)XRNW | Maximum time for DSP to release the data bus after XR/ W inactive-high | 4 |  | ns |

- During inactive cycles, the XINTF address bus will always hold the last address put out on the bus. This includes alignment cycles.

- TW = Trail period, write access. See Table 8-3.

Lead

Active

Trail

td(XCOH-XZCSL)

td(XCOHL-XZCSH)

td(XCOH-XA)

td(XCOHL-XWEL)

td(XCOHL-XWEH)

td(XCOH-XRNWL)

td(XCOHL-XRNWH)

td(XWEL-XD)

ten(XD)XWEL

tdis(XD)XRNW th(XD)XWEH

DOUT

XCLKOUT = XTIMCLK XCLKOUT = 1/2 XTIMCLK

XZCS0AND1, XZCS2,

XZCS6AND7

XA[0:18]

XRD

XWE XR/W

XD[0:15]

XREADY

- All XINTF accesses (lead period) begin on the rising edge of XCLKOUT. When necessary, the device will insert an alignment cycle before an access to meet this requirement.

- During alignment cycles, all signals will transition to their inactive state.

- For USEREADY = 0, the external XREADY input signal is ignored.

- XA[0:18] will hold the last address put on the bus during inactive cycles, including alignment cycles.

Figure 8-28. Example Write Access

XTIMING register parameters used for this example:

| XRDLEAD | XRDACTIVE | XRDTRAIL | USEREADY | X2TIMING | XWRLEAD | XWRACTIVE | XWRTRAIL | READYMODE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N/A(1) | N/A(1) | N/A(1) | 0 | 0 | ≥ 1 | ≥ 0 | ≥ 0 | N/A(1) |

(1)	N/A = “Don’t care” for this example

      - External Interface Ready-on-Read Timing With One External Wait State

        - External Memory Interface Read Switching Characteristics (Ready-on-Read, 1 Wait State)

| PARAMETER |  | MIN | MAX | UNIT |
| --- | --- | --- | --- | --- |
| td(XCOH-XZCSL) | Delay time, XCLKOUT high to zone chip-select active-low | 1 |  | ns |
| td(XCOHL-XZCSH) | Delay time, XCLKOUT high/low to zone chip-select inactive-high | –2 | 3 | ns |
| td(XCOH-XA) | Delay time, XCLKOUT high to address valid | 2 |  | ns |
| td(XCOHL-XRDL) | Delay time, XCLKOUT high/low to XRD active-low | 1 |  | ns |
| td(XCOHL-XRDH) | Delay time, XCLKOUT high/low to XRD inactive-high | –2 | 1 | ns |
| th(XA)XZCSH | Hold time, address valid after zone chip-select inactive-high | (1) |  | ns |
| th(XA)XRD | Hold time, address valid after XRD inactive-high | (1) |  | ns |

(1)	During inactive cycles, the XINTF address bus will always hold the last address put out on the bus. This includes alignment cycles.

        - External Memory Interface Read Timing Requirements (Ready-on-Read, 1 Wait State)

|  |  | MIN MAX | UNIT |
| --- | --- | --- | --- |
| ta(A) | Access time, read data from address valid | (LR + AR) – 14(1) | ns |
| ta(XRD) | Access time, read data valid from XRD active-low | AR – 12(1) | ns |
| tsu(XD)XRD | Setup time, read data valid before XRD strobe inactive-high | 12 | ns |
| th(XD)XRD | Hold time, read data valid after XRD inactive-high | 0 | ns |

(1)	LR = Lead period, read access. AR = Active period, read access. See Table 8-3.

        - Synchronous XREADY Timing Requirements (Ready-on-Read, 1 Wait State)

|  |  | MIN(1) MAX | UNIT |
| --- | --- | --- | --- |
| tsu(XRDYsynchL)XCOHL | Setup time, XREADY (synchronous) low before XCLKOUT high/low | 15 | ns |
| th(XRDYsynchL) | Hold time, XREADY (synchronous) low | 12 | ns |
| te(XRDYsynchH) | Earliest time XREADY (synchronous) can go high before the sampling XCLKOUT edge | 3 | ns |
| tsu(XRDYsynchH)XCOHL | Setup time, XREADY (synchronous) high before XCLKOUT high/low | 15 | ns |
| th(XRDYsynchH)XZCSH | Hold time, XREADY (synchronous) held high after zone chip-select high | 0 | ns |

(1)	The first XREADY (synchronous) sample occurs with respect to E in Figure 8-29: E = (XRDLEAD + XRDACTIVE) tc(XTIM)

When first sampled, if XREADY (synchronous) is found to be high, then the access will complete. If XREADY (synchronous) is found to be low, it will be sampled again each tc(XTIM) until it is found to be high.

For each sample (n), the setup time (D) with respect to the beginning of the access can be calculated as:

D = (XRDLEAD + XRDACTIVE + n – 1) tc(XTIM) – tsu(XRDYsynchL)XCOHL

where n is the sample number (n = 1, 2, 3, and so forth).

        - Asynchronous XREADY Timing Requirements (Ready-on-Read, 1 Wait State)

|  |  | MIN(1) MAX | UNIT |
| --- | --- | --- | --- |
| tsu(XRDYAsynchL)XCOHL | Setup time, XREADY (asynchronous) low before XCLKOUT high/low | 11 | ns |
| th(XRDYAsynchL) | Hold time, XREADY (asynchronous) low | 8 | ns |
| te(XRDYAsynchH) | Earliest time XREADY (asynchronous) can go high before the sampling XCLKOUT edge | 3 | ns |
| tsu(XRDYAsynchH)XCOHL | Setup time, XREADY (asynchronous) high before XCLKOUT high/low | 11 | ns |
| th(XRDYAsynchH)XZCSH | Hold time, XREADY (asynchronous) held high after zone chip-select high | 0 | ns |

(1)	The first XREADY (asynchronous) sample occurs with respect to E in Figure 8-30: E = (XRDLEAD + XRDACTIVE – 2) tc(XTIM)

When first sampled, if XREADY (asynchronous) is found to be high, then the access will complete. If XREADY (asynchronous) is found to be low, it will be sampled again each tc(XTIM) until it is found to be high.

For each sample, setup time from the beginning of the access can be calculated as: D = (XRDLEAD + XRDACTIVE – 3 + n) tc(XTIM) – tsu(XRDYAsynchL)XCOHL

where n is the sample number (n = 1, 2, 3, and so forth).

See Notes (A) and (B)

Lead

Active

WS (Synch)

Trail

See Note (C)

XCLKOUT = XTIMCLK

XCLKOUT = 1/2 XTIMCLK

XZCS0AND1, XZCS2,

XZCS6AND7

XA[0:18]

XRD

XWE XR/W

td(XCOH-XZCSL) td(XCOH-XA)

td(XCOHL-XRDL)

tsu(XD)XRD ta(XRD)

td(XCOHL-XZCSH)

td(XCOHL-XRDH)

XD[0:15]

ta(A)

tsu(XRDYsynchL)XCOHL

DIN

th(XD)XRD

th(XRDYsynchL)

t

te(XRDYsynchH)

th(XRDYsynchH)XZCSH

XREADY(Synch)

su(XRDHsynchH)XCOHL

See Note (D)

See Note (E)

Legend:

= Don’t care. Signal can be high or low during this time.

- All XINTF accesses (lead period) begin on the rising edge of XCLKOUT. When necessary, the device will insert an alignment cycle before an access to meet this requirement.

- During alignment cycles, all signals will transition to their inactive state.

- During inactive cycles, the XINTF address bus will always hold the last address put out on the bus. This includes alignment cycles.

- For each sample, setup time from the beginning of the access (D) can be calculated as: D = (XRDLEAD + XRDACTIVE + n – 1) tc(XTIM) – tsu(XRDYsynchL)XCOHL

- Reference for the first sample is with respect to this point E = (XRDLEAD + XRDACTIVE) tc(XTIM) where n is the sample number (n = 1, 2, 3, and so forth).

Figure 8-29. Example Read With Synchronous XREADY Access

XTIMING register parameters used for this example:

| XRDLEAD | XRDACTIVE | XRDTRAIL | USEREADY | X2TIMING | XWRLEAD | XWRACTIVE | XWRTRAIL | READYMODE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ≥ 1 | 3 | ≥ 1 | 1 | 0 | N/A(1) | N/A(1) | N/A(1) | 0 = XREADY (Synch) |

(1)	N/A = “Don’t care” for this example

See

Notes (A) and (B)

WS (Async)

Active

Lead

Trail

See Note (C)

td(XCOH-XZCSL)

td(XCOHL-XZCSH)

td(XCOH-XA)

td(XCOHL-XRDL)

tsu(XD)XRD ta(XRD)

td(XCOHL-XRDH)

ta(A)

th(XD)XRD

DIN

tsu(XRDYasynchL)XCOHL

te(XRDYasynchH)

th(XRDYasynchH)XZCSH

th(XRDYasynchL)

tsu(XRDYasynchH)XCOHL

See Note (D)

See Note (E)

XCLKOUT = XTIMCLK XCLKOUT = 1/2 XTIMCLK

XZCS0AND1, XZCS2,

XZCS6AND7

XA[0:18]

XRD XWE XR/W

XD[0:15]

XREADY(Asynch)

Legend:

= Don’t care. Signal can be high or low during this time.

- All XINTF accesses (lead period) begin on the rising edge of XCLKOUT. When necessary, the device will insert an alignment cycle before an access to meet this requirement.

- During alignment cycles, all signals will transition to their inactive state.

- During inactive cycles, the XINTF address bus will always hold the last address put out on the bus. This includes alignment cycles.

- For each sample, setup time from the beginning of the access can be calculated as: D = (XRDLEAD + XRDACTIVE – 3 + n) tc(XTIM) – tsu(XRDYAsynchL)XCOHL where n is the sample number (n = 1, 2, 3, and so forth).

- Reference for the first sample is with respect to this point: E = (XRDLEAD + XRDACTIVE – 2) tc(XTIM)

Figure 8-30. Example Read With Asynchronous XREADY Access

XTIMING register parameters used for this example:

| XRDLEAD | XRDACTIVE | XRDTRAIL | USEREADY | X2TIMING | XWRLEAD | XWRACTIVE | XWRTRAIL | READYMODE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ≥ 1 | 3 | ≥ 1 | 1 | 0 | N/A(1) | N/A(1) | N/A(1) | 1 = XREADY (Async) |

(1)	N/A = “Don’t care” for this example

      - External Interface Ready-on-Write Timing With One External Wait State

        - External Memory Interface Write Switching Characteristics (Ready-on-Write, 1 Wait State)

| PARAMETER |  | MIN | MAX | UNIT |
| --- | --- | --- | --- | --- |
| td(XCOH-XZCSL) | Delay time, XCLKOUT high to zone chip-select active-low | 1 |  | ns |
| td(XCOHL-XZCSH) | Delay time, XCLKOUT high or low to zone chip-select inactive-high | –2 | 3 | ns |
| td(XCOH-XA) | Delay time, XCLKOUT high to address valid | 2 |  | ns |
| td(XCOHL-XWEL) | Delay time, XCLKOUT high/low to XWE low | 2 |  | ns |
| td(XCOHL-XWEH) | Delay time, XCLKOUT high/low to XWE high | 2 |  | ns |
| td(XCOH-XRNWL) | Delay time, XCLKOUT high to XR/ W low | 1 |  | ns |
| td(XCOHL-XRNWH) | Delay time, XCLKOUT high/low to XR/ W high | –2 | 1 | ns |
| ten(XD)XWEL | Enable time, data bus driven from XWE low | 0 |  | ns |
| td(XWEL-XD) | Delay time, data valid after XWE active-low | 4 |  | ns |
| th(XA)XZCSH | Hold time, address valid after zone chip-select inactive-high | (1) |  | ns |
| th(XD)XWE | Hold time, write data valid after XWE inactive-high | TW – 2(2) |  | ns |
| tdis(XD)XRNW | Maximum time for DSP to release the data bus after XR/ W inactive-high | 4 |  | ns |

- During inactive cycles, the XINTF address bus will always hold the last address put out on the bus. This includes alignment cycles.

- TW = trail period, write access. See Table 8-3.

        - Synchronous XREADY Timing Requirements (Ready-on-Write, 1 Wait State)

|  |  | MIN(1) MAX | UNIT |
| --- | --- | --- | --- |
| tsu(XRDYsynchL)XCOHL | Setup time, XREADY (synchronous) low before XCLKOUT high/low | 15 | ns |
| th(XRDYsynchL) | Hold time, XREADY (synchronous) low | 12 | ns |
| te(XRDYsynchH) | Earliest time XREADY (synchronous) can go high before the sampling XCLKOUT edge | 3 | ns |
| tsu(XRDYsynchH)XCOHL | Setup time, XREADY (synchronous) high before XCLKOUT high/low | 15 | ns |
| th(XRDYsynchH)XZCSH | Hold time, XREADY (synchronous) held high after zone chip-select high | 0 | ns |

(1)	The first XREADY (synchronous) sample occurs with respect to E in Figure 8-31: E = (XWRLEAD + XWRACTIVE) tc(XTIM)

When first sampled, if XREADY (synchronous) is found to be high, then the access will complete. If XREADY (synchronous) is found to be low, it will be sampled again each tc(XTIM) until it is found to be high.

For each sample, setup time from the beginning of the access can be calculated as:

D = (XWRLEAD + XWRACTIVE + n – 1) tc(XTIM) – tsu(XRDYsynchL)XCOHL

where n is the sample number (n = 1, 2, 3, and so forth).

        - Asynchronous XREADY Timing Requirements (Ready-on-Write, 1 Wait State)

|  |  | MIN(1) MAX | UNIT |
| --- | --- | --- | --- |
| tsu(XRDYasynchL)XCOHL | Setup time, XREADY (asynchronous) low before XCLKOUT high/low | 11 | ns |
| th(XRDYasynchL) | Hold time, XREADY (asynchronous) low | 8 | ns |
| te(XRDYasynchH) | Earliest time XREADY (asynchronous) can go high before the sampling XCLKOUT edge | 3 | ns |
| tsu(XRDYasynchH)XCOHL | Setup time, XREADY (asynchronous) high before XCLKOUT high/low | 11 | ns |
| th(XRDYasynchH)XZCSH | Hold time, XREADY (asynchronous) held high after zone chip-select high | 0 | ns |

(1)	The first XREADY (synchronous) sample occurs with respect to E in Figure 8-32: E = (XWRLEAD + XWRACTIVE – 2) tc(XTIM)

When first sampled, if XREADY (asynchronous) is found to be high, then the access will complete. If XREADY (asynchronous) is found to be low, it will be sampled again each tc(XTIM) until it is found to be high.

For each sample, setup time from the beginning of the access can be calculated as:

D = (XWRLEAD + XWRACTIVE – 3 + n) tc(XTIM) – tsu(XRDYasynchL)XCOHL

where n is the sample number (n = 1, 2, 3, and so forth).

See

Notes (A) and (B)

WS (Synch)

Active

Trail

See Note (C)

Lead 1

td(XCOH-XZCSL)

td(XCOHL-XZCSH)

td(XCOH-XA)

th(XRDYsynchH)XZCSH

td(XCOHL-XWEL)

td(XCOHL-XWEH)

td(XCOH-XRNWL)

td(XCOHL-XRNWH)

td(XWEL-XD)

t

tdis(XD)XRNW th(XD)XWEH

en(XD)XWEL

DOUT

tsu(XRDYsynchL)XCOHL

te(XRDYsynchH)

th(XRDYsynchL)

tsu(XRDHsynchH)XCOHL

See Note (D)

See Note (E)

XCLKOUT = XTIMCLK XCLKOUT = 1/2 XTIMCLK

XZCS0AND1, XZCS2,

XZCS6AND7

XA[0:18]

XRD

XWE

XR/W

XD[0:15]

XREADY(Synch)

Legend:

= Don’t care. Signal can be high or low during this time.

- All XINTF accesses (lead period) begin on the rising edge of XCLKOUT. When necessary, the device will insert an alignment cycle before an access to meet this requirement.

- During alignment cycles, all signals will transition to their inactive state.

- During inactive cycles, the XINTF address bus will always hold the last address put out on the bus. This includes alignment cycles.

- For each sample, setup time from the beginning of the access can be calculated as: D = (XWRLEAD + XWRACTIVE + n – 1) tc(XTIM) – tsu(XRDYsynchL)XCOHL where n is the sample number (n = 1, 2, 3 and so forth).

- Reference for the first sample is with respect to this point E = (XWRLEAD + XWRACTIVE) tc(XTIM)

Figure 8-31. Write With Synchronous XREADY Access

XTIMING register parameters used for this example:

| XRDLEAD | XRDACTIVE | XRDTRAIL | USEREADY | X2TIMING | XWRLEAD | XWRACTIVE | XWRTRAIL | READYMODE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N/A(1) | N/A(1) | N/A(1) | 1 | 0 | ≥ 1 | 3 | ≥ 1 | 0 = XREADY (Synch) |

(1)	N/A = “Don’t care” for this example

See Notes (A) and (B)

Lead 1

WS (Async) Active

Trail

See Note (C)

XCLKOUT = XTIMCLK

XCLKOUT = 1/2 XTIMCLK

XZCS0AND1, XZCS2,

XZCS6AND7

XA[0:18]

td(XCOH-XZCSL) td(XCOH-XA)

td(XCOHL-XZCSH)

th(XRDYasynchH)XZCSH

XRD

XWE XR/W

XD[0:15]

td(XCOH-XRNWL)

ten(XD)XWEL

td(XCOHL-XWEL)

td(XWEL-XD)

DOUT

td(XCOHL-XWEH) td(XCOHL-XRNWH)

tdis(XD)XRNW th(XD)XWEH

tsu(XRDYasynchL)XCOHL th(XRDYasynchL)

te(XRDYasynchH)

tsu(XRDYasynchH)XCOHL

XREADY(Asynch)

See Note (D)

See Note (E)

Legend:

= Don’t care. Signal can be high or low during this time.

- All XINTF accesses (lead period) begin on the rising edge of XCLKOUT. When necessary, the device will insert an alignment cycle before an access to meet this requirement.

- During alignment cycles, all signals will transition to their inactive state.

- During inactive cycles, the XINTF address bus will always hold the last address put out on the bus. This includes alignment cycles.

- For each sample, setup time from the beginning of the access can be calculated as: D = (XWRLEAD + XWRACTIVE – 3 + n) tc(XTIM) – tsu(XRDYasynchL)XCOHL where n is the sample number (n = 1, 2, 3 and so forth).

- Reference for the first sample is with respect to this point E = (XWRLEAD + XWRACTIVE – 2) tc(XTIM)

Figure 8-32. Write With Asynchronous XREADY Access

XTIMING register parameters used for this example:

| XRDLEAD | XRDACTIVE | XRDTRAIL | USEREADY | X2TIMING | XWRLEAD | XWRACTIVE | XWRTRAIL | READYMODE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N/A(1) | N/A(1) | N/A(1) | 1 | 0 | ≥ 1 | 3 | ≥ 1 | 1 = XREADY (Async) |

(1)	N/A = “Don’t care” for this example

      - XHOLD and XHOLDA

If the HOLD mode bit is set while XHOLD and XHOLDA are both low (external bus accesses granted), the XHOLDA signal is forced high (at the end of the current cycle) and the external interface is taken out of high- impedance mode.

On a reset ( XRS), the HOLD mode bit is set to 0. If the XHOLD signal is active low on a system reset, the bus and all signal strobes must be in high-impedance mode, and the XHOLDA signal is also driven active low.

When HOLD mode is enabled and XHOLDA is active-low (external bus grant active), the CPU can still execute code from internal memory. If an access is made to the external interface, the CPU is stalled until the XHOLD signal is removed.

An external DMA request, when granted, places the following signals in a high-impedance mode:

XA[18:0]	XZCS0AND1

XD[15:0]	XZCS2

XWE, XRD	XZCS6AND7

XR/ W

All other signals not listed in this group remain in their default or functional operational modes during these signal events.

      - XHOLD/ XHOLDA Timing

        - XHOLD/ XHOLDA Timing Requirements (XCLKOUT = XTIMCLK)

|  |  | MIN(1) (2) MAX | UNIT |
| --- | --- | --- | --- |
| td(HL-HiZ) | Delay time, XHOLD low to Hi-Z on all Address, Data, and Control | 4tc(XTIM) | ns |
| td(HL-HAL) | Delay time, XHOLD low to XHOLDA low | 5tc(XTIM) | ns |
| td(HH-HAH) | Delay time, XHOLD high to XHOLDA high | 3tc(XTIM) | ns |
| td(HH-BV) | Delay time, XHOLD high to Bus valid | 4tc(XTIM) | ns |

- When a low signal is detected on XHOLD, all pending XINTF accesses will be completed before the bus is placed in a high-impedance state.

- The state of XHOLD is latched on the rising edge of XTIMCLK.

td(HL-Hiz)

td(HH-HAH)

td(HL-HAL)

td(HH-BV)

High-Impedance

Valid

High-Impedance

Valid

Valid

See Note (A)

See Note (B)

XCLKOUT

(/1 Mode)

XHOLD

XHOLDA

XR/W, XZCS0AND1,

XZCS2, XZCS6AND7

XA[18:0]

XD[15:0]

- All pending XINTF accesses are completed.

- Normal XINTF operation resumes.

Figure 8-33. External Interface Hold Waveform

        - XHOLD/ XHOLDA Timing Requirements (XCLKOUT = 1/2 XTIMCLK)

|  |  | MIN(1) (2) MAX (3) | UNIT |
| --- | --- | --- | --- |
| td(HL-HiZ) | Delay time, XHOLD low to Hi-Z on all Address, Data, and Control | 4tc(XTIM) + tc(XCO) | ns |
| td(HL-HAL) | Delay time, XHOLD low to XHOLDA low | 4tc(XTIM) + 2tc(XCO) | ns |
| td(HH-HAH) | Delay time, XHOLD high to XHOLDA high | 4tc(XTIM) | ns |
| td(HH-BV) | Delay time, XHOLD high to Bus valid | 6tc(XTIM) | ns |

- When a low signal is detected on XHOLD, all pending XINTF accesses will be completed before the bus is placed in a high-impedance state.

- The state of XHOLD is latched on the rising edge of XTIMCLK.

- After the XHOLD is detected low or high, all bus transitions and XHOLDA transitions will occur with respect to the rising edge of XCLKOUT. Thus, for this mode where XCLKOUT = 1/2 XTIMCLK, the transitions can occur up to 1 XTIMCLK cycle earlier than the maximum value specified.

td(HL-HAL)

td(HH-HAH)

td(HL-HiZ)

td(HH-BV)

High-Impedance

Valid

High-Impedance

Valid

Valid

High-Impedance

See Note (A)

See Note (B)

XCLKOUT (1/2 XTIMCLK)

XHOLD

XHOLDA

XR/W, XZCS0AND1,

XZCS2, XZCS6AND7

XA[18:0]

XD[15:0]

- All pending XINTF accesses are completed.

- Normal XINTF operation resumes.

Figure 8-34. XHOLD/ XHOLDA Timing Requirements (XCLKOUT = 1/2 XTIMCLK)

      - On-Chip Analog-to-Digital Converter

        - ADC Absolute Maximum Ratings Over Recommended Operating Conditions (Unless Otherwise Noted)

|  |  | MIN(1) | MAX | UNIT |
| --- | --- | --- | --- | --- |
| Supply voltage range | VSSA1/VSSA2 to VDDA1/VDDA2/AVDDREFBG | –0.3 | 4.6 | V |
| Supply voltage range | VSS1 to VDD1 | –0.3 | 2.5 | V |
| Analog Input (ADCIN) Clamp Current, total (max) |  | –20(2) | 20(2) | mA |

- Unless otherwise noted, the list of absolute maximum ratings are specified over recommended operating conditions. Stresses beyond those listed under Absolute Maximum Ratings may cause permanent damage to the device. These are stress ratings only, and functional operation of the device at these or any other conditions beyond those indicated under Recommended Operating Conditions is not implied. Exposure to absolute-maximum-rated conditions for extended periods may affect device reliability.

- The analog inputs have an internal clamping circuit that clamps the voltage to a diode drop above VDDA or below VSS. The continuous clamp current per pin is ±2 mA.

        - ADC Electrical Characteristics Over Recommended Operating Conditions (Unless Otherwise Noted)—AC Specifications

| PARAMETER |  | MIN TYP MAX | UNIT |
| --- | --- | --- | --- |
| SINAD | Signal-to-noise ratio + distortion | 62 | dB |
| SNR | Signal-to-noise ratio | 62 | dB |
| THD (100 kHz) | Total harmonic distortion | –68 | dB |
| ENOB (SNR) | Effective number of bits | 10.1 | Bits |
| SFDR | Spurious free dynamic range | 69 | dB |

        - ADC Electrical Characteristics Over Recommended Operating Conditions (Unless Otherwise Noted)—DC Specifications

| PARAMETER(1) |  | MIN | TYP | MAX | UNIT |
| --- | --- | --- | --- | --- | --- |
| Resolution |  | 12 |  |  | Bits |
| ADC clock(2) |  | 1 |  |  | kHz |
|  |  | 25 |  |  | MHz |
| ACCURACY |  |  |  |  |  |
| INL (Integral nonlinearity)(3) | 1–18.75 MHz ADC clock | ±1.5 |  |  | LSB |
| DNL (Differential nonlinearity)(3) | 1–18.75 MHz ADC clock | ±1 |  |  | LSB |
| Offset error(4) |  | –80 |  | 80 | LSB |
| Overall gain error with internal reference(5) |  | –200 |  | 200 | LSB |
| Overall gain error with external reference(6) | If ADCREFP – ADCREFM = 1 V ± 0.1% | –50 |  | 50 | LSB |
| Channel-to-channel offset variation |  | ±8 |  |  | LSB |
| Channel-to-channel Gain variation |  | ±8 |  |  | LSB |
| ANALOG INPUT |  |  |  |  |  |
| Analog input voltage (ADCINx to ADCLO)(7) |  | 0 |  | 3 | V |
| ADCLO |  | –5 | 0 | 5 | mV |
| Input capacitance |  | 10 |  |  | pF |
| Input leakage current |  |  | 3 | ±5 | µA |
| INTERNAL VOLTAGE REFERENCE (5) |  |  |  |  |  |
| Accuracy, ADCVREFP |  | 1.9 | 2 | 2.1 | V |
| Accuracy, ADCVREFM |  | 0.95 | 1 | 1.05 | V |
| Voltage difference, ADCREFP – ADCREFM |  | 1 |  |  | V |
| Temperature coefficient |  | 50 |  |  | PPM/°C |
| Reference noise |  | 100 |  |  | µV |
| EXTERNAL VOLTAGE REFERENCE (6) |  |  |  |  |  |
| Accuracy, ADCVREFP |  | 1.9 | 2 | 2.1 | V |
| Accuracy, ADCVREFM |  | 0.95 | 1 | 1.05 | V |
| Input voltage difference, ADCREFP – ADCREFM |  | 0.99 | 1 | 1.01 | V |

- Tested at 12.5-MHz ADCCLK.

- If SYSCLKOUT ≤ 25 MHz, ADC clock ≤ SYSCLKOUT/2.

- The INL degrades for frequencies beyond 18.75 MHz–25 MHz. Applications that require these sampling rates should use a 20K resistor as bias resistor on the ADCRESEXT pin. This improves overall linearity and typical current drawn by the ADC will be a few mA more than 24.9-kΩ bias.

- 1 LSB has the weighted value of 3.0/4096 = 0.732 mV.

- A single internal band gap reference (±5% accuracy) sources both ADCREFP and ADCREFM signals, and hence, these voltages track together. The ADC converter uses the difference between these two as its reference. The total gain error will be the combination of the gain error shown here and the voltage reference accuracy (ADCREFP – ADCREFM). A software-based calibration procedure is recommended for better accuracy. See the F2810, F2811, and F2812 ADC Calibration Application Report and Section 11.4 for relevant documents.

- In this mode, the accuracy of external reference is critical for overall gain. The voltage difference (ADCREFP – ADCREFM) will determine the overall accuracy.

- Voltages above VDDA + 0.3 V or below VSS – 0.3 V applied to an analog input pin may temporarily affect the conversion of another pin. To avoid this, the analog inputs should be kept within these limits.

        - Current Consumption for Different ADC Configurations

          - Current Consumption for Different ADC Configurations (at 25-MHz ADCCLK)

| IDDA (TYP)(1) (2) | IDDAIO (TYP) | IDD1 (TYP) | ADC OPERATING MODE/CONDITIONS |
| --- | --- | --- | --- |
| 40 mA | 1 µA | 0.5 mA | Mode A (Operational Mode): - BG and REF enabled - PWD disabled |
| 7 mA | 0 | 5 µA | Mode B: - ADC clock enabled - BG and REF enabled - PWD enabled |
| 1 µA | 0 | 5 µA | Mode C: - ADC clock enabled - BG and REF disabled - PWD enabled |
| 1 µA | 0 | 0 | Mode D: - ADC clock disabled - BG and REF disabled - PWD enabled |

- Test Conditions:

  - SYSCLKOUT = 150 MHz

  - ADC module clock = 25 MHz

  - ADC performing a continuous conversion of all 16 channels in Mode A

- IDDA – includes current into VDDA1/VDDA2 and AVDDREFBG

Rs

ADCIN0

Ron 1 k

Switch

ac

Cp

10 pF

Ch

1.25 pF

28x DSP

Source Signal

Typical Values of the Input Circuit Components:

Switch Resistance (Ron): 1 k Sampling Capacitor (Ch): 1.25 pF Parasitic Capacitance (Cp): 10 pF Source Resistance (Rs): 50 

Figure 8-35. ADC Analog Input Impedance Model

        - ADC Power-Up Control Bit Timing

ADC Power Up Delay

ADC Ready for Conversions

td(BGR)

td(PWD)

PWDNBG

PWDNREF

PWDNADC

Request for ADC Conversion

Figure 8-36. ADC Power-Up Control Bit Timing

          - ADC Power-Up Delays

|  |  | MIN(1) | TYP | MAX | UNIT |
| --- | --- | --- | --- | --- | --- |
| td(BGR) | Delay time for band gap reference to be stable. Bits 7 and 6 of the ADCTRL3 register (ADCBGRFDN1/0) are to be set to 1 before the ADCPWDN bit is enabled. | 7 | 8 | 10 | ms |
| td(PWD) | Delay time for power-down control to be stable. Bit 5 of the ADCTRL3 register (ADCPWDN) is to be set to 1 before any ADC conversions are initiated. | 20 | 50 |  | µs |
|  |  | 1 |  |  | ms |

(1)	These delays are necessary and recommended to make the ADC analog reference circuit stable before conversions are initiated. If conversions are started without these delays, the ADC results will show a higher gain. For power down, all three bits can be cleared at the same time.

        - Detailed Description

          - Reference Voltage

The on-chip ADC has a built-in reference, which provides the reference voltages for the ADC. ADCVREFP is set to 2.0 V and ADCVREFM is set to 1.0 V.

          - Analog Inputs

The on-chip ADC consists of 16 analog inputs, which are sampled either one at a time or two channels at a time. These inputs are software-selectable.

          - Converter

The on-chip ADC uses a 12-bit four-stage pipeline architecture, which achieves a high sample rate with low power consumption.

          - Conversion Modes

The conversion can be performed in two different conversion modes:

            - Sequential sampling mode (SMODE = 0)

            - Simultaneous sampling mode (SMODE = 1)

        - Sequential Sampling Mode (Single-Channel) (SMODE = 0)

In sequential sampling mode, the ADC can continuously convert input signals on any of the channels (Ax to Bx). The ADC can start conversions on event triggers from the Event Managers (EVA/EVB), software trigger, or from an external ADCSOC signal. If the SMODE bit is 0, the ADC will do conversions on the selected channel on every Sample/Hold pulse. The conversion time and latency of the Result register update are explained below. The ADC interrupt flags are set a few SYSCLKOUT cycles after the Result register update. The selected channels will be sampled at every falling edge of the Sample/Hold pulse. The Sample/Hold pulse width can be programmed to be 1 ADC clock wide (minimum) or 16 ADC clocks wide (maximum).

Sample n+2

Sample n+1

Sample n

td(SH)

tdschx_n+1

tdschx_n

tSH

Analog Input on Channel Ax or Bx

ADC Clock Sample and Hold

SH Pulse SMODE Bit

ADC Event Trigger from EV

or Other Sources

Figure 8-37. Sequential Sampling Mode (Single-Channel) Timing

          - Sequential Sampling Mode Timing

|  |  | SAMPLE n | SAMPLE n + 1 | AT 25-MHz ADC CLOCK, tc(ADCCLK) = 40 ns | REMARKS |
| --- | --- | --- | --- | --- | --- |
| td(SH) | Delay time from event trigger to sampling | 2.5tc(ADCCLK) |  |  |  |
| tSH | Sample/ Hold width/ Acquisition width | (1 + Acqps) * tc(ADCCLK) |  | 40 ns with Acqps = 0 | Acqps value = 0–15 ADCTRL1[8:11] |
| td(schx_n) | Delay time for first result to appear in the Result register | 4tc(ADCCLK) |  | 160 ns |  |
| td(schx_n+1) | Delay time for successive results to appear in the Result register |  | (2 + Acqps) * tc(ADCCLK) | 80 ns |  |

        - Simultaneous Sampling Mode (Dual-Channel) (SMODE = 1)

In simultaneous mode, the ADC can continuously convert input signals on any one pair of channels (A0/B0 to A7/B7). The ADC can start conversions on event triggers from the Event Managers (EVA/EVB), software trigger, or from an external ADCSOC signal. If the SMODE bit is 1, the ADC will do conversions on two selected channels on every Sample/Hold pulse. The conversion time and latency of the Result register update are explained below. The ADC interrupt flags are set a few SYSCLKOUT cycles after the Result register update. The selected channels will be sampled simultaneously at the falling edge of the Sample/Hold pulse. The Sample/ Hold pulse width can be programmed to be 1 ADC clock wide (minimum) or 16 ADC clocks wide (maximum).

Note

In Simultaneous Mode, the ADCIN channel pair select has to be A0/B0, A1/B1, ..., A7/B7, and not in other combinations (such as A1/B3, and so forth).

Sample n

Sample n+1

Sample n+2

td(SH)

tSH

tdschA0_n+1

tdschA0_n

tdschB0_n

tdschB0_n+1

Analog Input on Channel Ax Analog Input on Channel Bv

ADC Clock Sample and Hold

SH Pulse

SMODE Bit

ADC Event Trigger from EV

or Other Sources

Figure 8-38. Simultaneous Sampling Mode Timing

          - Simultaneous Sampling Mode Timing

|  |  | SAMPLE n | SAMPLE n + 1 | AT 25-MHz ADC CLOCK, tc(ADCCLK) = 40 ns | REMARKS |
| --- | --- | --- | --- | --- | --- |
| td(SH) | Delay time from event trigger to sampling | 2.5tc(ADCCLK) |  |  |  |
| tSH | Sample/Hold width/ Acquisition Width | (1 + Acqps) * tc(ADCCLK) |  | 40 ns with Acqps = 0 | Acqps value = 0–15 ADCTRL1[8:11] |
| td(schA0_n) | Delay time for first result to appear in Result register | 4tc(ADCCLK) |  | 160 ns |  |
| td(schB0_n) | Delay time for first result to appear in Result register | 5tc(ADCCLK) |  | 200 ns |  |
| td(schA0_n+1) | Delay time for successive results to appear in Result register |  | (3 + Acqps) * tc(ADCCLK) | 120 ns |  |
| td(schB0_n+1) | Delay time for successive results to appear in Result register |  | (3 + Acqps) * tc(ADCCLK) | 120 ns |  |

        - Definitions of Specifications and Terminology

Integral Nonlinearity

Integral nonlinearity refers to the deviation of each individual code from a line drawn from zero through full scale. The point used as zero occurs 1/2 LSB before the first code transition. The full-scale point is defined as level 1/2 LSB beyond the last code transition. The deviation is measured from the center of each particular code to the true straight line between these two points.

Differential Nonlinearity

An ideal ADC exhibits code transitions that are exactly 1 LSB apart. DNL is the deviation from this ideal value. A differential nonlinearity error of less than ±1 LSB ensures no missing codes.

Zero Offset

The major carry transition should occur when the analog input is at zero volt. Zero error is defined as the deviation of the actual transition from that point.

Gain Error

The first code transition should occur at an analog value 1/2 LSB above negative full scale. The last transition should occur at an analog value 1 1/2 LSB below the nominal full scale. Gain error is the deviation of the actual difference between first and last code transitions and the ideal difference between first and last code transitions.

Signal-to-Noise Ratio + Distortion (SINAD)

SINAD is the ratio of the rms value of the measured input signal to the rms sum of all other spectral components below the Nyquist frequency, including harmonics but excluding dc. The value for SINAD is expressed in decibels.

Effective Number of Bits (ENOB)

For a sine wave, SINAD can be expressed in terms of the number of bits. Using the following formula,

N  (SINAD  1.76)

6.02

it is possible to get a measure of performance expressed as N, the effective number of bits. Thus, effective number of bits for a device for sine wave inputs at a given input frequency can be calculated directly from its measured SINAD.

Total Harmonic Distortion (THD)

THD is the ratio of the rms sum of the first six harmonic components to the rms value of the measured input signal and is expressed as a percentage or in decibels.

Spurious Free Dynamic Range (SFDR)

SFDR is the difference in dB between the rms amplitude of the input signal and the peak spurious signal.

      - Multichannel Buffered Serial Port (McBSP) Timing

        - McBSP Transmit and Receive Timing

          - McBSP Timing Requirements

| NO.(1) (2) |  |  |  | MIN MAX | UNIT |
| --- | --- | --- | --- | --- | --- |
|  | McBSP module clock (CLKG, CLKX, CLKR) range |  |  | 1 | kHz |
|  |  |  |  | 20(3) | MHz |
|  | McBSP module cycle time (CLKG, CLKX, CLKR) range |  |  | 50 | ns |
|  |  |  |  | 1 | ms |
| M11 | tc(CKRX) | Cycle time, CLKR/X | CLKR/X ext | 2P | ns |
| M12 | tw(CKRX) | Pulse duration, CLKR/X high or CLKR/X low | CLKR/X ext | P – 7 | ns |
| M13 | tr(CKRX) | Rise time, CLKR/X | CLKR/X ext | 7 | ns |
| M14 | tf(CKRX) | Fall time, CLKR/X | CLKR/X ext | 7 | ns |
| M15 | tsu(FRH-CKRL) | Setup time, external FSR high before CLKR low | CLKR int | 18 | ns |
|  |  |  | CLKR ext | 2 |  |
| M16 | th(CKRL-FRH) | Hold time, external FSR high after CLKR low | CLKR int | 0 | ns |
|  |  |  | CLKR ext | 6 |  |
| M17 | tsu(DRV-CKRL) | Setup time, DR valid before CLKR low | CLKR int | 18 | ns |
|  |  |  | CLKR ext | 2 |  |
| M18 | th(CKRL-DRV) | Hold time, DR valid after CLKR low | CLKR int | 0 | ns |
|  |  |  | CLKR ext | 6 |  |
| M19 | tsu(FXH-CKXL) | Setup time, external FSX high before CLKX low | CLKX int | 18 | ns |
|  |  |  | CLKX ext | 2 |  |
| M20 | th(CKXL-FXH) | Hold time, external FSX high after CLKX low | CLKX int | 0 | ns |
|  |  |  | CLKX ext | 6 |  |

- Polarity bits CLKRP = CLKXP = FSRP = FSXP = 0. If the polarity of any of the signals is inverted, then the timing references of that signal are also inverted.

- 2P = 1/CLKG in ns. CLKG is the output of sample rate generator mux. CLKG = CLKSRG/(1 + CLKGDV).

CLKSRG can be LSPCLK, CLKX, CLKR as source. CLKSRG ≤ (SYSCLKOUT/2). McBSP performance is limited by I/O buffer switching speed.

- Internal clock prescalers must be adjusted such that the McBSP clock (CLKG, CLKX, CLKR) speeds are not greater than the I/O buffer speed limit (20 MHz).

          - McBSP Switching Characteristics

| NO.(1) (2) | PARAMETER |  |  |  | MIN | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M1 | tc(CKRX) | Cycle time, CLKR/X |  | CLKR/X int | 2P |  | ns |
| M2 | tw(CKRXH) | Pulse duration, CLKR/X high |  | CLKR/X int | D – 5(3) | D + 5(3) | ns |
| M3 | tw(CKRXL) | Pulse duration, CLKR/X low |  | CLKR/X int | C – 5(3) | C + 5(3) | ns |
| M4 | td(CKRH-FRV) | Delay time, CLKR high to internal FSR valid |  | CLKR int | 0 | 4 | ns |
|  |  |  |  | CLKR ext | 3 | 27 |  |
| M5 | td(CKXH-FXV) | Delay time, CLKX high to internal FSX valid |  | CLKX int | 0 | 4 | ns |
|  |  |  |  | CLKX ext | 3 | 27 |  |
| M6 | tdis(CKXH-DXHZ) | Disable time, CLKX high to DX high impedance following last data bit |  | CLKX int | 8 |  | ns |
|  |  |  |  | CLKX ext | 14 |  |  |
| M7 | td(CKXH-DXV) | Delay time, CLKX high to DX valid. This applies to all bits except the first bit transmitted. |  | CLKX int | 9 |  | ns |
|  |  |  |  | CLKX ext | 28 |  |  |
|  |  | Delay time, CLKX high to DX valid. Only applies to first bit transmitted when in Data Delay 1 or 2 (XDATDLY = 01b or 10b) modes. | DXENA = 0 | CLKX int | 8 |  |  |
|  |  |  |  | CLKX ext | 14 |  |  |
|  |  |  | DXENA = 1 | CLKX int | P + 8 |  |  |
|  |  |  |  | CLKX ext | P + 14 |  |  |
| M8 | ten(CKXH-DX) | Enable time, CLKX high to DX driven. Only applies to first bit transmitted when in Data Delay 1 or 2 (XDATDLY = 01b or 10b) modes. | DXENA = 0 | CLKX int | 0 |  | ns |
|  |  |  |  | CLKX ext | 6 |  |  |
|  |  |  | DXENA = 1 | CLKX int | P |  |  |
|  |  |  |  | CLKX ext | P + 6 |  |  |
| M9 | td(FXH-DXV) | Delay time, FSX high to DX valid. Only applies to first bit transmitted when in Data Delay 0 (XDATDLY = 00b) mode. | DXENA = 0 | FSX int | 8 |  | ns |
|  |  |  |  | FSX ext | 14 |  |  |
|  |  |  | DXENA = 1 | FSX int | P + 8 |  |  |
|  |  |  |  | FSX ext | P + 14 |  |  |
| M10 | ten(FXH-DX) | Enable time, FSX high to DX driven. Only applies to first bit transmitted when in Data Delay 0 (XDATDLY = 00b) mode. | DXENA = 0 | FSX int | 0 |  | ns |
|  |  |  |  | FSX ext | 6 |  |  |
|  |  |  | DXENA = 1 | FSX int | P |  |  |
|  |  |  |  | FSX ext | P + 6 |  |  |

- Polarity bits CLKRP = CLKXP = FSRP = FSXP = 0. If the polarity of any of the signals is inverted, then the timing references of that signal are also inverted.

- 2P = 1/CLKG in ns.

- C = CLKRX low pulse width = P D = CLKRX high pulse width = P

M13

M3, M12

M4

M4

M14

M15

M16

M18

M17

Bit (n-1)

M17

(n-2)

(n-3)

(n-4)

M18

Bit (n-1)

(n-2)

(n-3)

M17

M18

Bit (n-1)

(n-2)

M1, M11 M2, M12

CLKR

FSR (int)

FSR (ext)

DR (RDATDLY=00b)

DR (RDATDLY=01b)

DR (RDATDLY=10b)

Figure 8-39. McBSP Receive Timing

M1, M11

M2, M12

M3, M12

M13

M14

M5

M5

M19

M20

M9

M10

Bit 0

M7

Bit (n-1)

(n-2)

(n-3)

(n-4)

M7

M8

Bit 0

Bit (n-1)

(n-2)

(n-3)

M6

M7

M8

Bit 0

Bit (n-1)

(n-2)

CLKX

FSX (int)

FSX (ext)

DX (XDATDLY=00b)

DX (XDATDLY=01b)

DX (XDATDLY=10b)

Figure 8-40. McBSP Transmit Timing

        - McBSP as SPI Master or Slave Timing

          - McBSP as SPI Master or Slave Timing Requirements (CLKSTP = 10b, CLKXP = 0)

| NO.(1) (2) |  |  | MASTER | SLAVE | UNIT |
| --- | --- | --- | --- | --- | --- |
|  |  |  | MIN MAX | MIN MAX |  |
| M30 | tsu(DRV-CKXL) | Setup time, DR valid before CLKX low | 30 | 8P – 10 | ns |
| M31 | th(CKXL-DRV) | Hold time, DR valid after CLKX low | 1 | 8P – 10 | ns |
| M32 | tsu(BFXL-CKXH) | Setup time, FSX low before CLKX high |  | 8P + 10 | ns |
| M33 | tc(CKX) | Cycle time, CLKX | 2P | 16P | ns |

- For all SPI slave modes, CLKX has to be a minimum of 8 CLKG cycles. Furthermore, CLKG should be LSPCLK/2 by setting CLKSM = CLKGDV = 1.

- 2P = 1/CLKG

          - McBSP as SPI Master or Slave Switching Characteristics (CLKSTP = 10b, CLKXP = 0)

| NO.(1) | PARAMETER |  | MASTER | SLAVE | UNIT |
| --- | --- | --- | --- | --- | --- |
|  |  |  | MIN MAX | MIN MAX |  |
| M24 | th(CKXL-FXL) | Hold time, FSX low after CLKX low | 2P |  | ns |
| M25 | td(FXL-CKXH) | Delay time, FSX low to CLKX high | P |  | ns |
| M28 | tdis(FXH-DXHZ) | Disable time, DX high impedance following last data bit from FSX high | 6 | 6P + 6 | ns |
| M29 | td(FXL-DXV) | Delay time, FSX low to DX valid | 6 | 4P + 6 | ns |

- 2P = 1/CLKG

LSB

M32

MSB

M33

M24

M25

M28

M29

Bit 0

Bit(n-1)

(n-2)

(n-3)

(n-4)

M30

Bit 0

Bit(n-1)

M31

(n-2)

(n-3)

(n-4)

CLKX

FSX

DX

DR

Figure 8-41. McBSP Timing as SPI Master or Slave: CLKSTP = 10b, CLKXP = 0

| NO.(1) (2) |  |  | MASTER | SLAVE | UNIT |
| --- | --- | --- | --- | --- | --- |
|  |  |  | MIN MAX | MIN MAX |  |
| M39 | tsu(DRV-CKXH) | Setup time, DR valid before CLKX high | 30 | 8P – 10 | ns |
| M40 | th(CKXH-DRV) | Hold time, DR valid after CLKX high | 1 | 8P – 10 | ns |
| M41 | tsu(FXL-CKXH) | Setup time, FSX low before CLKX high |  | 16P + 10 | ns |
| M42 | tc(CKX) | Cycle time, CLKX | 2P | 16P | ns |

- For all SPI slave modes, CLKX has to be a minimum of 8 CLKG cycles. Furthermore, CLKG should be LSPCLK/2 by setting CLKSM = CLKGDV = 1.

- 2P = 1/CLKG

          - McBSP as SPI Master or Slave Switching Characteristics (CLKSTP = 11b, CLKXP = 0)

| NO.(1) | PARAMETER |  | MASTER | SLAVE | UNIT |
| --- | --- | --- | --- | --- | --- |
|  |  |  | MIN MAX | MIN MAX |  |
| M34 | th(CKXL-FXL) | Hold time, FSX low after CLKX low | P |  | ns |
| M35 | td(FXL-CKXH) | Delay time, FSX low to CLKX high | 2P |  | ns |
| M37 | tdis(CKXL-DXHZ) | Disable time, DX high impedance following last data bit from CLKX low | P + 6 | 7P + 6 | ns |
| M38 | td(FXL-DXV) | Delay time, FSX low to DX valid | 6 | 4P + 6 | ns |

(1)	2P = 1/CLKG

LSB

M41

MSB

M42

M34

M35

M37

Bit 0

Bit(n-1)

(n-2)

(n-3)

(n-4)

M39

Bit 0

Bit(n-1)

M40

(n-2)

(n-3)

(n-4)

CLKX

FSX

DX

DR

Figure 8-42. McBSP Timing as SPI Master or Slave: CLKSTP = 11b, CLKXP = 0

          - McBSP as SPI Master or Slave Timing Requirements (CLKSTP = 10b, CLKXP = 1)

| NO.(1) (2) |  |  | MASTER | SLAVE | UNIT |
| --- | --- | --- | --- | --- | --- |
|  |  |  | MIN MAX | MIN MAX |  |
| M49 | tsu(DRV-CKXH) | Setup time, DR valid before CLKX high | 30 | 8P – 10 | ns |
| M50 | th(CKXH-DRV) | Hold time, DR valid after CLKX high | 1 | 8P – 10 | ns |
| M51 | tsu(FXL-CKXL) | Setup time, FSX low before CLKX low |  | 8P + 10 | ns |
| M52 | tc(CKX) | Cycle time, CLKX | 2P | 16P | ns |

- For all SPI slave modes, CLKX has to be a minimum of 8 CLKG cycles. Furthermore, CLKG should be LSPCLK/2 by setting CLKSM = CLKGDV = 1.

- 2P = 1/CLKG

          - McBSP as SPI Master or Slave Switching Characteristics (CLKSTP = 10b, CLKXP = 1)

| NO.(1) | PARAMETER |  | MASTER | SLAVE | UNIT |
| --- | --- | --- | --- | --- | --- |
|  |  |  | MIN MAX | MIN MAX |  |
| M43 | th(CKXH-FXL) | Hold time, FSX low after CLKX high | 2P |  | ns |
| M44 | td(FXL-CKXL) | Delay time, FSX low to CLKX low | P |  | ns |
| M47 | tdis(FXH-DXHZ) | Disable time, DX high impedance following last data bit from FSX high | 6 | 6P + 6 | ns |
| M48 | td(FXL-DXV) | Delay time, FSX low to DX valid | 6 | 4P + 6 | ns |

- 2P = 1/CLKG

LSB

M51

MSB

M52

M43

M44

M47

M48

Bit 0

Bit(n-1)

(n-2)

(n-3)

(n-4)

M49

Bit 0

Bit(n-1)

M50

(n-2)

(n-3)

(n-4)

CLKX

FSX

DX

DR

Figure 8-43. McBSP Timing as SPI Master or Slave: CLKSTP = 10b, CLKXP = 1

| NO.(1) (2) |  |  | MASTER | SLAVE | UNIT |
| --- | --- | --- | --- | --- | --- |
|  |  |  | MIN MAX | MIN MAX |  |
| M58 | tsu(DRV-CKXL) | Setup time, DR valid before CLKX low | 30 | 8P – 10 | ns |
| M59 | th(CKXL-DRV) | Hold time, DR valid after CLKX low | 1 | 8P – 10 | ns |
| M60 | tsu(FXL-CKXL) | Setup time, FSX low before CLKX low |  | 16P + 10 | ns |
| M61 | tc(CKX) | Cycle time, CLKX | 2P | 16P | ns |

- For all SPI slave modes, CLKX has to be a minimum of 8 CLKG cycles. Furthermore, CLKG should be LSPCLK/2 by setting CLKSM = CLKGDV = 1.

- 2P = 1/CLKG

8.12.5.16.2.8 McBSP as SPI Master or Slave Switching Characteristics (CLKSTP = 11b, CLKXP = 1)

| NO.(1) | PARAMETER |  | MASTER | SLAVE | UNIT |
| --- | --- | --- | --- | --- | --- |
|  |  |  | MIN MAX | MIN MAX |  |
| M53 | th(CKXH-FXL) | Hold time, FSX low after CLKX high | P |  | ns |
| M54 | td(FXL-CKXL) | Delay time, FSX low to CLKX low | 2P |  | ns |
| M55 | td(CLKXH-DXV) | Delay time, CLKX high to DX valid | –2 0 | 3P + 6 5P + 20 | ns |
| M56 | tdis(CKXH-DXHZ) | Disable time, DX high impedance following last data bit from CLKX high | P + 6 | 7P + 6 | ns |
| M57 | td(FXL-DXV) | Delay time, FSX low to DX valid | 6 | 4P + 6 | ns |

(1)	2P = 1/CLKG

LSB

M60

MSB

M61

M53

M54

Bit 0

Bit(n-1)

M55

(n-2)

(n-3)

(n-4)

M58

Bit 0

Bit(n-1)

M59

(n-2)

(n-3)

(n-4)

CLKX

FSX

DX

DR

Figure 8-44. McBSP Timing as SPI Master or Slave: CLKSTP = 11b, CLKXP = 1

    - Emulator Connection Without Signal Buffering for the DSP

Figure 8-45 shows the connection between the DSP and JTAG header for a single-processor configuration. If the distance between the JTAG header and the DSP is greater than 6 inches, the emulation signals must be buffered. If the distance is less than 6 inches, buffering is typically not needed. Figure 8-45 shows the simpler, no-buffering situation. For the pullup/pulldown resistor values, see the pin description section.

EMU0 EMU1 TRST TMS TDI TDO TCK

EMU0

EMU1 TRST TMS TDI TDO TCK

TCK_RET

PD

GND

GND GND GND

GND

14

2

1

3

7

11

9

JTAG Header

4

6

8

10

12

5

13

VDDIO

VDDIO

6 inches or less

DSP

Figure 8-45. Emulator Connection Without Signal Buffering for the DSP

    - Interrupt Timing

      - Interrupt Switching Characteristics

| PARAMETER |  | MIN MAX | UNIT |
| --- | --- | --- | --- |
| td(PDP-PWM)HZ Delay time, PDPINTx low to PWM high- impedance state | Without input qualifier | 12 | ns |
|  | With input qualifier | 1tc(SCO) + IQT + 12(1) |  |
| td(TRIP-PWM)HZ Delay time, CxTRIP/ TxCTRIP signals low to PWM high-impedance state | Without input qualifier | 3 * tc(SCO) | ns |
|  | With input qualifier | 2tc(SCO) + IQT(1) |  |
| td(INT) Delay time, INT low/high to interrupt-vector fetch |  | IQT + 12tc(SCO) (1) | ns |

(1)	Input Qualification Time (IQT) = [tc(SCO) × 2 × QUALPRD] × 5 + [tc(SCO) × 2 × QUALPRD].

      - Interrupt Timing Requirements

|  |  |  | MIN MAX | UNIT |
| --- | --- | --- | --- | --- |
| tw(INT) | Pulse duration, INT input low/high | With no qualifier | 2tc(SCO) | cycles |
|  |  | With qualifier | 1tc(SCO) + IQT(1) |  |
| tw(PDP) | Pulse duration, PDPINTx input low | With no qualifier | 2tc(SCO) | cycles |
|  |  | With qualifier | 1tc(SCO) + IQT(1) |  |
| tw(CxTRIP) | Pulse duration, CxTRIP input low | With no qualifier | 2tc(SCO) | cycles |
|  |  | With qualifier | 1tc(SCO) + IQT(1) |  |
| tw(TxCTRIP) | Pulse duration, TxCTRIP input low | With no qualifier | 2tc(SCO) | cycles |
|  |  | With qualifier | 1tc(SCO) + IQT(1) |  |

tw(PDP), tw(CxTRIP), tw(TxCTRIP)

td(PDP-PWM)HZ, td(TRIP-PWM)HZ

(1)	Input Qualification Time (IQT) = [tc(SCO) × 2 × QUALPRD] × 5 + [tc(SCO) × 2 × QUALPRD].

XCLKOUT

(A)

TxCTRIP, CxTRIP,

(B)

PDPINTx

(C)

PWM

tw(INT)

td(INT)

Interrupt Vector

XNMI, XINT1, XINT2

A0-A15

- XCLKOUT = SYSCLKOUT

- TxCTRIP – T1CTRIP, T2CTRIP, T3CTRIP, T4CTRIP CxTRIP – C1TRIP, C2TRIP, C3TRIP, C4TRIP, C5TRIP, or C6TRIP PDPINTx – PDPINTA or PDPINTB

- PWM refers to all the PWM pins in the device (that is, PWMn and TnPWM pins or PWM pin pair relevant to each CxTRIP pin). The state of the PWM pins after PDPINTx is taken high depends on the state of the FCOMPOE bit.

Figure 8-46. External Interrupt Timing

    - Flash Timing

      - Flash Endurance for A and S Temperature Material

|  |  | ERASE/PROGRAM TEMPERATURE(1) | MIN | TYP | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| Nf | Flash endurance for the array (Write/Erase cycles) | 0°C to 85°C (ambient) | 20000(2) | 50000(2) |  | cycles |
| NOTP | OTP endurance for the array (Write cycles) | 0°C to 85°C (ambient) | 1 |  |  | write |

- Write/erase operations outside of the temperature ranges indicated are not specified and may affect the endurance numbers.

- The Write/Erase cycle numbers of 20000 (MIN) and 50000 (TYP) are applicable only for silicon revision G. For older silicon revisions, the Write/Erase cycle numbers of 100 (MIN) and 1000 (TYP) are applicable.

      - Flash Endurance for Q Temperature Material

|  |  | ERASE/PROGRAM TEMPERATURE(1) | MIN | TYP | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| Nf | Flash endurance for the array (Write/Erase cycles) | –40°C to 125°C (ambient) | 20000(2) | 50000(2) |  | cycles |
| NOTP | OTP endurance for the array (Write cycles) | –40°C to 125°C (ambient) | 1 |  |  | write |

- Write/erase operations outside of the temperature ranges indicated are not specified and may affect the endurance numbers.

- The Write/Erase cycle numbers of 20000 (MIN) and 50000 (TYP) are applicable only for silicon revision G. For older silicon revisions, the Write/Erase cycle numbers of 100 (MIN) and 1000 (TYP) are applicable.

      - Flash Parameters at 150-MHz SYSCLKOUT

| PARAMETER(1) |  |  |  | MIN TYP MAX | UNIT |
| --- | --- | --- | --- | --- | --- |
| Program Time | 16-Bit Word | Using Flash API v1(2) |  | 35 | µs |
|  |  | Using Flash API v2.10 |  | 50 |  |
|  | 8K Sector | Using Flash API v1(2) |  | 170 | ms |
|  |  | Using Flash API v2.10 |  | 250 |  |
|  | 16K Sector | Using Flash API v1(2) |  | 320 | ms |
|  |  | Using Flash API v2.10 |  | 500 |  |
| Erase Time(3) | 8K Sector |  |  | 10 | s |
|  | 16K Sector |  |  | 11 |  |
| IDD3VFLP | VDD3VFL current consumption during the Erase/Program cycle |  | Erase | 75 | mA |
|  |  |  | Program | 35 |  |
| IDDP | VDD current consumption during Erase/Program cycle |  |  | 140 | mA |
| IDDIOP | VDDIO current consumption during Erase/Program cycle |  |  | 20 | mA |

- Typical parameters as seen at room temperature including function call overhead, with all peripherals off. It is important to maintain a stable power supply during the entire flash programming process. It is conceivable that device current consumption during flash programming could be higher than normal operating conditions. The power supply used should ensure VMIN on the supply rails at all times, as specified in the Recommended Operating Conditions of the data sheet. Any brown-out or interruption to power during erasing/programming could potentially corrupt the password locations and lock the device permanently. Powering a target board (during flash programming) through the USB port is not recommended, as the port may be unable to respond to the power demands placed during the programming process.

- Flash API v1.00 is useable on rev. C silicon only.

- The on-chip flash memory is in an erased state when the device is shipped from TI. As such, erasing the flash memory is not required prior to programming, when programming the device for the first time. However, the erase operation is needed on all subsequent programming operations.

      - Flash/OTP Access Timing

| PARAMETER | MIN MAX | UNIT |
| --- | --- | --- |
| ta(fp) Paged Flash access time | 36 | ns |
| ta(fr) Random Flash access time | 36 | ns |
| ta(OTP) OTP access time | 60 | ns |

      - Flash Data Retention Duration

| PARAMETER | TEST CONDITIONS | MIN MAX | UNIT |
| --- | --- | --- | --- |
| tretention Data retention duration | TJ = 55°C | 15 | years |

Table 8-5. Minimum Required Flash Wait States at Different Frequencies

| SYSCLKOUT (MHz) | SYSCLKOUT (ns) | PAGE WAIT STATE(1) | RANDOM WAIT STATE(1) (2) | OTP |
| --- | --- | --- | --- | --- |
| 150 | 6.67 | 5 | 5 | 8 |
| 120 | 8.33 | 4 | 4 | 7 |
| 100 | 10 | 3 | 3 | 5 |
| 75 | 13.33 | 2 | 2 | 4 |
| 50 | 20 | 1 | 1 | 2 |
| 30 | 33.33 | 1 | 1 | 1 |
| 25 | 40 | 0 | 1 | 1 |
| 15 | 66.67 | 0 | 1 | 1 |
| 4 | 250 | 0 | 1 | 1 |

- Formulas to compute page wait state and random wait state:

 ta(fp) 	

Flash Page Wait State   	 1 (round up to the next highest integer, or 0, whichever is larger)

 tc(SCO) 	

		

 ta(fr) 	

Flash Random Wait State   	 1 (round up to the next highest integer, or 1, whichever is larger)

 tc(SCO) 	

		

 ta(OTP) 	

OTP Wait State   	 1 (round up to the next highest integer, or 1, whichever is larger)

 tc(SCO) 	

		

- Random wait state must be greater than or equal to 1.

#### Detailed Description

###### Brief Descriptions

    - C28x CPU

The C28x DSP generation is the newest member of the TMS320C2000™ DSP platform. The C28x is source code compatible to the 24x/240x DSP devices, hence existing 240x users can leverage their significant software investment. Additionally, the C28x is a very efficient C/C++ engine, enabling users to develop not only their system control software in a high-level language, but also enables math algorithms to be developed using C/C+

+. The C28x is as efficient in DSP math tasks as it is in system control tasks that typically are handled by microcontroller devices. This efficiency removes the need for a second processor in many systems. The 32 x 32- bit MAC capabilities of the C28x and its 64-bit processing capabilities, enable the C28x to efficiently handle higher numerical resolution problems that would otherwise demand a more expensive floating-point processor solution. Add to this the fast interrupt response with automatic context save of critical registers, resulting in a device that is capable of servicing many asynchronous events with minimal latency. The C28x has an 8-level- deep protected pipeline with pipelined memory accesses. This pipelining enables the C28x to execute at high speeds without resorting to expensive high-speed memories. Special branch-look-ahead hardware minimizes the latency for conditional discontinuities. Special store conditional operations further improve performance.

    - Memory Bus (Harvard Bus Architecture)

As with many DSP type devices, multiple busses are used to move data between the memories and peripherals and the CPU. The C28x memory bus architecture contains a program read bus, data read bus and data write bus. The program read bus consists of 22 address lines and 32 data lines. The data read and write busses consist of 32 address lines and 32 data lines each. The 32-bit-wide data busses enable single cycle 32-bit operations. The multiple bus architecture, commonly termed “Harvard Bus”, enables the C28x to fetch an instruction, read a data value and write a data value in a single cycle. All peripherals and memories attached to the memory bus will prioritize memory accesses. Generally, the priority of Memory Bus accesses can be summarized as follows:

Highest:	Data Writes (Simultaneous data and program writes cannot occur on the memory bus.) Program Writes (Simultaneous data and program writes cannot occur on the memory bus.) Data Reads

Program Reads (Simultaneous program reads and fetches cannot occur on the memory bus.)

Lowest:   Fetches (Simultaneous program reads and fetches cannot occur on the memory bus.)

    - Peripheral Bus

To enable migration of peripherals between various Texas Instruments (TI) DSP family of devices, the F281x adopts a peripheral bus standard for peripheral interconnect. The peripheral bus bridge multiplexes the various busses that make up the processor “Memory Bus” into a single bus consisting of 16 address lines and 16 or 32 data lines and associated control signals. Two versions of the peripheral bus are supported on the F281x. One version only supports 16-bit accesses (called peripheral frame 2). The other version supports both 16- and 32-bit accesses (called peripheral frame 1).

    - Real-Time JTAG and Analysis

The F281x implements the standard IEEE 1149.1 JTAG interface. Additionally, the F281x supports real-time mode of operation whereby the contents of memory, peripheral, and register locations can be modified while the processor is running and executing code and servicing interrupts. The user can also single step through non- time critical code while enabling time-critical interrupts to be serviced without interference. The F281x implements the real-time mode in hardware within the CPU. This is a unique feature to the F281x, no software monitor is required. Additionally, special analysis hardware is provided that allows the user to set hardware breakpoint or data/address watch-points and generate various user selectable break events when a match occurs.

    - External Interface (XINTF) (F2812 Only)

This asynchronous interface consists of 19 address lines, 16 data lines, and three chip-select lines. The chip- select lines are mapped to five external zones, Zones 0, 1, 2, 6, and 7. Zones 0 and 1 share a single chip-select; Zones 6 and 7 also share a single chip-select. Each of the five zones can be programmed with a different number of wait states, strobe signal setup and hold timing and each zone can be programmed for extending wait states externally or not. The programmable wait-state, chip-select and programmable strobe timing enables glueless interface to external memories and peripherals.

    - Flash

The F2812 and F2811 contain 128K x 16 of embedded flash memory, segregated into four 8K x 16 sectors, and six 16K x 16 sectors. The F2810 has 64K x 16 of embedded flash, segregated into two 8K x 16 sectors, and three 16K x 16 sectors. All three devices also contain a single 1K x 16 of OTP memory at address range 0x3D 7800–0x3D 7BFF. The user can individually erase, program, and validate a flash sector while leaving other sectors untouched. However, it is not possible to use one sector of the flash or the OTP to execute flash algorithms that erase/program other sectors. Special memory pipelining is provided to enable the flash module to achieve higher performance. The flash/OTP is mapped to both program and data space; therefore, it can be used to execute code or store data information.

Note

The F2810/F2811/F2812 Flash and OTP wait states can be configured by the application. This allows applications running at slower frequencies to configure the flash to use fewer wait states.

Flash effective performance can be improved by enabling the flash pipeline mode in the Flash options register. With this mode enabled, effective performance of linear code execution will be much faster than the raw performance indicated by the wait state configuration alone. The exact performance gain when using the Flash pipeline mode is application-dependent.

For more information on the Flash options, Flash wait-state, and OTP wait-state registers, see the

TMS320x281x DSP System Control and Interrupts Reference Guide.

    - M0, M1 SARAMs

All C28x devices contain these two blocks of single access memory, each 1K x 16 in size. The stack pointer points to the beginning of block M1 on reset. The M0 block overlaps the 240x device B0, B1, B2 RAM blocks and hence the mapping of data variables on the 240x devices can remain at the same physical address on C28x devices. The M0 and M1 blocks, like all other memory blocks on C28x devices, are mapped to both program and data space. Hence, the user can use M0 and M1 to execute code or for data variables. The partitioning is performed within the linker. The C28x device presents a unified memory map to the programmer. This makes for easier programming in high-level languages.

    - L0, L1, H0 SARAMs

The F281x contains an additional 16K x 16 of single-access RAM, divided into three blocks (4K + 4K + 8K). Each block can be independently accessed hence minimizing pipeline stalls. Each block is mapped to both program and data space.

    - Boot ROM

The Boot ROM is factory-programmed with boot-loading software. The Boot ROM program executes after device reset and checks several GPIO pins to determine which boot mode to enter. For example, the user can select to execute code already present in the internal Flash or download new software to internal RAM through one of several serial ports. Other boot modes exist as well. The Boot ROM also contains standard tables, such as SIN/COS waveforms, for use in math-related algorithms. Table 9-1 shows the details of how various boot modes may be invoked. See the TMS320x281x DSP Boot ROM Reference Guide, for more information.

Table 9-1. Boot Mode Selection

| BOOT MODE SELECTED(1) (2) | GPIOF4 (SCITXDA) | GPIOF12 (MDXA) | GPIOF3 (SPISTEA) | GPIOF2 (SPICLK) |
| --- | --- | --- | --- | --- |
| GPIO PU status(3) | PU | No PU | No PU | No PU |
| Jump to Flash address 0x3F 7FF6. A branch instruction must have been programmed here prior to reset to re-direct code execution as desired. | 1 | x | x | x |
| Call SPI_Boot to load from an external serial SPI EEPROM | 0 | 1 | x | x |
| Call SCI_Boot to load from SCI-A | 0 | 0 | 1 | 1 |
| Jump to H0 SARAM address 0x3F 8000 | 0 | 0 | 1 | 0 |
| Jump to OTP address 0x3D 7800 | 0 | 0 | 0 | 1 |
| Call Parallel_Boot to load from GPIO Port B | 0 | 0 | 0 | 0 |

- Extra care must be taken due to any effect toggling SPICLK to select a boot mode may have on external logic.

- If the boot mode selected is Flash, H0, or OTP, then no external code is loaded by the bootloader.

- PU = pin has an internal pullup. No PU = pin does not have an internal pullup.

    - Security

The F281x supports high levels of security to protect the user firmware from being reverse-engineered. The security features a 128-bit password (hardcoded for 16 wait states), which the user programs into the flash. One code security module (CSM) is used to protect the flash/OTP and the L0/L1 SARAM blocks. The security feature prevents unauthorized users from examining the memory contents via the JTAG port, executing code from external memory or trying to boot-load some undesirable software that would export the secure memory contents. To enable access to the secure blocks, the user must write the correct 128-bit ”KEY” value, which matches the value stored in the password locations within the Flash.

Note

- When the code-security passwords are programmed, all addresses between 0x3F 7F80 and 0x3F 7FF5 cannot be used as program code or data. These locations must be programmed to 0x0000.

- If the code security feature is not used, addresses 0x3F 7F80 through 0x3F 7FEF may be used for code or data.

- The 128-bit password (at 0x3F 7FF8–0x3F 7FFF) must not be programmed to zeros. Doing so would permanently lock the device.

Table 9-2. Impact of Using the Code Security Module

| ADDRESS | CODE SECURITY STATUS |  |
| --- | --- | --- |
|  | Code Security Enabled | Code Security Disabled |
| 0x3F 7F80 – 0x3F 7FEF | Fill with 0x0000 | Application code and data(1) |
| 0x3F 7FF0 – 0x3F 7FF5 |  |  |
| 0x3D 7BFC – 0x3D 7BFF | Application code and data |  |

(1)	See the TMS320F281x DSPs Silicon Errata for some restrictions.

Note

Code Security Module Disclaimer

THE CODE SECURITY MODULE (CSM) INCLUDED ON THIS DEVICE WAS DESIGNED TO PASSWORD PROTECT THE DATA STORED IN THE ASSOCIATED MEMORY (EITHER ROM OR FLASH) AND IS WARRANTED BY TEXAS INSTRUMENTS (TI), IN ACCORDANCE WITH ITS STANDARD TERMS AND CONDITIONS, TO CONFORM TO TI'S PUBLISHED SPECIFICATIONS FOR THE WARRANTY PERIOD APPLICABLE FOR THIS DEVICE.

TI DOES NOT, HOWEVER, WARRANT OR REPRESENT THAT THE CSM CANNOT BE COMPROMISED OR BREACHED OR THAT THE DATA STORED IN THE ASSOCIATED MEMORY CANNOT BE ACCESSED THROUGH OTHER MEANS. MOREOVER, EXCEPT AS SET FORTH ABOVE, TI MAKES NO WARRANTIES OR REPRESENTATIONS CONCERNING THE CSM OR OPERATION OF THIS DEVICE, INCLUDING ANY IMPLIED WARRANTIES OF MERCHANT ABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

IN NO EVENT SHALL TI BE LIABLE FOR ANY CONSEQUENTIAL, SPECIAL, INDIRECT, INCIDENTAL, OR PUNITIVE DAMAGES, HOWEVER CAUSED, ARISING IN ANY WAY OUT OF YOUR USE OF THE CSM OR THIS DEVICE, WHETHER OR NOT TI HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES. EXCLUDED DAMAGES INCLUDE, BUT ARE NOT LIMITED TO LOSS OF DATA, LOSS OF GOODWILL, LOSS OF USE OR INTERRUPTION OF BUSINESS OR OTHER ECONOMIC LOSS.

    - Peripheral Interrupt Expansion (PIE) Block

The PIE block serves to multiplex numerous interrupt sources into a smaller set of interrupt inputs. The PIE block can support up to 96 peripheral interrupts. On the F281x, 45 of the possible 96 interrupts are used by peripherals. The 96 interrupts are grouped into blocks of 8 and each group is fed into 1 of 12 CPU interrupt lines (INT1 to INT12). Each of the 96 interrupts is supported by its own vector stored in a dedicated RAM block that can be overwritten by the user. The vector is automatically fetched by the CPU on servicing the interrupt. It takes 8 CPU clock cycles to fetch the vector and save critical CPU registers. Hence the CPU can quickly respond to interrupt events. Prioritization of interrupts is controlled in hardware and software. Each individual interrupt can be enabled/disabled within the PIE block.

    - External Interrupts (XINT1, XINT2, XINT13, XNMI)

The F281x supports three masked external interrupts (XINT1, 2, 13). XINT13 is combined with one non-masked external interrupt (XNMI). The combined signal name is XNMI_XINT13. Each of the interrupts can be selected for negative or positive edge triggering and can also be enabled/disabled (including the XNMI). The masked interrupts also contain a 16-bit free-running up-counter, which is reset to zero when a valid interrupt edge is detected. This counter can be used to accurately time-stamp the interrupt.

    - Oscillator and PLL

The F281x can be clocked by an external oscillator or by a crystal attached to the on-chip oscillator circuit. A PLL is provided supporting up to 10-input clock-scaling ratios. The PLL ratios can be changed on-the-fly in software, enabling the user to scale back on operating frequency if lower power operation is desired. Refer to Section 8 for timing details. The PLL block can be set in bypass mode.

    - Watchdog

The F281x supports a watchdog timer. The user software must regularly reset the watchdog counter within a certain time frame; otherwise, the watchdog will generate a reset to the processor. The watchdog can be disabled if necessary.

    - Peripheral Clocking

The clocks to each individual peripheral can be enabled/disabled to reduce power consumption when a peripheral is not in use. Additionally, the system clock to the serial ports (except eCAN) and the event managers, CAP and QEP blocks can be scaled relative to the CPU clock. This enables the timing of peripherals to be decoupled from increasing CPU clock speeds.

    - Low-Power Modes

The F281x devices are fully static CMOS devices. Three low-power modes are provided:

IDLE:	Place CPU in low-power mode. Peripheral clocks may be turned off selectively and only those peripherals that must function during IDLE are left operating. An enabled interrupt from an active peripheral will wake the processor from IDLE mode.

STANDBY:	Turns off clock to CPU and peripherals. This mode leaves the oscillator and PLL functional. An external interrupt event will wake the processor and the peripherals. Execution begins on the next valid cycle after detection of the interrupt event.

HALT:	Turns off the internal oscillator. This mode basically shuts down the device and places it in the lowest possible power consumption mode. Only a reset or XNMI can wake the device from this mode.

    - Peripheral Frames 0, 1, 2 (PFn)

The F281x segregates peripherals into three sections. The mapping of peripherals is as follows:

| PF0: | XINTF: | External Interface Configuration Registers (F2812 only) |
| --- | --- | --- |
|  | PIE: | PIE Interrupt Enable and Control Registers Plus PIE Vector Table |
|  | Flash: | Flash Control, Programming, Erase, Verify Registers |
|  | Timers: | CPU-Timers 0, 1, 2 Registers |
|  | CSM: | Code Security Module KEY Registers |
| PF1: | eCAN: | eCAN Mailbox and Control Registers |
| PF2: | SYS: | System Control Registers |
|  | GPIO: | GPIO Mux Configuration and Control Registers |
|  | EV: | Event Manager (EVA/EVB) Control Registers |
|  | McBSP: | McBSP Control and TX/RX Registers |
|  | SCI: | Serial Communications Interface (SCI) Control and RX/TX Registers |
|  | SPI: | Serial Peripheral Interface (SPI) Control and RX/TX Registers |
|  | ADC: | 12-Bit ADC Registers |

    - General-Purpose Input/Output (GPIO) Multiplexer

Most of the peripheral signals are multiplexed with general-purpose I/O (GPIO) signals. This multiplexing enables use of a pin as GPIO if the peripheral signal or function is not used. On reset, all GPIO pins are configured as inputs. The user can then individually program each pin for GPIO mode or peripheral signal mode. For specific inputs, the user can also select the number of input qualification cycles to filter unwanted noise glitches.

    - 32-Bit CPU-Timers (0, 1, 2)

CPU-Timers 0, 1, and 2 are identical 32-bit timers with presettable periods and with 16-bit clock prescaling. The timers have a 32-bit count-down register, which generates an interrupt when the counter reaches zero. The counter is decremented at the CPU clock speed divided by the prescale value setting. When the counter reaches zero, it is automatically reloaded with a 32-bit period value. CPU-Timer 2 is reserved for the DSP/BIOS Real- Time OS, and is connected to INT14 of the CPU. If DSP/BIOS is not being used, CPU-Timer 2 is available for general use. CPU-Timer 1 is for general use and can be connected to INT13 of the CPU. CPU-Timer 0 is also for general use and is connected to the PIE block.

    - Control Peripherals

The F281x supports the following peripherals that are used for embedded control and communication:

EV:	The event manager module includes general-purpose timers, full-compare/PWM units, capture inputs (CAP) and quadrature-encoder pulse (QEP) circuits. Two such event managers are provided which enable two three-phase motors to be driven or four two-phase motors. The event managers on the F281x are compatible to the event managers on the 240x devices (with some minor enhancements).

ADC:	The ADC block is a 12-bit converter, single ended, 16-channels. It contains two sample-and-hold units for simultaneous sampling.

    - Serial Port Peripherals

The F281x supports the following serial communication peripherals:

eCAN:	This is the enhanced version of the CAN peripheral. It supports 32 mailboxes, time stamping of messages, and is compliant with ISO 11898-1 (CAN 2.0B).

McBSP:	The multichannel buffered serial port (McBSP) connects to E1/T1 lines, phone-quality codecs for modem applications or high-quality stereo audio DAC devices. The McBSP receive and transmit registers are supported by a 16-level FIFO that significantly reduces the overhead for servicing this peripheral.

SPI:	The SPI is a high-speed, synchronous serial I/O port that allows a serial bit stream of programmed length (one to sixteen bits) to be shifted into and out of the device at a programmable bit-transfer rate. Normally, the SPI is used for communications between the DSP controller and external peripherals or another processor. Typical applications include external I/O or peripheral expansion through devices such as shift registers, display drivers, and ADCs. Multi- device communications are supported by the master/slave operation of the SPI. On the F281x, the port supports a 16- level, receive-and-transmit FIFO for reducing servicing overhead.

SCI:	The serial communications interface is a two-wire asynchronous serial port, commonly known as UART. On the F281x, the port supports a 16-level, receive-and-transmit FIFO for reducing servicing overhead.

###### Peripherals

The integrated peripherals of the F281x are described in the following subsections:

- Three 32-bit CPU-Timers

- Two event-manager modules (EVA, EVB)

- Enhanced analog-to-digital converter (ADC) module

- Enhanced controller area network (eCAN) module

- Multichannel buffered serial port (McBSP) module

- Serial communications interface modules (SCI-A, SCI-B)

- Serial peripheral interface (SPI) module

- Digital I/O and shared pin functions

    - 32-Bit CPU-Timers 0/1/2

There are three 32-bit CPU-timers on the F281x devices (CPU-TIMER0/1/2).

Timer 2 is reserved for DSP/BIOS. CPU-Timer 0 and CPU-Timer 1 can be used in user applications. These timers are different from the general-purpose (GP) timers that are present in the Event Manager modules (EVA, EVB).

Note

If the application is not using DSP/BIOS, then CPU-Timer 2 can be used in the application.

32-Bit Timer Period

PRDH:PRD

32-Bit Counter

TIMH:TIM

Borrow

Borrow

16-Bit Prescale Counter PSCH:PSC

16-Bit Timer Divide-Down TDDRH:TDDR

Reset Timer Reload

SYSCLKOUT

TCR.4

(Timer Start Status)

TINT

Figure 9-1. CPU-Timers

In the F281x devices, the timer interrupt signals (TINT0, TINT1, TINT2) are connected as shown in Figure 9-2.

TINT2

CPU-TIMER 2

(Reserved for DSP/BIOS)

CPU-TIMER 1

INT14

INT13

C28x

CPU

INT1

to INT12

TINT1

XINT13

TINT0

CPU-TIMER 0

PIE

- The timer registers are connected to the memory bus of the C28x processor.

- The timing of the timers is synchronized to SYSCLKOUT of the processor clock.

Figure 9-2. CPU-Timer Interrupts Signals and Output Signal

The general operation of the timer is as follows: The 32-bit counter register “TIMH:TIM” is loaded with the value in the period register “PRDH:PRD”. The counter register decrements at the SYSCLKOUT rate of the C28x. When the counter reaches 0, a timer interrupt output signal generates an interrupt pulse. The registers listed in Table 9-3 are used to configure the timers. For more information, see the TMS320x281x DSP System Control and Interrupts Reference Guide.

Table 9-3. CPU-Timers 0, 1, 2 Configuration and Control Registers

| NAME | ADDRESS | SIZE (x16) | DESCRIPTION |
| --- | --- | --- | --- |
| TIMER0TIM | 0x00 0C00 | 1 | CPU-Timer 0, Counter Register |
| TIMER0TIMH | 0x00 0C01 | 1 | CPU-Timer 0, Counter Register High |
| TIMER0PRD | 0x00 0C02 | 1 | CPU-Timer 0, Period Register |
| TIMER0PRDH | 0x00 0C03 | 1 | CPU-Timer 0, Period Register High |
| TIMER0TCR | 0x00 0C04 | 1 | CPU-Timer 0, Control Register |
| Reserved | 0x00 0C05 | 1 |  |
| TIMER0TPR | 0x00 0C06 | 1 | CPU-Timer 0, Prescale Register |
| TIMER0TPRH | 0x00 0C07 | 1 | CPU-Timer 0, Prescale Register High |
| TIMER1TIM | 0x00 0C08 | 1 | CPU-Timer 1, Counter Register |
| TIMER1TIMH | 0x00 0C09 | 1 | CPU-Timer 1, Counter Register High |
| TIMER1PRD | 0x00 0C0A | 1 | CPU-Timer 1, Period Register |
| TIMER1PRDH | 0x00 0C0B | 1 | CPU-Timer 1, Period Register High |
| TIMER1TCR | 0x00 0C0C | 1 | CPU-Timer 1, Control Register |
| Reserved | 0x00 0C0D | 1 |  |
| TIMER1TPR | 0x00 0C0E | 1 | CPU-Timer 1, Prescale Register |
| TIMER1TPRH | 0x00 0C0F | 1 | CPU-Timer 1, Prescale Register High |
| TIMER2TIM | 0x00 0C10 | 1 | CPU-Timer 2, Counter Register |
| TIMER2TIMH | 0x00 0C11 | 1 | CPU-Timer 2, Counter Register High |
| TIMER2PRD | 0x00 0C12 | 1 | CPU-Timer 2, Period Register |
| TIMER2PRDH | 0x00 0C13 | 1 | CPU-Timer 2, Period Register High |
| TIMER2TCR | 0x00 0C14 | 1 | CPU-Timer 2, Control Register |
| Reserved | 0x00 0C15 | 1 |  |
| TIMER2TPR | 0x00 0C16 | 1 | CPU-Timer 2, Prescale Register |
| TIMER2TPRH | 0x00 0C17 | 1 | CPU-Timer 2, Prescale Register High |
| Reserved | 0x00 0C18 – 0x00 0C3F | 40 |  |

    - Event Manager Modules (EVA, EVB)

The event-manager modules include general-purpose (GP) timers, full-compare/PWM units, capture units, and quadrature-encoder pulse (QEP) circuits. EVA and EVB timers, compare units, and capture units function identically. However, timer/unit names differ for EVA and EVB. Table 9-4 shows the module and signal names used. Table 9-4 shows the features and functionality available for the event-manager modules and highlights EVA nomenclature.

Event managers A and B have identical peripheral register sets with EVA starting at 7400h and EVB starting at 7500h. The paragraphs in this section describe the function of GP timers, compare units, capture units, and QEPs using EVA nomenclature. These paragraphs are applicable to EVB with regard to function—however, module/signal names would differ. Table 9-5 lists the EVA registers. For more information, see the TMS320x281x DSP Event Manager (EV) Reference Guide.

Table 9-4. Module and Signal Names for EVA and EVB

| EVENT MANAGER MODULES | EVA |  | EVB |  |
| --- | --- | --- | --- | --- |
|  | MODULE | SIGNAL | MODULE | SIGNAL |
| GP Timers | GP Timer 1 GP Timer 2 | T1PWM/T1CMP T2PWM/T2CMP | GP Timer 3 GP Timer 4 | T3PWM/T3CMP T4PWM/T4CMP |
| Compare Units | Compare 1 Compare 2 Compare 3 | PWM1/2 PWM3/4 PWM5/6 | Compare 4 Compare 5 Compare 6 | PWM7/8 PWM9/10 PWM11/12 |
| Capture Units | Capture 1 Capture 2 Capture 3 | CAP1 CAP2 CAP3 | Capture 4 Capture 5 Capture 6 | CAP4 CAP5 CAP6 |
| QEP Channels | QEP1 QEP2 QEPI1 | QEP1 QEP2 | QEP3 QEP4 QEPI2 | QEP3 QEP4 |
| External Clock Inputs | Direction External Clock | TDIRA TCLKINA | Direction External Clock | TDIRB TCLKINB |
| External Trip Inputs | Compare | C1TRIP C2TRIP C3TRIP | Compare | C4TRIP C5TRIP C6TRIP |
| External Trip Inputs |  | T1CTRIP_PDPINTA (1) T2CTRIP/ EVASOC |  | T3CTRIP_PDPINTB (1) T4CTRIP/ EVBSOC |

- In the 24x/240x-compatible mode, the T1CTRIP_PDPINTA pin functions as PDPINTA and the T3CTRIP_PDPINTB pin functions as PDPINTB.

Table 9-5. EVA Registers

| NAME(1) | ADDRESS | SIZE (x16) | DESCRIPTION |
| --- | --- | --- | --- |
| GPTCONA | 0x00 7400 | 1 | GP Timer Control Register A |
| T1CNT | 0x00 7401 | 1 | GP Timer 1 Counter Register |
| T1CMPR | 0x00 7402 | 1 | GP Timer 1 Compare Register |
| T1PR | 0x00 7403 | 1 | GP Timer 1 Period Register |
| T1CON | 0x00 7404 | 1 | GP Timer 1 Control Register |
| T2CNT | 0x00 7405 | 1 | GP Timer 2 Counter Register |
| T2CMPR | 0x00 7406 | 1 | GP Timer 2 Compare Register |
| T2PR | 0x00 7407 | 1 | GP Timer 2 Period Register |
| T2CON | 0x00 7408 | 1 | GP Timer 2 Control Register |
| EXTCONA(2) | 0x00 7409 | 1 | GP Extension Control Register A |
| COMCONA | 0x00 7411 | 1 | Compare Control Register A |
| ACTRA | 0x00 7413 | 1 | Compare Action Control Register A |
| DBTCONA | 0x00 7415 | 1 | Dead-Band Timer Control Register A |
| CMPR1 | 0x00 7417 | 1 | Compare Register 1 |
| CMPR2 | 0x00 7418 | 1 | Compare Register 2 |
| CMPR3 | 0x00 7419 | 1 | Compare Register 3 |
| CAPCONA | 0x00 7420 | 1 | Capture Control Register A |
| CAPFIFOA | 0x00 7422 | 1 | Capture FIFO Status Register A |
| CAP1FIFO | 0x00 7423 | 1 | Two-Level-Deep Capture FIFO Stack 1 |
| CAP2FIFO | 0x00 7424 | 1 | Two-Level-Deep Capture FIFO Stack 2 |
| CAP3FIFO | 0x00 7425 | 1 | Two-Level-Deep Capture FIFO Stack 3 |
| CAP1FBOT | 0x00 7427 | 1 | Bottom Register of Capture FIFO Stack 1 |
| CAP2FBOT | 0x00 7428 | 1 | Bottom Register of Capture FIFO Stack 2 |
| CAP3FBOT | 0x00 7429 | 1 | Bottom Register of Capture FIFO Stack 3 |
| EVAIMRA | 0x00 742C | 1 | Interrupt Mask Register A |
| EVAIMRB | 0x00 742D | 1 | Interrupt Mask Register B |
| EVAIMRC | 0x00 742E | 1 | Interrupt Mask Register C |
| EVAIFRA | 0x00 742F | 1 | Interrupt Flag Register A |
| EVAIFRB | 0x00 7430 | 1 | Interrupt Flag Register B |
| EVAIFRC | 0x00 7431 | 1 | Interrupt Flag Register C |

- The EV-B register set is identical except the address range is from 0x00 7500 to 0x00 753F. The above registers are mapped to Zone

2. This space allows only 16-bit accesses. 32-bit accesses produce undefined results.

- New register compared to 24x/240x

Peripheral Bus

16

16

16

16

GP Timer 2

T2PWM_T2CMP

QEP

Logic

Capture Units

TDIRA

CAP1_QEP1 CAP2_QEP2

CAP3_QEPI1

DBTCONA[15:0]

QEPDIR

QEPCLK

HSPCLK

clock

dir reset

TCLKINA

EXTCONA[1:2]

T1CON[1]

dir

clock

T2CON[5,4]

CAPCONA[10,9]

Prescaler

Output Logic

Timer 2 Compare

ACTRA[11:0]

COMCONA[15:5,2:0]

Full Compare 3

Index Qual

T2CON[1]

Full Compare 1

Output Logic

Dead-Band Logic

Prescaler

GP Timer 1

T1CON[5,4]

Output Logic

Control Logic

CAPCONA[15:12,7:0]

T2CON[15:11,7,6,3,2,0]

T2CON[10:8]

GPTCONA[3,2]

ACTRA[15:12],

COMCONA[12], T1CON[13:11]

T1CON[15:11,6,3,2]

Full Compare 2

T1CON[10:8]

GPTCONA[1,0]

GPTCONA[12:4], CAPCONA[8], EXTCONA[0]

SVPWM

State Machine

EVAENCLK

EVATO ADC (Internal) T1CTRIP/PDPINTA, T2CTRIP, C1TRIP, C2TRIP, C3TRIP

EVASOC ADC (External)

TDIRA

T1PWM_T1CMP

Timer 1 Compare

PWM1

PWM2 PWM3

PWM4 PWM5 PWM6

TCLKINA

HSPCLK

16

The EVB module is similar to the EVA module.

Figure 9-3. Event Manager A Functional Block Diagram

      - General-Purpose (GP) Timers

There are two GP timers. The GP timer x (x = 1 or 2 for EVA; x = 3 or 4 for EVB) includes:

- A 16-bit timer, up-/down-counter, TxCNT, for reads or writes

- A 16-bit timer-compare register, TxCMPR (double-buffered with shadow register), for reads or writes

- A 16-bit timer-period register, TxPR (double-buffered with shadow register), for reads or writes

- A 16-bit timer-control register, TxCON, for reads or writes

- Selectable internal or external input clocks

- A programmable prescaler for internal or external clock inputs

- Control and interrupt logic, for four maskable interrupts: underflow, overflow, timer compare, and period interrupts

- A selectable direction input pin (TDIRx) (to count up or down when directional up-/down-count mode is selected)

The GP timers can be operated independently or synchronized with each other. The compare register associated with each GP timer can be used for compare function and PWM-waveform generation. There are three continuous modes of operations for each GP timer in up- or up/down-counting operations. Internal or external input clocks with programmable prescaler are used for each GP timer. GP timers also provide the time base for the other event-manager submodules: GP timer 1 for all the compares and PWM circuits, GP timer 2/1 for the capture units and the quadrature-pulse counting operations. Double-buffering of the period and compare registers allows programmable change of the timer (PWM) period and the compare/PWM pulse width as needed.

      - Full-Compare Units

There are three full-compare units on each event manager. These compare units use GP timer1 as the time base and generate six outputs for compare and PWM-waveform generation using programmable deadband circuit. The state of each of the six outputs is configured independently. The compare registers of the compare units are double-buffered, allowing programmable change of the compare/PWM pulse widths as needed.

      - Programmable Deadband Generator

Deadband generation can be enabled/disabled for each compare unit output individually. The deadband- generator circuit produces two outputs (with or without deadband zone) for each compare unit output signal. The output states of the deadband generator are configurable and changeable as needed by way of the double- buffered ACTRx register.

      - PWM Waveform Generation

Up to eight PWM waveforms (outputs) can be generated simultaneously by each event manager: three independent pairs (six outputs) by the three full-compare units with programmable deadbands, and two independent PWMs by the GP-timer compares.

      - Double Update PWM Mode

The F281x Event Manager supports “Double Update PWM Mode.” This mode refers to a PWM operation mode in which the position of the leading edge and the position of the trailing edge of a PWM pulse are independently modifiable in each PWM period. To support this mode, the compare register that determines the position of the edges of a PWM pulse must allow (buffered) compare value update once at the beginning of a PWM period and another time in the middle of a PWM period. The compare registers in F281x Event Managers are all buffered and support three compare value reload/update (value in buffer becoming active) modes. These modes have earlier been documented as compare value reload conditions. The reload condition that supports double update PWM mode is reloaded on Underflow (beginning of PWM period) OR Period (middle of PWM period). Double update PWM mode can be achieved by using this condition for compare value reload.

      - PWM Characteristics

Characteristics of the PWMs are as follows:

- 16-bit registers

- Wide range of programmable deadband for the PWM output pairs

- Change of the PWM carrier frequency for PWM frequency wobbling as needed

- Change of the PWM pulse widths within and after each PWM period as needed

- External-maskable power and drive-protection interrupts

- Pulse-pattern-generator circuit, for programmable generation of asymmetric, symmetric, and four-space vector PWM waveforms

- Minimized CPU overhead using auto-reload of the compare and period registers

- The PWM pins are driven to a high-impedance state when the PDPINTx pin is driven low and after PDPINTx signal qualification. The PDPINTx pin (after qualification) is reflected in bit 8 of the COMCONx register.

  - PDPINTA pin status is reflected in bit 8 of COMCONA register.

  - PDPINTB pin status is reflected in bit 8 of COMCONB register.

- EXTCON register bits provide options to individually trip control for each PWM pair of signals

      - Capture Unit

The capture unit provides a logging function for different events or transitions. The values of the selected GP timer counter is captured and stored in the two-level-deep FIFO stacks when selected transitions are detected on capture input pins, CAPx (x = 1, 2, or 3 for EVA; and x = 4, 5, or 6 for EVB). The capture unit consists of three capture circuits.

Capture units include the following features:

- One 16-bit capture control register, CAPCONx (R/W)

- One 16-bit capture FIFO status register, CAPFIFOx

- Selection of GP timer 1/2 (for EVA) or 3/4 (for EVB) as the time base

- Three 16-bit 2-level-deep FIFO stacks, one for each capture unit

- Three capture input pins (CAP1/2/3 for EVA, CAP4/5/6 for EVB)—one input pin per capture unit. [All inputs are synchronized with the device (CPU) clock. In order for a transition to be captured, the input must hold at its current level to meet the input qualification circuitry requirements. The input pins CAP1/2 and CAP4/5 can also be used as QEP inputs to the QEP circuit.]

- User-specified transition (rising edge, falling edge, or both edges) detection

- Three maskable interrupt flags, one for each capture unit

- The capture pins can also be used as general-purpose interrupt pins, if they are not used for the capture function.

      - Quadrature-Encoder Pulse (QEP) Circuit

Two capture inputs (CAP1 and CAP2 for EVA; CAP4 and CAP5 for EVB) can be used to interface the on-chip QEP circuit with a quadrature encoder pulse. Full synchronization of these inputs is performed on-chip. Direction or leading-quadrature pulse sequence is detected, and GP timer 2/4 is incremented or decremented by the rising and falling edges of the two input signals (four times the frequency of either input pulse).

With EXTCONA register bits, the EVA QEP circuit can use CAP3 as a capture index pin as well. Similarly, with EXTCONB register bits, the EVB QEP circuit can use CAP6 as a capture index pin.

      - External ADC Start-of-Conversion

EVA/EVB start-of-conversion (SOC) can be sent to an external pin ( EVASOC/ EVBSOC) for external ADC interface. EVASOC and EVBSOC are MUXed with T2CTRIP and T4CTRIP, respectively.

    - Enhanced Analog-to-Digital Converter (ADC) Module

A simplified functional block diagram of the ADC module is shown in Figure 9-4. The ADC module consists of a 12-bit ADC with a built-in sample-and-hold (S/H) circuit. Functions of the ADC module include:

- 12-bit ADC core with built-in S/H

- Analog input: 0.0 V to 3.0 V (voltages above 3.0 V produce full-scale conversion results)

- Fast conversion rate: 80 ns at 25-MHz ADC clock, 12.5 MSPS

- 16-channel, MUXed inputs

- Autosequencing capability provides up to 16 “autoconversions” in a single session. Each conversion can be programmed to select any 1 of 16 input channels

- Sequencer can be operated as two independent 8-state sequencers or as one large 16-state sequencer (that is, two cascaded 8-state sequencers)

- Sixteen result registers (individually addressable) to store conversion values – The digital value of the input analog voltage is derived by:

Digital Value = 0	, when ADCIN  ADCLO

(

Digital Value = floor 4096   ADCIN  ADCLO

3

, when ADCLO  ADCIN  3 V

Digital Value = 4095	, when ADCIN  3 V

- Multiple triggers as sources for the start-of-conversion (SOC) sequence

  - S/W – software immediate start

  - EVA – Event manager A (multiple event sources within EVA)

  - EVB – Event manager B (multiple event sources within EVB)

- Flexible interrupt control allows interrupt request on every end-of-sequence (EOS) or every other EOS

- Sequencer can operate in “start/stop” mode, allowing multiple “time-sequenced triggers” to synchronize conversions

- EVA and EVB triggers can operate independently in dual-sequencer mode

- Sample-and-hold (S/H) acquisition time window has separate prescale control

The ADC module in the F281x has been enhanced to provide flexible interface to event managers A and B. The ADC interface is built around a fast, 12-bit ADC module with a fast conversion rate of 80 ns at 25-MHz ADC clock. The ADC module has 16 channels, configurable as two independent 8-channel modules to service event managers A and B. The two independent 8-channel modules can be cascaded to form a 16-channel module. Although there are multiple input channels and two sequencers, there is only one converter in the ADC module. Figure 9-4 shows the block diagram of the F281x ADC module.

The two 8-channel modules have the capability to autosequence a series of conversions, each module has the choice of selecting any one of the respective eight channels available through an analog MUX. In the cascaded mode, the autosequencer functions as a single 16-channel sequencer. On each sequencer, once the conversion is complete, the selected channel value is stored in its respective RESULT register. Autosequencing allows the system to convert the same channel multiple times, allowing the user to perform oversampling algorithms. This gives increased resolution over traditional single-sampled conversion results.

ADCSOC

ADCINB0

High-Speed

Prescaler

SYSCLKOUT

C28x

ADCENCLK

HSPCLK

Analog

MUX

Result Registers

ADCINA0

Result Reg 0

Result Reg 1

70A8h

S/H

ADCINA7

12-Bit

ADC

Module

Result Reg 7

Result Reg 8

70AFh

70B0h

S/H

System

Control Block

ADCINB7

Result Reg 15

70B7h

ADC Control Registers

S/W

EVA

SOC

Sequencer 1

Sequencer 2

SOC

S/W EVB

Figure 9-4. Block Diagram of the F281x ADC Module

To obtain the specified accuracy of the ADC, proper board layout is critical. To the best extent possible, traces leading to the ADCIN pins should not run in close proximity to the digital signal paths. This is to minimize switching noise on the digital lines from getting coupled to the ADC inputs. Furthermore, proper isolation techniques must be used to isolate the ADC module power pins (VDDA1/VDDA2, AVDDREFBG) from the digital supply. For better accuracy and ESD protection, unused ADC inputs should be connected to analog ground.

Notes:

- The ADC registers are accessed at the SYSCLKOUT rate. The internal timing of the ADC module is controlled by the high-speed peripheral clock (HSPCLK).

- The behavior of the ADC module based on the state of the ADCENCLK and HALT signals is as follows:

ADCENCLK: On reset, this signal will be low. While reset is active-low ( XRS) the clock to the register will still function. This is necessary to make sure all registers and modes go into their default reset state. The analog module will, however, be in a low-power inactive state. As soon as reset goes high, then the clock to the registers will be disabled. When the user sets the ADCENCLK signal high, then the clocks to the registers will be enabled and the analog module will be enabled. There will be a certain time delay (ms range) before the ADC is stable and can be used.

HALT: This signal only affects the analog module. It does not affect the registers. If low, the ADC module is powered. If high, the ADC module goes into low-power mode. The HALT mode will stop the clock to the CPU, which will stop the HSPCLK. Therefore the ADC register logic will be turned off indirectly.

Figure 9-5 shows the ADC pin-biasing for internal reference and Figure 9-6 shows the ADC pin-biasing for external reference.

ADC 16-Channel Analog Inputs

ADCINA[7:0] ADCINB[7:0]

ADCLO

(A)

Analog input 0-3 V with respect to ADCLO Connect to Analog Ground

Test Pin	ADCBGREFIN

ADC External Current Bias Resistor	ADCRESEXT

24.9 k /20 k (B)

10 μF(C)

ADC Reference Positive Output

ADC Reference Medium Output

ADC Analog Power

ADC Reference Power ADC Analog I/O Power

ADC Digital Power

ADCREFP

ADCREFM

VDDA1 VDDA2 VSSA1 VSSA2

AVDDREFBG AVSSREFBG

VDDAIO

VSSAIO

VDD1

10 μF(C)

ADCREFP and ADCREFM should not be loaded by external circuitry

Analog 3.3 V

Analog 3.3 V

Analog 3.3 V

Analog 3.3 V Analog Ground

1.8 V can use the same 1.8-V (or 1.9-V) supply as the digital core but separate the two with a

VSS1

Digital Ground

ferrite bead or a filter

- Provide access to this pin in PCB layouts. Intended for test purposes only.

- Use 24.9 kΩ for ADC clock range 1–18.75 MHz; use 20 kΩ for ADC clock range 18.75–25 MHz.

- TAIYO YUDEN EMK325F106ZH, EMK325BJ106MD, or equivalent ceramic capacitor

- External decoupling capacitors are recommended on all power pins.

- Analog inputs must be driven from an operational amplifier that does not degrade the ADC performance.

Figure 9-5. ADC Pin Connections With Internal Reference

Note

The temperature rating of any recommended component must match the rating of the end product.

ADC 16-Channel Analog Inputs

Test Pin

ADCINA[7:0] ADCINB[7:0]

ADCLO ADCBGREFIN

Analog Input 0-3 V With Respect to ADCLO Connect to Analog Ground

24.9 k /20 k (C)

ADC External Current Bias Resistor	ADCRESEXT

ADC Reference Positive Input ADC Reference Medium Input

ADCREFP ADCREFM

2 V

(D)

1 V

1 μF - 10 μF

1 μF - 10 μF

ADC Analog Power

ADC Reference Power

ADC Analog I/O Power

ADC Digital Power

VDDA1 VDDA2 VSSA1 VSSA2

AVDDREFBG AVSSREFBG

VDDAIO VSSAIO

VDD1 VSS1

Analog 3.3 V

Analog 3.3 V

Analog 3.3 V

Analog 3.3 V Analog Ground

1.8 V can use the same 1.8-V (or 1.9-V)

Digital Ground	supply as the digital core but separate the

two with a ferrite bead or a filter

- External decoupling capacitors are recommended on all power pins.

- Analog inputs must be driven from an operational amplifier that does not degrade the ADC performance.

- Use 24.9 kΩ for ADC clock range 1–18.75 MHz; use 20 kΩ for ADC clock range 18.75–25 MHz.

- It is recommended that buffered external references be provided with a voltage difference of (ADCREFP – ADCREFM) = 1 V ± 0.1% or better. External reference is enabled using bit 8 in the ADCTRL3 Register at ADC power up. In this mode, the accuracy of external reference is critical for overall gain. The voltage ADCREFP – ADCREFM will determine the overall accuracy. Do not enable internal references when external references are connected to ADCREFP and ADCREFM. See the TMS320x281x DSP Analog-to-Digital Converter (ADC) Reference Guide for more information.

Figure 9-6. ADC Pin Connections With External Reference

The ADC operation is configured, controlled, and monitored by the registers listed in Table 9-6.

Table 9-6. ADC Registers

| NAME(1) | ADDRESS | SIZE (x16) | DESCRIPTION |
| --- | --- | --- | --- |
| ADCTRL1 | 0x00 7100 | 1 | ADC Control Register 1 |
| ADCTRL2 | 0x00 7101 | 1 | ADC Control Register 2 |
| ADCMAXCONV | 0x00 7102 | 1 | ADC Maximum Conversion Channels Register |
| ADCCHSELSEQ1 | 0x00 7103 | 1 | ADC Channel Select Sequencing Control Register 1 |
| ADCCHSELSEQ2 | 0x00 7104 | 1 | ADC Channel Select Sequencing Control Register 2 |
| ADCCHSELSEQ3 | 0x00 7105 | 1 | ADC Channel Select Sequencing Control Register 3 |
| ADCCHSELSEQ4 | 0x00 7106 | 1 | ADC Channel Select Sequencing Control Register 4 |
| ADCASEQSR | 0x00 7107 | 1 | ADC Auto-Sequence Status Register |
| ADCRESULT0 | 0x00 7108 | 1 | ADC Conversion Result Buffer Register 0 |
| ADCRESULT1 | 0x00 7109 | 1 | ADC Conversion Result Buffer Register 1 |
| ADCRESULT2 | 0x00 710A | 1 | ADC Conversion Result Buffer Register 2 |
| ADCRESULT3 | 0x00 710B | 1 | ADC Conversion Result Buffer Register 3 |
| ADCRESULT4 | 0x00 710C | 1 | ADC Conversion Result Buffer Register 4 |
| ADCRESULT5 | 0x00 710D | 1 | ADC Conversion Result Buffer Register 5 |
| ADCRESULT6 | 0x00 710E | 1 | ADC Conversion Result Buffer Register 6 |
| ADCRESULT7 | 0x00 710F | 1 | ADC Conversion Result Buffer Register 7 |
| ADCRESULT8 | 0x00 7110 | 1 | ADC Conversion Result Buffer Register 8 |
| ADCRESULT9 | 0x00 7111 | 1 | ADC Conversion Result Buffer Register 9 |
| ADCRESULT10 | 0x00 7112 | 1 | ADC Conversion Result Buffer Register 10 |
| ADCRESULT11 | 0x00 7113 | 1 | ADC Conversion Result Buffer Register 11 |
| ADCRESULT12 | 0x00 7114 | 1 | ADC Conversion Result Buffer Register 12 |
| ADCRESULT13 | 0x00 7115 | 1 | ADC Conversion Result Buffer Register 13 |
| ADCRESULT14 | 0x00 7116 | 1 | ADC Conversion Result Buffer Register 14 |
| ADCRESULT15 | 0x00 7117 | 1 | ADC Conversion Result Buffer Register 15 |
| ADCTRL3 | 0x00 7118 | 1 | ADC Control Register 3 |
| ADCST | 0x00 7119 | 1 | ADC Status Register |
| Reserved | 0x00 711C – 0x00 711F | 4 |  |

(1)	The above registers are Peripheral Frame 2 Registers.

    - Enhanced Controller Area Network (eCAN) Module

The CAN module has the following features:

- Fully compliant with ISO 11898-1 (CAN 2.0B)

- Supports data rates up to 1 Mbps

- Thirty-two mailboxes, each with the following properties:

  - Configurable as receive or transmit

  - Configurable with standard or extended identifier

  - Has a programmable receive mask

  - Supports data and remote frame

  - Composed of 0 to 8 bytes of data

  - Uses a 32-bit time stamp on receive and transmit message

  - Protects against reception of new message

  - Holds the dynamically programmable priority of transmit message

  - Employs a programmable interrupt scheme with two interrupt levels

  - Employs a programmable alarm on transmission or reception time-out

- Low-power mode

- Programmable wake-up on bus activity

- Automatic reply to a remote request message

- Automatic retransmission of a frame in case of loss of arbitration or error

- 32-bit local network time counter synchronized by a specific message (communication in conjunction with mailbox 16)

- Self-test mode

  - Operates in a loopback mode receiving its own message. A “dummy” acknowledge is provided, thereby eliminating the need for another node to provide the acknowledge bit.

Note

NOTE: For a SYSCLKOUT of 150 MHz, the smallest bit rate possible is 23.4 kbps. The 28x CAN has passed the conformance test per ISO/DIS 16845. Contact TI for details.

eCAN0INT

eCAN1INT

Controls Address	Data

32

Message Controller

32

32

eCAN Memory (512 Bytes) Registers and

Message Objects Control

Enhanced CAN Controller

Mailbox RAM (512 Bytes)

32-Message Mailbox of 4 x 32-Bit Words

Memory Management Unit

CPU Interface, Receive Control Unit,

Timer Management Unit

32

eCAN Protocol Kernel

SN65HVD23x

3.3-V CAN Transceiver

Status Buffer

Control Buffer

Transmit Buffer

Receive Buffer

CAN Bus

Figure 9-7. eCAN Block Diagram and Interface Circuit

Table 9-7. 3.3-V eCAN Transceivers for the TMS320F281x DSPs

| PART NUMBER | SUPPLY VOLTAGE | LOW-POWER MODE | SLOPE CONTROL | VREF | OTHER | TA |
| --- | --- | --- | --- | --- | --- | --- |
| SN65HVD230 | 3.3 V | Standby | Adjustable | Yes | – | –40°C to 85°C |
| SN65HVD230Q | 3.3 V | Standby | Adjustable | Yes | – | –40°C to 125°C |
| SN65HVD231 | 3.3 V | Sleep | Adjustable | Yes | – | –40°C to 85°C |
| SN65HVD231Q | 3.3 V | Sleep | Adjustable | Yes | – | –40°C to 125°C |
| SN65HVD232 | 3.3 V | None | None | None | – | –40°C to 85°C |
| SN65HVD232Q | 3.3 V | None | None | None | – | –40°C to 125°C |
| SN65HVD233 | 3.3 V | Standby | Adjustable | None | Diagnostic Loopback | –40°C to 125°C |
| SN65HVD234 | 3.3 V | Standby and Sleep | Adjustable | None | – | –40°C to 125°C |
| SN65HVD235 | 3.3 V | Standby | Adjustable | None | Autobaud Loopback | –40°C to 125°C |
| ISO1050 | 3–5.5 V | None | None | None | Built-in Isolation Low Prop Delay Thermal Shutdown Failsafe Operation Dominant Time-out | –55°C to 105°C |

eCAN Control and Status Registers

6000h

603Fh

6040h

607Fh

6080h

60BFh

60C0h

60FFh

6100h-6107h

6108h-610Fh

6110h-6117h

6118h-611Fh

6120h-6127h

eCAN Memory (512 Bytes)

Control and Status Registers

Local Acceptance Masks (LAM) (32 x 32-Bit RAM)

Message Object Time Stamps (MOTS) (32 x 32-Bit RAM)

Message Object Time-Out (MOTO) (32 x 32-Bit RAM)

|  | Mailbox Enable - CANME |
| --- | --- |
|  | Mailbox Direction - CANMD |
|  | Transmission Request Set - CANTRS |
|  | Transmission Request Reset - CANTRR |
|  | Transmission Acknowledge - CANTA |
|  | Abort Acknowledge - CANAA |
|  | Received Message Pending - CANRMP |
|  | Received Message Lost - CANRML |
|  | Remote Frame Pending - CANRFP |
|  | Global Acceptance Mask - CANGAM |
|  | Master Control - CANMC |
|  | Bit-Timing Configuration - CANBTC |
|  | Error and Status - CANES |
|  | Transmit Error Counter - CANTEC |
|  | Receive Error Counter - CANREC |
|  | Global Interrupt Flag 0 - CANGIF0 |
|  | Global Interrupt Mask - CANGIM |
|  | Global Interrupt Flag 1 - CANGIF1 |
|  | Mailbox Interrupt Mask - CANMIM |
|  | Mailbox Interrupt Level - CANMIL |
|  | Overwrite Protection Control - CANOPC |
|  | TX I/O Control - CANTIOC |
|  | RX I/O Control - CANRIOC |
|  | Time Stamp Counter - CANTSC |
|  | Time-Out Control - CANTOC |
|  | Time-Out Status - CANTOS |
|  | Reserved |

eCAN Memory RAM (512 Bytes)

Mailbox 0

Mailbox 1

Mailbox 2

Mailbox 3

Mailbox 4

Mailbox 28

Mailbox 29

Mailbox 30

Mailbox 31

61E0h-61E7h

61E8h-61EFh

61F0h-61F7h

61F8h-61FFh

|  | Message Identifier - MSGID |
| --- | --- |
|  | Message Control - MSGCTRL |
|  | Message Data Low - MDL |
|  | Message Data High - MDH |

Message Mailbox (16 Bytes)

61EAh-61EBh

61ECh-61EDh

61EEh-61EFh

61E8h-61E9h

Figure 9-8. eCAN Memory Map

Note

If the eCAN module is not used in an application, the RAM available (LAM, MOTS, MOTO, and mailbox RAM) can be used as general-purpose RAM. The CAN module clock should be enabled for this.

The CAN registers listed in Table 9-8 are used by the CPU to configure and control the CAN controller and the message objects. eCAN control registers only support 32-bit read/write operations. Mailbox RAM can be accessed as 16 bits or 32 bits. 32-bit accesses are aligned to an even boundary.

Table 9-8. CAN Registers

| NAME(1) | ADDRESS | SIZE (x32) | DESCRIPTION |
| --- | --- | --- | --- |
| CANME | 0x00 6000 | 1 | Mailbox enable |
| CANMD | 0x00 6002 | 1 | Mailbox direction |
| CANTRS | 0x00 6004 | 1 | Transmit request set |
| CANTRR | 0x00 6006 | 1 | Transmit request reset |
| CANTA | 0x00 6008 | 1 | Transmission acknowledge |
| CANAA | 0x00 600A | 1 | Abort acknowledge |
| CANRMP | 0x00 600C | 1 | Receive message pending |
| CANRML | 0x00 600E | 1 | Receive message lost |
| CANRFP | 0x00 6010 | 1 | Remote frame pending |
| CANGAM | 0x00 6012 | 1 | Global acceptance mask |
| CANMC | 0x00 6014 | 1 | Master control |
| CANBTC | 0x00 6016 | 1 | Bit-timing configuration |
| CANES | 0x00 6018 | 1 | Error and status |
| CANTEC | 0x00 601A | 1 | Transmit error counter |
| CANREC | 0x00 601C | 1 | Receive error counter |
| CANGIF0 | 0x00 601E | 1 | Global interrupt flag 0 |
| CANGIM | 0x00 6020 | 1 | Global interrupt mask |
| CANGIF1 | 0x00 6022 | 1 | Global interrupt flag 1 |
| CANMIM | 0x00 6024 | 1 | Mailbox interrupt mask |
| CANMIL | 0x00 6026 | 1 | Mailbox interrupt level |
| CANOPC | 0x00 6028 | 1 | Overwrite protection control |
| CANTIOC | 0x00 602A | 1 | TX I/O control |
| CANRIOC | 0x00 602C | 1 | RX I/O control |
| CANTSC | 0x00 602E | 1 | Time stamp counter (Reserved in SCC mode) |
| CANTOC | 0x00 6030 | 1 | Time-out control (Reserved in SCC mode) |
| CANTOS | 0x00 6032 | 1 | Time-out status (Reserved in SCC mode) |

(1)	These registers are mapped to Peripheral Frame 1.

    - Multichannel Buffered Serial Port (McBSP) Module

The McBSP module has the following features:

- Compatible to McBSP in TMS320C54x/TMS320C55x DSP devices, except the DMA features

- Full-duplex communication

- Double-buffered data registers which allow a continuous data stream

- Independent framing and clocking for receive and transmit

- External shift clock generation or an internal programmable frequency shift clock

- A wide selection of data sizes including 8-, 12-, 16-, 20-, 24-, or 32-bits

- 8-bit data transfers with LSB or MSB first

- Programmable polarity for both frame synchronization and data clocks

- Highly programmable internal clock and frame generation

- Support A-bis mode

- Direct interface to industry-standard CODECs, Analog Interface Chips (AICs), and other serially connected A/D and D/A devices

- Works with SPI-compatible devices

- Two 16 x 16-level FIFO for Transmit channel

- Two 16 x 16-level FIFO for Receive channel

The following application interfaces can be supported on the McBSP:

- T1/E1 framers

- MVIP switching-compatible and ST-BUS-compliant devices including:

  - MVIP framers

  - H.100 framers

  - SCSA framers

  - IOM-2 compliant devices

  - AC97-compliant devices (the necessary multiphase frame synchronization capability is provided.)

  - IIS-compliant devices

- McBSP clock rate = CLKG = CLKSRG/(1 + CLKGDIV) , where CLKSRG source could be LSPCLK, CLKX, or CLKR. 1

1 Serial port performance is limited by I/O buffer switching speed. Internal prescalers must be adjusted such that the peripheral speed is less than the I/O buffer speed limit—20-MHz maximum.

Peripheral Write Bus

MXINT

To CPU

TX FIFO

Interrupt

TX FIFO Registers

16	16

LSPCLK

16

FSX

16

CLKX

DX

DR

16

CLKR

16

FSR

16

16

McBSP

16

16

MRINT

To CPU

RX FIFO

Interrupt

RX FIFO Registers

Peripheral Read Bus

RX Interrupt Logic

McBSP Receive Interrupt Select Logic

McBSP Transmit Interrupt Select Logic

TX Interrupt Logic

RX FIFO _1

RX FIFO _0

RX FIFO _15

RBR1 Register

Expand Logic

RSR1

RSR2

TX FIFO _0

TX FIFO _1

TX FIFO _15

DRR1 Receive Buffer

DRR2 Receive Buffer

RBR2 Register

XSR1

XSR2

Compand Logic

McBSP Registers and

Control Logic

Figure 9-9 shows the block diagram of the McBSP module with FIFO, interfaced to the F281x version of Peripheral Frame 2.

TX FIFO _15

TX FIFO _1

TX FIFO _0

| DXR2 Transmit Buffer |  | DXR1 Transmit Buffer |
| --- | --- | --- |

RX FIFO _15

RX FIFO _1

RX FIFO _0

Figure 9-9. McBSP Module With FIFO

Table 9-9 provides a summary of the McBSP registers.

Table 9-9. McBSP Registers

| NAME | ADDRESS 0x00 78xxh | TYPE (R/W) | RESET VALUE (HEX) | DESCRIPTION |
| --- | --- | --- | --- | --- |
| DATA REGISTERS, RECEIVE, TRANSMIT (1) |  |  |  |  |
| – | – | – | 0x0000 | McBSP Receive Buffer Register |
| – | – | – | 0x0000 | McBSP Receive Shift Register |
| – | – | – | 0x0000 | McBSP Transmit Shift Register |
| DRR2 | 00 | R | 0x0000 | McBSP Data Receive Register 2 - Read First if the word size is greater than 16 bits, else ignore DRR2 |
| DRR1 | 01 | R | 0x0000 | McBSP Data Receive Register 1 - Read Second if the word size is greater than 16 bits, else read DRR1 only |
| DXR2 | 02 | W | 0x0000 | McBSP Data Transmit Register 2 - Write First if the word size is greater than 16 bits, else ignore DXR2 |
| DXR1 | 03 | W | 0x0000 | McBSP Data Transmit Register 1 - Write Second if the word size is greater than 16 bits, else write to DXR1 only |
| McBSP CONTROL REGISTERS |  |  |  |  |
| SPCR2 | 04 | R/W | 0x0000 | McBSP Serial Port Control Register 2 |
| SPCR1 | 05 | R/W | 0x0000 | McBSP Serial Port Control Register 1 |
| RCR2 | 06 | R/W | 0x0000 | McBSP Receive Control Register 2 |
| RCR1 | 07 | R/W | 0x0000 | McBSP Receive Control Register 1 |
| XCR2 | 08 | R/W | 0x0000 | McBSP Transmit Control Register 2 |
| XCR1 | 09 | R/W | 0x0000 | McBSP Transmit Control Register 1 |
| SRGR2 | 0A | R/W | 0x0000 | McBSP Sample Rate Generator Register 2 |
| SRGR1 | 0B | R/W | 0x0000 | McBSP Sample Rate Generator Register 1 |
| MULTICHANNEL CONTROL REGISTERS |  |  |  |  |
| MCR2 | 0C | R/W | 0x0000 | McBSP Multichannel Register 2 |
| MCR1 | 0D | R/W | 0x0000 | McBSP Multichannel Register 1 |
| RCERA | 0E | R/W | 0x0000 | McBSP Receive Channel Enable Register Partition A |
| RCERB | 0F | R/W | 0x0000 | McBSP Receive Channel Enable Register Partition B |
| XCERA | 10 | R/W | 0x0000 | McBSP Transmit Channel Enable Register Partition A |
| XCERB | 11 | R/W | 0x0000 | McBSP Transmit Channel Enable Register Partition B |
| PCR | 12 | R/W | 0x0000 | McBSP Pin Control Register |
| RCERC | 13 | R/W | 0x0000 | McBSP Receive Channel Enable Register Partition C |
| RCERD | 14 | R/W | 0x0000 | McBSP Receive Channel Enable Register Partition D |
| XCERC | 15 | R/W | 0x0000 | McBSP Transmit Channel Enable Register Partition C |
| XCERD | 16 | R/W | 0x0000 | McBSP Transmit Channel Enable Register Partition D |
| RCERE | 17 | R/W | 0x0000 | McBSP Receive Channel Enable Register Partition E |
| RCERF | 18 | R/W | 0x0000 | McBSP Receive Channel Enable Register Partition F |
| XCERE | 19 | R/W | 0x0000 | McBSP Transmit Channel Enable Register Partition E |
| XCERF | 1A | R/W | 0x0000 | McBSP Transmit Channel Enable Register Partition F |
| RCERG | 1B | R/W | 0x0000 | McBSP Receive Channel Enable Register Partition G |
| RCERH | 1C | R/W | 0x0000 | McBSP Receive Channel Enable Register Partition H |
| XCERG | 1D | R/W | 0x0000 | McBSP Transmit Channel Enable Register Partition G |

Table 9-9. McBSP Registers (continued)

| NAME | ADDRESS 0x00 78xxh | TYPE (R/W) | RESET VALUE (HEX) | DESCRIPTION |
| --- | --- | --- | --- | --- |
| XCERH | 1E | R/W | 0x0000 | McBSP Transmit Channel Enable Register Partition H |

Table 9-9. McBSP Registers (continued)

| NAME | ADDRESS 0x00 78xxh | TYPE (R/W) | RESET VALUE (HEX) | DESCRIPTION |
| --- | --- | --- | --- | --- |
| FIFO MODE REGISTERS (applicable only in FIFO mode) |  |  |  |  |
| FIFO Data Registers (2) |  |  |  |  |
| DRR2 | 00 | R | 0x0000 | McBSP Data Receive Register 2 – Top of receive FIFO - Read First FIFO pointers will not advance |
| DRR1 | 01 | R | 0x0000 | McBSP Data Receive Register 1 – Top of receive FIFO - Read Second for FIFO pointers to advance |
| DXR2 | 02 | W | 0x0000 | McBSP Data Transmit Register 2 – Top of transmit FIFO - Write First FIFO pointers will not advance |
| DXR1 | 03 | W | 0x0000 | McBSP Data Transmit Register 1 – Top of transmit FIFO - Write Second for FIFO pointers to advance |
| FIFO Control Registers |  |  |  |  |
| MFFTX | 20 | R/W | 0xA000 | McBSP Transmit FIFO Register |
| MFFRX | 21 | R/W | 0x201F | McBSP Receive FIFO Register |
| MFFCT | 22 | R/W | 0x0000 | McBSP FIFO Control Register |
| MFFINT | 23 | R/W | 0x0000 | McBSP FIFO Interrupt Register |
| MFFST | 24 | R/W | 0x0000 | McBSP FIFO Status Register |

- DRR2/DRR1 and DXR2/DXR1 share the same addresses of receive and transmit FIFO registers in FIFO mode.

- FIFO pointers advancing is based on order of access to DRR2/DRR1 and DXR2/DXR1 registers.

    - Serial Communications Interface (SCI) Module

The F281x devices include two serial communications interface (SCI) modules. The SCI modules support digital communications between the CPU and other asynchronous peripherals that use the standard non-return-to-zero (NRZ) format. The SCI receiver and transmitter are double-buffered, and each has its own separate enable and interrupt bits. Both can be operated independently or simultaneously in the full-duplex mode. To ensure data integrity, the SCI checks received data for break detection, parity, overrun, and framing errors. The bit rate is programmable to over 65000 different speeds through a 16-bit baud-select register.

Features of each SCI module include:

- Two external pins:

  - SCITXD: SCI transmit-output pin

  - SCIRXD: SCI receive-input pin

NOTE: Both pins can be used as GPIO if not used for SCI.

- Baud rate programmable to 64K different rates 2

Baud rate 

LSPCLK (BRR  1) * 8

when BRR  0

- Data-word format – One start bit

 LSPCLK 16

when BRR  0

  - Data-word length programmable from one to eight bits

  - Optional even/odd/no parity bit

  - One or two stop bits

- Four error-detection flags: parity, overrun, framing, and break detection

- Two wake-up multiprocessor modes: idle-line and address bit

- Half- or full-duplex operation

- Double-buffered receive and transmit functions

- Transmitter and receiver operations can be accomplished through interrupt-driven or polled algorithms with status flags.

  - Transmitter: TXRDY flag (transmitter-buffer register is ready to receive another character) and TX EMPTY flag (transmitter-shift register is empty)

  - Receiver: RXRDY flag (receiver-buffer register is ready to receive another character), BRKDT flag (break condition occurred), and RX ERROR flag (monitoring four interrupt conditions)

- Separate enable bits for transmitter and receiver interrupts (except BRKDT)

- Max bit rate = 75 MHz/16 = 4.688 x 106 b/s

- NRZ (non-return-to-zero) format

- Ten SCI module control registers located in the control register frame beginning at address 7050h

NOTE: All registers in this module are 8-bit registers that are connected to Peripheral Frame 2. When a register is accessed, the register data is in the lower byte (7–0), and the upper byte (15–8) is read as zeros. Writing to the upper byte has no effect.

Enhanced features:

- Auto baud-detect hardware logic

- 16-level transmit/receive FIFO

2 Serial port performance is limited by I/O buffer switching speed. Internal prescalers must be adjusted such that the peripheral speed is less than the I/O buffer speed limit—20 MHz maximum.

The SCI port operation is configured and controlled by the registers listed in Table 9-10 and Table 9-11.

Table 9-10. SCI-A Registers

| NAME | ADDRESS | SIZE (x16) | DESCRIPTION |
| --- | --- | --- | --- |
| SCICCRA | 0x00 7050 | 1 | SCI-A Communications Control Register |
| SCICTL1A | 0x00 7051 | 1 | SCI-A Control Register 1 |
| SCIHBAUDA | 0x00 7052 | 1 | SCI-A Baud Register, High Bits |
| SCILBAUDA | 0x00 7053 | 1 | SCI-A Baud Register, Low Bits |
| SCICTL2A | 0x00 7054 | 1 | SCI-A Control Register 2 |
| SCIRXSTA | 0x00 7055 | 1 | SCI-A Receive Status Register |
| SCIRXEMUA | 0x00 7056 | 1 | SCI-A Receive Emulation Data Buffer Register |
| SCIRXBUFA | 0x00 7057 | 1 | SCI-A Receive Data Buffer Register |
| SCITXBUFA | 0x00 7059 | 1 | SCI-A Transmit Data Buffer Register |
| SCIFFTXA(1) | 0x00 705A | 1 | SCI-A FIFO Transmit Register |
| SCIFFRXA(1) | 0x00 705B | 1 | SCI-A FIFO Receive Register |
| SCIFFCTA(1) | 0x00 705C | 1 | SCI-A FIFO Control Register |
| SCIPRIA | 0x00 705F | 1 | SCI-A Priority Control Register |

- These registers are new registers for the FIFO mode.

Table 9-11. SCI-B Registers

| NAME(1) | ADDRESS | SIZE (x16) | DESCRIPTION |
| --- | --- | --- | --- |
| SCICCRB | 0x00 7750 | 1 | SCI-B Communications Control Register |
| SCICTL1B | 0x00 7751 | 1 | SCI-B Control Register 1 |
| SCIHBAUDB | 0x00 7752 | 1 | SCI-B Baud Register, High Bits |
| SCILBAUDB | 0x00 7753 | 1 | SCI-B Baud Register, Low Bits |
| SCICTL2B | 0x00 7754 | 1 | SCI-B Control Register 2 |
| SCIRXSTB | 0x00 7755 | 1 | SCI-B Receive Status Register |
| SCIRXEMUB | 0x00 7756 | 1 | SCI-B Receive Emulation Data Buffer Register |
| SCIRXBUFB | 0x00 7757 | 1 | SCI-B Receive Data Buffer Register |
| SCITXBUFB | 0x00 7759 | 1 | SCI-B Transmit Data Buffer Register |
| SCIFFTXB(2) | 0x00 775A | 1 | SCI-B FIFO Transmit Register |
| SCIFFRXB(2) | 0x00 775B | 1 | SCI-B FIFO Receive Register |
| SCIFFCTB(2) | 0x00 775C | 1 | SCI-B FIFO Control Register |
| SCIPRIB | 0x00 775F | 1 | SCI-B Priority Control Register |

- Registers in this table are mapped to peripheral bus 16 space. This space only allows 16-bit accesses. 32-bit accesses produce undefined results.

- These registers are new registers for the FIFO mode.

Figure 9-10 shows the SCI module block diagram.

TX FIFO _0

TX FIFO _1

- - - - -

TX FIFO _15

RX FIFO _15

- - - - -

RX FIFO _1

RX FIFO _0

| FE | OE | PE |
| --- | --- | --- |

SCICTL1.1

SCITXD

Frame Format and Mode

SCITXD

TXENA

8

TXWAKE

SCICTL1.3

8

TXINT

To CPU

1

TX FIFO

Interrupts

SCITXBUF.7-0

TX FIFO Registers

SCIFFENA

SCIHBAUD. 15 - 8	SCIFFTX.14

SCIRXD

LSPCLK

RXWAKE

SCIRXST.1

SCILBAUD. 7 - 0

RXENA

8

SCICTL1.0

Receive-Data Buffer Register SCIRXBUF.7-0

8

SCIRXST.6

BRKDT

SCIRXST.5

RX Interrupt Logic

RXINT

SCIRXST.7  SCIRXST.4 - 2

SCIRXBUF.7-0

RX FIFO Registers

RXFFOVF

SCIFFRX.15

To CPU

RX Error

RX ERR INT ENA

SCICTL1.6

SCI RX Interrupt Select Logic

RX FIFO

Interrupts

SCICTL2.1

RXRDY	RX/BK INT ENA

RXSHF Register

AutoBaud Detect Logic

WUT

TXSHF

Register

SCIRXD

RX Error

Baud Rate LSbyte Register

Baud Rate MSbyte Register

Transmitter-Data Buffer Register

Parity Even/Odd	Enable

SCICCR.6 SCICCR.5

TX EMPTY

SCICTL2.6

TXRDY	TX INT ENA

SCICTL2.7

SCICTL2.0

TX Interrupt Logic SCI TX Interrupt Select Logic

Figure 9-10. Serial Communications Interface (SCI) Module Block Diagram

    - Serial Peripheral Interface (SPI) Module

The F281x devices include the four-pin serial peripheral interface (SPI) module. The SPI is a high-speed, synchronous serial I/O port that allows a serial bit stream of programmed length (one to sixteen bits) to be shifted into and out of the device at a programmable bit-transfer rate. Normally, the SPI is used for communications between the DSP controller and external peripherals or another processor. Typical applications include external I/O or peripheral expansion through devices such as shift registers, display drivers, and ADCs. Multidevice communications are supported by the master/slave operation of the SPI.

The SPI module features include:

- Four external pins:

  - SPISOMI: SPI slave-output/master-input pin

  - SPISIMO: SPI slave-input/master-output pin

  - SPISTE: SPI slave transmit-enable pin

  - SPICLK: SPI serial-clock pin

NOTE: All four pins can be used as GPIO, if the SPI module is not used.

- Two operational modes: master and slave

- Baud rate: 125 different programmable rates

Baud rate 

LSPCLK (SPIBRR  1)

when SPIBRR  0

 LSPCLK 4

when SPIBRR  0, 1, 2, 3

Serial port performance is limited by I/O buffer switching speed. Internal prescalers must be adjusted such that the peripheral speed is less than the I/O buffer speed limit—20 MHz maximum.

- Data word length: one to sixteen data bits

- Four clocking schemes (controlled by clock polarity and clock phase bits) include:

  - Falling edge without phase delay: SPICLK active-high. SPI transmits data on the falling edge of the SPICLK signal and receives data on the rising edge of the SPICLK signal.

  - Falling edge with phase delay: SPICLK active-high. SPI transmits data one half-cycle ahead of the falling edge of the SPICLK signal and receives data on the falling edge of the SPICLK signal.

  - Rising edge without phase delay: SPICLK inactive-low. SPI transmits data on the rising edge of the SPICLK signal and receives data on the falling edge of the SPICLK signal.

  - Rising edge with phase delay: SPICLK inactive-low. SPI transmits data one half-cycle ahead of the rising edge of the SPICLK signal and receives data on the rising edge of the SPICLK signal.

- Simultaneous receive and transmit operation (transmit function can be disabled in software)

- Transmitter and receiver operations are accomplished through either interrupt-driven or polled algorithms.

- Nine SPI module control registers: Located in control register frame beginning at address 7040h.

NOTE: All registers in this module are 16-bit registers that are connected to Peripheral Frame 2. When a register is accessed, the register data is in the lower byte (7–0), and the upper byte (15–8) is read as zeros. Writing to the upper byte has no effect.

Enhanced features:

- 16-level transmit/receive FIFO

- Delayed transmit control

The SPI port operation is configured and controlled by the registers listed in Table 9-12.

Table 9-12. SPI Registers

| NAME(1) | ADDRESS | SIZE (x16) | DESCRIPTION |
| --- | --- | --- | --- |
| SPICCR | 0x00 7040 | 1 | SPI Configuration Control Register |
| SPICTL | 0x00 7041 | 1 | SPI Operation Control Register |
| SPISTS | 0x00 7042 | 1 | SPI Status Register |
| SPIBRR | 0x00 7044 | 1 | SPI Baud Rate Register |
| SPIRXEMU | 0x00 7046 | 1 | SPI Receive Emulation Buffer Register |
| SPIRXBUF | 0x00 7047 | 1 | SPI Serial Input Buffer Register |
| SPITXBUF | 0x00 7048 | 1 | SPI Serial Output Buffer Register |
| SPIDAT | 0x00 7049 | 1 | SPI Serial Data Register |
| SPIFFTX | 0x00 704A | 1 | SPI FIFO Transmit Register |
| SPIFFRX | 0x00 704B | 1 | SPI FIFO Receive Register |
| SPIFFCT | 0x00 704C | 1 | SPI FIFO Control Register |
| SPIPRI | 0x00 704F | 1 | SPI Priority Control Register |

(1)	These registers are mapped to Peripheral Frame 2. This space only allows 16-bit accesses. 32-bit accesses produce undefined results.

Figure 9-11 is a block diagram of the SPI in slave mode.

SPIFFENA SPIFFTX.14

RX FIFO Registers

RX FIFO _0

RX FIFO _1

- - - - -

RX FIFO _15

SPIRXBUF

Receiver Overrun Flag

RX FIFO

Interrupt

SPISTS.7

Overrun INT ENA

SPICTL.4

16

SPIRXBUF Buffer Register

TX FIFO Registers

SPITXBUF

TX FIFO _15

- - - - -

TX FIFO _0

TX FIFO _1

16

16

TX FIFO

Interrupt

SPIFFOVF FLAG

SPIFFRX.15

SPI INT FLAG

SPISTS.6

SPI INT ENA

SPICTL.0

SPIINT/SPIRXINT

RX Interrupt Logic

To CPU

TX Interrupt Logic

SPITXINT

SPITXBUF Buffer Register

16	M	M

SPIDAT Data Register

SPIDAT.15 - 0

Talk

SPICTL.1

State Control

S	SW1

M

S	SW2

S

M S

Master/Slave

SPISIMO

SPISOMI

(A)

SPISTE

SPI Char

SPICCR.3 - 0	S

SPICTL.2

3  2  1  0

SW3

Clock

Clock

LSPCLK

SPI Bit Rate

SPIBRR.6 - 0

M	S	Polarity

SPICCR.6

M

Phase

SPICTL.3

SPICLK

6  5  4  3  2  1  0

A. SPISTE is driven low by the master for a slave device.

Figure 9-11. Serial Peripheral Interface Module Block Diagram (Slave Mode)

    - GPIO MUX

The GPIO Mux registers are used to select the operation of shared pins on the F281x devices. The pins can be individually selected to operate as “Digital I/O” or connected to “Peripheral I/O” signals (via the GPxMUX registers). If selected for “Digital I/O”mode, registers are provided to configure the pin direction (via the GPxDIR registers) and to qualify the input signal to remove unwanted noise (via the GPxQUAL) registers). Table 9-13 lists the GPIO Mux Registers.

Table 9-13. GPIO Mux Registers

| NAME(1) (2) (3) | ADDRESS | SIZE (x16) | DESCRIPTION |
| --- | --- | --- | --- |
| GPAMUX | 0x00 70C0 | 1 | GPIO A Mux Control Register |
| GPADIR | 0x00 70C1 | 1 | GPIO A Direction Control Register |
| GPAQUAL | 0x00 70C2 | 1 | GPIO A Input Qualification Control Register |
| Reserved | 0x00 70C3 | 1 |  |
| GPBMUX | 0x00 70C4 | 1 | GPIO B Mux Control Register |
| GPBDIR | 0x00 70C5 | 1 | GPIO B Direction Control Register |
| GPBQUAL | 0x00 70C6 | 1 | GPIO B Input Qualification Control Register |
| Reserved | 0x00 70C7 | 1 |  |
| Reserved | 0x00 70C8 | 1 |  |
| Reserved | 0x00 70C9 | 1 |  |
| Reserved | 0x00 70CA | 1 |  |
| Reserved | 0x00 70CB | 1 |  |
| GPDMUX | 0x00 70CC | 1 | GPIO D Mux Control Register |
| GPDDIR | 0x00 70CD | 1 | GPIO D Direction Control Register |
| GPDQUAL | 0x00 70CE | 1 | GPIO D Input Qualification Control Register |
| Reserved | 0x00 70CF | 1 |  |
| GPEMUX | 0x00 70D0 | 1 | GPIO E Mux Control Register |
| GPEDIR | 0x00 70D1 | 1 | GPIO E Direction Control Register |
| GPEQUAL | 0x00 70D2 | 1 | GPIO E Input Qualification Control Register |
| Reserved | 0x00 70D3 | 1 |  |
| GPFMUX | 0x00 70D4 | 1 | GPIO F Mux Control Register |
| GPFDIR | 0x00 70D5 | 1 | GPIO F Direction Control Register |
| Reserved | 0x00 70D6 | 1 |  |
| Reserved | 0x00 70D7 | 1 |  |
| GPGMUX | 0x00 70D8 | 1 | GPIO G Mux Control Register |
| GPGDIR | 0x00 70D9 | 1 | GPIO G Direction Control Register |
| Reserved | 0x00 70DA | 1 |  |
| Reserved | 0x00 70DB | 1 |  |
| Reserved | 0x00 70DC – 0x00 70DF | 4 |  |

- Reserved locations return undefined values and writes are ignored.

- Not all inputs support input signal qualification.

- These registers are EALLOW protected. This prevents spurious writes from overwriting the contents and corrupting the system.

If configured for ”Digital I/O” mode, additional registers are provided for setting individual I/O signals (via the GPxSET registers), for clearing individual I/O signals (via the GPxCLEAR registers), for toggling individual I/O signals (via the GPxTOGGLE registers), or for reading/writing to the individual I/O signals (via the GPxDAT registers). Table 9-14 lists the GPIO Data Registers. For more information, see the TMS320x281x DSP System Control and Interrupts Reference Guide.

Table 9-14. GPIO Data Registers

| NAME(1) (2) | ADDRESS | SIZE (x16) | DESCRIPTION |
| --- | --- | --- | --- |
| GPADAT | 0x00 70E0 | 1 | GPIO A Data Register |
| GPASET | 0x00 70E1 | 1 | GPIO A Set Register |
| GPACLEAR | 0x00 70E2 | 1 | GPIO A Clear Register |
| GPATOGGLE | 0x00 70E3 | 1 | GPIO A Toggle Register |
| GPBDAT | 0x00 70E4 | 1 | GPIO B Data Register |
| GPBSET | 0x00 70E5 | 1 | GPIO B Set Register |
| GPBCLEAR | 0x00 70E6 | 1 | GPIO B Clear Register |
| GPBTOGGLE | 0x00 70E7 | 1 | GPIO B Toggle Register |
| Reserved | 0x00 70E8 | 1 |  |
| Reserved | 0x00 70E9 | 1 |  |
| Reserved | 0x00 70EA | 1 |  |
| Reserved | 0x00 70EB | 1 |  |
| GPDDAT | 0x00 70EC | 1 | GPIO D Data Register |
| GPDSET | 0x00 70ED | 1 | GPIO D Set Register |
| GPDCLEAR | 0x00 70EE | 1 | GPIO D Clear Register |
| GPDTOGGLE | 0x00 70EF | 1 | GPIO D Toggle Register |
| GPEDAT | 0x00 70F0 | 1 | GPIO E Data Register |
| GPESET | 0x00 70F1 | 1 | GPIO E Set Register |
| GPECLEAR | 0x00 70F2 | 1 | GPIO E Clear Register |
| GPETOGGLE | 0x00 70F3 | 1 | GPIO E Toggle Register |
| GPFDAT | 0x00 70F4 | 1 | GPIO F Data Register |
| GPFSET | 0x00 70F5 | 1 | GPIO F Set Register |
| GPFCLEAR | 0x00 70F6 | 1 | GPIO F Clear Register |
| GPFTOGGLE | 0x00 70F7 | 1 | GPIO F Toggle Register |
| GPGDAT | 0x00 70F8 | 1 | GPIO G Data Register |
| GPGSET | 0x00 70F9 | 1 | GPIO G Set Register |
| GPGCLEAR | 0x00 70FA | 1 | GPIO G Clear Register |
| GPGTOGGLE | 0x00 70FB | 1 | GPIO G Toggle Register |
| Reserved | 0x00 70FC – 0x00 70FF | 4 |  |

- Reserved locations will return undefined values and writes will be ignored.

- These registers are NOT EALLOW protected. The above registers will typically be accessed regularly by the user.

Figure 9-12 shows how the various register bits select the various modes of operation for GPIO function.

Digital I/O

Peripheral I/O

0

1

0

1

MUX

MUX

SYSCLKOUT

High-Impedance

Enable (1)

XRS

Internal (Pullup or Pulldown)

PIN

Input Qualification

High- Impedance Control

GPxDIR

Register Bit

GPxMUX

Register Bit

GPxQUAL

Register

GPxDAT/SET/CLEAR/TOGGLE

Register Bit(s)

- In the GPIO mode, when the GPIO pin is configured for output operation, reading the GPxDAT data register only gives the value written, not the value at the pin. In the peripheral mode, the state of the pin can be read through the GPxDAT register, provided the corresponding direction bit is zero (input mode).

- Some selected input signals are qualified by the SYSCLKOUT. The GPxQUAL register specifies the qualification sampling period. The sampling window is 6 samples wide and the output is only changed when all samples are the same (all 0's or all 1's). This feature removes unwanted spikes from the input signal.

Figure 9-12. GPIO/Peripheral Pin Multiplexing

Note

The input function of the GPIO pin and the input path to the peripheral are always enabled. It is the output function of the GPIO pin that is multiplexed with the output path of the primary (peripheral) function. Since the output buffer of a pin connects back to the input buffer, any GPIO signal present at the pin will be propagated to the peripheral module as well. Therefore, when a pin is configured for GPIO operation, the corresponding peripheral functionality (and interrupt-generating capability) must be disabled. Otherwise, interrupts may be inadvertently triggered. This is especially critical when the PDPINTA and PDPINTB pins are used as GPIO pins, since a value of zero for GPDDAT.0 or GPDDAT.5 ( PDPINTx) will put PWM pins in a high-impedance state. The CxTRIP and TxCTRIP pins will also put the corresponding PWM pins in high impedance, if they are driven low (as GPIO pins) and bit EXTCONx.0 = 1.

###### Memory Maps

Block Start Address

0x00 0000

0x00 0040

Low 64K

(24x/240x Equivalent Data Space)

0x00 0400

0x00 0800

0x00 0D00

0x00 0E00

0x00 2000

0x00 6000

0x00 7000

0x00 8000

0x00 9000

0x00 A000

0x3D 7800

0x3D 7C00

0x3D 8000

High 64K (24x/240x Equivalent

Program Space)

0x3F 7FF8

0x3F 8000

0x3F A000

0x3F F000

0x3F FFC0

LEGEND:

On-Chip Memory	External Memory XINTF

| Data Space | Prog Space | Data Space | Prog Space |
| --- | --- | --- | --- |
| M0 Vector - RAM (32 x 32) (Enabled if VMAP = 0) |  | Reserved |  |
| M0 SARAM (1K x 16) |  |  |  |
| M1 SARAM (1K x 16) |  |  |  |
| Peripheral Frame 0 | Reserved |  |  |
| PIE Vector - RAM (256 x 16) (Enabled if VMAP = 1, ENPIE = 1) |  |  |  |
| Reserved |  |  |  |
| Reserved |  | XINTF Zone 0 (8K x 16, XZCS0AND1) |  |
|  |  | XINTF Zone 1 (8K x 16, XZCS0AND1) (Protected) |  |
| Peripheral Frame 1 (Protected) | Reserved | Reserved |  |
| Peripheral Frame 2 (Protected) |  |  |  |
| L0 SARAM (4K x 16, Secure Block) |  |  |  |
| L1 SARAM (4K x 16, Secure Block) |  |  |  |
| Reserved |  |  |  |
|  |  | XINTF Zone 2 (0.5M x 16, XZCS2) |  |
|  |  | XINTF Zone 6 (0.5M x 16, XZCS6AND7) |  |
|  |  | Reserved |  |
| OTP (1K x 16, Secure Block) |  |  |  |
| Reserved (1K) |  |  |  |
| Flash (128K x 16, Secure Block) |  |  |  |
| 128-Bit Password |  |  |  |
| H0 SARAM (8K x 16) |  |  |  |
| Reserved |  |  |  |
|  |  | XINTF Zone 7 (16K x 16, XZCS6AND7) (Enabled if MP/MC = 1) |  |
| Boot ROM (4K x 16) (Enabled if MP/MC = 0) |  |  |  |
| BROM Vector - ROM (32 x 32) (Enabled if VMAP = 1, MP/MC = 0, ENPIE = 0) |  | XINTF Vector - RAM (32 x 32) (Enabled if VMAP = 1, MP/MC = 1, ENPIE = 0) |  |

0x00 2000

0x00 4000

0x08 0000

0x10 0000

0x18 0000

0x3F C000

Only one of these vector maps - M0 vector, PIE vector, BROM vector, XINTF vector - should be enabled at a time.

- Memory blocks are not to scale.

- Reserved locations are reserved for future expansion. Application should not access these areas.

- Boot ROM and Zone 7 memory maps are active either in on-chip or XINTF zone depending on MP/ MC, not in both.

- Peripheral Frame 0, Peripheral Frame 1, and Peripheral Frame 2 memory maps are restricted to data memory only. User program cannot access these memory maps in program space.

- “Protected” means the order of Write followed by Read operations is preserved rather than the pipeline order.

- Certain memory ranges are EALLOW protected against spurious writes after configuration.

- Zones 0 and 1 and Zones 6 and 7 share the same chip select; hence, these memory blocks have mirrored locations.

Figure 9-13. F2812 Memory Map

LEGEND:

Block Start Address

0x00 0000

0x00 0040

Low 64K

(24x/240x Equivalent Data Space)

0x00 0400

0x00 0800

0x00 0D00

0x00 0E00

0x00 2000

0x00 6000

0x00 7000

0x00 8000

0x00 9000

0x00 A000

0x3D 7800

0x3D 7C00

0x3D 8000

High 64K (24x/240x Equivalent

Program Space)

0x3F 7FF8

0x3F 8000

0x3F A000

0x3F F000

0x3F FFC0

On-Chip Memory

| Data Space | Prog Space |
| --- | --- |
| M0 Vector - RAM (32 x 32) (Enabled if VMAP = 0) |  |
| M0 SARAM (1K x 16) |  |
| M1 SARAM (1K x 16) |  |
| Peripheral Frame 0 | Reserved |
| PIE Vector - RAM (256 x 16) (Enabled if VMAP = 1, ENPIE = 1) |  |
| Reserved |  |
| Reserved |  |
| Peripheral Frame 1 (Protected) | Reserved |
| Peripheral Frame 2 (Protected) |  |
| L0 SARAM (4K x 16, Secure Block) |  |
| L1 SARAM (4K x 16, Secure Block) |  |
| Reserved |  |
| OTP (1K x 16, Secure Block) |  |
| Reserved (1K) |  |
| Flash (128K x 16, Secure Block) |  |
| 128-Bit Password |  |
| H0 SARAM (8K x 16) |  |
| Reserved |  |
| Boot ROM (4K x 16) (Enabled if MP/MC = 0) |  |
| BROM Vector - ROM (32 x 32) (Enabled if VMAP = 1, MP/MC = 0, ENPIE = 0) |  |

Only one of these vector maps - M0 vector, PIE vector, BROM vector - should be enabled at a time.

- Memory blocks are not to scale.

- Reserved locations are reserved for future expansion. Application should not access these areas.

- Peripheral Frame 0, Peripheral Frame 1, and Peripheral Frame 2 memory maps are restricted to data memory only. User program cannot access these memory maps in program space.

- “Protected” means the order of Write followed by Read operations is preserved rather than the pipeline order.

- Certain memory ranges are EALLOW protected against spurious writes after configuration.

Figure 9-14. F2811 Memory Map

LEGEND:

Block Start Address

0x00 0000

0x00 0040

Low 64K

(24x/240x Equivalent Data Space)

0x00 0400

0x00 0800

0x00 0D00

0x00 0E00

0x00 2000

0x00 6000

0x00 7000

0x00 8000

0x00 9000

0x00 A000

0x3D 7800

0x3D 7C00

0x3E 8000

High 64K (24x/240x Equivalent

Program Space)

0x3F 7FF8

0x3F 8000

0x3F A000

0x3F F000

0x3F FFC0

On-Chip Memory

| Data Space | Prog Space |
| --- | --- |
| M0 Vector - RAM (32 x 32) (Enabled if VMAP = 0) |  |
| M0 SARAM (1K x 16) |  |
| M1 SARAM (1K x 16) |  |
| Peripheral Frame 0 | Reserved |
| PIE Vector - RAM (256 x 16) (Enabled if VMAP = 1, ENPIE = 1) |  |
| Reserved |  |
| Reserved |  |
| Peripheral Frame 1 (Protected) | Reserved |
| Peripheral Frame 2 (Protected) |  |
| L0 SARAM (4K x 16, Secure Block) |  |
| L1 SARAM (4K x 16, Secure Block) |  |
| Reserved |  |
| OTP (1K x 16, Secure Block) |  |
| Reserved |  |
| Flash (64K x 16, Secure Block) |  |
| 128-Bit Password |  |
| H0 SARAM (8K x 16) |  |
| Reserved |  |
| Boot ROM (4K x 16) (Enabled if MP/MC = 0) |  |
| BROM Vector - ROM (32 x 32) (Enabled if VMAP = 1, MP/MC = 0, ENPIE = 0) |  |

Only one of these vector maps - M0 vector, PIE vector, BROM vector - should be enabled at a time.

- Memory blocks are not to scale.

- Reserved locations are reserved for future expansion. Application should not access these areas.

- Peripheral Frame 0, Peripheral Frame 1, and Peripheral Frame 2 memory maps are restricted to data memory only. User program cannot access these memory maps in program space.

- “Protected” means the order of Write followed by Read operations is preserved rather than the pipeline order.

- Certain memory ranges are EALLOW protected against spurious writes after configuration.

Figure 9-15. F2810 Memory Map

Table 9-15. Addresses of Flash Sectors in F2812 and F2811

| ADDRESS RANGE | PROGRAM AND DATA SPACE |
| --- | --- |
| 0x3D 8000 0x3D 9FFF | Sector J, 8K x 16 |
| 0x3D A000 0x3D BFFF | Sector I, 8K x 16 |
| 0x3D C000 0x3D FFFF | Sector H, 16K x 16 |
| 0x3E 0000 0x3E 3FFF | Sector G, 16K x 16 |
| 0x3E 4000 0x3E 7FFF | Sector F, 16K x 16 |
| 0x3E 8000 0x3E BFFF | Sector E, 16K x 16 |
| 0x3E C000 0x3E FFFF | Sector D, 16K x 16 |
| 0x3F 0000 0x3F 3FFF | Sector C, 16K x 16 |
| 0x3F 4000 0x3F 5FFF | Sector B, 8K x 16 |
| 0x3F 6000 0x3F 7F80 0x3F 7FF5 0x3F 7FF6 0x3F 7FF7 0x3F 7FF8 0x3F 7FFF | Sector A, 8K x 16 Program to 0x0000 when using the Code Security Module Boot-to-Flash Entry Point (program branch instruction here) Security Password (128-Bit) (Do not program to all zeros) |

Table 9-16. Addresses of Flash Sectors in F2810

| ADDRESS RANGE | PROGRAM AND DATA SPACE |
| --- | --- |
| 0x3E 8000 0x3E BFFF | Sector E, 16K x 16 |
| 0x3E C000 0x3E FFFF | Sector D, 16K x 16 |
| 0x3F 0000 0x3F 3FFF | Sector C, 16K x 16 |
| 0x3F 4000 0x3F 5FFF | Sector B, 8K x 16 |
| 0x3F 6000 0x3F 7F80 0x3F 7FF5 0x3F 7FF6 0x3F 7FF7 0x3F 7FF8 0x3F 7FFF | Sector A, 8K x 16 Program to 0x0000 when using the Code Security Module Boot-to-Flash Entry Point (program branch instruction here) Security Password (128-Bit) (Do not program to all zeros) |

The “Low 64K” of the memory address range maps into the data space of the 240x. The “High 64K” of the memory address range maps into the program space of the 24x/240x. 24x/240x-compatible code will execute only from the “High 64K”memory area. Hence, the top 32K of Flash and H0 SARAM block can be used to run 24x/240x-compatible code (if MP/ MC mode is low) or, on the F2812, code can be executed from XINTF Zone 7 (if MP/ MC mode is high).

The XINTF consists of five independent zones. One zone has its own chip select and the remaining four zones share two chip selects. Each zone can be programmed with its own timing (wait states) and to either sample or ignore external ready signal. This makes interfacing to external peripherals easy and glueless.

Note

The chip selects of XINTF Zone 0 and Zone 1 are merged into a single chip select ( XZCS0AND1); and the chip selects of XINTF Zone 6 and Zone 7 are merged into a single chip select ( XZCS6AND7). See Section 9.6, External Interface, XINTF (F2812 only), for details.

Peripheral Frame 1, Peripheral Frame 2, and XINTF Zone 1 are grouped together to enable these blocks to be “write/read peripheral block protected”. The “protected” mode ensures that all accesses to these blocks happen as written. Because of the C28x pipeline, a write immediately followed by a read, to different memory locations, will appear in reverse order on the memory bus of the CPU. This can cause problems in certain peripheral applications where the user expected the write to occur first (as written). The C28x CPU supports a block protection mode where a region of memory can be protected to make sure that operations occur as written (the penalty is extra cycles that are added to align the operations). This mode is programmable and, by default, it will protect the selected zones.

On the F2812, at reset, XINTF Zone 7 is accessed if the XMP/ MC pin is pulled high. This signal selects microprocessor or microcomputer mode of operation. In microprocessor mode, Zone 7 is mapped to high memory such that the vector table is fetched externally. The Boot ROM is disabled in this mode. In microcomputer mode, Zone 7 is disabled such that the vectors are fetched from Boot ROM. This allows the user to either boot from on-chip memory or from off-chip memory. The state of the XMP/ MC signal on reset is stored in an MP/ MC mode bit in the XINTCNF2 register. The user can change this mode in software and hence control the mapping of Boot ROM and XINTF Zone 7. No other memory blocks are affected by XMP/ MC.

I/O space is not supported on the F2812 XINTF.

The wait states for the various spaces in the memory map area are listed in Table 9-17.

Table 9-17. Wait States

| AREA | WAIT-STATES | COMMENTS |
| --- | --- | --- |
| M0 and M1 SARAMs | 0-wait | Fixed |
| Peripheral Frame 0 | 0-wait | Fixed |
| Peripheral Frame 1 | 0-wait (writes) 2-wait (reads) | Fixed |
| Peripheral Frame 2 | 0-wait (writes) 2-wait (reads) | Fixed |
| L0 and L1 SARAMs | 0-wait | Fixed |
| OTP | Programmable, 1-wait minimum | Programmed via the Flash registers. 1-wait-state operation is possible at a reduced CPU frequency. See Section 9.1.6, Flash, for more information. |
| Flash | Programmable, 0-wait minimum | Programmed via the Flash registers. 0-wait-state operation is possible at reduced CPU frequency. The CSM password locations are hardwired for 16 wait states. See Section 9.1.6, Flash, for more information. |
| H0 SARAM | 0-wait | Fixed |
| Boot-ROM | 1-wait | Fixed |
| XINTF | Programmable, 1-wait minimum | Programmed via the XINTF registers. Cycles can be extended by external memory or peripheral. 0-wait operation is not possible. |

###### Register Map

The F281x devices contain three peripheral register spaces. The spaces are categorized as follows:

Peripheral Frame 0:	These are peripherals that are mapped directly to the CPU memory bus.

See Table 9-18.

Peripheral Frame 1:	These are peripherals that are mapped to the 32-bit peripheral bus.

See Table 9-19.

Peripheral Frame 2:	These are peripherals that are mapped to the 16-bit peripheral bus.

See Table 9-20.

Table 9-18. Peripheral Frame 0 Registers

| NAME(1) | ADDRESS RANGE | SIZE (x16) | ACCESS TYPE(2) |
| --- | --- | --- | --- |
| Device Emulation Registers | 0x00 0880 – 0x00 09FF | 384 | EALLOW protected |
| Reserved | 0x00 0A00 – 0x00 0A7F | 128 |  |
| FLASH Registers(3) | 0x00 0A80 – 0x00 0ADF | 96 | EALLOW protected CSM Protected |
| Code Security Module Registers | 0x00 0AE0 – 0x00 0AEF | 16 | EALLOW protected |
| Reserved | 0x00 0AF0 – 0x00 0B1F | 48 |  |
| XINTF Registers | 0x00 0B20 – 0x00 0B3F | 32 | Not EALLOW protected |
| Reserved | 0x00 0B40 – 0x00 0BFF | 192 |  |
| CPU-TIMER0/1/2 Registers | 0x00 0C00 – 0x00 0C3F | 64 | Not EALLOW protected |
| Reserved | 0x00 0C40 – 0x00 0CDF | 160 |  |
| PIE Registers | 0x00 0CE0 – 0x00 0CFF | 32 | Not EALLOW protected |
| PIE Vector Table | 0x00 0D00 – 0x00 0DFF | 256 | EALLOW protected |
| Reserved | 0x00 0E00 – 0x00 0FFF | 512 |  |

- Registers in Frame 0 support 16-bit and 32-bit accesses.

- If registers are EALLOW protected, then writes cannot be performed until the user executes the EALLOW instruction. The EDIS instruction disables writes. This prevents stray code or pointers from corrupting register contents.

- The Flash Registers are also protected by the Code Security Module (CSM).

Table 9-19. Peripheral Frame 1 Registers

| NAME(1) | ADDRESS RANGE | SIZE (x16) | ACCESS TYPE |
| --- | --- | --- | --- |
| eCAN Registers | 0x00 6000 – 0x00 60FF | 256 (128 x 32) | Some eCAN control registers (and selected bits in other eCAN control registers) are EALLOW-protected. |
| eCAN Mailbox RAM | 0x00 6100 – 0x00 61FF | 256 (128 x 32) | Not EALLOW-protected |
| Reserved | 0x00 6200 – 0x00 6FFF | 3584 |  |

- The eCAN control registers only support 32-bit read/write operations. All 32-bit accesses are aligned to even address boundaries.

Table 9-20. Peripheral Frame 2 Registers

| NAME(1) | ADDRESS RANGE | SIZE (x16) | ACCESS TYPE |
| --- | --- | --- | --- |
| Reserved | 0x00 7000 – 0x00 700F | 16 |  |
| System Control Registers | 0x00 7010 – 0x00 702F | 32 | EALLOW Protected |
| Reserved | 0x00 7030 – 0x00 703F | 16 |  |
| SPI-A Registers | 0x00 7040 – 0x00 704F | 16 | Not EALLOW Protected |
| SCI-A Registers | 0x00 7050 – 0x00 705F | 16 | Not EALLOW Protected |
| Reserved | 0x00 7060 – 0x00 706F | 16 |  |
| External Interrupt Registers | 0x00 7070 – 0x00 707F | 16 | Not EALLOW Protected |
| Reserved | 0x00 7080 – 0x00 70BF | 64 |  |
| GPIO Mux Registers | 0x00 70C0 – 0x00 70DF | 32 | EALLOW Protected |
| GPIO Data Registers | 0x00 70E0 – 0x00 70FF | 32 | Not EALLOW Protected |
| ADC Registers | 0x00 7100 – 0x00 711F | 32 | Not EALLOW Protected |
| Reserved | 0x00 7120 – 0x00 73FF | 736 |  |
| EV-A Registers | 0x00 7400 – 0x00 743F | 64 | Not EALLOW Protected |
| Reserved | 0x00 7440 – 0x00 74FF | 192 |  |
| EV-B Registers | 0x00 7500 – 0x00 753F | 64 | Not EALLOW Protected |
| Reserved | 0x00 7540 – 0x00 774F | 528 |  |
| SCI-B Registers | 0x00 7750 – 0x00 775F | 16 | Not EALLOW Protected |
| Reserved | 0x00 7760 – 0x00 77FF | 160 |  |
| McBSP Registers | 0x00 7800 – 0x00 783F | 64 | Not EALLOW Protected |
| Reserved | 0x00 7840 – 0x00 7FFF | 1984 |  |

- Peripheral Frame 2 only allows 16-bit accesses. All 32-bit accesses are ignored (invalid data may be returned or written).

###### Device Emulation Registers

These registers are used to control the protection mode of the C28x CPU and to monitor some critical device signals. The registers are defined in Table 9-21.

Table 9-21. Device Emulation Registers

| NAME | ADDRESS RANGE | SIZE (x16) | DESCRIPTION |
| --- | --- | --- | --- |
| DEVICECNF | 0x00 0880 – 0x00 0881 | 2 | Device Configuration Register |
| PARTID | 0x00 0882 | 1 | Part ID Register 0x0001 or 0x0002 – F281x |
| REVID | 0x00 0883 | 1 | 0x0001 – Silicon Revision A 0x0002 – Silicon Revision B 0x0003 – Silicon Revisions C, D Revision ID Register 0x0004 – Reserved 0x0005 – Silicon Revision E 0x0006 – Silicon Revision F 0x0007 – Silicon Revision G |
| PROTSTART | 0x00 0884 | 1 | Block Protection Start Address Register |
| PROTRANGE | 0x00 0885 | 1 | Block Protection Range Address Register |
| Reserved | 0x00 0886 – 0x00 09FF | 378 |  |

###### External Interface, XINTF (F2812 Only)

This section gives a top-level view of the external interface (XINTF) that is implemented on the F2812 device.

The external interface is a non-multiplexed asynchronous bus, similar to the C240x external interface. The external interface on the F2812 is mapped into five fixed zones shown in Figure 9-16.

Figure 9-16 shows the F2812 XINTF signals.

Data Space	Prog Space

XD[15:0]

XA[18:0]

XINTF Zone 0

(8K x 16)

XINTF Zone 1

(8K x 16)

XZCS0

XZCS1

XZCS0AND1

XINTF Zone 2

(512K x 16)

XZCS2

XINTF Zone 6

(512K x 16)

XZCS6

XZCS6AND7

XINTF Zone 7

(16K x 16)

(mapped here if MP/MC = 1)

XZCS7

XWE

XRD XR/W XREADY XMP/MC XHOLD

XHOLDA

(E)

XCLKOUT

0x00 0000

0x00 2000

0x00 4000

0x00 6000

0x08 0000

0x10 0000

0x18 0000

0x3F C000

0x40 0000

- The mapping of XINTF Zone 7 is dependent on the XMP/ MC device input signal and the MP/ MC mode bit (bit 8 of XINTCNF2 register). Zones 0, 1, 2, and 6 are always enabled.

- Each zone can be programmed with different wait states, setup and hold timing, and is supported by zone chip selects ( XZCS0AND1, XZCS2, XZCS6AND7), which toggle when an access to a particular zone is performed. These features enable glueless connection to many external memories and peripherals.

- The chip selects for Zone 0 and Zone 1 are ANDed internally together to form one chip select ( XZCS0AND1). Any external memory that is connected to XZCS0AND1 is dually mapped to both Zone 0 and Zone 1.

- The chip selects for Zone 6 and Zone 7 are ANDed internally together to form one chip select ( XZCS6AND7). Any external memory that is connected to XZCS6AND7 is dually mapped to both Zone 6 and Zone 7. This means that if Zone 7 is disabled (via the MP/ MC mode), then any external memory is still accessible via Zone 6 address space.

- XCLKOUT is also pinned out on the F2810 and F2811 devices.

Figure 9-16. External Interface Block Diagram

The operation and timing of the external interface, can be controlled by the registers listed in Table 9-22.

Table 9-22. XINTF Configuration and Control Register Mappings

| NAME | ADDRESS | SIZE (x16) | DESCRIPTION |
| --- | --- | --- | --- |
| XTIMING0 | 0x00 0B20 | 2 | XINTF Timing Register, Zone 0 can access as two 16-bit registers or one 32-bit register. |
| XTIMING1 | 0x00 0B22 | 2 | XINTF Timing Register, Zone 1 can access as two 16-bit registers or one 32-bit register. |
| XTIMING2 | 0x00 0B24 | 2 | XINTF Timing Register, Zone 2 can access as two 16-bit registers or one 32-bit register. |
| XTIMING6 | 0x00 0B2C | 2 | XINTF Timing Register, Zone 6 can access as two 16-bit registers or one 32-bit register. |
| XTIMING7 | 0x00 0B2E | 2 | XINTF Timing Register, Zone 7 can access as two 16-bit registers or one 32-bit register. |
| XINTCNF2 | 0x00 0B34 | 2 | XINTF Configuration Register can access as two 16-bit registers or one 32-bit register. |
| XBANK | 0x00 0B38 | 1 | XINTF Bank Control Register |
| XREVISION | 0x00 0B3A | 1 | XINTF Revision Register |

    - Timing Registers

XINTF signal timing can be tuned to match specific external device requirements such as setup and hold times to strobe signals for contention avoidance and maximizing bus efficiency. The XINTF timing parameters can be configured individually for each zone based on the requirements of the memory or peripheral accessed by that particular zone. This allows the programmer to maximize the efficiency of the bus on a per-zone basis. All XINTF timing values are with respect to XTIMCLK, which is equal to or one-half of the SYSCLKOUT rate, as shown in Figure 8-26.

For detailed information on the XINTF timing and configuration register bit fields, see the TMS320x281x DSP External Interface (XINTF) Reference Guide.

    - XREVISION Register

The XREVISION register contains a unique number to identify the particular version of XINTF used in the product. For the F2812, this register will be configured as described in Table 9-23.

Table 9-23. XREVISION Register Bit Definitions

| BIT(S) | NAME | TYPE | RESET | DESCRIPTION |
| --- | --- | --- | --- | --- |
| 15–0 | REVISION | R | 0x0004 | Current XINTF Revision. For internal use/reference. Test purposes only. Subject to change. |

MUX

(A)

96 Interrupts

Interrupts

WDINT

WAKEINT

LPMINT

XINT1

XINT2

TINT0

TINT2

TINT1

select

enable

XNMI_XINT13

XNMICTR[15:0]

XNMICR[15:0]

Interrupt Control

TIMER 1

TIMER 2

(Reserved for DSP/BIOS)

TIMER 0

XINT2CTR[15:0]

XINT2CR[15:0]

Interrupt Control

XINT1CTR[15:0]

XINT1CR[15:0]

Interrupt Control

Low-Power Modes

Watchdog

PIE

GPIO MUX

Peripherals

(SPI, SCI, McBSP, CAN, EV, ADC)

(41 Interrupts)

INT1

to INT12

C28x CPU

INT14

INT13

NMI

Figure 9-17 shows how the various interrupt sources are multiplexed within the F281x devices.

A. Out of a possible 96 interrupts, 45 are currently used by peripherals.

Figure 9-17. Interrupt Sources

Eight PIE block interrupts are grouped into one CPU interrupt. In total, 12 CPU interrupt groups, with 8 interrupts per group equals 96 possible interrupts. On the F281x, 45 of these are used by peripherals as shown in Table 9-24.

The TRAP #VectorNumber instruction transfers program control to the interrupt service routine corresponding to the vector specified. TRAP #0 attempts to transfer program control to the address pointed to by the reset vector. The PIE vector table does not, however, include a reset vector. Therefore, TRAP #0 should not be used when the PIE is enabled. Doing so will result in undefined behavior.

When the PIE is enabled, TRAP #1 through TRAP #12 will transfer program control to the interrupt service routine corresponding to the first vector within the PIE group. For example: TRAP #1 fetches the vector from INT1.1, TRAP #2 fetches the vector from INT2.1 and so forth.

IFR[12:1]

IER[12:1]

INTM

INT1 INT2

MUX

1

CPU

0

INT11 INT12

(Flag)

(Enable)

Global Enable

INTx

PIEACKx

(Enable/Flag)

MUX

(Enable)		(Flag) PIEIERx[8:1]	PIEIFRx[8:1]

INTx.1 INTx.2 INTx.3 INTx.4 INTx.5

INTx.6 INTx.7 INTx.8

From Peripherals or

External Interrupts

| CPU INTERRUPTS(1) | PIE INTERRUPTS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | INTx.8 | INTx.7 | INTx.6 | INTx.5 | INTx.4 | INTx.3 | INTx.2 | INTx.1 |
| INT1 | WAKEINT (LPM/WD) | TINT0 (TIMER 0) | ADCINT (ADC) | XINT2 | XINT1 | Reserved | PDPINTB (EV-B) | PDPINTA (EV-A) |
| INT2 | Reserved | T1OFINT (EV-A) | T1UFINT (EV-A) | T1CINT (EV-A) | T1PINT (EV-A) | CMP3INT (EV-A) | CMP2INT (EV-A) | CMP1INT (EV-A) |
| INT3 | Reserved | CAPINT3 (EV-A) | CAPINT2 (EV-A) | CAPINT1 (EV-A) | T2OFINT (EV-A) | T2UFINT (EV-A) | T2CINT (EV-A) | T2PINT (EV-A) |
| INT4 | Reserved | T3OFINT (EV-B) | T3UFINT (EV-B) | T3CINT (EV-B) | T3PINT (EV-B) | CMP6INT (EV-B) | CMP5INT (EV-B) | CMP4INT (EV-B) |
| INT5 | Reserved | CAPINT6 (EV-B) | CAPINT5 (EV-B) | CAPINT4 (EV-B) | T4OFINT (EV-B) | T4UFINT (EV-B) | T4CINT (EV-B) | T4PINT (EV-B) |
| INT6 | Reserved | Reserved | MXINT (McBSP) | MRINT (McBSP) | Reserved | Reserved | SPITXINTA (SPI) | SPIRXINTA (SPI) |
| INT7 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |
| INT8 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |
| INT9 | Reserved | Reserved | ECAN1INT (CAN) | ECAN0INT (CAN) | SCITXINTB (SCI-B) | SCIRXINTB (SCI-B) | SCITXINTA (SCI-A) | SCIRXINTA (SCI-A) |
| INT10 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |
| INT11 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |
| INT12 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

Figure 9-18. Multiplexing of Interrupts Using the PIE Block Table 9-24. PIE Peripheral Interrupts

- Out of the 96 possible interrupts, 45 interrupts are currently used. The remaining interrupts are reserved for future devices. These interrupts can be used as software interrupts if they are enabled at the PIEIFRx level, provided none of the interrupts within the group is being used by a peripheral. Otherwise, interrupts coming in from peripherals may be lost by accidentally clearing their flag while modifying the PIEIFR.

To summarize, there are two safe cases when the reserved interrupts could be used as software interrupts:

  - No peripheral within the group is asserting interrupts.

  - No peripheral interrupts are assigned to the group (example PIE group 12).

Table 9-25. PIE Configuration and Control Registers

| NAME(1) | ADDRESS | SIZE (x16) | DESCRIPTION |
| --- | --- | --- | --- |
| PIECTRL | 0x0000 0CE0 | 1 | PIE, Control Register |
| PIEACK | 0x0000 0CE1 | 1 | PIE, Acknowledge Register |
| PIEIER1 | 0x0000 0CE2 | 1 | PIE, INT1 Group Enable Register |
| PIEIFR1 | 0x0000 0CE3 | 1 | PIE, INT1 Group Flag Register |
| PIEIER2 | 0x0000 0CE4 | 1 | PIE, INT2 Group Enable Register |
| PIEIFR2 | 0x0000 0CE5 | 1 | PIE, INT2 Group Flag Register |
| PIEIER3 | 0x0000 0CE6 | 1 | PIE, INT3 Group Enable Register |
| PIEIFR3 | 0x0000 0CE7 | 1 | PIE, INT3 Group Flag Register |
| PIEIER4 | 0x0000 0CE8 | 1 | PIE, INT4 Group Enable Register |
| PIEIFR4 | 0x0000 0CE9 | 1 | PIE, INT4 Group Flag Register |
| PIEIER5 | 0x0000 0CEA | 1 | PIE, INT5 Group Enable Register |
| PIEIFR5 | 0x0000 0CEB | 1 | PIE, INT5 Group Flag Register |
| PIEIER6 | 0x0000 0CEC | 1 | PIE, INT6 Group Enable Register |
| PIEIFR6 | 0x0000 0CED | 1 | PIE, INT6 Group Flag Register |
| PIEIER7 | 0x0000 0CEE | 1 | PIE, INT7 Group Enable Register |
| PIEIFR7 | 0x0000 0CEF | 1 | PIE, INT7 Group Flag Register |
| PIEIER8 | 0x0000 0CF0 | 1 | PIE, INT8 Group Enable Register |
| PIEIFR8 | 0x0000 0CF1 | 1 | PIE, INT8 Group Flag Register |
| PIEIER9 | 0x0000 0CF2 | 1 | PIE, INT9 Group Enable Register |
| PIEIFR9 | 0x0000 0CF3 | 1 | PIE, INT9 Group Flag Register |
| PIEIER10 | 0x0000 0CF4 | 1 | PIE, INT10 Group Enable Register |
| PIEIFR10 | 0x0000 0CF5 | 1 | PIE, INT10 Group Flag Register |
| PIEIER11 | 0x0000 0CF6 | 1 | PIE, INT11 Group Enable Register |
| PIEIFR11 | 0x0000 0CF7 | 1 | PIE, INT11 Group Flag Register |
| PIEIER12 | 0x0000 0CF8 | 1 | PIE, INT12 Group Enable Register |
| PIEIFR12 | 0x0000 0CF9 | 1 | PIE, INT12 Group Flag Register |
| Reserved | 0x0000 0CFA – 0x0000 0CFF | 6 | Reserved |

(1)	The PIE configuration and control registers are not protected by EALLOW mode. The PIE vector table is protected.

    - External Interrupts

Table 9-26. External Interrupts Registers

| NAME | ADDRESS | SIZE (x16) | DESCRIPTION |
| --- | --- | --- | --- |
| XINT1CR | 0x00 7070 | 1 | XINT1 control register |
| XINT2CR | 0x00 7071 | 1 | XINT2 control register |
| Reserved | 0x00 7072 – 0x00 7076 | 5 |  |
| XNMICR | 0x00 7077 | 1 | XNMI control register |
| XINT1CTR | 0x00 7078 | 1 | XINT1 counter register |
| XINT2CTR | 0x00 7079 | 1 | XINT2 counter register |
| Reserved | 0x00 707A – 0x00 707E | 5 |  |
| XNMICTR | 0x00 707F | 1 | XNMI counter register |

Each external interrupt can be enabled/disabled or qualified using positive or negative going edge. For more information, see the TMS320x281x DSP System Control and Interrupts Reference Guide.

Peripheral Bus

System Control

Reset

XRS

SYSCLKOUT

Peripheral Reset

(A)

CLKIN

X1/XCLKIN

C28x CPU

X2

XF_XPLLDIS

System

Control Registers

Clock Enables

Peripheral

Registers

eCAN

I/O

Low-Speed Prescaler

LSPCLK

Peripheral

Registers

Low-Speed Peripherals

SCI-A/B, SPI, McBSP

I/O

GPIO

MUX

GPIOs

High-Speed Prescaler

HSPCLK

Peripheral

Registers

High-Speed Peripherals

EV-A/B

I/O

HSPCLK

ADC

Registers

12-Bit ADC

16 ADC Inputs

Power Modes Control

OSC

PLL

Watchdog Block

This section describes the F281x oscillator, PLL and clocking mechanisms, the watchdog function and the low- power modes. Figure 9-19 shows the various clock and reset domains in the F281x devices that will be discussed.

A. CLKIN is the clock input to the CPU. SYSCLKOUT is the output clock of the CPU. They are of the same frequency.

Figure 9-19. Clock and Reset Domains

The PLL, clocking, watchdog and low-power modes, are controlled by the registers listed in Table 9-27.

Table 9-27. PLL, Clocking, Watchdog, and Low-Power Mode Registers

| NAME(1) | ADDRESS | SIZE (x16) | DESCRIPTION |
| --- | --- | --- | --- |
| Reserved | 0x00 7010 – 0x00 7017 | 8 |  |
| Reserved | 0x00 7018 | 1 |  |
| Reserved | 0x00 7019 | 1 |  |
| HISPCP | 0x00 701A | 1 | High-Speed Peripheral Clock Prescaler Register for HSPCLK clock |
| LOSPCP | 0x00 701B | 1 | Low-Speed Peripheral Clock Prescaler Register for LSPCLK clock |
| PCLKCR | 0x00 701C | 1 | Peripheral Clock Control Register |
| Reserved | 0x00 701D | 1 |  |
| LPMCR0 | 0x00 701E | 1 | Low-Power Mode Control Register 0 |
| LPMCR1 | 0x00 701F | 1 | Low-Power Mode Control Register 1 |
| Reserved | 0x00 7020 | 1 |  |
| PLLCR | 0x00 7021 | 1 | PLL Control Register(2) |
| SCSR | 0x00 7022 | 1 | System Control and Status Register |
| WDCNTR | 0x00 7023 | 1 | Watchdog Counter Register |
| Reserved | 0x00 7024 | 1 |  |
| WDKEY | 0x00 7025 | 1 | Watchdog Reset Key Register |
| Reserved | 0x00 7026 – 0x00 7028 | 3 |  |
| WDCR | 0x00 7029 | 1 | Watchdog Control Register |
| Reserved | 0x00 702A – 0x00 702F | 6 |  |

- All of the above registers can only be accessed by executing the EALLOW instruction.

- The PLL control register (PLLCR) is reset to a known state by the XRS signal only. Emulation reset (through Code Composer Studio) will not reset PLLCR.

###### OSC and PLL Block

Figure 9-20 shows the OSC and PLL block on F281x.

XF_XPLLDIS

XPLLDIS

Latch

XRS

XCLKIN

OSCCLK (PLL Disabled)

0

CLKIN

CPU

/2

1

4-Bit PLL Select

4-Bit PLL Select

PLL Block

PLL

PLL

Bypass

X2

On-Chip

Oscillator (OSC)

X1/XCLKIN

SYSCLKOUT

Figure 9-20. OSC and PLL Block

The on-chip oscillator circuit enables a crystal to be attached to the F281x devices using the X1/XCLKIN and X2 pins. If a crystal is not used, then an external oscillator can be directly connected to the X1/XCLKIN pin and the X2 pin is left unconnected. The logic-high level in this case should not exceed VDD. The PLLCR bits [3:0] set the clocking ratio.

Table 9-28. PLLCR Register Bit Definitions

| BIT(S) | NAME | TYPE | XRS RESET(1) | DESCRIPTION |
| --- | --- | --- | --- | --- |
| 15:4 | Reserved | R = 0 | 0:0 |  |
| 3:0 | DIV | R/W | 0,0,0,0 | SYSCLKOUT = (XCLKIN * n)/2, where n is the PLL multiplication factor. Bit Value n SYSCLKOUT 0000 PLL Bypassed XCLKIN/2 0001 1 XCLKIN/2 0010 2 XCLKIN 0011 3 XCLKIN * 1.5 0100 4 XCLKIN * 2 0101 5 XCLKIN * 2.5 0110 6 XCLKIN * 3 0111 7 XCLKIN * 3.5 1000 8 XCLKIN * 4 1001 9 XCLKIN * 4.5 1010 10 XCLKIN * 5 1011 11 Reserved 1100 12 Reserved 1101 13 Reserved 1110 14 Reserved 1111 15 Reserved |

(1)	The PLLCR register is reset to a known state by the XRS reset line. If a reset is issued by the debugger, the PLL clocking ratio is not changed.

    - Loss of Input Clock

In PLL enabled mode, if the input clock XCLKIN or the oscillator clock is removed or absent, the PLL will still issue a “limp-mode” clock. The limp-mode clock will continue to clock the CPU and peripherals at a typical frequency of 1–4 MHz. The PLLCR register should have been written to with a non-zero value for this feature to work.

Normally, when the input clocks are present, the watchdog counter will decrement to initiate a watchdog reset or WDINT interrupt. However, when the external input clock fails, the watchdog counter will stop decrementing (that is, the watchdog counter does not change with the limp-mode clock). This condition could be used by the application firmware to detect the input clock failure and initiate necessary shut-down procedure for the system.

Note

Applications in which the correct CPU operating frequency is absolutely critical must implement a mechanism by which the DSP will be held in reset, should the input clocks ever fail. For example, an R-C circuit may be used to trigger the XRS pin of the DSP, should the capacitor ever get fully charged. An I/O pin may be used to discharge the capacitor on a periodic basis to prevent it from getting fully charged. Such a circuit would also help in detecting failure of the VDD3VFL rail.

###### PLL-Based Clock Module

The F281x has an on-chip, PLL-based clock module. This module provides all the necessary clocking signals for the device, as well as control for low-power mode entry. The PLL has a 4-bit ratio control to select different CPU clock rates. The watchdog module should be disabled before writing to the PLLCR register. It can be re-enabled (if need be) after the PLL module has stabilized, which takes 131072 XCLKIN cycles.

The PLL-based clock module provides two modes of operation:

- Crystal operation: This mode allows the use of an external crystal/resonator to provide the time base to the device.

- External clock source operation: This mode allows the internal oscillator to be bypassed. The device clocks are generated from an external clock source input on the X1/XCLKIN pin.

X1/XCLKIN

X2

External Clock Signal

X1/XCLKIN

X2

(A)

(A)

CL1	CL2

Crystal

(Toggling 0-VDD)	NC

(a)	(b)

A. TI recommends that customers have the resonator/crystal vendor characterize the operation of their device with the DSP chip. The resonator/crystal vendor has the equipment and expertise to tune the tank circuit. The vendor can also advise the customer regarding the proper tank component values that will ensure start-up and stability over the entire operating range.

| PLL MODE | REMARKS | SYSCLKOUT |
| --- | --- | --- |
| PLL Disabled | Invoked by tying XPLLDIS pin low upon reset. PLL block is completely disabled. Clock input to the CPU (CLKIN) is directly derived from the clock signal present at the X1/XCLKIN pin. | XCLKIN |
| PLL Bypassed | Default PLL configuration upon power-up, if PLL is not disabled. The PLL itself is bypassed. However, the /2 module in the PLL block divides the clock input at the X1/ XCLKIN pin by two before feeding it to the CPU. | XCLKIN/2 |
| PLL Enabled | Achieved by writing a non-zero value “n” into PLLCR register. The /2 module in the PLL block now divides the output of the PLL by two before feeding it to the CPU. | (XCLKIN * n) / 2 |

Figure 9-21. Recommended Crystal/Clock Connection Table 9-29. Possible PLL Configuration Modes

###### External Reference Oscillator Clock Option

The typical specifications for the external quartz crystal for a frequency of 30 MHz are listed below:

- Fundamental mode, parallel resonant

- CL (load capacitance) = 12 pF

- CL1 = CL2 = 24 pF

- Cshunt = 6 pF

- ESR range = 25 to 40 Ω

###### Watchdog Block

The watchdog block on the F281x is identical to the one used on the 240x devices. The watchdog module generates an output pulse, 512 oscillator clocks wide (OSCCLK), whenever the 8-bit watchdog up counter has reached its maximum value. To prevent this, the user disables the counter or the software must periodically write a 0x55 + 0xAA sequence into the watchdog key register which will reset the watchdog counter. Figure 9-22 shows the various functional blocks within the watchdog module.

WDCNTR[7:0]

OSCCLK

/512

Watchdog

Prescaler

WDCLK

8-Bit

Watchdog Counter CLR

Clear Counter

Internal

Pullup

WDKEY[7:0]

Bad Key

Watchdog

55 + AA

Key Detector

Generate

Output Pulse (512 OSCCLKs)

WDRST

WDINT

Good Key

XRS

Core-reset

Bad

WDCHK

Key

(A)

WDRST

1

0

1

WDCR (WDCHK[2:0])

WDCR (WDDIS)

SCSR (WDENINT)

WDCR (WDPS[2:0])

A. The WDRST signal is driven low for 512 OSCCLK cycles.

Figure 9-22. Watchdog Module

The WDINT signal enables the watchdog to be used as a wakeup from IDLE/STANDBY mode timer.

In STANDBY mode, all peripherals are turned off on the device. The only peripheral that remains functional is the watchdog. The WATCHDOG module will run off the PLL clock or the oscillator clock. The WDINT signal is fed to the LPM block so that it can wake the device from STANDBY (if enabled). See Section 9.13, Low-Power Modes Block, for more details.

In IDLE mode, the WDINT signal can generate an interrupt to the CPU, via the PIE, to take the CPU out of IDLE mode.

In HALT mode, this feature cannot be used because the oscillator (and PLL) are turned off and hence, so is the WATCHDOG.

###### Low-Power Modes Block

The low-power modes on F281x are similar to the 240x devices. Table 9-30 summarizes the various modes.

Table 9-30. Low-Power Modes

| MODE | LPM[1:0] | OSCCLK | CLKIN | SYSCLKOUT | EXIT(1) |
| --- | --- | --- | --- | --- | --- |
| Normal | X,X | on | on | on | – |
| IDLE | 0,0 | on | on | on(2) | XRS, WDINT, Any Enabled Interrupt, XNMI, Debugger(3) |
| STANDBY | 0,1 | on (watchdog still running) | off | off | XRS, WDINT, XINT1, XNMI, T1/2/3/4CTRIP, C1/2/3/4/5/6TRIP, SCIRXDA, SCIRXDB, CANRX, Debugger(3) |
| HALT | 1,X | off (oscillator and PLL turned off, watchdog not functional) | off | off | XRS, XNMI, Debugger(3) |

- The Exit column lists which signals or under what conditions the low-power mode will be exited. A low signal, on any of the signals, will exit the low-power condition. This signal must be kept low long enough for an interrupt to be recognized by the device. Otherwise, the IDLE mode will not be exited and the device will go back into the indicated low-power mode.

- The IDLE mode on the C28x behaves differently than on the 24x/240x. On the C28x, the clock output from the core (SYSCLKOUT) is still functional; while on the 24x/240x, the clock is turned off.

- On the C28x, the JTAG port can still function even if the core clock (CLKIN) is turned off.

The various low-power modes operate as follows:

IDLE Mode	This mode is exited by any enabled interrupt or an XNMI that is recognized by the processor. The LPM block performs no tasks during this mode as long as the LPMCR0(LPM) bits are set to 0,0.

STANDBY Mode	All other signals (including XNMI) will wake the device from STANDBY mode if selected by the LPMCR1 register. The user will need to select which signal(s) will wake the device. The selected signal(s) are also qualified by the OSCCLK before waking the device. The number of OSCCLKs is specified in the LPMCR0 register.

HALT Mode	Only the XRS and XNMI external signals can wake the device from HALT mode. The XNMI input to the core has an enable/disable bit. Hence, it is safe to use the XNMI signal for this function.

Note

The low-power modes do not affect the state of the output pins (PWM pins included). They will be in whatever state the code left them when the IDLE instruction was executed.

#### Applications, Implementation, and Layout

Note

Information in the following applications sections is not part of the TI component specification, and TI does not warrant its accuracy or completeness. TI’s customers are responsible for determining suitability of components for their purposes. Customers should validate and test their design implementation to confirm system functionality.

###### TI Reference Design

The TI Reference Design Library is a robust reference design library spanning analog, embedded processor, and connectivity. Created by TI experts to help you jump start your system design, all reference designs include schematic or block diagrams, BOMs, and design files to speed your time to market. Search and download designs at Select TI reference designs.

#### Device and Documentation Support

###### Getting Started

This section gives a brief overview of the steps to take when first developing for a C28x device. For more detail on each of these steps, see the following:

- C2000 real-time control MCUs – Tools & software

###### Device and Development Support Tool Nomenclature

To designate the stages in the product development cycle, TI assigns prefixes to the part numbers of all TMS320™ DSP devices and support tools. Each TMS320 DSP commercial family member has one of three prefixes: TMX, TMP, or TMS (for example, TMS320F2810). Texas Instruments recommends two of three possible prefix designators for its support tools: TMDX and TMDS. These prefixes represent evolutionary stages of product development from engineering prototypes (TMX/TMDX) through fully qualified production devices/ tools (TMS/TMDS).

TMX	Experimental device that is not necessarily representative of the final device’s electrical specifications

TMP	Final silicon die that conforms to the device’s electrical specifications but has not completed quality and reliability verification

TMS	Fully qualified production device

Support tool development evolutionary flow:

TMDX	Development-support product that has not yet completed Texas Instruments internal qualification testing

TMDS	Fully qualified development-support product

TMX and TMP devices and TMDX development-support tools are shipped against the following disclaimer: "Developmental product is intended for internal evaluation purposes."

TMS devices and TMDS development-support tools have been characterized fully, and the quality and reliability of the device have been demonstrated fully. TI’s standard warranty applies.

Predictions show that prototype devices (TMX or TMP) have a greater failure rate than the standard production devices. Texas Instruments recommends that these devices not be used in any production system because their expected end-use failure rate still is undefined. Only qualified production devices are to be used.

TI device nomenclature also includes a suffix with the device family name. This suffix indicates the package type (for example, PBK) and temperature range (for example, A). Figure 11-1 provides a legend for reading the complete device name for any TMS320F281x family member.

For device part numbers and further ordering information, see the Package Option Addendum of this document, the TI website (www.ti.com), or contact your TI sales representative.

For additional description of the device nomenclature markings on the die, see the TMS320F281x DSPs Silicon Errata.

Figure 11-1. TMS320F281x Device Nomenclature

###### Tools and Software

TI offers an extensive line of development tools. Some of the tools and software to evaluate the performance of the device, generate code, and develop solutions are listed below. To view all available tools and software, visit the Tools & software page for each device.

Development Tools

TMS320F2812 eZdsp Start Kit (DSK)

The F2812 eZdsp™ is a standalone module that lets evaluators examine certain characteristics of the F2812 digital signal processor (DSP) to determine if this DSP meets their application requirements. This module has a single chip parallel port to JTAG scan controller. Therefore the module can be operated without additional development tools such as an emulator. Furthermore, the module is an excellent platform to develop, demonstrate, and run software for the F2812 processor.

Uniflash Standalone Flash Tool

CCS Uniflash is a standalone tool used to program on-chip flash memory on TI MCUs.

Software Tools

controlSUITE™ Software Suite: Software and Development Tools for C2000™ Microcontrollers

controlSUITE™ for C2000™ microcontrollers is a cohesive set of software infrastructure, tools, and documentation designed to minimize system development time. From device-specific drivers and support software to complete examples in sophisticated system applications, controlSUITE provides the needed resources at every stage of development and evaluation.

Code Composer Studio™ (CCS) Integrated Development Environment (IDE) for C2000 Microcontrollers

Code Composer Studio is an integrated development environment (IDE) that supports TI's Microcontroller and Embedded Processors portfolio. Code Composer Studio comprises a suite of tools used to develop and debug embedded applications. It includes an optimizing C/C++ compiler, source code editor, project build environment, debugger, profiler, and many other features. The intuitive IDE provides a single user interface taking the user through each step of the application development flow. Familiar tools and interfaces allow users to get started faster than ever before. Code Composer Studio combines the advantages of the Eclipse software framework with advanced embedded debug capabilities from TI resulting in a compelling feature-rich development environment for embedded developers.

Models

Various models are available for download from the product Tools & Software pages. These include I/O Buffer Information Specification (IBIS) Models and Boundary-Scan Description Language (BSDL) Models. To view all available models, visit the Models section of the Tools & Software page for each device.

###### Documentation Support

To receive notification of documentation updates, navigate to the device product folder on ti.com. In the upper right corner, click on Alert me to register and receive a weekly digest of any product information that has changed. For change details, review the revision history included in any revised document.

The current documentation that describes the processor, related peripherals, and other technical collateral is listed below.

Errata

TMS320F281x DSPs Silicon Errata describes the advisories and usage notes for different versions of silicon.

CPU User's Guides

TMS320C28x CPU and Instruction Set Reference Guide describes the central processing unit (CPU) and the assembly language instructions of the TMS320C28x fixed-point digital signal processors (DSPs). It also describes emulation features available on these DSPs.

Peripheral Guides

C2000 Real-Time Control Peripherals Reference Guide describes the peripheral reference guides of the 28x digital signal processors (DSPs).

TMS320x281x DSP Analog-to-Digital Converter (ADC) Reference Guide describes the ADC module. The module is a 12-bit pipelined ADC. The analog circuits of this converter, referred to as the core in this document, include the front-end analog multiplexers (MUXs), sample-and-hold (S/H) circuits, the conversion core, voltage regulators, and other analog supporting circuits. Digital circuits, referred to as the wrapper in this document, include programmable conversion sequencer, result registers, interface to analog circuits, interface to device peripheral bus, and interface to other on-chip modules.

TMS320x281x DSP Boot ROM Reference Guide describes the purpose and features of the bootloader (factory- programmed boot-loading software). It also describes other contents of the device on-chip boot ROM and identifies where all of the information is located within that memory.

TMS320x281x DSP Event Manager (EV) Reference Guide describes the EV modules that provide a broad range of functions and features that are particularly useful in motion control and motor control applications. The EV modules include general-purpose (GP) timers, full-compare/PWM units, capture units, and quadrature-encoder pulse (QEP) circuits.

TMS320x281x DSP External Interface (XINTF) Reference Guide describes the external interface (XINTF) of the 281x digital signal processors (DSPs).

TMS320x281x DSP Multichannel Buffered Serial Port (McBSP) Reference Guide describes the McBSP available on the 281x devices. The McBSPs allow direct interface between a DSP and other devices in a system.

TMS320x281x DSP System Control and Interrupts Reference Guide describes the various interrupts and system control features of the 281x digital signal processors (DSPs).

TMS320x281x Enhanced Controller Area Network (eCAN) Reference Guide describes the eCAN that uses established protocol to communicate serially with other controllers in electrically noisy environments. With 32 fully configurable mailboxes and time-stamping feature, the eCAN module provides a versatile and robust serial communication interface. The eCAN module implemented in the C28x DSP is compatible with the ISO11898-1 (CAN 2.0B) standard (active).

TMS320x281x Serial Communications Interface (SCI) Reference Guide describes the SCI that is a two-wire asynchronous serial port, commonly known as a UART. The SCI modules support digital communications between the CPU and other asynchronous peripherals that use the standard non-return-to-zero (NRZ) format.

TMS320x281x Serial Peripheral Interface Reference Guide describes the SPI—a high-speed synchronous serial input/output (I/O) port that allows a serial bit stream of programmed length (one to sixteen bits) to be shifted into and out of the device at a programmed bit-transfer rate. The SPI is used for communications between the DSP controller and external peripherals or another controller.

Tools Guides

TMS320C28x Assembly Language Tools v18.12.0.LTS User's Guide describes the assembly language tools (assembler and other tools used to develop assembly language code), assembler directives, macros, common object file format, and symbolic debugging directives for the TMS320C28x device.

TMS320C28x Optimizing C/C++ Compiler v18.12.0.LTS User's Guide describes the TMS320C28x C/C++ compiler. This compiler accepts ANSI standard C/C++ source code and produces TMS320 DSP assembly language source code for the TMS320C28x device.

TMS320C28x DSP/BIOS 5.x Application Programming Interface (API) Reference Guide describes development using DSP/BIOS.

Application Reports

The SMT & packaging application notes website lists documentation on TI’s surface mount technology (SMT) and application notes on a variety of packaging-related topics.

Programming Examples for the TMS320x28xx eCAN Application Report contains several programming examples to illustrate how the eCAN module is set up for different modes of operation. The objective is to help you come up to speed quickly in programming the eCAN. All programs have been extensively commented to aid easy understanding.

F2810, F2811, and F2812 ADC Calibration Application Report describes a method for improving the absolute accuracy of the 12-bit analog-to-digital converter (ADC) found on the F2810/F2811/F2812 devices. Due to inherent gain and offset errors, the absolute accuracy of the ADC is impacted. The methods described in this application note can improve the absolute accuracy of the ADC to achieve levels better than 0.5%. This application note is accompanied by an example program (ADCcalibration, spra989.zip) that executes from RAM on the F2812 eZdsp.

An Introduction to IBIS (I/O Buffer Information Specification) Modeling discusses various aspects of IBIS including its history, advantages, compatibility, model generation flow, data requirements in modeling the input/ output structures and future trends.

Semiconductor Packing Methodology describes the packing methodologies employed to prepare semiconductor devices for shipment to end users.

Calculating Useful Lifetimes of Embedded Processors provides a methodology for calculating the useful lifetime of TI embedded processors (EPs) under power when used in electronic systems. It is aimed at general engineers who wish to determine if the reliability of the TI EP meets the end system reliability requirement.

Calculating FIT for a Mission Profile explains how to use TI's reliability de-rating tools to calculate a component- level FIT under power-on conditions for a system mission profile.

Semiconductor and IC Package Thermal Metrics describes traditional and new thermal metrics and puts their application in perspective with respect to system-level junction temperature estimation.

Serial Flash Programming of C2000™ Microcontrollers discusses using a flash kernel and ROM loaders for serial programming a device.

###### Support Resources

TI E2E™ support forums are an engineer's go-to source for fast, verified answers and design help — straight from the experts. Search existing answers or ask your own question to get the quick design help you need.

Linked content is provided "AS IS" by the respective contributors. They do not constitute TI specifications and do not necessarily reflect TI's views; see TI's Terms of Use.

###### Trademarks

Code Composer Studio™, DSP/BIOS™, MicroStar BGA™, and are trademarks of TI.

C2000™, Piccolo™, TMS320C2000™, TMS320™, controlSUITE™, TI E2E™ are trademarks of Texas Instruments. eZdsp™ is a trademark of Spectrum Digital Incorporated.

All trademarks are the property of their respective owners.

###### Electrostatic Discharge Caution

This integrated circuit can be damaged by ESD. Texas Instruments recommends that all integrated circuits be handled with appropriate precautions. Failure to observe proper handling and installation procedures can cause damage.

ESD damage can range from subtle performance degradation to complete device failure. Precision integrated circuits may be more susceptible to damage because very small parametric changes could cause the device not to meet its published specifications.

###### Glossary

TI Glossary	This glossary lists and explains terms, acronyms, and definitions.

#### Mechanical, Packaging, and Orderable Information

###### Packaging Information

The following pages include mechanical, packaging, and orderable information. This information is the most current data available for the designated devices. This data is subject to change without notice and revision of this document. For browser-based versions of this data sheet, refer to the left-hand navigation.

To learn more about TI packaging, visit the Packaging information website.

###### PACKAGING INFORMATION

| Orderable part number | Status Material type (1) (2) |  | Package ¦ Pins | Package qty ¦ Carrier | RoHS (3) | Lead finish/ Ball material (4) | MSL rating/ Peak reflow (5) | Op temp (°C) | Part marking (6) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TMS320F2810PBKA | Active | Production | LQFP (PBK) ¦ 128 | 90 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-2-260C-1 YEAR | -40 to 85 | 320F2810PBKA TMS |
| TMS320F2810PBKA.A | Active | Production | LQFP (PBK) ¦ 128 | 90 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-2-260C-1 YEAR | -40 to 85 | 320F2810PBKA TMS |
| TMS320F2810PBKAG4 | Active | Production | LQFP (PBK) ¦ 128 | 90 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-2-260C-1 YEAR | -40 to 125 | 320F2810PBKA TMS |
| TMS320F2810PBKAG4.A | Active | Production | LQFP (PBK) ¦ 128 | 90 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-2-260C-1 YEAR | -40 to 125 | 320F2810PBKA TMS |
| TMS320F2810PBKQ | Active | Production | LQFP (PBK) ¦ 128 | 90 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-2-260C-1 YEAR | -40 to 125 | 320F2810PBKQ TMS |
| TMS320F2810PBKQ.A | Active | Production | LQFP (PBK) ¦ 128 | 90 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-2-260C-1 YEAR | -40 to 125 | 320F2810PBKQ TMS |
| TMS320F2810PBKQR | Active | Production | LQFP (PBK) ¦ 128 | 1000 ¦ LARGE T&R | Yes | NIPDAU | Level-2-260C-1 YEAR | -40 to 125 | 320F2810PBKQ TMS |
| TMS320F2810PBKQR.A | Active | Production | LQFP (PBK) ¦ 128 | 1000 ¦ LARGE T&R | Yes | NIPDAU | Level-2-260C-1 YEAR | -40 to 125 | 320F2810PBKQ TMS |
| TMS320F2810PBKS | Active | Production | LQFP (PBK) ¦ 128 | 90 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-2-260C-1 YEAR | -40 to 125 | 320F2810PBKS TMS |
| TMS320F2810PBKS.A | Active | Production | LQFP (PBK) ¦ 128 | 90 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-2-260C-1 YEAR | -40 to 125 | 320F2810PBKS TMS |
| TMS320F2810PBKSG4 | Active | Production | LQFP (PBK) ¦ 128 | 90 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-2-260C-1 YEAR | -40 to 125 | 320F2810PBKS TMS |
| TMS320F2810PBKSG4.A | Active | Production | LQFP (PBK) ¦ 128 | 90 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-2-260C-1 YEAR | -40 to 125 | 320F2810PBKS TMS |
| TMS320F2811PBKA | Active | Production | LQFP (PBK) ¦ 128 | 90 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-2-260C-1 YEAR | -40 to 85 | 320F2811PBKA TMS |
| TMS320F2811PBKA.A | Active | Production | LQFP (PBK) ¦ 128 | 90 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-2-260C-1 YEAR | -40 to 85 | 320F2811PBKA TMS |
| TMS320F2811PBKAG4 | Active | Production | LQFP (PBK) ¦ 128 | 90 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-2-260C-1 YEAR | -40 to 125 | 320F2811PBKA TMS |
| TMS320F2811PBKAG4.A | Active | Production | LQFP (PBK) ¦ 128 | 90 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-2-260C-1 YEAR | -40 to 125 | 320F2811PBKA TMS |

| Orderable part number | Status Material type (1) (2) |  | Package ¦ Pins | Package qty ¦ Carrier | RoHS (3) | Lead finish/ Ball material (4) | MSL rating/ Peak reflow (5) | Op temp (°C) | Part marking (6) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TMS320F2811PBKQ | Active | Production | LQFP (PBK) ¦ 128 | 90 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-2-260C-1 YEAR | -40 to 125 | 320F2811PBKQ TMS |
| TMS320F2811PBKQ.A | Active | Production | LQFP (PBK) ¦ 128 | 90 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-2-260C-1 YEAR | -40 to 125 | 320F2811PBKQ TMS |
| TMS320F2811PBKS | Active | Production | LQFP (PBK) ¦ 128 | 90 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-2-260C-1 YEAR | -40 to 125 | 320F2811PBKS TMS |
| TMS320F2811PBKS.A | Active | Production | LQFP (PBK) ¦ 128 | 90 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-2-260C-1 YEAR | -40 to 125 | 320F2811PBKS TMS |
| TMS320F2812GBBA | Active | Production | NFBGA (GBB) ¦ 179 | 160 ¦ JEDEC TRAY (5+1) | No | SNPB | Level-3-220C-168 HR | -40 to 85 | TMS 320F2812GBBA |
| TMS320F2812GBBA.A | Active | Production | NFBGA (GBB) ¦ 179 | 160 ¦ JEDEC TRAY (5+1) | No | SNPB | Level-3-220C-168 HR | -40 to 85 | TMS 320F2812GBBA |
| TMS320F2812GBBAR | Active | Production | NFBGA (GBB) ¦ 179 | 1000 ¦ LARGE T&R | No | SNPB | Level-3-220C-168 HR | -40 to 85 | TMS 320F2812GBBA |
| TMS320F2812GBBAR.A | Active | Production | NFBGA (GBB) ¦ 179 | 1000 ¦ LARGE T&R | No | SNPB | Level-3-220C-168 HR | -40 to 85 | TMS 320F2812GBBA |
| TMS320F2812GBBS | Active | Production | NFBGA (GBB) ¦ 179 | 160 ¦ EIAJ TRAY (5+1) | No | SNPB | Level-3-220C-168 HR | -40 to 125 | TMS 320F2812GBBS |
| TMS320F2812GBBS.A | Active | Production | NFBGA (GBB) ¦ 179 | 160 ¦ EIAJ TRAY (5+1) | No | SNPB | Level-3-220C-168 HR | -40 to 125 | TMS 320F2812GBBS |
| TMS320F2812PGFA | Active | Production | LQFP (PGF) ¦ 176 | 40 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-3-260C-168 HR | -40 to 85 | 320F2812PGFA TMS |
| TMS320F2812PGFA.A | Active | Production | LQFP (PGF) ¦ 176 | 40 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-3-260C-168 HR | -40 to 85 | 320F2812PGFA TMS |
| TMS320F2812PGFAG4 | Active | Production | LQFP (PGF) ¦ 176 | 40 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-3-260C-168 HR | -40 to 85 | 320F2812PGFA TMS |
| TMS320F2812PGFQ | Active | Production | LQFP (PGF) ¦ 176 | 40 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-3-260C-168 HR | -40 to 125 | 320F2812PGFQ TMS |
| TMS320F2812PGFQ.A | Active | Production | LQFP (PGF) ¦ 176 | 40 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-3-260C-168 HR | -40 to 125 | 320F2812PGFQ TMS |
| TMS320F2812PGFS | Active | Production | LQFP (PGF) ¦ 176 | 40 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-3-260C-168 HR | -40 to 125 | 320F2812PGFS TMS |
| TMS320F2812PGFS.A | Active | Production | LQFP (PGF) ¦ 176 | 40 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-3-260C-168 HR | -40 to 125 | 320F2812PGFS TMS |

| Orderable part number | Status (1) | Material type (2) | Package ¦ Pins | Package qty ¦ Carrier | RoHS (3) | Lead finish/ Ball material (4) | MSL rating/ Peak reflow (5) | Op temp (°C) | Part marking (6) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TMS320F2812PGFSG4 | Active | Production | LQFP (PGF) ¦ 176 | 40 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-3-260C-168 HR | -40 to 125 | 320F2812PGFS TMS |
| TMS320F2812PGFSG4.A | Active | Production | LQFP (PGF) ¦ 176 | 40 ¦ JEDEC TRAY (5+1) | Yes | NIPDAU | Level-3-260C-168 HR | -40 to 125 | 320F2812PGFS TMS |
| TMS320F2812ZAYA | Active | Production | NFBGA (ZAY) ¦ 179 | 160 ¦ EIAJ TRAY (5+1) | Yes | SNAGCU | Level-3-260C-168 HR | -40 to 85 | TMS 320F2812ZAYA |
| TMS320F2812ZAYA.A | Active | Production | NFBGA (ZAY) ¦ 179 | 160 ¦ EIAJ TRAY (5+1) | Yes | SNAGCU | Level-3-260C-168 HR | -40 to 85 | TMS 320F2812ZAYA |
| TMS320F2812ZAYAR | Active | Production | NFBGA (ZAY) ¦ 179 | 1000 ¦ LARGE T&R | Yes | SNAGCU | Level-3-260C-168 HR | -40 to 85 | TMS 320F2812ZAYA |
| TMS320F2812ZAYAR.A | Active | Production | NFBGA (ZAY) ¦ 179 | 1000 ¦ LARGE T&R | Yes | SNAGCU | Level-3-260C-168 HR | -40 to 85 | TMS 320F2812ZAYA |
| TMS320F2812ZAYS | Active | Production | NFBGA (ZAY) ¦ 179 | 160 ¦ EIAJ TRAY (5+1) | Yes | SNAGCU | Level-3-260C-168 HR | -40 to 125 | TMS 320F2812ZAYS |
| TMS320F2812ZAYS.A | Active | Production | NFBGA (ZAY) ¦ 179 | 160 ¦ EIAJ TRAY (5+1) | Yes | SNAGCU | Level-3-260C-168 HR | -40 to 125 | TMS 320F2812ZAYS |

(1) Status: For more details on status, see our product life cycle.

(2) Material type: When designated, preproduction parts are prototypes/experimental devices, and are not yet approved or released for full production. Testing and final process, including without limitation quality assurance, reliability performance testing, and/or process qualification, may not yet be complete, and this item is subject to further changes or possible discontinuation. If available for ordering, purchases will be subject to an additional waiver at checkout, and are intended for early internal evaluation purposes only. These items are sold without warranties of any kind.

(3) RoHS values: Yes, No, RoHS Exempt. See the TI RoHS Statement for additional information and value definition.

(4) Lead finish/Ball material: Parts may have multiple material finish options. Finish options are separated by a vertical ruled line. Lead finish/Ball material values may wrap to two lines if the finish value exceeds the maximum column width.

(5) MSL rating/Peak reflow: The moisture sensitivity level ratings and peak solder (reflow) temperatures. In the event that a part has multiple moisture sensitivity ratings, only the lowest level per JEDEC standards is shown. Refer to the shipping label for the actual reflow temperature that will be used to mount the part to the printed circuit board.

(6) Part marking: There may be an additional marking, which relates to the logo, the lot trace code information, or the environmental category of the part.

Multiple part markings will be inside parentheses. Only one part marking contained in parentheses and separated by a "~" will appear on a part. If a line is indented then it is a continuation of the previous line and the two combined represent the entire part marking for that device.

Important Information and Disclaimer:The information provided on this page represents TI's knowledge and belief as of the date that it is provided. TI bases its knowledge and belief on information provided by third parties, and makes no representation or warranty as to the accuracy of such information. Efforts are underway to better integrate information from third parties. TI has taken and continues to take reasonable steps to provide representative and accurate information but may not have conducted destructive testing or chemical analysis on incoming materials and chemicals. TI and TI suppliers consider certain information to be proprietary, and thus CAS numbers and other limited information may not be available for release.

In no event shall TI's liability arising out of such information exceed the total purchase price of the TI part(s) at issue in this document sold by TI to Customer on an annual basis.

OTHER QUALIFIED VERSIONS OF TMS320F2810, TMS320F2810-Q1, TMS320F2811, TMS320F2811-Q1, TMS320F2812, TMS320F2812-Q1 :

- Catalog : TMS320F2810, TMS320F2811, SM320F2812, TMS320F2812

- Automotive : TMS320F2810-Q1, TMS320F2811-Q1, TMS320F2812-Q1

- Enhanced Product : SM320F2812-EP

- Military : SMJ320F2812, SMJ320F2812

NOTE: Qualified Version Definitions:

  - Catalog - TI's standard catalog product

  - Automotive - Q100 devices qualified for high-reliability automotive applications targeting zero defects

  - Enhanced Product - Supports Defense, Aerospace and Medical Applications

  - Military - QML certified for Military and Defense Applications

www.ti.com	31-Oct-2025

###### TAPE AND REEL INFORMATION

Reel Diameter

K0	P1

B0 W

Cavity

A0

REEL DIMENSIONS	TAPE DIMENSIONS

| A0 | Dimension designed to accommodate the component width |
| --- | --- |
| B0 | Dimension designed to accommodate the component length |
| K0 | Dimension designed to accommodate the component thickness |
| W | Overall width of the carrier tape |
| P1 | Pitch between successive cavity centers |

Reel Width (W1)

QUADRANT ASSIGNMENTS FOR PIN 1 ORIENTATION IN TAPE

Sprocket Holes

Q1	Q2	Q1	Q2

Q3	Q4	Q3	Q4	User Direction of Feed

Pocket Quadrants

*All dimensions are nominal

| Device | Package Type | Package Drawing | Pins | SPQ | Reel Diameter (mm) | Reel Width W1 (mm) | A0 (mm) | B0 (mm) | K0 (mm) | P1 (mm) | W (mm) | Pin1 Quadrant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TMS320F2812GBBAR | NFBGA | GBB | 179 | 1000 | 330.0 | 24.4 | 12.35 | 12.35 | 2.3 | 16.0 | 24.0 | Q1 |
| TMS320F2812ZAYAR | NFBGA | ZAY | 179 | 1000 | 330.0 | 24.4 | 12.35 | 12.35 | 2.3 | 16.0 | 24.0 | Q1 |

H

TAPE AND REEL BOX DIMENSIONS

W

L

*All dimensions are nominal

| Device | Package Type | Package Drawing | Pins | SPQ | Length (mm) | Width (mm) | Height (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TMS320F2812GBBAR | NFBGA | GBB | 179 | 1000 | 336.6 | 336.6 | 41.3 |
| TMS320F2812ZAYAR | NFBGA | ZAY | 179 | 1000 | 336.6 | 336.6 | 41.3 |

www.ti.com	31-Oct-2025

TRAY

L - Outer tray length without tabs

W -

Outer tray width

P1 - Tray unit pocket pitch

CW - Measurement for tray edge (Y direction) to corner pocket center CL - Measurement for tray edge (X direction) to corner pocket center

KO -

Outer tray height

Chamfer on Tray corner indicates Pin 1 orientation of packed units.

*All dimensions are nominal

| Device | Package Name | Package Type | Pins | SPQ | Unit array matrix | Max temperature (°C) | L (mm) | W (mm) | K0 (µm) | P1 (mm) | CL (mm) | CW (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TMS320F2810PBKA | PBK | LQFP | 128 | 90 | 6 x 15 | 150 | 315 | 135.9 | 7620 | 20.3 | 15.4 | 15.4 |
| TMS320F2810PBKA.A | PBK | LQFP | 128 | 90 | 6 x 15 | 150 | 315 | 135.9 | 7620 | 20.3 | 15.4 | 15.4 |
| TMS320F2810PBKAG4 | PBK | LQFP | 128 | 90 | 6 x 15 | 150 | 315 | 135.9 | 7620 | 20.3 | 15.4 | 15.4 |
| TMS320F2810PBKAG4.A | PBK | LQFP | 128 | 90 | 6 x 15 | 150 | 315 | 135.9 | 7620 | 20.3 | 15.4 | 15.4 |
| TMS320F2810PBKQ | PBK | LQFP | 128 | 90 | 6 x 15 | 150 | 315 | 135.9 | 7620 | 20.3 | 15.4 | 15.4 |
| TMS320F2810PBKQ.A | PBK | LQFP | 128 | 90 | 6 x 15 | 150 | 315 | 135.9 | 7620 | 20.3 | 15.4 | 15.4 |
| TMS320F2810PBKS | PBK | LQFP | 128 | 90 | 6 x 15 | 150 | 315 | 135.9 | 7620 | 20.3 | 15.4 | 15.4 |
| TMS320F2810PBKS.A | PBK | LQFP | 128 | 90 | 6 x 15 | 150 | 315 | 135.9 | 7620 | 20.3 | 15.4 | 15.4 |
| TMS320F2810PBKSG4 | PBK | LQFP | 128 | 90 | 6 x 15 | 150 | 315 | 135.9 | 7620 | 20.3 | 15.4 | 15.4 |
| TMS320F2810PBKSG4.A | PBK | LQFP | 128 | 90 | 6 x 15 | 150 | 315 | 135.9 | 7620 | 20.3 | 15.4 | 15.4 |
| TMS320F2811PBKA | PBK | LQFP | 128 | 90 | 6 x 15 | 150 | 315 | 135.9 | 7620 | 20.3 | 15.4 | 15.4 |
| TMS320F2811PBKA.A | PBK | LQFP | 128 | 90 | 6 x 15 | 150 | 315 | 135.9 | 7620 | 20.3 | 15.4 | 15.4 |
| TMS320F2811PBKAG4 | PBK | LQFP | 128 | 90 | 6 x 15 | 150 | 315 | 135.9 | 7620 | 20.3 | 15.4 | 15.4 |
| TMS320F2811PBKAG4.A | PBK | LQFP | 128 | 90 | 6 x 15 | 150 | 315 | 135.9 | 7620 | 20.3 | 15.4 | 15.4 |
| TMS320F2811PBKQ | PBK | LQFP | 128 | 90 | 6 x 15 | 150 | 315 | 135.9 | 7620 | 20.3 | 15.4 | 15.4 |
| TMS320F2811PBKQ.A | PBK | LQFP | 128 | 90 | 6 x 15 | 150 | 315 | 135.9 | 7620 | 20.3 | 15.4 | 15.4 |
| TMS320F2811PBKS | PBK | LQFP | 128 | 90 | 6 x 15 | 150 | 315 | 135.9 | 7620 | 20.3 | 15.4 | 15.4 |

| Device | Package Name | Package Type | Pins | SPQ | Unit array matrix | Max temperature (°C) | L (mm) | W (mm) | K0 (µm) | P1 (mm) | CL (mm) | CW (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TMS320F2811PBKS.A | PBK | LQFP | 128 | 90 | 6 x 15 | 150 | 315 | 135.9 | 7620 | 20.3 | 15.4 | 15.4 |
| TMS320F2812GBBA | GBB | NFBGA | 179 | 160 | 8 x 20 | 150 | 315 | 135.9 | 7620 | 15.4 | 11.2 | 19.65 |
| TMS320F2812GBBA.A | GBB | NFBGA | 179 | 160 | 8 x 20 | 150 | 315 | 135.9 | 7620 | 15.4 | 11.2 | 19.65 |
| TMS320F2812GBBS | GBB | NFBGA | 179 | 160 | 8 x 20 | 150 | 315 | 135.9 | 7620 | 15.4 | 11.2 | 19.65 |
| TMS320F2812GBBS.A | GBB | NFBGA | 179 | 160 | 8 x 20 | 150 | 315 | 135.9 | 7620 | 15.4 | 11.2 | 19.65 |
| TMS320F2812PGFA | PGF | LQFP | 176 | 40 | 4x10 | 150 | 315 | 135.9 | 7620 | 20.7 | 30.4 | 20.7 |
| TMS320F2812PGFA.A | PGF | LQFP | 176 | 40 | 4x10 | 150 | 315 | 135.9 | 7620 | 20.7 | 30.4 | 20.7 |
| TMS320F2812PGFAG4 | PGF | LQFP | 176 | 40 | 4x10 | 150 | 315 | 135.9 | 7620 | 20.7 | 30.4 | 20.7 |
| TMS320F2812PGFQ | PGF | LQFP | 176 | 40 | 4x10 | 150 | 315 | 135.9 | 7620 | 20.7 | 30.4 | 20.7 |
| TMS320F2812PGFQ.A | PGF | LQFP | 176 | 40 | 4x10 | 150 | 315 | 135.9 | 7620 | 20.7 | 30.4 | 20.7 |
| TMS320F2812PGFS | PGF | LQFP | 176 | 40 | 4x10 | 150 | 315 | 135.9 | 7620 | 20.7 | 30.4 | 20.7 |
| TMS320F2812PGFS.A | PGF | LQFP | 176 | 40 | 4x10 | 150 | 315 | 135.9 | 7620 | 20.7 | 30.4 | 20.7 |
| TMS320F2812PGFSG4 | PGF | LQFP | 176 | 40 | 4x10 | 150 | 315 | 135.9 | 7620 | 20.7 | 30.4 | 20.7 |
| TMS320F2812PGFSG4.A | PGF | LQFP | 176 | 40 | 4x10 | 150 | 315 | 135.9 | 7620 | 20.7 | 30.4 | 20.7 |
| TMS320F2812ZAYA | ZAY | NFBGA | 179 | 160 | 8 x 20 | 150 | 315 | 135.9 | 7620 | 15.4 | 11.2 | 19.65 |
| TMS320F2812ZAYA.A | ZAY | NFBGA | 179 | 160 | 8 x 20 | 150 | 315 | 135.9 | 7620 | 15.4 | 11.2 | 19.65 |
| TMS320F2812ZAYS | ZAY | NFBGA | 179 | 160 | 8 x 20 | 150 | 315 | 135.9 | 7620 | 15.4 | 11.2 | 19.65 |
| TMS320F2812ZAYS.A | ZAY | NFBGA | 179 | 160 | 8 x 20 | 150 | 315 | 135.9 | 7620 | 15.4 | 11.2 | 19.65 |

# PGF0176A

### LQFP - 1.6 mm max height

SCALE 0.550

PLASTIC QUAD FLATPACK

PIN 1 ID

23.8

176

24.2 NOTE 3

B

133

1	132

24.2

23.8

NOTE 3

26.2

TYP

25.8

44

89

A	45	88

172X 0.5

0.27

176X

0.17

|  | 0.08 | C | A | B |
| --- | --- | --- | --- | --- |

4X 21.5

C

(0.13) TYP

SEE DETAIL A

SEATING PLANE

1.6 MAX

0.25

GAGE PLANE

(1.4)

NOTES:

0 -7

0.75

0.45

C

0.08

DETA 12 AIL A

TYPICAL

0.15

0.05

4215177/A 05/2017

- All linear dimensions are in millimeters. Any dimensions in parenthesis are for reference only. Dimensioning and tolerancing per ASME Y14.5M.

- This drawing is subject to change without notice.

- This dimension does not include mold flash, protrusions, or gate burrs.

- Reference JEDEC registration MS-026.

# PGF0176A

## EXAMPLE BOARD LAYOUT

### LQFP - 1.6 mm max height

PLASTIC QUAD FLATPACK

176X (1.5)

176

SYMM

133

1

176X (0.3)

132

172X (0.5)

SYMM

(25.4)

(R0.05) TYP

44	89

SEE DETAILS

45	88

0.05 MAX ALL AROUND

(25.4)

LAND PATTERN EXAMPLE

EXPOSED METAL SHOWN SCALE:4X

METAL

0.05 MIN ALL AROUND

SOLDER MASK OPENING

EXPOSED METAL

SOLDER MASK OPENING

EXPOSED METAL

METAL UNDER SOLDER MASK

NOTES: (continued)

NON SOLDER MASK

DEFINED	SOLDER MASK DETAILS

SOLDER MASK DEFINED

4215177/A 05/2017

- Publication IPC-7351 may have alternate designs.

- Solder mask tolerances between and around signal pads can vary based on board fabrication site.

# PGF0176A

### LQFP - 1.6 mm max height

PLASTIC QUAD FLATPACK

176X (1.5)

176

SYMM

133

1

176X (0.3)

132

172X (0.5)

SYMM

(25.4)

(R0.05) TYP

44	89

45	88

(25.4)

SOLDER PASTE EXAMPLE

BASED ON 0.125 mm THICK STENCIL SCALE:4X

4215177/A 05/2017

NOTES: (continued)

- Laser cutting apertures with trapezoidal walls and rounded corners may offer better paste release. IPC-7525 may have alternate design recommendations.

- Board assembly site may have different recommendations for stencil design.

B

12.1	A

11.9

BALL A1 CORNER

12.1

11.9

1.4 MAX

C

0.45

0.35

BALL TYP

SEATING PLANE

C

0.12

10.4 TYP	(0.8) TYP

10.4

TYP

P

N M L K J H G F E D C

(0.8) TYP

SYMM

0.45

|  | 0.15 | C | A | B |
| --- | --- | --- | --- | --- |
|  | 0.05 | C |  |  |

179X Ø 0.55

0.8 TYP

B A

1  2	3  4  5  6  7

8  9 10 11 12 13 14

NOTES:

0.8 TYP

SYMM

4225682/A 02/2020

NanoFree is a trademark of Texas Instruments.

- All linear dimensions are in millimeters. Any dimensions in parenthesis are for reference only. Dimensioning and tolerancing per ASME Y14.5M.

- This drawing is subject to change without notice.

## GBB0179A

EXAMPLE BOARD LAYOUT

### NFBGA - 1.4 mm max height

PLASTIC BALL GRID ARRAY

(0.8) TYP

(0.8) TYP

A  1	2 3	4	5	6	7	8	9	10  11 12 13 14 B

C D E F G H J K L M N

SYMM

179X (Ø 0.4)

P

SYMM

LAND PATTERN EXAMPLE

SCALE: 8X

0.05 MAX ALL AROUND

EXPOSED METAL

0.05 MIN ALL AROUND

METAL UNDER SOLDER MASK

SOLDER MASK

OPENING

(Ø 0.40) METAL

NON- SOLDER MASK DEFINED (PREFERRED)

EXPOSED

METAL

SOLDER MASK DEFINED

(Ø 0.40) SOLDER MASK OPENING

NOTES: (continued)

SOLDER MASK DETAILS

NOT TO SCALE

4225682/A 02/2020

- Final dimensions may vary due to manufacturing tolerance considerations and also routing constraints. Refer to Texas Instruments Literature number SNVA009 (www.ti.com/lit/snva009).

(0.8) TYP

(0.8) TYP

A  1	2 3	4	5	6	7	8	9	10  11 12 13 14 B

C D E F G H J K L M N

SYMM

179X (Ø 0.4)

P

SYMM

SOLDER PASTE EXAMPLE

BASED ON 0.150 mm THICK STENCIL SCALE: 8X

4225682/A 02/2020

NOTES: (continued)

- Laser cutting apertures with trapezoidal walls and rounded corners may offer better paste release.

# ZAY0179A

### NFBGA - 1.4 mm max height

SCALE 1.200

PLASTIC BALL GRID ARRAY

B

12.1

11.9	A

BALL A1 CORNER

12.1

11.9

1.4 MAX

C

0.45

0.35

P

C

0.12

⌓

10.4 TYP SYMM

℄

(0.8)

(0.8)

SEATING PLANE

N

M

L

K

10.4

J

H

TYP

G

SYMM

℄

F

E

D C

179X

B

0.55

0.45

0.8

TYP

A

1  2	3  4  5  6  7  8  9  10 11  12  13 14

0.8 TYP

⌖ ⌀0.15Ⓜ C A B

⌀0.08Ⓜ C

4225014/C 07/2020

NOTES:

- All linear dimensions are in millimeters. Any dimensions in parenthesis are for reference only. Dimensioning and tolerancing per ASME Y14.5M.

- This drawing is subject to change without notice.

# ZAY0179A

### NFBGA - 1.4 mm max height

PLASTIC BALL GRID ARRAY

179X ( 0.4)

A

(0.8) TYP

(0.8) TYP

1

2	3	4	5	6	7

8	9	10	11	12	13	14

B C

D

E

F

G	SYMM

℄

H

J

K

L

M

N

P

SYMM

℄

##### LAND PATTERN EXAMPLE

EXPOSED METAL SHOWN SCALE: 10X

0.05 MAX ALL AROUND

SOLDER MASK

OPENING

EXPOSED METAL

( 0.4) METAL EDGE

0.05 MIN ALL AROUND

EXPOSED METAL

METAL UNDER SOLDER MASK

( 0.4) SOLDER MASK OPENING

NOTES: (continued)

NON-SOLDER MASK DEFINED (PREFERRED)

##### SOLDER MASK DETAILS

NOT TO SCALE

SOLDER MASK DEFINED

4225014/C 07/2020

- Final dimensions may vary due to manufacturing tolerance considerations and also routing constraints. For information, see Texas Instruments literature number SPRAA99 (www.ti.com/lit/spraa99).

# ZAY0179A

### NFBGA - 1.4 mm max height

PLASTIC BALL GRID ARRAY

179X ( 0.4)

(0.8) TYP

A

(0.8) TYP

1	2	3	4	5	6	7

8	9	10	11	12	13	14

B C

D

E

F

G	SYMM

℄

H

J

K

L

M

N

P

SYMM

℄

##### SOLDER PASTE EXAMPLE

BASED ON 0.150 mm THICK STENCIL SCALE: 10X

4225014/C 07/2020

NOTES: (continued)

- Laser cutting apertures with trapezoidal walls and rounded corners may offer better paste release.

#### IMPORTANT NOTICE AND DISCLAIMER

TI PROVIDES TECHNICAL AND RELIABILITY DATA (INCLUDING DATASHEETS), DESIGN RESOURCES (INCLUDING REFERENCE DESIGNS), APPLICATION OR OTHER DESIGN ADVICE, WEB TOOLS, SAFETY INFORMATION, AND OTHER RESOURCES “AS IS” AND WITH ALL FAULTS, AND DISCLAIMS ALL WARRANTIES, EXPRESS AND IMPLIED, INCLUDING WITHOUT LIMITATION ANY IMPLIED WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE OR NON-INFRINGEMENT OF THIRD PARTY INTELLECTUAL PROPERTY RIGHTS.

These resources are intended for skilled developers designing with TI products. You are solely responsible for (1) selecting the appropriate TI products for your application, (2) designing, validating and testing your application, and (3) ensuring your application meets applicable standards, and any other safety, security, regulatory or other requirements.

These resources are subject to change without notice. TI grants you permission to use these resources only for development of an application that uses the TI products described in the resource. Other reproduction and display of these resources is prohibited. No license is granted to any other TI intellectual property right or to any third party intellectual property right. TI disclaims responsibility for, and you fully indemnify TI and its representatives against any claims, damages, costs, losses, and liabilities arising out of your use of these resources.

TI’s products are provided subject to TI’s Terms of Sale, TI’s General Quality Guidelines, or other applicable terms available either on ti.com or provided in conjunction with such TI products. TI’s provision of these resources does not expand or otherwise alter TI’s applicable warranties or warranty disclaimers for TI products. Unless TI explicitly designates a product as custom or customer-specified, TI products are standard, catalog, general purpose devices.

TI objects to and rejects any additional or different terms you may propose.

Copyright © 2025, Texas Instruments Incorporated Last updated 10/2025