Test Requirements

| SCU_STC_SRS_101 | The software shall execute on a single-core DSP processor with a maximum CPU frequency of 100 MHz. |
| --- | --- |
| SCU_STC_SRS_102 | The software shall operate within the available on-chip RAM capacity of the target hardware. |
| SCU_STC_SRS_103 | The software shall not exceed the available on-chip flash memory during normal operation. |
| SCU_STC_SRS_104 | The software shall execute without reliance on external memory devices. |
| SCU_STC_SRS_105 | The software shall use only hardware-supported GPIO pins for digital input and output operations. |
| SCU_STC_SRS_106 | The software shall access memory-mapped registers only within the address ranges defined by the target hardware. |
| SCU_STC_SRS_107 | The software shall not require hardware features not supported by the target processor. |
| SCU_STC_SRS_108 | The software shall ensure CPU utilization remains within allowable limits during nominal operation. |
| SCU_STC_SRS_109 | The software shall operate within the electrical and timing constraints of the target hardware platform. |
| SCU_STC_SRS_110 | The software shall execute the main control loop with a period of 10 milliseconds. |
| SCU_STC_SRS_111 | The software shall complete sensor data acquisition within 2 milliseconds |
| SCU_STC_SRS_112 | The software shall process received communication frames within 5 milliseconds of reception. |
| SCU_STC_SRS_113 | The software shall complete initialization within 500 milliseconds after power-up. |
| SCU_STC_SRS_114 | The software shall detect watchdog timeout conditions within 100 milliseconds. |
| SCU_STC_SRS_115 | The software shall sample analog input signals at a rate of 1 kHz. |
| SCU_STC_SRS_116 | The software shall meet worst-case execution time requirements under peak load conditions. |
| SCU_STC_SRS_117 | The software shall ensure deterministic execution of time-critical functions. |
| SCU_STC_SRS_118 | The software shall communicate with external systems using the CAN interface supported by the target hardware |
| SCU_STC_SRS_119 | The software shall use the SPI interface for sensor data exchange. |
| SCU_STC_SRS_120 | The software shall use the UART interface for diagnostic communication during startup. |
| SCU_STC_SRS_121 | The software shall not require Ethernet communication on the target hardware. |
| SCU_STC_SRS_122 | The software shall interface with GPIO inputs and outputs as defined by the system architecture. |
| SCU_STC_SRS_123 | The software shall handle interface communication errors in accordance with system design. |
| SCU_STC_SRS_124 | The software shall ensure all interface configurations comply with hardware specifications. |
| SCU_STC_SRS_125 | The software shall transmit synchronization frames over the CAN bus during startup. Verification Method: Analysis |

| SCU_STC_SRS_126 | The software shall statically allocate all memory required for runtime operation |
| --- | --- |
| SCU_STC_SRS_127 | The software shall not use dynamic memory allocation during execution. |
| SCU_STC_SRS_128 | The software shall limit stack usage to predefined bounds. |
| SCU_STC_SRS_129 | The software shall ensure deterministic execution of time-critical functions. |
| SCU_STC_SRS_130 | The software shall limit heap usage to zero bytes. |
| SCU_STC_SRS_131 | The software shall ensure resource usage remains within system limits during fault conditions. |
| SCU_STC_SRS_120 | The software shall ensure buffer sizes are bounded and defined at compile time. |
| SCU_STC_SRS_132 | The software shall not require Ethernet communication on the target hardware. |
| SCU_STC_SRS_144 | The software shall interface with GPIO inputs and outputs as defined by the system architecture. |
| SCU_STC_SRS_134 | The software shall handle interface communication errors in accordance with system design. |
| SCU_STC_SRS_135 | The software shall ensure all interface configurations comply with hardware specifications. |
| SCU_STC_SRS_136 | The software shall transmit synchronization frames over the CAN bus during startup. Verification Method: Analysis |